3GPP TSG-RAN WG3#126                                             	                     R3-247761

Orlando, USA, 18 – 22 November 2024

Agenda item:	16.1

Source:	Nokia

Title: 	[TP for TR 38.769] Response to R3-247684

Document for:	Approval

# Introduction

In Tdoc R3-247684, it is proposed to conclude the study item with one solution for topology 1 and 3 solutions for topology 2. Also, the solutions mentioned here are the ones with several remaining open issues and associated unresolved editor’s notes, notably the unresolved open issue of XXAP protocol which are obstacles to conclude the study item.

This paper proposes a minimum set of agreements to be reached to enable concluding the RAN3 part of the study item.

# Discussion for topology 2

It should first be noted that SA2 only has temporary conclusions

For topology 2 SA2 has down-selected RRC based option and UP based option. NAS option is FFS.

Two options will be specified for Topology 2: - User-plane option as defined in clause 8.1.3.2 - RRC-based option as defined in clause 8.1.3.3 Editor’s note: Whether a NAS-based option is needed in addition is FFS.

The NAS option can be summarized as gNB fully transparent and ignorant of the AIoT request sent from AIoTF to UE. Otherwise, this would be an RRC based option. But this full transparency obviously creates the issue of how to allocate the radio resources involving the gNB for the coordination. Therefore, the NAS based solution is obviously more complicated than the RRC based solution for same result: it is also unnecessary to specify all options.

Proposal 1: Remove NAS based solution for topology 2.

In order to complete the RAN3 study item, at a minimum also an agreement on the protocol stack of the XX interface must be achieved.

In SA2 interim agreements for the RRC based option the AIoTF connects directly or indirectly to the gNB. One of the main use cases of AIoT is when AF/AIoT CN knows the UE readers to be involved and the UE readers are moving (e.g. fleet management). This use case requires the AMF involvement to translate the list of SUCIs to be reached into a list of UE NGAP IDs understandable for the gNB and also for AMF to identify the relevant gNB(s) to reach. AMF involvement may also bring the UE to RRC connected if necessary.

To support this use case the gNB will necessarily connect to the AMF using NGAP and especially the NGAP setup procedure at the beginning. Then AMF will then typically send to gNB a new NGAP AIoT request procedure which RAN3 is defining in TR 38.769 section 6.5 and including a list of UE NGAP IDs.

Using a direct interface in topology 2 is in theory possible if UE readers are fixed and if the AIoTF would know in advance their location i.e. the gNBs of the list of fixed readers. But it still raises additional complexity:

- How the AIoTF can identify the fixed UE readers to gNB if doesn’t know the UE NGAP IDs,

- How to bring the fixed readers to RRC connected if not,

- Need to define a new interface between gNB and AIoTF with associated interoperability testing.

This complexity can however be easily avoided by relaying the request by the AMF, reusing the same AIoT Request procedure which is anyway needed for the other “indirect” use case explained above.

Therefore, at a very minimum, if SA2 would still agree a direct XX (Nx) interface this extra complexity should be mitigated for the gNB by reusing the same protocol than for the indirect case which is a NGAP protocol and this leads to the following conclusion for RAN3:

Proposal 2: For topology 2, if SA2 agrees a direct interface between the AIoTF and the gNB, XX should be NG interface and XXAP should be NGAP.

# Consequence for topology 1

The agreement of SA2 for topology 1 is:

The principles and aspects in this clause are agreed to support Topology 1: - The new core network function (AIOTF) is introduced to support Ambient IoT functionality, described in clause 8.1.1, with the following features for topology 1: - Communicate with BS Reader (i.e., AIoT RAN), either directly or via an AMF, to trigger e.g., inventory request and command request).

In addition, RAN3 has already the following agreement at last RAN3#125bis:

In Topology 1 the signalling transport for XXAP termination at the AIoT RAN node is SCTP/IP. Capture in the TP other transport layer options had been discussed in this study is not pursued.

In topology 1 the typical use case would be an inventory using an area (warehouse) and the AIoT RAN node using the TRPs according to their location. Again, such request can easily be added reusing the AMF in the (same) NGAP AIoT Request procedure needed for topology 2, simply adding an “Area field” as shown below:

Again, specifying and deploying an additional direct interface would just require additional complexity on top (see above). Therefore, similarly as above, at a very minimum, if SA2 would still agree this direct XX (Nx) interface this extra complexity should be mitigated by reusing the same protocol than for the indirect case which is a NGAP protocol and this leads to the following conclusion for RAN3:

Proposal 3: For topology 1, if SA2 agrees a direct interface for topology 1 between the AIoTF and the AIoT RAN node, XX should be NG interface and XXAP should be NGAP.

# Conclusion and proposals

This paper has explained that the study item cannot be completed from RAN3 point of view without a minimum level of basic agreements, at least including the AIoT resource allocation and the XX and XXAP protocol. This leads to following proposals:

Proposal 1: Remove the NAS based solution for topology 2.

Proposal 2: For topology 2, if SA2 agrees a direct interface between the AIoTF and the gNB, XX should be NG interface and XXAP should be NGAP.

Proposal 3: For topology 1, if SA2 agrees a direct interface for topology 1 between the AIoTF and the AIoT RAN node, XX should be NG interface and XXAP should be NGAP.

Agree the TP for TR 38.769 presented in annex A below to reflect the above conclusion instead of the conclusion in R3-247684.

# References

- RP-234058, New Study Item: Study on Solutions for Ambient IOT (Internet of Things), Huawei

- RP-240826, Revised Study Item: Study on Solutions for Ambient IOT (Internet of Things), CMCC

# Annex A: Text Proposal for TR 38.769

# 8	Conclusions and recommendations

The logical system architectures have been studied and concluded as follows:

- For topology 1, one solution was concluded using NG interface between the AIoT RAN node and the AIoT CN.

- For topology 2, the RRC-based solution was concluded using the NG interface between the gNB and the AIoT CN. The NAS-based solution is excluded. UP solution may be supported based on SA2 agreement.

About the impacts on AIoT CN - AIoT RAN interface, signaling and procedures have been developed to support A-IoT Inventory operation and Command operation for topology 1 and topology 2 respectively, as documented in clause 6.5.

Based on the SID, solutions for locating an Ambient IoT device with minimal specification impact are identified, as documented in clause 6.9.