3GPP TSG RAN WG3#126	R3-247824

18th November – 22nd November - 2024

Orlando, USA

Agenda Item:	13.2

Source:	China Telecom (moderator)

Title:	Summary of offline discussion on MobilityEnh

Document for:	Approval

Introduction

CB: # ME

- Continue discussing the stage-2 and stage-3, and capture the agreements in the CRs.

- Discuss on the LS on CSI-RS sending to RAN1.

- Offline discussion on single reference configuration and multiple reference configuration.

(moderator - CT)

Offline discussion in R3-247824

For the Chairman’s Notes

Propose the following:

Reply LS to RAN1 in [R3-247822] – agreed

TP for TS38.300, R3-247262 rev [in R3-247889] – agreed

TP for TS38.423, R3-247484 rev [in R3-247890] – agreed

BLCR for TS38.420, R3-247181 rev [in R3-247887]– endorsed

BLCR for TS37.483, R3-247590 – endorsed

Propose to capture the following:

The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message.

The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation.

Turn the WA into agreement: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB.

Normal data forwarding may be initiated after the source gNB sends the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation.

Discussion

In this offline, we plan to continue to discuss the remaining issues on support of inter-CU LTM.

Support of LTM in Standalone scenario

CSI-RS configuration

At this meeting, RAN3 received an incoming LS from RAN1 in R3-247006, with a request to provide feedback on the feasibility for a Working Assumption to support semi-persistent CSI-RS for LTM candidate cells.

Working Assumption In addition to periodic CSI-RS, semi-persistent CSI-RS is supported for candidate cell L1-RSRP measurement for gNB scheduled reporting from RAN1 perspective The following working assumption was made in RAN1#118bis for candidate cell L1-RSRP measurement based on CSI-RS: RAN1 foresees the RAN3 spec impact to enable this functionality for inter-DU/inter-CU case, and hence RAN3’s view is needed to confirm the working assumption. Therefore, RAN1 would like to ask RAN3 for the feasibility of specifying the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s) and any other potential issues (e.g. RAN3 workload).

Issue1: Solution to support semi-persistent CSI-RS:

In order to support semi-persistent CSI-RS measurement in LTM, the source node and the target node shall inform each other when the candidate cell active the semi-persistent CSI-RS, which will incur RAN3 signalling design.

In [7261, HW], it provides three options to support SP CSI-RS measurement in intra-CU inter-DU case:

As illustrated in Figure 1, after receiving SSB measurement report from the UE, the source gNB-DU determines to start semi-persistent CSI-RS measurement for the candidate cell. Before sending the SP CSI-RS measurement indication to the UE, the source gNB-DU has three options to trigger the candidate gNB-DU to transmit the CSI-RS correspondingly.

- Option 1: The source gNB-DU determines the CSI-RS resources that to be transmitted, and sends the CSI-RS resource (set) IDs to the candidate gNB-DU via the gNB-CU to request the candidate gNB-DU to transmit the CSI-RS. In this option, the source gNB-DU needs to know the SSB and CSI-RS association of the candidate cell, to determine the suitable CSI-RS resources.

- Option 2: The source gNB-DU forwards the SSB measurement report to the candidate gNB-DU and the candidate gNB-DU determines the CSI-RS resources. The candidate gNB-DU sends that information back to the source gNB-DU.

- Option 3: The gNB-CU decides to trigger semi-persistent CSI-RS measurement and sends the selected CSI-RS resource (set) IDs to the candidate gNB-DU and the source gNB-DU based on L3 measurement results. The candidate gNB-DU starts to transmit CSI-RS following the indication of the gNB-CU, and the source gNB-DU triggers the UE measurement.

In this section, we can discuss the basic signaling framework to support SP CSI-RS coordination for the intra-CU inter-DU case first. The message to be used and the detailed IE design can be further discussed in the next meeting.

Question: Which node makes the decision on SP CSI-RS transmission: source DU, candidate DU or CU for the intra-CU inter-DU case?

Moderator summary:

LG: It is a common understanding that the semi-persistent and periodic CSI-RS are per-UE specific.

After we reach a consensus on the intra-CU case, we can further discuss the solution to support SP CSI-RS for inter-CU case.

- Option 1: source node-initiated coordination

- Option 2: candidate node -initiated coordination

Question: Which node makes the decision on SP CSI-RS transmission for inter-CU case: source node or candidate node?

Moderator summary:

Issue2: Reply LS to RAN1:

During the online session, we agreed to send a reply LS to RAN1 to confirm the feasibility of coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s).

CATT prepares the draft reply LS [draft LS in R3-247822] to in inbox for companies to review:

1. Overall Description: RAN3 would like to thank RAN1 for their LS about supporting semi-persistent CSI-RS resource for LTM candidate cells. RAN3 confirmed it is feasible to specify the signalling for coordination between serving cell and candidate cell(s) on the transmission of semi-persistent CSI-RS(s), and RAN3 will continue to work on the details. 2. Actions: To RAN1: ACTION: RAN3 respectfully asks RAN1 to take the above into account. 3. Date of Next TSG-RAN WG3 Meetings: TSG RAN WG3 Meeting #127 17th– 21st February 2025 Greece TSG RAN WG3 Meeting #127bis 7th– 11st April 2025 China TSG RAN WG3 Meeting # 128 19th– 23rd May 2025 Malta

Moderator summary:

Agree to send the reply LS to RAN1.

Issue3: Signaling enhancement on XnAP to support CSI-RS configuration during LTM preparation phase:

Rel-19 LTM has agreed to support CSI-RS (i.e. at least periodic CSI-RS), However, since CSI-RSs are typically configured as UE-specific resources (unlike SSBs, which are cell-specific), RAN3 should discuss how to generate the common CSI-RS configuration and the CSI-RS report configuration.

According to [7102, Nokia], the procedure will consist of the following four steps:

- Step # 1: Source CU sends HANDOVER REQUEST message to target CU based on L3 measurements.

- Step # 2: Target CU sends CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message.

- Step # 3: Source CU will generate common CSI-Resource Configuration based on CSI-RS configuration received from all target cells and sends to target CU in LTM CONFIGURATION UPDATE message.

- Step # 4: Target CU will send CSI-Report Configuration to source CU in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

Here we can check whether the above steps are reasonable, and can we agree with the proposals as below:

Proposal 1: The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU shall contain the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message.

Proposal 2: It is the source CU to generate common CSI-RS Resource Configuration and sent it to candidate CU via LTM Configuration update message, the candidate CU shall contain the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

Moderator summary:

The source CU can request candidate CU to provide CSI-RS configuration in HANDOVER REQUEST message, and candidate CU signals the CSI-RS configuration in HANDOVER REQUEST ACKNOWLEDGEMENT message.

The source CU generates common CSI-RS Resource Configuration and sends it to candidate CU in LTM CONFIGURATION UPDATE message, the candidate CU signals the CSI-RS Report Configuration in LTM CONFIGURATION UPDATE ACKNOWLEDGEMENT message.

Reference configuration

Issue 1: Reply LS to RAN2:

During the online session, we discussed the multiple reference configurations for inter-CU LTM, some companies think it has benefits compared to single reference configuration from RAN3 signaling and standardization effort point of view, some companies think this issue has been decided in RAN2 and we can follow RAN2 agreements.

Since no consensus reached online, we can further check whether to send the reply LS to RAN2 to revisit the reference configuration discussion.

Some background information toward this issue is cited as below:

Agreement in RAN2#127: Agreements on inter-CU LTM Reference configuration: - Inter-CU LTM re-uses the reference configuration from Rel-18 LTM. No additional reference configurations (no multiple reference configurations) are supported. The example comparison between single reference configuration and multiple reference configurations. [7762, LGE]

Question: Whether to send the reply LS to RAN2 to revisit the reference configuration discussion?

Moderator summary:

Some companies think that multiple reference configuration has benefits on multi-vendor scenario.

Send the reply LS: LG, NTT Docomo, Nokia, ZTE, Vodafone

Do not send the reply LS: QC, HW, E///, CATT, NEC, SS

No consensus.

Issue2: Signaling enhancement to support reference configuration:

In [7126, Google], it provides several proposals for RAN3 signaling design to support reference configuration:

For the inter-CU MCG LTM, there could be ways for the source gNB to obtain the reference configuration. It can be based on the serving/source gNB configuration so it is available from the beginning of the inter-CU MCG LTM preparation. Or alternatively, the source gNB can request the reference configuration from the first candidate gNB and provide it to the other candidate gNB(s) in the following preparations. Therefore, the source gNB, if available, may provide a reference configuration in a Handover Request message. Or instead the source gNB can request one by the Handover Request message and obtain one in the Handover Request Acknowledge message from a candidate gNB. Proposal 1: The source gNB can either request a reference configuration or, if available, provide a reference configuration for LTM in a Handover Request message. Proposal 2: A candidate gNB, if requested by the source gNB, provides one reference configuration for LTM in a Handover Request Acknowledge message. Proposal 3: If a reference configuration for LTM is provided in the Handover Request message, the candidate gNB indicates whether the LTM candidate configuration is a full/complete candidate configuration in the Handover Request Acknowledge message.

In this session, we can check whether the above proposal is acceptable.

Moderator summary:

The source gNB can either request a reference configuration or, if available, provide a reference configuration for LTM in a Handover Request message.

A candidate gNB, if requested by the source gNB, provides one reference configuration for LTM in a Handover Request Acknowledge message.

If a reference configuration for LTM is provided in the Handover Request message, the candidate gNB indicates whether the LTM candidate configuration is a full/complete candidate configuration in the Handover Request Acknowledge message.

No consensus on the above proposals.

Error handling of multiple UE associations

In the last meeting, we reached the working assumption as below:

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell. The error handling of multiple UE associations need to be considered.

In [7483, E///], it provides a compromise solution to avoid the complex error handling, and still use one Handover Preparation to prepare only one LTM candidate cell:

When the first Handover Preparation is done, to prepare the first LTM candidate cell controlled by the target gNB, the UE association is established and the pair “Source NG-RAN node UE XnAP ID, Target NG-RAN node UE XnAP ID” is created. When the following Handover Preparation procedures are done, to prepare the other LTM candidate cells controlled by the same target CU (second, third, etc.), the source CU signals to the target CU the Target NG-RAN node UE XnAP ID already assigned by the target CU for the UE. With this information, the target CU can understand that there is an already existing UE association for the UE and avoids assigning a new Target NG-RAN node UE XnAP ID. It seems therefore possible, to resolve the error handling concerning multiple UE associations in stage 3.

We can check whether we can change the WA into agreement in this session:

Proposal: Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

Moderator summary:

Turn the WA into agreement: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

FFS on how the target node to identify the UE association in parallel LTM request.

UE association setup

For subsequent LTM, after each cell switch, the UE association needs to be established between the new serving gNB and candidate gNB(s). During the offline discussion in the last meeting, we summarized following options:

- Option 1: During the LTM preparation phase, the candidate gNB allocates UE XnAP ID and sends it to other candidate gNBs (e.g. via initial source gNB).

- Option 2: After each cell switch, the new serving gNB updates the Source node UE XnAP ID and sends it to other candidate gNBs, and then the candidate gNB-CUs reply with the Target node UE XnAP ID.

- Option 3: After each cell switch, the old serving gNB transfers the UE XnAP ID(s) used by the candidate gNBs to the new serving gNB.

In addition, we reached the following agreement:

Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.

In this meeting, companies have following proposals:

In [7102, Nokia]: Proposal 9: For Subsequent LTM, the new Source gNB signals its XnAP identifiers to all Candidate gNBs within the XnAP LTM CONFIGURATION UPDATE message to update the information related to a given UE context. In [7183, ZTE]: Proposal 5: Old serving gNB transfers the gNB ID of other candidate gNBs and Xn AP ID used by the candidate gNB to new serving gNB. FFS which procedure is used. In [7211, NEC]: Proposal 7: For the UE association between new serving gNB and other candidate gNB, from efficiency perspective, to exchange the AP ID between gNBs after the serving gNB change. Proposal 8: For the UE association between new serving gNB and other candidate gNB, from principle perspective that APID is under control of the node who allocated the APID, it is proposed that the new serving gNB signals its XnAP ID to other candidate gNBs i.e. option 2. In [7234, QC]: Proposal 6: To establish the necessary pairwise UE associations between target gNB and candidate gNBs after cell switch, RAN3 to consider reusing solutions introduced for mobile IAB. In [7261, HW]: Proposal 16: Reusing the serving gNB change notification procedure to setup the UE associated Xn signalling connection between the new serving gNB and the candidate gNBs after each cell switch. In [7389, CATT]: Proposal 3: After each cell switch, the new serving gNB and candidate gNBs establish the UE association. i.e., option 2. In [7483, Ericsson]: Proposal 11: To support subsequent LTM in the RAN, the LTM Configuration Update is used to allocate UE XnAP IDs between the new source gNB and the candidate gNBs. In [7432, Lenovo]: Proposal 6: The new source gNB establishes the UE association with other candidate gNBs after each cell switch. Proposal 7: The source gNB transfers the UE XnAP ID used by the candidate gNBs to the new source gNB. In [7350, SS]: Proposal 7: UE association information between current serving gNB and each candidate CUs will be transferred to all the candidate CUs during LTM preparation phase (Alt 1) or to the new serving CU when cell switch occurs (Alt 2). In [7367, CT]: Proposal 9: The new introduced UE associated Class-1 XnAP procedure can be reused to build the UE association after each LTM cell switch. In [5568, LGE]: Proposal 9: For direct UE-associations between C-CUs, first discuss two aspects before deep dive: 1)How C-CU (who became a new serving CU, i.e. “new source” due to Inter-CU LTM execution from S-CU) identifies other CUs (as potential target, excluding S-CU) to establish UE-associations with? 2)How to identify the same UE? Proposal 10: For (1), consider enhancing the LTM Configuration ID Mapping List IE to include the information of which candidate cell belongs to which CU, which shall be maintained up-to-date (whenever update happens) and synced to all the involved C-CUs for correct LTM operations, thereby no need to worry about the timing of information update. Proposal 11: For (2), agree to use “UE XnAP ID” as baseline for identifying the same UE, when establishing direct UE-associations in-between C-CUs. •FFS on which UE XnAP ID to be used, either one allocated by S-CU during initial preparation toward the receiving C-CU, or allocated by the receiving C-CU during initial preparation (for interaction with S-CU), or both. •FFS on when to be shared across C-CUs.

Based on companies’ contributions this meeting, majority companies think that option 2 is more reasonable and the LTM Configuration Update message can be used to establish the UE associations.

Proposal: To support subsequent LTM, the LTM Configuration Update procedure is reused to allocate UE XnAP IDs between the new source gNB and the candidate gNBs after each LTM Cell Switch.

Moderator summary:

To support subsequent LTM, the LTM Configuration Update procedure is reused to establish UE association between the new source gNB and the other candidate gNB(s) after each inter-CU LTM Cell Switch.

Collected LTM information transfer for subsequent LTM

To support subsequent inter-CU LTM, the LTM information (e.g., early sync configuration, LTM configuration ID, etc) may need to sync up among candidate gNBs.

In Rel-18 inter-DU intra-CU LTM, the following note is captured in stage 2:

NOTE 2: Step 7 may also be triggered after step 19, or after step 22 by implementation for subsequent LTM.

(the step 7 is CU->DU: UE CONTEXT MODICATION REQUEST; the step 19 is DU->CU: DU-CU CELL SWITCH NOTIFICATION; the step 22 is DU->CU: ACCESS SUCCESS)

RAN3 to discuss how the LTM configurations are synced up among candidate gNBs in case of subsequent LTM.  Similar as F1AP, there are three possible options to support LTM information transfer to support subsequent LTM:

- Option 1: Send the collected LTM information to the candidate gNB(s) during the handover preparation phase.

- Option 2: Send the collected LTM information to the target gNB when the source gNB sends the LTM Cell Switch Command to the UE.

- Option 3: Send the collected LTM information to the target gNB when the source gNB receives the Handover Success message from the target gNB.

Proposal 1: It is up to the network implementation when to transmit the collected LTM information to the candidate CUs: during the handover preparation step, or after the Cell Switch Command step, or after Handover Success step.

Proposal 2: Reuse the LTM Configuration Update procedure to transmit the collected LTM information among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID(s).

Moderator summary:

What the detailed collected LTM info should be discussed.

TA Information Transfer

In PDCCH ordered early RACH procedure for inter-CU LTM, in order to transfer the TA information between the candidate gNB and the source gNB. In RAN3#124 meeting, we agreed to introduce a new procedure on Xn to transfer the TA information.

Introduce a new procedure on Xn to transfer the TA information.

Based on the companies’ contribution, moderator summarized the following options to be further discussed:

- Option 1: Introduce a new non-UE associated class 2 procedure

- Option 2: Introduce a new UE associated class 2 procedure

Option1 is similar as what we have done in R18 intra-CU LTM. In [7483, Ericsson], it implies that Option1 may delay TA values transmitted to the source node, and proposal to have a UE associated message to transfer the TA values of certain UE without having to wait for other TA values associated to other UEs.

Moderator wants to check companies’ views towards this issue and whether we can reach some agreements.

Moderator summary:

Moreover, if we reach consensus on the message type, we can further discuss what information can be included in the TA Information Transfer message (e.g. candidate Cell ID, TA value, Preamble index, RA-RNTI, Tag ID Pointier and SSB Positions in Burst).

In [7483, E///], it proposed to keep the existing principle that XnAP Identities and F1AP Identities are independent. While there are companies propose to include the source gNB ID and source gNB-DU ID(s) [7680, CMCC][7261,HW][7350,SS].

In this session, we can discuss including what information can be included in the TA Information Transfer message, the candidate information is listed as below:

- Candidate Cell ID

- TA value

- Preamble index

- RA-RNTI

- Tag ID Pointier

- SSB Positions in Burst

- Early UL Sync Configuration Provider ID

- source gNB ID and source gNB-DU ID

Moderator summary:

Candidate-initiated LTM Cancellation

In the last RAN3 meeting, we agreed to introduce a new procedure for candidate gNB-initiated LTM cancellation. But left the name of the procedure for further discussion.

Introduce a new procedure for candidate gNB-initiated LTM cancellation.

In the contributions to this meeting, many companies provide the name of the new class-2 procedure, for example:

- LTM Handover Cancel, proposed by E///.

- LTM Cancel, proposed by HW.

- Mobility Cancel, proposed by CATT.

- Candidate Cancel, proposed by CATT.

- Configuration Cancel, proposed by CT.

Here, moderator wants to check companies’ views for the above proposed names, and which name is acceptable.

Question: which procedure name is acceptable?

Moderator summary:

Confirm the message name as LTM Cancel for candidate gNB-initiated LTM cancellation.

In section 3.1.5, we discussed the UE association establishment after each cell switch, which means that after the new UE association established, the candidate gNB is aware of the current serving gNB (new source gNB) of the UE. On this premise, the candidate gNB can initiate the LTM Cancel procedure to the new source gNB, which then sends the Candidate Cells To Be Cancelled List IE to all other candidate gNB(s) via current Handover Cancel message.

Proposal: For subsequent LTM, the candidate gNB can initiate the LTM Cancel procedure to the new source gNB, which then sends the Candidate Cells To Be Cancelled List IE to all other candidate gNB(s) via current Handover Cancel message.

Moderator summary:

LTM Release

In RAN3#124 meeting, we left an open issue on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

Currently there is an FFS in the current BLCR for TS38.300 (the editor notes under Step20).

20. The target gNB may send the UE CONTEXT RELEASE to inform the source gNB to release radio and C-plane related resources associated to the UE context. Any ongoing data forwarding may continue. Editor’s Note: Details are FFS on step 20.

In the last RAN3 meeting, we reached following working assumption:

WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB

Moderator wants to check whether we can change the Working Assumption into agreement and remove the FFS in current BLCR.

Moderator summary:

Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exists in the source gNB.

Data forwarding

In current BLCR for TS38.300, there is a note related to the late data forwarding:

NOTE : Late data forwarding may be initiated as soon as the source gNB receives the HANDOVER SUCCESS message.

It is our common understanding that late data forwarding can be initiated after source gNB receives Handover Success message from target gNB. However, for LTM, the source gNB can also initiate data forwarding as soon as it sends the LTM Cell Switch Command to the UE, which is similar as what we have done in R18 LTM.

Proposal: Normal data forwarding may be initiated after the source gNB sends the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation.

Moderator summary:

Normal data forwarding may be initiated after the source gNB sends the LTM Cell Switch Command to the UE, when exactly it is initiated is left to implementation.

Based on companies’ contribution, another data forwarding issue that need to be solved is the data forwarding address transfer among candidate gNBs for subsequent LTM:

In [7351, SS]: Proposal 4: Candidate gNB provides the data forwarding proposal and TNL addresses information for admitted PDU sessions or DRBs in HO REQ ACK in advance during LTM preparation phase. Proposal 5: Source gNB provides the collected all the data forwarding addresses information (including source and candidate gNB) to other each candidate gNB if this candidate gNB proposes data forwarding proposal. In [7102, Nokia]: Proposal 6: The Source gNB includes the PDU Session Resource Information List information of all Candidate gNBs in the XnAP LTM CELL SWITCH NOTIFICATION message. Proposal 7: The Source gNB may also provide PDU Session Resource Information List information of all Candidate gNBs prior to LTM cell switch operation. In [7351, ZTE]: Proposal 3: After LTM completion, for subsequent LTM, the new serving gNB can request other candidate node(s) to perform data forwarding per PDU session/DRB. New serving gNB and other candidate gNB negotiate data forwarding after LTM completion and before next subsequent LTM. Proposal 4: RAN3 to discuss whether and when a candidate gNB flush UE data that is received in early data forwarding after a LTM procedure. In [7432, Lenovo]: Proposal 8: The source gNB collects the data forwarding information associated with all candidate cells, and then sends the collected data forwarding information to all candidate gNBs during the LTM preparation, e.g., via LTM Configuration Update message. In [7261, HW]: Proposal 12: To reuse the LTM Configuration Update procedure to sync up configurations among candidate gNBs for subsequent LTM, including early sync configuration, configuration ID, and data forwarding addresses. It is up to the network implementation when the sync up is performed: during the preparation step, or during the cell switch execution step, or after successful cell switch. In [7367, CT]: Proposal 4: The data forwarding address can be send together with other collected LTM information (e.g. early sync configuration, LTM configuration IDs) to the candidate gNB(s) in case of subsequent LTM.

Moderator summarized above proposals in to following options to be further discussed:

- Option 1: The data forwarding address is sent to the candidate node during LTM preparation phase.

- Option 2: The data forwarding address is sent to the candidate node in the XnAP CELL SWITCH NOTIFICATION message

- Option 3: The data forwarding address is sent together with the collected LTM information during the LTM preparation phase or late steps (up to implementation)

- Option 4: The new serving gNB and other candidate gNB negotiate data forwarding after LTM completion and before next subsequent LTM.

Question: Which option is agreeable? And which message to use?

Moderator summary:

Support of inter-CU LTM in NR-DC scenario.

If time allows, we can open the discussion on support of inter-CU LTM in DC scenario in the offline.

Applicable scenario

Based on [7483, E///], the following are NR-DC scenarios are proposed to be supported for inter-CU LTM for NR-DC:

- Inter-MN LTM without SN Release

- Inter-MN LTM with SN Release

- SN LTM initiated SN Change

In this session, we can check whether the above proposed NR-DC scenario should be supported.

Proposal: RAN3 to discuss the following scenarios to support inter-CU LTM with NR-DC.

- Inter-MN LTM without SN Release, with/without intra-SN LTM

- Inter-MN LTM with SN Release

- SN LTM initiated SN Change

Moderator summary:

SN initiated inter-CU LTM

In this meeting, we received an LS(R3-247111 / R2-2409378) from RAN2 containing the following RAN2 agreements related to inter-CU LTM.

RAN2 #127: 10. Inter-CU SCG LTM preparation can be initiated by source SN. 11. The inter-CU SCG LTM configuration, SN generates SCG part configuration, MN includes it into its MN RRC configuration message. 12. For inter-CU SCG LTM, the LTM cell switch command MAC CE is sent by source SN. 13. RAN2 understands for the security key update of inter-CU SCG LTM, SCPAC security key update mechanism is taken as baseline. We will send LS to SA3 to ask them to take it into account for their works. 14. Only SN-initiated inter-SN LTM (including LTM configuration, early DL/UL synch and LTM execution) is supported in Rel-19. RAN2 #127bis: 3. The SCPAC-similar security update configuration is introduced for inter-CU SCG LTM, i.e. similar to IEs sk-CounterConfiguration, servingSecurityCellSetId and securityCellSetId. The names of the new IEs are to be discussed in stage3. 4. Regarding the candidate and reference configuration generation and signaling design, the following SCPAC-similar principles can be applied for inter-CU SCG LTM as baseline: - The reference configuration for inter-CU SCG LTM at least include SCG part, FFS on MCG part. - FFS: Network ensures that when UE combines the reference and candidate configuration for inter-CU SCG LTM, the configuration generated by UE must contain both MCG and SCG part configurations. - The candidate configuration and reference configuration are modeled as an MN RRCReconfiguration message. - Upon inter-CU SCG LTM, the UE performs reconfiguration with sync towards SCG, but the reconfiguration with sync in MCG is not allowed. - The MN generates the MCG part of the reference configuration (if any), while the SN (source or candidate) generates the SCG part of the reference configuration. - The MN is responsible for the reference configuration generation for SN initiated inter-CU SCG LTM. It can be up to the NW implementation whether to include the MCG part. - The MN can request an SCG reference configuration from any of the involved SNs. 5.For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN. 6.The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells. 7.The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN. 8.In order to support subsequent inter-CU SCG LTM, the MN needs to transfer, during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN. 9.Upon execution of inter-SN SCG LTM, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. 10.Re-use legacy LTM Cell Switch Command MAC CE for inter-SN LTM.

In this session, we can discuss the stage signaling design for SN-initiated inter-CU SCG LTM by taking into account the above RAN2 agreements. An example of the stage-2 signaling flows for SN initiated inter-CU SCG LTM is provided in [7182, ZTE] and cited as below:

Figure 1. SN initiated inter-CU SCG LTM procedure

LTM Preparation:

In step 1~3a, the source SN initiates the inter-CU SCG LTM preparation procedure by sending a SN Change Required message to the MN. Then the MN requests each candidate SN to allocate resources for the UE by means of the SN Addition procedure. Within the list of cells suggested by the source SN, the candidate SN decides the list of PSCell(s) to prepare for SCG LTM, considering of the maximum number indicated by the MN. For each prepared PSCell, the candidate SN provides the corresponding SCG radio resource configuration, and may also provide the L1 RS (e.g. SSB or CSI-RS) configuration for L1 measurement, early UL sync configuration, or TCI-state configuration. The candidate SN sends those configurations to the MN via SN Addition Request Acknowledge message. And then the MN may send the Xn-U Address Indication message to the candidate SN(s) to provide the information for data forwarding and/or trigger the early data forwarding.

5. For SN initiated inter-CU SCG LTM, the candidate SN provides the SCG part configuration of each candidate PSCell, and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN.

Proposal 1: For SN initiated inter-CU SCG LTM, the source SN initiates the inter-CU SCG LTM preparation procedure by sending an SN Change Required message to the MN.

Proposal 2: Then the MN requests each candidate SN to allocate resources for the UE via SN Addition request message.

Proposal 3: Within the list of cells suggested by the source SN, the candidate SN provides the SCG part configuration of each candidate PSCell and may also provide the L1 RS (e.g. a list of SSB or a list of CSI-RS) configuration for L1 measurement, early UL sync configuration or TCI-state configuration, to the MN, via SN addition request ACK message.

Moderator summary:

In step 4~5a, the MN initiates the SN modification procedure towards the source SN and candidate SN(s), to transfer the LTM related configuration of candidate cells.

6.	The source SN is responsible to generate the common CSI resource configuration for L1 measurement on candidate SCG cells.

7. The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN. And the source SN responds with the common CSI resource configuration to the MN.

Proposal 4: The MN sends the received L1 RS configuration, early UL sync configuration, or TCI-state configuration of candidate cells to the source SN, e.g. via SN modification request message, and the source SN responds with the common CSI resource configuration to the MN via SN modification request ACK message.

Moderator summary:

In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), e.g. via MN initiated SN modification procedure. And then the candidate SN can update the candidate SCG configuration (e.g. generating the L1 CSI report configuration), accordingly.

8.	In order to support subsequent inter-CU SCG LTM, the MN needs to transfer ,during the LTM preparation phase, the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s). Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN.

Proposal 5: In order to support subsequent inter-CU SCG LTM, the MN needs to transfer the common CSI resource configuration and the collected information of candidate cells to the candidate SN(s), via SN modification request message. Accordingly, the candidate SN(s) responds with the updated candidate SCG configuration to the MN via SN modification request ACK message.

Moderator summary:

In step 6/7, the MN sends the RRCReconfiguration message to the UE including the SCG LTM configuration. And the UE responses an RRCReconfigurationComplete message to the MN. The MN RRCReconfigurationComplete message includes an SN RRCReconfigurationComplete message to the source SN.

In step 8, the MN informs the source SN with the SN RRCReconfigurationComplete message via SN Change Confirm message, to indicate that SCG LTM is prepared.

Early sync:

In step 9a/9b, the UE performs DL/UL sync with the LTM candidate cell(s) before receiving the cell switch command. For the PDCCH order triggered early RACH, the candidate SN(s) needs to transfer the calculated TA value of candidate cell(s) to the source SN via the MN. The existing TA information transfer procedure via F1 interface can be extended to Xn interface to transfer the TA information between the MN and the source/candidate SN.

Proposal 6: The TA information transfer message can be reused to transfer the TA information from the candidate SN to the source SN via the MN.

Moderator summary:

LTM cell switch execution:

In step 10~11, the UE sends the L1 measurement report(s) to the source SN (i.e. source SN DU). The source SN decides to execute LTM cell switch to the target cell by sending an LTM cell switch command MAC CE to the UE.

In step 12, the source SN sends a notification message to the target SN via the MN, to indicate the triggering of LTM to the UE. The message may include the target cell ID, the selected TCI-state ID(s) of the target cell. We can simply extend the existing cell switch notification procedure via F1 interface to Xn interface.

Proposal 7: The Cell Switch Notification message can be reused to inform the selected target cell from the source SN to the target SN via the MN.

Moderator summary:

LTM cell switch completion:

In step 13/14, the UE sends an MN RRCReconfigurationComplete message to the MN, which includes an SN RRCReconfigurationComplete message. And then the MN informs the target SN that the UE has completed the reconfiguration procedure successfully via SN Reconfiguration Complete message, including the SN RRCReconfigurationComplete message.

In step 15, the UE performs the random access procedure towards the target cell, if UE does not have valid TA of the target cell. If the UE has valid TA of the target cell, the UE skips random access procedure towards the target cell, i.e. RACH-less LTM. The order the UE sends the MN RRCReconfigurationComplete message and performs the Random Access procedure towards the target SCG is not defined. If the UE has performed a RA procedure in step 15, the UE considers that LTM cell switch execution is successfully completed when the random access procedure towards the target SCG is successfully completed. For RACH-less LTM, the UE considers that LTM cell switch execution is successfully completed when the UE determines that the target SN has successfully received its first UL data.

In step 16, the MN initiates the SN modification or release procedure to the source SN, to inform the execution of SCG LTM. The detailed procedure is similar to SN initiated inter-SN SCPAC.

In step 17~18a/b, the data forwarding procedure is performed.

In step 19, if applicable, the update of the UP path towards the core network is performed via a PDU session path update procedure.

The above flow charts and procedures for SN initiated inter-CU SCG LTM in Figure 1 can be taken as the baseline.

Avoid simultaneous MCG and SCG LTM

In RAN3 #123 meeting, RAN3 agreed that inter-CU LTM is not configured in both MCG and SCG at the same time, which is also confirmed by RAN2 agreement and the updated WID.

For coexistence of MCG and SCG LTM, following agreements were reached in RAN2#127bis meeting:

- RAN2 confirms to support coexistence of following cases, it is up to network implementation to ensure simultaneous execution for both MCG and SCG will not happen:

- Inter-MN LTM and intra-SN LTM

- Inter-SN LTM and intra-MN LTM

Besides, in RAN3#123bis meeting, we reached following agreement related to inter-CU LTM in NR-DC:

- Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Regarding this issue, we needs to design solutions to avoid simultaneous configuration for inter-CU MCG and SCG LTM. It is straightforward to enable the MN/source SN to inform the other node once it decides to initiate an inter-CU LTM, the detailed coordination signaling can be further discussed.

Proposal: To avoid simultaneous configuration for inter-CU MCG and inter-CU SCG LTM, the MN/source SN needs to inform the other node once it decides to initiate an inter-CU LTM.

Moderator summary:

TP/BLCRs

Based on the chairman notes’ s guidance, in this offline discussion, we should work on stage2/3 TPs, since we have already endorsed the BLCR for TS38.300 and TS38.423, we can work on the TPs for these two BLCR first, and if time allows, we can further check whether to adopt other stage2/3 BLCR/TPs (i.e. TS38.401, TS 38.420, TS38.473).

TPs for TS38.300

The TP for TS38.300 is proposed in [7262, HW], [7432, Lenovo], [7369, CT], [7484, E///], [5522, CMCC], [5653, ZTE], we can check the contribution if possible.

TPs for TS38.423

The TP for TS38.423 is proposed in [7484, E///], [7433, Lenovo], [7213, NEC], [5459, Lenovo], we can check the contributions if possible.

BLCR to TS38.420

The BLCR for TS38.420 is proposed in [7181, ZTE], we can check the contribution if possible

BLCR to TS37.483

The BLCR for TS37.483 is proposed in [7590, LGE], we can check the contribution if possible

Others

The TP for TS38.401 is proposed in [7352, Samsung], we can check the contribution if possible.

The TP for TS37.340 is proposed in [7182, ZTE], we can check the contribution if possible.

The TP for TS38.473 is proposed in [7261, HW], we can check the contribution if possible.

Conclusion, Recommendations [if needed]

If needed

References

| R3-247006 | LS on the support of semi-persistent CSI-RS resource for LTM candidate cells (RAN1(Fujitsu)) | LS in |
| --- | --- | --- |
| R3-247389 | (draft reply LS) Discussion on support of semi-persistent CSI-RS resource (CATT) | discussion |
| R3-247111 | LS on RAN2 agreements for inter-CU LTM (RAN2(ZTE)) | LS in Resp in R3-247762 |
| R3-247757 | Reply LS on security handling for inter-CU LTM in non-DC cases (SA3(Samsung)) | LS in |
| R3-247758 | Reply LS on security key update of inter-CU SCG LTM (SA3(Xiaomi)) | LS in |
| R3-247709 | Initial considerations on Inter-CU LTM (Rakuten Mobile, Inc) | discussion |
| R3-247747 | Inter-gNB/CU LTM typical deployment scenarios and operational requirements (Jio, NTT DOCOMO, Qualcomm Incorporated, Vodafone, Sony, Bharti Airtel, Telstra) | discussion |
| R3-247234 | Impact analysis of coupling LTM cell switch and CU anchor changes on subsequent inter-gNB LTM (Qualcomm Incorporated, Reliance JIO, Vodafone, NTT DoCoMo, Sony, Bharti Airtel) | discussion |
| R3-247483 | Comprehensive analysis of inter-CU LTM (Ericsson) | discussion |
| R3-247350 | Discussion on inter-gNB-CU LTM (Samsung) | discussion |
| R3-247112 | Support of Multiple Reference Configurations (Nokia. KDDI, LG Electronics, ZTE, Rakuten, Fujitsu) | Discussion Resp in R3-247762 |
| R3-247126 | (TP to BL CR to 38.300 and 38.423) Support for reference configurations in the inter-CU MCG LTM (Google) | other |
| R3-247260 | Further analysis on the PDCP anchor based solution (Huawei, Ericsson, CATT, China Telecom, CMCC,Samsung) | Discussion Resp in R3-247755 |
| R3-247235 | Signalling enhancements for inter-CU LTM HO (Qualcomm Incorporated, Reliance JIO, Vodafone, NTT DoCoMo, Sony, Bharti Airtel) | discussion |
| R3-247661 | Comparison of RRC/PDCP anchor and anchor switch-based solutions for inter-gNB LTM (Qualcomm Incorporated, Reliance JIO, Vodafone, NTT DOCOMO, Sony, Bharti Airtel) | discussion |
| R3-247368 | Discussion on support of inter-CU LTM in NR-DC scenario (China Telecom) | discussion |
| R3-247102 | Discussion on Inter-gNB-CU LTM Procedure (Nokia) | discussion |
| R3-247367 | Discussion on signaling enhancement for inter-CU LTM (China Telecom) | discussion |
| R3-247630 | [Draft] Reply LS on the support of semi-persistent CSI-RS resource for LTM candidate cells (NTT DOCOMO INC..) | LS out To: RAN1 CC: RAN2 |
| R3-247485 | Network coordination for transmission of semi-persistent CSI-RS (Ericsson) | discussion |
| R3-247108 | Discussion on semi-persistent CSI-RS resource for LTM (Nokia) | discussion |
| R3-247263 | [DRAFT] Reply LS on the support of semi-persistent CSI-RS resource for LTM candidate cells (Huawei) | LS out To: RAN1 CC: RAN2 |
| R3-247182 | TP for SN initiated inter-SN LTM (ZTE Corporation) | other |
| R3-247212 | Rel-19 inter-CU LTM in DC scenario (NEC) | discussion |
| R3-247183 | Discussion on inter-CU LTM (ZTE Corporation) | discussion |
| R3-247211 | discussion of Rel-19 inter-CU LTM issues (NEC) | discussion |
| R3-247434 | General aspects of inter-CU SCG LTM (Lenovo) | discussion |
| R3-247591 | Keep continuing discussions on Inter-CU LTM signaling design (LG Electronics Inc.) | discussion |
| R3-247680 | Discussion on inter-CU LTM (CMCC) | discussion |
| R3-247629 | Discussion on inter-CU LTM non-DC case (NTT DOCOMO INC..) | discussion |
| R3-247390 | Discussion for Inter-CU LTM (CATT) | discussion |
| R3-247710 | Data forwarding and transmission in Inter-CU LTM (Rakuten Mobile, Inc) | discussion |
| R3-247351 | Additional discussion on Inter-gNB-CU LTM (Samsung) | discussion |
| R3-247103 | Discussion on Inter-CU LTM Dual Connectivity (Nokia) | discussion |
| R3-247751 | Discussion on inter-CU LTM DC case (NTT DOCOMO INC..) | discussion |
| R3-247391 | Discussion for Inter-CU LTM in DC (CATT) | discussion |
| R3-247590 | Baseline CR for introducing Rel-19 Mobility enhancement in E1AP (LG Electronics Inc., Nokia, China Telecom, Google, Ericsson, CATT, Qualcomm, Samsung, CMCC, ZTE, Huawei, NTT Docomo, Lenovo, NEC) | CR0130r4, TS 37.483 v18.2.0, Rel-19, Cat. B |
| R3-247484 | (TPs to BL CRs for TS 38.300 and 38.423) – Support for inter-CU LTM (Ericsson) | other |
| R3-247432 | [TP to BLCR for TS 38.300] Inter-CU LTM (Lenovo) | other |
| R3-247262 | (TP for LTM BLCR for TS38.300): Update on inter-CU LTM basic procedure (Huawei) | other |
| R3-247369 | (TP for TS38.300) On support of inter-CU LTM (China Telecom) | other |
| R3-247352 | (TP to BLCR for TS 38.401) Procedure on Inter-gNB-CU LTM (Samsung) | other |
| R3-247181 | (BL CR to 38.420) Support for Inter-CU LTM (ZTE Corporation) | CR0046r, TS 38.420 v18.1.0, Rel-19, Cat. B |
| R3-247433 | [TP to BLCR for TS 38.423] Inter-CU LTM (Lenovo) | other |
| R3-247592 | (TP for NR_Mob_Ph4 TS 38.423 BL CR) (LG Electronics Inc.) | other |
| R3-247213 | (TP to 38.423 on inter-CU LTM) inter-CU LTM related information (NEC) | other |
| R3-247261 | (TP for LTM BLCR for TS38.473): Further discussion on inter-CU LTM procedure (Huawei) | other |

Appendix

Previous meeting agreements:

RAN3#123bis:

Prioritize to support inter-CU LTM over Xn interface, and RAN3 specify the inter-CU LTM solutions for standalone scenario first.

Reuse existing Xn Handover Request and Handover Request ACK for Inter-CU LTM initial preparation.

Confirm the case that inter-CU LTM is not configured in both MCG and SCG at the same time.

Early data forwarding can be supported for inter-CU LTM.

Cell Switch Notification from source DU to target DU (in different gNB from source) for LTM execution.

RAN3#124:

Source gNB-CU initiates the handover preparation procedure for inter-gNB-CU LTM.

Introduce a new procedure on Xn to transfer the TA information.

A new XnAP class 2 procedure, namely LTM Cell Switch Notification is introduced on Xn to forward the target Cell ID and target TCI state ID(s) from the source gNB-CU to the target gNB-CU.

Reuse Handover Success procedure over Xn for Rel-19 Inter-CU LTM, to tell the source CU that the UE has accessed to the target Cell.

The Handover Cancel message is reused by the source gNB to release the reserved resource for LTM candidate cells in the candidate gNBs.

Early data forwarding can be triggered before the Source gNB triggers a MAC CE Command to the UE to change cells, timing is left to implementation.

Early sync configuration (TCI state and RACH configuration) can be obtained during the LTM preparation phase through the handover request and handover request acknowledge messages.

A candidate gNB can initiate cancellation of configured LTM candidate cell(s) of its own. Details are FFS.

FFS on the LTM modification procedures.

FFS on whether to reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

RAN3#125:

Introduce a new UE associated Class-1 XnAP procedure to update the LTM configurations for subsequent LTM.

Change the name of LTM Cell Switch Notification message to Cell Switch Notification message.

Reuse the Early Status Transfer and SN Status Transfer message for inter-CU LTM.

Adopt Class-2 procedure for candidate gNB-initiated LTM cancellation. Down select from Option1 and Option3 in the next meeting:

Option 1: Reuse CHO Cancel

Option 2: Rename CHO Cancel

Option 3: Introduce new procedure

RAN3#125bis:

Current SSB information in Xn Setup and Configuration Update procedures can be reused for LTM preparation phase.

WA: For inter-CU LTM mobility, a separate LTM request message (i.e. HANDOVER REQUEST message) is used for each candidate cell.

The error handling of multiple UE associations need to be considered.

The LTM Configuration IDs are allocated by the source CU.

WA: Reuse the existing XnAP UE CONTEXT RELEASE message at the source gNB if no LTM candidate cell(s) exist in the source gNB.

Follow F1AP, the source gNB-CU sends the CSI resource configuration of candidate cells to candidate gNB-CUs via Handover Request message for subsequent LTM, and the candidate gNB-CU sends the CSI report configuration to the source gNB-CU via Handover Request ACK message.

How the source gNB-CU sends the reference configuration to all candidate gNBs is pending on RAN2 progress.

Confirm the name of the new procedure as “LTM Configuration Update”.

Introduce a new procedure for candidate gNB-initiated LTM cancellation.

- Allow UE association in-between candidate CUs (in case the Xn connectivity existed) for subsequent LTM. When and how to establish the UE association is FFS.