3GPP TSG-RAN WG3 Meeting #126				  	                  R3-247854

Orlando, USA, 18th – 22nd November 2024

Title:	[Draft]LS on QMC Coordination for RRC Segmentation in NR-DC

Response to:	-

Release:	Rel-18

Work Item:	NR_QoE_enh-Core

Source:	ZTE[Shall be RAN3]

To:	RAN2

Cc:

Contact person:	Yansheng Liu

liu.yansheng@zte.com.cn

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:

1	Overall description

RAN3 is currently discussing introduction, as correction to Rel-18, of inter-gNB coordination in support of enabling and disabling of RRC segmentation for QoE reporting on SRB4 and/or SRB5 in the NR-DC scenario.

Based on the current mechanism for inter-gNB coordination for QMC in NR-DC, the SN is not aware of whether RRC segmentation is allowed for SRB4. Similarly, the MN is not aware of whether RRC segmentation is allowed for SRB5.

With respect to the above, RAN3 kindly asks RAN2 the following:

- Question 1: Can the UE receive an AppLayerMeasConfig-r17 IE for MCG configuration containing rrc-SegAllowedSRB5-r18 parameter?

- Question 2: Can the UE receive an AppLayerMeasConfig-r17 IE for SCG configuration containing rrc-SegAllowedSRB4-r17 parameter?

- Question 3: If the UE receives from the MN the AppLayerMeasConfig-r17 IE which does not include the rrc-SegAllowedSRB5-r18 parameter, how does the UE treat its RRC segmentation state for SRB5 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB5 or does the UE consider that the RRC segmentation function for SRB5 shall be disabled?

- Question 4: If the UE receives from the SN the AppLayerMeasConfig-r17 parameter which does not include the rrc-SegAllowedSRB4-r17 parameter, how does the UE treat its RRC segmentation state for SRB4 after receiving this AppLayerMeasConfig-r17 parameter? Does the UE keep its previous RRC segmentation state for SRB4 or does the UE consider that the RRC segmentation function for SRB4 shall be disabled?

2	Actions

To RAN2

ACTION: 	RAN3 kindly asks RAN2 to reply to the questions above.

3	Dates of next RAN3 meetings

RAN3#127			17th – 21st  February 2025 		Athens, GR

RAN3#127-bis			7th – 11th April 2025 			Wuhan, CN