3GPP TSG-RAN WG4 Meeting #112bis	R4-2415370

Hefei, China, 14th-18th, October, 2024

Title: 	MPR applicability for FR1 intra-band UL CA

Agenda Item:	6.1.1.2.2

Source: 	Nokia

Document for:	Approval

# Introduction

In previous RAN4 meeting WF of power domain enhancements was approved [1]. This contribution discussed about topic MPR applicability for FR1 intra-band UL CA2 in that WF. Following issues were listed as FFS in WF[1].

#### Issue 2-1-1: Applicable MPR for intra-band contiguous CA with single activated cell

- Proposals

  - Proposal 1: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the single CC MPR requirements can apply. (Samsung)

  - Proposal 2: For PC3/PC2 intra-band contiguous carrier aggregation with single CC with activated cell, the following MPR requirements are applied (Samsung, CATT,, Nokia, Huawei, LGE)

    - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

    - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated;

    - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

- Proposal 3: For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated and refer to the following MPR tables and corresponding configured Tx power requirements (Qualcomm).

        - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C, along with configured Tx power requirements for 6.2.4

        - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated, along with configured Tx power requirements for 6.2D.4

        - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent, along with configured Tx power requirements for 6.2.4

- WF

  - FFS in next meeting

#### Issue 2-1-2: Single CC CBW or aggregated CBW for applying requirements of ACLR/SEM/SE

- Proposals

  - Proposal 1: Integral region and boundary of spurious emissions/ACLR/SEM should be based on the activated CC CBW instead of aggregated CBW. (Samsung)

  - Proposal 2: When only 1 CC is activated the aggregated BW for CA should be used for evaluating metrics such as spurious emissions, SEM and ACLR. (Qualcomm, Huawei)

- WF

  - FFS in next meeting

#### Issue 2-1-3: Single CC or CA requirements of ACLR/SEM/SE applied for single activated cell

- Proposals

  - Proposal 1: Even when transmitted RBs are allocated in only one of the CC, the emissions requirement based on the configured intra-band contiguous ULCA are used (SEM, ACLR, spurious emissions) and all allocations can use the single CC MPR for inner. FFS if a similar approach is applicable to some additional emission requirements. (Skyworks)

  - Proposal 2: Apply single carrier spurious emission/ACLR/SEM requirements for contiguous UL CA with only 1 CC transmitted. (CATT, Samsung, ZTE, vivo)

  - Proposal 3: For Rel-19 MPR applicability for FR1 intra-band contiguous UL CA, the spurious emissions/ACLR/SEM are kept for aggregated/configured CBW. (Huawei, Ericsson)

- WF

  - FFS in next meeting

#### Issue 2-1-4: Draft Rel-19 CR on MPR applicability for intra-band contiguous CA with single CC with activated cell

- Proposals

  - Proposal 1: Add the following description into clause 6.2A.2.1:

    - “For intra-band contiguous carrier aggregation with single CC with activated cell, MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C. MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated. MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.”

- WF

  - FFS the details of draft CR

# Discussion

Earlier in RAN4#111 it was already agreed that For PC3 and PC2 contiguous UL CA use the corresponding single CC MPR when only 1 CC is activated [2]. Open issues that need further decisions are in [1] and listed in clause 1 in this contribution.

Our proposal how to specify remaining open issues are presented in Figure 1 and are as follows.

- When only one CC is activated then

  - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

  - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated;

  - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

- Emission requirements are based on aggregated channel bandwidth also in case that only one CC is activated

  - the spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the edge of the aggregated channel bandwidth

  - the Adjacent Channel Leakage power Ratio is the ratio of the filtered mean power centred on the aggregated channel bandwidth to the filtered mean power centred on an adjacent aggregated channel bandwidth at nominal channel spacing.

  - the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5A.3.1-1 from the edge of the aggregated channel bandwidth

In short use single carrier MPR and UL CA emission requirements.

As the UL CA emission requirements are maintained then also the LO position can be in the middle of the aggregated CH BW thus no interruptions are needed for enabling single carrier MPR because LO re-tuning is not assumed.

Figure 1: MPR applicability for FR1 intra-band UL CA

MPR applicability for FR1 intra-band UL CA :

#### In RAN4#112 meeting [3], the applicable MPR for intra-band contiguous CA with single activated cell is discussed.

#### In case of single CC with activated cell, the following MPR requirements should be applied for PC3/PC2 intra-band contiguous carrier aggregation

- MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

- MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated;

- MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

However, the above MPR could be also applicable for other cases even with more than one activated cell or carrier with CA.

Proposal 4: RAN4 to study cases with more than one activated cell or carrier where single carrier MPR can be applicable.

# Conclusion

In this paper we have proposed

Proposal 1:

- When only one CC is activated then

  - MPR defined in Table 6.2.2-1 applies for UE power class 3 CA bandwidth classes B and C;

  - MPR defined in Table 6.2D.2-1 applies for power class 2 CA bandwidth classes B and C when TxD capability is indicated;

  - MPR defined in Table 6.2.2-2 applies for power class 2 CA bandwidth classes B and C when TxD capability is absent.

Proposal 2:

- Emission requirements are based on aggregated channel bandwidth also in case that only one CC is activated

  - the spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the edge of the aggregated channel bandwidth

  - the Adjacent Channel Leakage power Ratio is the ratio of the filtered mean power centred on the aggregated channel bandwidth to the filtered mean power centred on an adjacent aggregated channel bandwidth at nominal channel spacing.

  - the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.5A.3.1-1 from the edge of the aggregated channel bandwidth

Proposal 3: No interruptions are allowed for enabling single carrier MPR for the case when only one CC is activated in intraband contiguous UL CA.

Proposal 4: RAN4 to study cases with more than one activated cell or carrier where single carrier MPR can be applicable.

# 4	References

[1] R4-2414441, WF on power domain enhancements, RAN4#112

[2] R4-2410749, WF on power domain enhancements, RAN4#111