3GPP TSG-RAN WG4 Meeting # 112-bis					R4-2415417

Hefei Meeting, Oct. 14th – Oct 18th, 2024

Title:	TP to TR 38.719-02-01 CA_n41-n78 w ULCA

Source:	Nokia, Etisalat UAE

Agenda item:	5.3.3

Document for:	Approval

# 1	Introduction

This is a TP to TR 38.719-02-01 to add CA_n41-n78 with intra-band contiguous uplink in band n78.

************************************* Start of TP*****************************************

## 5.x	CA_n41-n78

### 5.x.1	Common for 1 band UL and 2 bands UL CA

#### 5.x.1.1	Operating bands for CA

Table 5.x.1.1-1: CA band combination constituent bands definition

| NR Band | Uplink (UL) band | Downlink (DL) band | Duplex |
| --- | --- | --- | --- |
|  | BS receive / UE transmit | BS transmit / UE receive | mode |
|  | FUL_low – FUL_high | FDL_low – FDL_high |  |
| n41 | 2496-2690 | 2496-2690 | TDD |
| n78 | 3300-3800 | 3300-3800 | TDD |
| NOTE 1: Applicable for UE supporting inter-band carrier aggregation with mandatory simultaneous Rx/Tx capability. |  |  |  |

#### 5.x.1.2	Channel bandwidths per operating band for CA

Table 5.x.1.2-1: Supported bandwidths per CA band combination

| CA operating/channel bandwidth (MHz) |  |  |  |  |
| --- | --- | --- | --- | --- |
| NR CA configuration | Uplink CA configuration or single uplink carrier | NR Band | Channel bandwidth (MHz) | Bandwidth combination set |
| CA_n41A-n78C | CA_n78C CA_n41A-n78A CA_n41A-n78C | n41 | 5,10,15,20,25,30,35,40,45,50,60,70,80,90,100 | 0 |
|  |  | n78 | CA_n78C_BSC4 & 5 |  |

#### 5.x.1.3	UE co-existence studies for 1 band UL

Table 5.X.1.3-3 summarizes frequency ranges where IMD products caused by one UL band with 2CC intra-band UL CA may occur for headline

Table 5.x.1.3-3: Intra-band ULCA IMD overlap with the other DL band analysis.

| All in MHz | flow | fhigh | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| n78 fUL | 3300 | 3800 | Order | flow | fhigh |
| n41 fDL | 2496 | 2690 |  |  |  |
| 2CCBW1 | Minimum | Maximum | IMD2 | Min2CCBW | Max2CCBW |
|  | 70 | 200 | (1-1) | 70 | 200 |
| Close to UL IMD range2 |  |  | IMD4 | 2*Min2CCBW | 2*Max2CCBW |
| Order | flow | fhigh | (2-2) | 140 | 400 |
| IMD3 | fULlow-Max2CCBW | fULhigh+Max2CCBW | IMD6 | 3*Min2CCBW | 3*Max2CCBW |
| (2-1) | 3100 | 4000 | (3-3) | 210 | 600 |
| IMD5 | fULlow-2*Max2CCLBW | fULhigh+2*Max2CCBW | Close to H2 IMD range4 |  |  |
| (3-2) | 2900 | 4200 | Order | flow | fhigh |
| IMD7 | fULlow-3*Max2CCBW | fULhigh+3*Max2CCBW | IMD4 | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
| (4-3) | 2700 | 4400 | (3-1) | 6400 | 7800 |
| IMD9 | fULlow-4*Max2CCBW | fULhigh+4*Max2CCBW | IMD6 | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
| (5-4) | 2500 | 4600 | (4-2) | 6200 | 8000 |
| IMD11 | fULlow-5*Max2CCBW | fULhigh+5*Max2CCBW | Close to H3 IMD range4 |  |  |
| (6-5) | 2300 | 4800 | Order | flow | fhigh |
| IMD13 | fULlow-6*Max2CCBW | fULhigh+6*Max2CCBW | IMD5 | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
| (7-6) | 2100 | 5000 | (4-1) | 9700 | 11600 |
| Analysis | IMD9, IMD11 and IMD13 of n78C may fall into n41 receive band |  |  |  |  |
| NOTE 1: 2CCBW is the instantaneous transmit bandwidth of the two intra-band UL CCs: - The minimum 2CCBW for contiguous / non-contiguous intra-band ULCA is 0 / minimum UL channel bandwidth - The maximum 2CCBW for contiguous / non-contiguous ULCA is Min(maximum aggregated bandwidth / maximum separation bandwidth(600MHz),fULhigh-fULlow) NOTE 2: The close to UL IMD range is the most critical when the victim DL band in proximity to the UL band: - For contiguous/non-contiguous intra-band ULCA within a TDD band, IMD order up to 9/7 should be considered and MPR assumed - For intra-band ULCA within a FDD band, IMD order up to 13 should be considered for bands in the same band group and MPR is not assumed. If justified by poor filtering performance, higher order IMD may need to be specified. NOTE 3: The BB IMD range should only be considered if the DL band is below the UL band and for non-contiguous ULCA within a TDD band >3GHz (assuming CA with 450MHz bands is not considered) - IMD2 is not considered assuming CA with 450MHz bands is not considered - IMD4 is considered for FDD or SimRx/Tx TDD bands <1GHz - IMD6 is considered for FDD or SimRx/Tx TDD bands <1.68GHz NOTE 4: The harmonic 2 and 3 IMD ranges should only be considered if the DL band is above the UL band |  |  |  |  |  |

#### 5.x.1.4	∆TIB,c and ∆RIB,c values

Already captured in fallbacks.

#### 5.x.1.5	REFSENS requirements

The impact of the IMD9 product is found similar to that or CA_n41C-n77A with uplink CA_n41C into n77 and the MSD value is re-used.

Table 5.x.1.5-1: 2DL/2UL inter-band Reference sensitivity QPSK PREFSENS and uplink/downlink configurations for PC3 CA

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n78 | n41 | N/A | 10 | N/A | 2500 | 2.7 | TDD | IMD9 |
|  | n78 | 3350 3450 | 100 | 1 (RBSTART=0) 1 (RBSTART=272) | N/A | N/A | TDD | N/A |

#### 5.x.1.6	OOB blocking exception requirements

Already captured in fallbacks.

### 5.x.2	Specific for 2 bands UL CA

#### 5.x.2.2	UE co-existence studies for 2 bands UL

Table 5.x.2.2-2 lists Band n41 + Band n78 2 bands UL CA(3CC) triple beat IMD analysis for Band n41- Band n78. Note that this is TDD-TDD, so there’s no active RX during TX operation of both uplink bands

Table 5.x.2.2-2: Band n41 and Band n78 for 3CC UL IMD products

| Band / CA1 | n41 |  | CA_n78B/C |  |
| --- | --- | --- | --- | --- |
| Frequency limit (all MHz) | fx_low | fx_high | fy_low | fy_high |
| fUL | 2496-2690 |  | 3300-3800 |  |
| fDL | 2496-2690 |  | N/A | N/A |
| 2CCBW2 | N/A | N/A | Minimum | Maximum |
|  |  |  | 0 -200 |  |
| IMD3 products | fxUL_low-max2CCBW | fxUL_low | fxUL_high | fxUL_high+max2CCBW |
| IMD3 (MHz) | 2296-2496 |  | 2690-2890 |  |
| Analysis | TDD-TDD combination so no RX bands are impacted. |  |  |  |
| NOTE 1: If the two bands are not part of the same or adjacent band groups as defined in table A.1, the analysis can be ignored. NOTE 2: For contiguous intra-band ULCA, the minimum and maximum separation BW are 0MHz and Min(fy_high-fy_low, maximum aggregated BW) respectively. |  |  |  |  |

#### 5.x.2.3	REFSENS requirements

No impact on Refsens in inter-band ULCA for these TDD bands.

************************************* End of TP******************************************