# 3GPP TSG-RAN WG4 Meeting # 112-bis                              R4-2415906

# Hefei, China, Oct.14th – 18th, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Further discussion on R19 2Tx/3Tx PC2/1.5 Inter-band NR CA/ENDC

# Agenda item:	6.1.1.1.3

# Document for:	Approval

# 1 Introduction

In last meeting, some agreements/open issues are included in the WF[2]. In this contribution, we give some further discussions on 2Tx/3Tx PC2/1.5 Inter-band NR CA/ENDC.

# 2	Discussion

- 3UL CC with 2Tx

- For the 3UL CC configuration with 3Tx, the agreements are:

## 2.3 3UL CC with 3Tx Adhoc agreement: Don not consider 3ULCC with 3Tx CA_nXA-nY(2A)/ DC_XA_nY(2A) scenario

- The above agreement is for 3Tx, and it seems the 3UL CC with 2Tx is missing. For 2Tx 3UL CC, we think the above agreements can also be applied since it originally comes from PC3 2Tx band combination basket WID, as we stated in [3].

- Meanwhile, 3UL CC with inter+intra-band contiguous like CA_nXA-nYC/ DC_XA_nYC scenario are not precluded, which means such configurations could be still valid. In this case, we must exclude more than 3UL CC configurations to align with the PC3 band combination, which is: Combinations with four UL CCs are not supported.

- So to make the configurations more clear, it is proposed:

- Proposal 1: Do not consider 3ULCC with PC2/PC1.5 2Tx CA_nXA-nY(2A)/ DC_XA_nY(2A) scenario.

- Proposal 2: Do not consider 4UL CC with PC2/PC1.5 2Tx/3Tx inter-band UL NR CA/ENDC scenario.

# 3 Conclusion

In this contribution, we give some further discussions on 2Tx/3Tx PC2/1.5 Inter-band NR CA/ENDC. The proposals and conclusions are:

- Proposal 1: Do not consider 3ULCC with PC2/PC1.5 2Tx CA_nXA-nY(2A)/ DC_XA_nY(2A) scenario.

- Proposal 2: Do not consider 4UL CC with PC2/PC1.5 2Tx/3Tx scenario.

# 4	Reference

[1] R4-2410750, WF on high power UE, Samsung

[2] R4-2414277, WF on HPUE for CA in TN, Samsung

[3] R4-2411881, Further discussion on R19 2Tx3Tx PC21.5 Inter-band NR CAENDC, ZTE Corporation, Sanechips