3GPP TSG-RAN WG4 Meeting #112bis	R4-2416455

Hefei, China, October 14th – 18th, 2024

Agenda Item: 6.4.3

Source: MediaTek inc.

Title: Discussion on core part for FR1 less than 5MHz

Document for: Discussion

# Introduction

The open issues for the discussion of FR1 <5MHz are captured in the way forward (WF). In this document, we provide our views on NW indication on PBCH BW in MO configuration and/or HO command [1].

Sub-topic 2-3 Other RRM impacts Issue 2-3-1: NW indication on PBCH BW in MO configuration and/or HO command: - Proposals - Option 1: RAN4 to discuss whether NW indication on PBCH BW in MO configuration is needed considering a cell with less than 5MHz can be SCell. - Option 2: If SCell can use 12PRB SSB bandwidth in R19, it’s necessary to provide assistance information to UE regarding whether the PBCH is 12 or 20 PRBs in either MO or HO command.

# Discussion on the indication of PBCH BW

The SSB index reading is based on reading the DMRS in PBCH signals. For legacy NR requirements, the SSB is made of 20 PRBs, which is the case for all previous NR releases for different features. For < 5MHz UE operating with 3 MHz, the UE is required to measure a truncated SSB, which is made of 12 PRBs. Given that the SSB index reading is impacted on whether the PBCH is 12 or 20 PRBs, thus, it is essential to the UE to know whether the PBCH is 12 PRBs or 20 PRBs when performing measurements and handover. Based on the discussion from the previous release, some companies highlighted that the UE could predict whether the PBCH is 12 PRBs or 20 PRBs using the sync raster information. Therefore, we provide discussion on whether it is sufficient to rely on the sync raster to provide the UE with information on whether the SSB is 12 or 20 PRBs.

First, the sync raster location and formulas are provided in clause 5.4.3 in TS 38.104 with minimum frequency spacing between any two consecutive points of sync raster equal to 100 kHz, as provided in the tables below and sketched in the figure 1.

Table 5.4.3.1-1: GSCN parameters for the global frequency raster for above 3 MHz channel bandwidth Range of frequencies (MHz); SS block frequency position SSREF; GSCN; Range of GSCN. 0 – 3000; N * 1200 kHz + M * 50 kHz, N = 1:2499, M ϵ {1,3,5} (Note); 3N + (M-3)/2; 2 – 7498. NOTE: The default value for operating bands which only support SCS spaced channel raster(s) is M=3.. Table 5.4.3.1-2: GSCN parameters for the global frequency raster for 3 MHz channel bandwidth Range of frequencies (MHz); SS block frequency position SSREF; GSCN; Range of GSCN. 0 – 1000; N * 600 kHz + M * 50 kHz + 300 kHz, N = 1:1665, M ϵ {1,3,5} (Note 1); 26638 + 3N + (M-3)/2; 26640 – 31634. NOTE 1: Only applicable for 15 PRB DCH transmission within 3 MHz channel bandwidth with punctured PBCH defined in TS 38.211 [9] clause 7.4.3.1..

Figure 1: Legacy (20 PRBs) for above 3 MHz vs new sync raster (12 RBs punctured SSB) for 3 MHz.

Observation 1: The frequency spacing between two consecutive points of sync raster is equal to 100 kHz.

On the other hand, the measurement object indicates the SSB frequency, as in TS 38.331 shown below, which indicated by ‘ARFCN-ValueNR’.

MeasObjectNR ::= SEQUENCE { ssbFrequency ARFCN-ValueNR OPTIONAL, -- Cond SSBorAssociatedSSB ssbSubcarrierSpacing SubcarrierSpacing OPTIONAL, -- Cond SSBorAssociatedSSB smtc1 SSB-MTC OPTIONAL, -- Cond SSBorAssociatedSSB smtc2 SSB-MTC2 OPTIONAL, -- Cond IntraFreqConnected

The ARFCN-ValueNR is defined in TS 38.101-1 in clause 5.4.2.1, as provided below:

#### 5.4.2.1 NR-ARFCN and channel raster The global frequency channel raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels, SS blocks and other elements. The global frequency raster is defined for all frequencies from 0 to 100 GHz. The granularity of the global frequency raster is ΔFGlobal. RF reference frequencies are designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range (0…2016666) on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in table 5.4.2.1-1 and NREF is the NR-ARFCN. FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs) Table 5.4.2.1-1: NR-ARFCN parameters for the global frequency raster Frequency range (MHz); ΔFGlobal (kHz); FREF-Offs (MHz); NREF-Offs; Range of NREF. 0 – 3000; 5; 0; 0; 0 – 599999. 3000 – 24250; 15; 3000; 600000; 600000 – 2016666. The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The RF reference frequency for an RF channel maps to a resource element on the carrier. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster with a granularity ΔFRaster, which may be equal to or larger than ΔFGlobal.

From above description, the granularity of the global frequency raster is ΔFGlobal is equal to 5 kHz, which is much smaller than the granularity of the sync raster, which is equal to 100 kHz. This means that the NW can configure the UE with SSB for SCell on different centre frequency than that used in sync raster. Therefore, the UE cannot predict whether this SSB configured in the measurement object (MO) is 12 PRBs or 20 PRBs.

Observation 2: the granularity of the global frequency raster is ΔFGlobal is equal to 5 kHz, which is much smaller than the granularity of the sync raster, which is equal to 100 kHz.

Observation 3: The SSB of SCell is not aligned with sync raster, hence UE cannot determine the SSB BW by simply checking the sync raster.

Furthermore, a UE is not expected to perform handover to a SCell, hence there is no need to add additional information in the handover command.

Observation 4: a UE is not expected to perform handover to a SCell.

Therefore, RAN4 shall ask RAN2 to add the information of whether the SSB is 12 PRBs or 20 PRBs in the MO.

- RAN4 shall ask RAN2 to add the information of whether the SSB is 12 or 20 PRBs in the measurement object (MO).

- There is no need to add the information of whether the SSB is 12 or 20 PRBs in the handover command (HO).

- RAN4 shall send the following LS as shown in this document:

- Overall Description: Based on discussions in RAN4#112-bis on the WI of NR_FR1_lessthan_5MHz_BW_Ph2, RAN4 has reached consensus to assist UE to differentiate whether the SSB is 12 PRBs or 20 PRBs during the measurements of SCell. Therefore, RAN4 would like to ask RAN2 to update the measurement object (MO) to include whether the SSB is 12 PRBs or 20 PRBs. 3. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account and add the indication on whether the SSB is 12 PRBs or 20 PRBs in the measurement object (MO).

# Summary

In this contribution, we have the following proposals:

Observation 1: The frequency spacing between two consecutive points of sync raster is equal to 100 kHz.

Observation 2: the granularity of the global frequency raster is ΔFGlobal is equal to 5 kHz, which is much smaller than the granularity of the sync raster, which is equal to 100 kHz.

Observation 3: the bands allowing <5MHz channel BW also supports other channel BW, UE cannot determine the SSB BW by simply checking the band index.

Observation 4: for handover unknown cell the UE cannot predict whether this SSB is punctured or not because the cell was not measured and the SSB in that cell was not indicated by the MO.

Proposal 1: RAN4 shall ask RAN2 to add the information of whether the SSB is 12 or 20 PRBs in the measurement object (MO).

Proposal 2: There is no need to add the information of whether the SSB is 12 or 20 PRBs in the handover command (HO).

Proposal 3: RAN4 shall send the following LS as shown in this document:

- Overall Description: Based on discussions in RAN4#112-bis on the WI of NR_FR1_lessthan_5MHz_BW_Ph2, RAN4 has reached consensus that for the UE to differentiate whether the SSB is 12 PRBs or 20 PRBs during the measurements of SCell. Therefore, RAN4 would like to ask RAN2 to update the measurement object (MO) to include whether the SSB is 12 PRBs or 20 PRBs. 3. Actions: To RAN2: RAN4 respectfully asks RAN2 to take the above information into account and add the indication on whether the SSB is 12 PRBs or 20 PRBs in the measurement object (MO).

# References

- R4-2413883, “WF for [112][214] NR_FR1_lessthan_5MHz_BW_Ph2”, Intel, Maastricht, NL, Aug 2024.