3GPP TSG-RAN4 Meeting #113	R4-2417522

# Orlando, United States, 18th Nov 2024 - 22nd Nov 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2515 | rev | - | Current version: | 18.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Core) Replacement of dual with 2Tx |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon, Ericsson, Qualcomm, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core, NR_RF_TxD-Core |  |  |  |  |  | Date: |  |  | 2024-11-01 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Two different terms with the same meaning have been used. That makes readers confugused. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replace dual with 2Tx |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | This makes readers confused as if they were different meanings and/or purposes. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1, 6.2.2, 6.2.3.2, 6.2.3.18, 6.2A.2.1, 6.2D.1, 6.2D.2, 6.2D.3, 6.2E.2.2, 6.2F.2, 6.2F.3.10, 6.2G.2, 7.3A.2.1, 7.3A.2.2, 7.3A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The corresponding Rel-16 Cat F CR is R4-2417520. Purpose of the corrections is the same across the releases. Rel-17 spec, however, started introducing explicitly TxD, more corrections are seen in Rel-17 and Rel-18. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of changes>

### 6.2.1	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth of NR carrier unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.1-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n2 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n3 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n5 |  |  |  |  |  |  | 23 | ±2 |
| n7 | 316 | +2/-3 |  |  | 26 | +2/-33 | 23 | ±23 |
| n8 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n12 |  |  |  |  |  |  | 23 | ±23 |
| n13 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n14 | 316,7 | +2/-3 |  |  | 26 | +2/-3 | 23 | ±2 |
| n18 |  |  |  |  |  |  | 23 | ±2 |
| n20 |  |  |  |  |  |  | 23 | ±23 |
| n24 |  |  |  |  |  |  | 23 | +2/-33 |
| n25 | 316 | +2/-3 |  |  | 26 | +2/-33 | 23 | ±23 |
| n26 |  |  |  |  |  |  | 23 | ±23 |
| n28 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n30 |  |  |  |  |  |  | 23 | ±2 |
| n31 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| n34 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n38 |  |  |  |  |  |  | 23 | ±2 |
| n39 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n40 | 316 | +2/-3 | 295 | +2/-3 | 26 | +2/-3 | 23 | ±2 |
| n41 | 316 | +2/-3 | 295 | +2/-33 | 26 | +2/-33 | 23 | ±23 |
| n47 |  |  |  |  |  |  | 23 | ±2 |
| n48 |  |  |  |  |  |  | 23 | +2/-3 |
| n50 |  |  |  |  |  |  | 23 | ±2 |
| n51 |  |  |  |  |  |  | 23 | ±2 |
| n53 |  |  |  |  |  |  | 23 | ±2 |
| n54 |  |  |  |  |  |  | 23 | ±2 |
| n65 |  |  |  |  |  |  | 23 | ±2 |
| n66 | 316 | +2/-3 |  |  | 26 | +2/-3 | 23 | ±2 |
| n70 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n71 | 316 | +2/-3 |  |  | 26 | +2/-3 | 23 | +2/-2.5 |
| n72 | 316 | +2/-3 |  |  |  |  | 23 | ±2 |
| n74 |  |  |  |  |  |  | 23 | ±2 |
| n77 | 316 | +2/-3 | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n78 | 316 | +2/-3 | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n79 |  |  | 295 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |
| n80 |  |  |  |  | 26 | +2/-33 | 23 | ±23 |
| n81 |  |  |  |  |  |  | 23 | ±2 |
| n82 |  |  |  |  |  |  | 23 | ±2 |
| n83 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n84 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n85 | 316 | +2/-3 |  |  | 26 | +2/-33 | 23 | ±23 |
| n86 |  |  |  |  |  |  | 23 | ±2 |
| n89 |  |  |  |  |  |  | 23 | ±2 |
| n91 |  |  |  |  |  |  | 23 | ±23, 4 |
| n92 |  |  |  |  |  |  | 23 | ±23, 4 |
| n93 |  |  |  |  |  |  | 23 | ±23, 4 |
| n94 |  |  |  |  |  |  | 23 | ±23, 4 |
| n95 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n97 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n98 |  |  |  |  | 26 | +2/-3 | 23 | ±2 |
| n99 |  |  |  |  |  |  | 23 | +2/-33 |
| n100 | 316, 8 | +2/-3 |  |  |  |  | 23 | ±2 |
| n101 | 316, 8 | +2/-3 |  |  |  |  | 23 | ±2 |
| n104 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |
| n105 |  |  |  |  |  |  | 23 | +2/-2.5 |
| n106 |  |  |  |  |  |  | 23 | ±2 |
| n109 |  |  |  |  |  |  | 23 | ±23, 4 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Power class 3 is default power class unless otherwise stated. NOTE 3: Refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 4: The maximum output power requirement is relaxed by reducing the lower tolerance limit by 0.3 dB. NOTE 5: Achieved via 2Tx. NOTE 6: Generally, PC1 UE is not targeted for smartphone form factor. NOTE 7: The UE power class 1 requirements for Band n14 are applicable for public safety scenario only. NOTE 8: PC1 in Band n100 and n101 is allowed only for FRMCS cab-radio’s, i.e. user equipments installed on trains with external antenna on top of the train roof at approximately 4m over ground. |  |  |  |  |  |  |  |  |

<Unnecessary part is omitted>

### 6.2.2	UE maximum output power reduction

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 2 and 3 and UE power class 1, the allowed maximum power reduction (MPR) is defined in Table 6.2.2-2, Table 6.2.2-1, Table 6.2.2-4b and Table 6.2.2-5, respectively for channel bandwidths  ≤ 100 MHz.  For UE power class 1.5 with 2Tx, the allowed maximum power reduction  (MPR) is defined in Table 6.2D.2-2 and Table 6.2D.2-3 in accordance with the indicated modifiedMPR-Behavior specified in Table L.1-1 for channel bandwidths ≤ 100 MHz. For UE power class 1.5 with 4 Tx, the allowed maximum power reduction is defined in Table 6.2D.2-4, 6.2D.2-5. When A UE that indicates PC1.5 for a given band is limited to PC2 by the rules in clause 6.2.1, the MPR requirements in Table 6.2.2-2 apply.

If the relative channel bandwidth ≤ 4% for TDD bands or ≤ 3% for FDD band, the ∆MPR is set to zero.

If the relative channel bandwidth > 4% for TDD bands or > 3% for FDD bands, the ∆MPR is defined in Table 6.2.2-3.

Where relative channel bandwidth = 2*BWChannel / (FUL_low + FUL_high)

The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation.

<Unnecessary part is omitted>

#### 6.2.3.2	A-MPR for NS_04

<Unnecessary part is omitted>

Table 6.2.3.2-2: A-MPR' values Access (Power Class 1.5, 2 and 3)

| Modulation/Waveform |  | A-MPR' (dB) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PC3_A1 | PC3_A2 | PC2_A3 | PC2_A4 | PC1.5_A51 | PC1.5_A61 |
| DFT-s-OFDM | Pi/2-BPSK | ≤ 3.5 | ≤ 3.5 | ≤ 3.5 | ≤ 5.5 | ≤ 5 | ≤ 7 |
|  | QPSK | ≤ 4 | ≤ 4 | ≤ 4.5 | ≤ 6 | ≤ 6 | ≤ 7.5 |
|  | 16 QAM | ≤ 4 | ≤ 4 | ≤ 5 | ≤ 6 | ≤ 6.5 | ≤ 7.5 |
|  | 64 QAM | ≤ 4 | ≤ 4.5 | ≤ 5 | ≤ 6.5 | ≤ 6.5 | ≤ 8 |
|  | 256 QAM | ≤ 4.5 | ≤ 6 | ≤ 6.5 | ≤ 8 | ≤ 8 | ≤ 9.5 |
| CP-OFDM | QPSK | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 16 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 64 QAM | ≤ 5.5 | ≤ 5.5 | ≤ 6.5 | ≤ 7.5 | ≤ 8 | ≤ 9 |
|  | 256 QAM | ≤ 6.5 | ≤ 8 | ≤ 7.5 | ≤ 10 | ≤ 9 | ≤ 11.5 |
| NOTE 1: PC1.5 assumes 2Tx. |  |  |  |  |  |  |  |

<Unnecessary part is omitted>

#### 6.2.3.18	A-MPR for NS_47

Table 6.2.3.18-1: A-MPR regions and types for NS_47 (Power Class 2 and 3)

| Channel Bandwidth, (MHz) | Carrier Centre Frequency, Fc, (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 30MHz | Fc=2560-2560.020 | ≤5.04 | ≤1.44 | A1 |
|  |  | >5.04, ≤9.6 | ≤1.44 | A2 |
|  |  | >24.48 | ≤1.44 | A3 |
|  |  | ≤9.6 | >21 | A2 |
|  |  |  | >14.4, <21 | A4 |
|  |  | ≤6.12 | >10, ≤14.4 | A4 |
|  |  |  | >1.44, <10 | A2 |
| NOTE: The A-MPR values are listed in Table 6.2.3.18-2. |  |  |  |  |

Table 6.2.3.18-2: A-MPR for modulation and waveform type (Power Class 2 and 3)

| Modulation/Waveform | A1(dB) |  | A2(dB) |  | A3(dB) |  | A4(dB) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PC3 | PC2 | PC3 | PC2 | PC3 | PC2 | PC3 | PC2 |
|  | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner | Outer/ Inner |
| DFT-s-OFDM PI/2 BPSK | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 | ≤ 2 | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 | ≤ 2 | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 5.5 | ≤ 8.5 |  | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 6 | ≤ 8.5 |  | ≤ 5 | ≤ 3 | ≤ 6 |
| DFT-s-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 6 | ≤ 8.5 |  | ≤ 5 |  | ≤ 6 |
| CP-OFDM QPSK | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 | ≤ 4 | ≤ 7 |
| CP-OFDM 16 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 | ≤ 4 | ≤ 7 |
| CP-OFDM 64 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  | ≤ 5 |  | ≤ 7 |
| CP-OFDM 256 QAM | ≤ 7 | ≤ 10 | ≤ 7 | ≤ 10 |  |  |  | ≤ 7 |

Table 6.2.3.18-3: A-MPR regions and types for NS_47 (Power Class 1.5)

| Channel Bandwidth, (MHz) | Carrier Centre Frequency, Fc, (MHz) | RBstart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 30MHz | Fc=2560-2560.020 | ≤5.04 | ≤1.44 | A1 |
|  |  | >5.04, ≤9.6 | ≤1.44 | A2 |
|  |  | >24.48 | ≤1.44 | A3 |
|  |  | ≤9.6 | >21 | A2 |
|  |  |  | >14.4, <21 | A4 |
|  |  | >6.12, ≤7.92 | >10, ≤14.4 | A5 |
|  |  | ≤6.12 | >10, ≤14.4 | A4 |
|  |  |  | >1.44, <10 | A2 |
| NOTE: The A-MPR values are listed in Table 6.2.3.18-4. |  |  |  |  |

Table 6.2.3.18-4: A-MPR for NS_47 (Power Class 1.5)

| Modulation/Waveform |  | A1(dB) | A2(dB) | A3(dB) | A4(dB) | A5(dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner | Outer/Inner |
| DFT-s-OFDM | PI/2 BPSK | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 | ≤ 3 |
|  | QPSK | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 | ≤ 3 |
|  | 16 QAM | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 | ≤ 3 |
|  | 64 QAM | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 |  |
|  | 256 QAM | ≤ 13 | ≤ 11 | ≤ 8 | ≤ 8.5 |  |
| CP-OFDM | QPSK | ≤ 13 | ≤ 12.5 | ≤ 8 | ≤ 9.5 | ≤ 4 |
|  | 16 QAM | ≤ 13 | ≤ 12.5 | ≤ 8 | ≤ 9.5 | ≤ 4 |
|  | 64 QAM | ≤ 13 | ≤ 12.5 | ≤ 8 | ≤ 9.5 |  |
|  | 256 QAM | ≤ 13 | ≤ 12.5 | ≤ 8 | ≤ 9.5 |  |
| NOTE 1: PC1.5 assumes 2Tx. |  |  |  |  |  |  |

<Unnecessary part is omitted>

#### 6.2A.2.1	UE maximum output power reduction for Intra-band contiguous CA

<Unnecessary part is omitted>

Table 6.2A.2.1-1b: Contiguous RB allocation for Power Class 2 with 2Tx2

| Modulation |  | MPR for bandwidth class B(dB) |  | MPR for bandwidth class C(dB) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer1 | inner | outer |
| DFT-s-OFDM | Pi/2 BPSK | 3.0 | 5.01 | 3.5 | 8 |
|  | QPSK | 3.0 | 5.01 | 3.5 | 8 |
|  | 16QAM | 3.5 | 5.01 | 3.5 | 8 |
|  | 64QAM | 4.0 | 5.51 | 6 | 8 |
|  | 256QAM | 6.5 | 7.0 | 8 | 8.5 |
| CP-OFDM | QPSK | 3.0 | 5.51 | 4.0 | 8.5 |
|  | 16QAM | 3.5 | 5.51 | 4.0 | 8.5 |
|  | 64QAM | 4.0 | 5.51 | 5.5 | 8.5 |
|  | 256QAM | 7.0 | 7.0 | 7.5 | 8.5 |
| NOTE 1: When 1 RB or 2 RB are allocated at the lower edge of lowest CC or upper edge of upper CC, MPR for outer is 5.5 dB. NOTE 2: UE indicating TxD supported |  |  |  |  |  |

<Unnecessary part is omitted>

Table 6.2A.2.1-4: non-contiguous RB allocation for Power Class 2 with 2Tx4

| Modulation |  | MPR for bandwidth class B(dB) |  |  | MPR for bandwidth class C(dB) |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | inner | Outer12 | Outer23 | Inner | Outer12 | Outer23 |
| DFT-s-OFDM | Pi/2 BPSK | 41 | 7.5 | 14 | 41 | 8.5 | 14.5 |
|  | QPSK | 41 | 7.5 |  | 41 | 8.5 |  |
|  | 16QAM | 41 | 7.5 |  | 41 | 8.5 |  |
|  | 64QAM | 6 | 7.5 |  | 6 | 8.5 |  |
|  | 256QAM | 7.5 | 8 |  | 7.5 | 8.5 |  |
| CP-OFDM | QPSK | 4.51 | 8 | 15 | 4.51 | 9 | 15.5 |
|  | 16QAM | 4.51 | 8 |  | 4.51 | 9 |  |
|  | 64QAM | 6 | 8 |  | 6 | 9 |  |
|  | 256QAM | 8.5 | 8.5 |  | 8.5 | 9 |  |
| NOTE 1: the allowed MPR is [4]dB for aggregated allocation bandwidth < [2MHz]. NOTE 2: Outer 1 MPR for Pi/2 BPSK and QPSK is reduced by 2dB for aggregated allocation bandwidth > 10MHz NOTE 3: Outer 2 MPR is reduced by 4.5dB for aggregated allocation bandwidth > 10MHz NOTE 4: UE indicating TxD supported |  |  |  |  |  |  |  |

<Unnecessary part is omitted>

### 6.2D.1	UE maximum output power for UL MIMO

<Unnecessary part is omitted>

If the UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for codebook based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211], the requirements in clause 6.2 apply for at least one antenna connector for the power class as indicated by the ue-PowerClass field in capability signalling with the following exception: for UEs indicating Tx diversity capability, the requirements in clause 6.2G for the power class indicated by the ue-PowerClass.

A UE with 2Tx indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the requirement in clause 6.2 for at least one antenna connector when scheduled for single antenna-port transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission with precoding matrix W=1 [6.3.1.5 TS 38.211].

### 6.2D.2	UE maximum output power reduction for UL MIMO

For UE with two or four transmit antenna connectors in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2D.1-1 is specified in Table 6.2.2-1for PC3, Table 6.2D.2-1 for 2Tx PC2 when the UE does not indicate ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for the band and Table 6.2.2-2 for 2Tx PC2 when the UE indicates ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for the band, Table 6.2D.2-2 and Table 6.2D.2-3 for PC1.5 with 2Tx, Table 6.2D.2-4, 6.2D.2-5 for PC1.5 with 4 Tx respectively. For UE power class 1.5 with 2Tx, the allowed maximum power reduction (MPR) defined in Table 6.2D.2-3 is in accordance with the indicated modifiedMPR-Behavior specified in Table L.1-1 for channel bandwidths ≤ 100 MHz. The requirements shall be met with UL MIMO configurations defined in Table 6.2D.1-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors.

For UE support uplink full power transmission (ULFPTx) for UL MIMO except the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16, the allowed MPR for the maximum output power in Table 6.2D.1-1 is specified in Table 6.2.2-1 for PC3, Table 6.2D.2-1 when TxD is indicated and Table 6.2.2-2  when TxD is not indicated for PC2 , Table 6.2D.2-2 and Table 6.2D.2-3 for PC1.5 with 2Tx, Table 6.2D.2-4, 6.2D.2-5 for PC1.5 with 4 Tx respectively, and the requirements shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode. A UE with 2Tx indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the maximum output power requirement with MPR according to clause 6.2.2. When a UE that indicates PC1.5 for a given band is limited to PC2 by the rules in clause 6.2.1, the MPR requirements in Table 6.2.2-2 apply. For UE support uplink full power transmission (ULFPTx) for UL MIMO, the maximum output power is defined as the sum of the maximum output power from all UE antenna connectors.

The same MPR requirements shall be applicable to UE with 1-layer UL MIMO transmission (either with or without ULPFTx) as with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of.

For the UE maximum output power modified by MPR, the power limits specified in clause 6.2D.4 apply.

If UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook based transmission, the corresponding requirements in clause 6.2D.1 apply for the power class as indicated by the ue-PowerClass field in capability signaling. A UE indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the requirement in clause 6.2 with MPR according to clause 6.2.2 for at least one antenna connector when scheduled for single antenna-port transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission on a single antenna port with precoding matrix W=1 [6.3.1.5 TS 38.211].

Table 6.2D.2-1: Maximum power reduction (MPR) for power class 2 with 2Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 1 | 01 |
|  | QPSK | ≤ 3.5 | ≤ 2 | 0.52 |
|  | 16 QAM | ≤ 3.5 | ≤ 2.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 3.5 | ≤ 3 |  |
|  | 256 QAM | ≤ 5.5 |  |  |
| CP-OFDM | QPSK | ≤ 4.0 | ≤ 3.5 | ≤ 2 |
|  | 16 QAM | ≤ 4.0 | ≤ 3.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |  |
|  | 256 QAM | ≤ 8.0 |  |  |
| NOTE 1: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostPi2BPSK-r18 is set to 1, the reference power is increased by ΔPPowerBoost. NOTE 2: For a UE indicating support for UE capability powerBoosting-pi2BPSK-QPSK-r18 or powerBoosting-pi2BPSK-QPSK-Modified-r18 and if the IE powerBoostQPSK-r18 is set to 1, the reference power is increased by ΔPPowerBoost. |  |  |  |  |

Table 6.2D.2-2: Maximum power reduction (MPR) for power class 1.5 with 2Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ [2] | ≤ 0.5 |
|  | QPSK | ≤ 6.5 | ≤ [2.5] | ≤ 0.5 |
|  | 16 QAM | ≤ 6.5 | ≤ [3.5] | ≤ 1.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [4] | ≤ 3.5 |
|  | 256 QAM | ≤ 6.5 | ≤ 6.5 | ≤ [6.5] |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ [4.5] | ≤ 2 |
|  | 16 QAM | ≤ 6.5 | ≤ [4.5] | ≤ 2.5 |
|  | 64 QAM | ≤ 6.5 | ≤ [5] | ≤ 4.5 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ [8.5] |

Table 6.2D.2-3: Maximum power reduction (MPR) for power class 1.5 with 2Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ 1.5 | ≤ 0 |
|  | QPSK | ≤ 6.5 | ≤ 2 | ≤ 0 |
|  | 16 QAM | ≤ 6.5 | ≤ 3 | ≤ 1 |
|  | 64 QAM | ≤ 6.5 | ≤ 3.5 | ≤ 3 |
|  | 256 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 4 | ≤ 1.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4 | ≤ 2 |
|  | 64 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Table 6.2D.2-4: Maximum power reduction (MPR) for power class 1.5 with 4 Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 8.0 | ≤ 3.0 | ≤ 2.0 |
|  | QPSK | ≤ 8.5 | ≤ 3.5 | ≤ 2.0 |
|  | 16 QAM | ≤ 8.5 | ≤ 4.0 | ≤ 2.5 |
|  | 64 QAM | ≤ 8.5 | ≤ 4.7 | ≤ 4.5 |
|  | 256 QAM | ≤ 9.5 | ≤ 7.0 | ≤ 7.0 |
| CP-OFDM | QPSK | ≤ 9.5 | ≤ 5.0 | ≤ 3.5 |
|  | 16 QAM | ≤ 9.5 | ≤ 5.0 | ≤ 4.0 |
|  | 64 QAM | ≤ 9.5 | ≤ 7.0 | ≤ 7.0 |
|  | 256 QAM | ≤ 9.5 | ≤ 9.5 | ≤ 9.5 |
| NOTE 1: This table is targeted to vehicular UE or other industrial device form factor with 10dB antenna isolation. |  |  |  |  |

Table 6.2D.2-5: Maximum power reduction (MPR) for power class 1.5 with 4 Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 7.5 | ≤ 2.0 | ≤ 0.5 |
|  | QPSK | ≤ 8.0 | ≤ 2.5 | ≤ 0.5 |
|  | 16 QAM | ≤ 8.0 | ≤ 3.5 | ≤ 1.5 |
|  | 64 QAM | ≤ 8.0 | ≤ 4.0 | ≤ 3.5 |
|  | 256 QAM | ≤ 8.0 | ≤ 6.5 | ≤ 6.5 |
| CP-OFDM | QPSK | ≤ 8.0 | ≤ 4.5 | ≤ 2.0 |
|  | 16 QAM | ≤ 8.0 | ≤ 4.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 8.0 | ≤ 5.0 | ≤ 5.0 |
|  | 256 QAM | ≤ 8.5 | ≤ 8.5 | ≤ 8.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Inner, outer and edge allocations are as defined in section 6.2.2 except for 	PC1.5 edge allocations which is for LCRB ≤ 4 RBs instead of LCRB ≤ 2 RBs for other power classes.

### 6.2D.3	UE additional maximum output power reduction for UL MIMO

For UE with two or four transmit antenna connectors in closed-loop spatial multiplexing scheme, the A-MPR values specified in clause 6.2.3 shall apply to the maximum output power specified in Table 6.2D.1-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE support uplink full power transmission (ULFPTx) for UL MIMO, the A-MPR values specified in clause 6.2.3 shall apply to the maximum output power specified in Table 6.2D.1-1. The requirements shall be met with the PUSCH configurations specified in Table 6.2D.1-3, based upon UE’s support of uplink full power transmission mode. For UE support uplink full power transmission (ULFPTx) for UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connector.

For the UE maximum output power modified by A-MPR, the power limits specified in clause 6.2D.4 apply.

If the UE is scheduled for single antenna-port PUSCH transmission by DCI format 0_0 or by DCI format 0_1 for single antenna port codebook-based transmission, the corresponding requirements in clause 6.2D.1 apply for the power class as indicated by the ue-PowerClass field in capability signaling. A UE with 2Tx indicating the feature ul-FullPwrMode-r16 or ul-FullPwrMode2-TPMIGroup-r16 for a band shall meet the requirement in clause 6.2 for at least one connector with A-MPR according to clause 6.2.3 when scheduled for single antenna-port transmission by DCI format 0_0 or by DCI format 0_1 for codebook-based transmission on a single antenna port with precoding matrix W=1 [6.3.1.5 TS 38.211].

<Unnecessary part is omitted>

#### 6.2E.2.2	MPR for Power class 2 and Power class 3 V2X UE

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for NR V2X physical channels PSCCH and PSSCH shall be as specified in Table 6.2E.2.2-1 for Power class 3 NR V2X UE and Table 6.2E.2.2-2 for power class 2 NR V2X UE.

Table 6.2E.2.2-1: Maximum Power Reduction (MPR) for power class 3 NR V2X

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK | ≤ 4.5 | ≤ 2.5 |
|  | 16QAM | ≤ 4.5 | ≤ 2.5 |
|  | 64 QAM | ≤ 4.5 |  |
|  | 256 QAM | ≤ 7.0 |  |

Table 6.2E.2.2-2: Maximum Power Reduction (MPR) for power class 2 NR V2X

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
|  | QPSK | ≤ 5.5 | ≤ 2.5 |
| CP-OFDM | 16QAM |  |  |
|  | 64 QAM | ≤ 6 | ≤ 4.5 |
|  | 256 QAM | ≤ 7.0 |  |

For NR V2X UE supporting SL MIMO or Tx diversity, the allowed MPR for the maximum output power for NR V2X physical channels PSCCH and PSSCH are specified in Table 6.2E.2.2-3 for power class 2 UE.

Table 6.2E.2.2-3: Maximum Power Reduction (MPR) for power class 2 NR V2X with 2Tx

| Modulation |  | Channel bandwidth/MPR (dB) |  |
| --- | --- | --- | --- |
|  |  | Outer RB allocations | Inner RB allocations |
| CP-OFDM | QPSK | ≤ 6.0 | ≤ 3.0 |
|  | 16QAM |  |  |
|  | 64 QAM | ≤ 7.0 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.0 |  |

<Unnecessary part is omitted>

### 6.2F.2	UE maximum output power reduction

<Unnecessary part is omitted>

Table 6.2F.2-4 Maximum power reduction (MPR)

for shared spectrum access UE power class 3 with 2Tx

| Pre-coding | Modulation | RB Allocation |  |  |
| --- | --- | --- | --- | --- |
|  |  | Full2 (dB) | Partial3 (dB) | Exception for 100MHz Full5 (dB) |
| DFT-s-ODFM | Pi/2 BPSK4 | ≤ [2.0] | ≤ [3.0] |  |
|  | QPSK | ≤ [2.0] | ≤ [3.0] |  |
|  | 16 QAM | ≤ [2.5] | ≤ [3.0] |  |
|  | 64 QAM | ≤ [2.5] | ≤ [3.5] |  |
|  | 256 QAM | ≤ [4.5] | ≤ [4.5] |  |
| CP-OFDM | QPSK | ≤ [3.5] | ≤ [4.0] | ≤ [5.0] |
|  | 16 QAM | ≤ [3.5] | ≤ [4.0] | ≤ [5.0] |
|  | 64 QAM | ≤ [4.5] | ≤ [4.5] |  |
|  | 256 QAM | ≤ [6.5] | ≤ [6.5] |  |
| NOTE 1: The MPR shall apply to all SCS in all active 20 MHz sub-bands contiguously allocated in the channel. NOTE 2: The MPR for Full RB allocation applies to all RB’s in all transmitted 20 MHz or larger channels that are fully allocated or all RB’s in all transmitted sub-bands for wideband operation that are fully allocated excluding the wideband configurations of Table 6.2F.2-2. NOTE 3: The MPR for Partial RB allocation applies to interlaced allocations with uplink resource allocation type 2 as specified in TS 38.214 [10] or transmitted sub-bands for wideband operation are transmitted according to the wideband configurations of Table 6.2F.2-2. NOTE 4: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. NOTE 5: Exception for 100MHz Full RB allocation MPR applies when all RB’s in all sub-bands for 100MHz wideband operation are fully allocated and sub-bands are transmitted according to the wideband configurations of Table 6.2F.2-2. |  |  |  |  |

<Unnecessary part is omitted>

#### 6.2F.3.10	A-MPR for NS_60

When "NS_60" is indicated in the cell, the A-MPR is specified in Table 6.2F.3.10-1 for power class 5. The Table 6.2F.3.10-2 is applicable for power class 3 with single Tx and Table 6.2F.3.10-2 is applicable for power class 3 with 2Tx.

<Unnecessary part is omitted>

Table 6.2F.3.10-2: A-MPR for NS_60 power class 3 with 2Tx

| Pre-coding | Modulation | Channel bandwidth (Sub-band allocation) / RB Allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 20 MHz |  | 40 MHz |  | 60 MHz |  | 80 MHz |  | 100 MHz |  |
|  |  | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) | Full (dB) | Partial (dB) |
| DFT-s-OFDM | PI/2 BPSK2 | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.5 |
|  | QPSK | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 3.0 | ≤ 6.0 | ≤ 3.0 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 4.5 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.5 |
|  | 16 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.5 |
|  | 64 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 4.5 | ≤ 6.0 | ≤ 4.5 | ≤ 5.5 |
|  | 256 QAM | ≤ 9.5 | ≤ 11.5 | ≤ 6.0 | ≤ 9.0 | ≤ 5.0 | ≤ 7.0 | ≤ 5.5 | ≤ 6.5 | ≤ 5.5 | ≤ 6.0 |
| NOTE 1: Full allocation A-MPR applies when all RB’s in a 20 MHz channel or all RB’s in all sub-bands for wideband operation are fully allocated and all sub-bands are transmitted. Partial allocation A-MPR applies when one or more RB’s in one or more sub-bands are not allocated but when all sub-bands within the channel are transmitted. When not all sub-bands within the channel are transmitted, the A-MPR associated with the channel bandwidth according to the bandwidth of the contiguously transmitted sub-bands and according to the allocation type applies NOTE 2: Applicable to Pi/2-BPSK modulation when IE powerBoostPi2BPSK is set to 0. |  |  |  |  |  |  |  |  |  |  |  |

<Unnecessary part is omitted>

### 6.2G.2	UE maximum output power reduction for Tx Diversity

For UE supporting Tx diversity, the allowed MPR for the maximum output power is specified in Table 6.2.2-1 for UE power class 3, in Table 6.2D.2-1 for UE power class 2, in Table 6.2D.2-2 and Table 6.2D.2-3 for UE power class 1.5 with 2Tx, in  Table 6.2D.2-4 and 6.2D.2-5 for UE power class 1.5 with 4 Tx. For UE power class 1.5 with 2Tx, the allowed maximum power reduction (MPR) defined in Table 6.2D.2-3 is in accordance with the indicated modifiedMPR-Behavior specified in Table L.1-1 for channel bandwidths ≤ 100 MHz. The maximum output power is defined as the sum of the maximum output power at each UE antenna connector. If a UE that supports PC1.5 has to apply the requirements of PC2 according to the rules in clause 6.2.1, the MPR requirements in Table 6.2.2-2 apply

<Unnecessary part is omitted>

#### 7.3A.2.1	Reference sensitivity power level for Intra-band contiguous CA

<Unnecessary part is omitted>

Table 7.3A.2.1-3: Power class 2 intra-band contiguous CA reference sensitivity with two uplink carriers.

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | UL PCC allocation | SCC ΔRIBC1 (dB) | SCC ΔRIBC2 (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- |
| CA_n71B | 15/15 | 30 MHz + 5 MHz | 20 (RBstart = 0) | 5.6 3 | 7.73 | FDD |
| NOTE 1: Applicable to UE supporting PC2 with single Tx. NOTE 2: Applicable to UE supporting PC2 with 2Tx. NOTE 3: Applicable only to BCS 4 and 5 and for UEs supporting the optional symmetrical UL/DL channel bandwidths. |  |  |  |  |  |  |

#### 7.3A.2.2	Reference sensitivity power level for Intra-band non-contiguous CA

<Unnecessary part is omitted>

Table 7.3A.2.2-1a: Power class 2 intra-band non-contiguous CA reference sensitivity with one uplink carrier.

| CA configuration | SCS (kHz) | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | SCC ΔRIBNC1 (dB) | SCC ΔRIBNC2 (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_n71(2A) | 15/15 | 15MHz + 10MHz | Wgap = 10.0 | 5 (RBstart = 2) | 24.8 | 29.3 | FDD |
|  |  | 25MHz + 5MHz1 | Wgap = 5.0 | 20 (RBstart = 8) | 27.23 | 31.83 |  |
| CA_n25(2A) 9 | 15/15 | 5MHz + 5MHz | Wgap = 55.0 | 105 | 7.3 | 10.0 | FDD |
|  |  |  | Wgap = 30.0 | 25 | 0.0 | 0.0 |  |
| CA_n25(2A) 10 CA_n25(3A) | 15/15 | 40MHz + 5MHz | Wgap = 20.0 | 40 (RBstart = 176) | [27.6] 8 | 32.68 | FDD |
| CA_n66(2A) CA_n66(3A) | N/A | NOTE 4 | NOTE 5 | NOTE 6, NOTE 7 | 0.0 | 0.0 | FDD |
| NOTE 1: Applicable to UE supporting PC2 with single Tx. NOTE 2: Applicable to UE supporting PC2 with 2Tx. NOTE 3: Applicable only to BCS 4 and 5 and for UEs supporting the optional symmetrical UL/DL channel bandwidths. NOTE 4: All combinations of channel bandwidths defined in Table 5.5A.2-1. NOTE 5: All applicable sub-block gap sizes. NOTE 6: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.3.2-1. NOTE 7: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 9: Bandwidth Combination Set 0. NOTE 10: Bandwidth Combination Set 1 |  |  |  |  |  |  |  |

<Unnecessary part is omitted>

### 7.3A.4	Reference sensitivity exceptions due to harmonic interference for CA

<Unnecessary part is omitted>

Table 7.3A.4-2a: Reference sensitivity exceptions and uplink/downlink configurations due to UL harmonic from a PC2 aggressor NR UL band for NR DL CA FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) | (dB) |  |  |
| n3 | n78 | 5 | 15 | 12 | 10 | 27.1 | 32.3 | NOTE 2 | UL2/DL1 direct-hit |
| n3 | n78 | 10 | 15 | 12 | 100 | 16.6 | 20.8 | NOTE 2 | UL2/DL1 direct-hit |
| n8 | n41 | 5 | 15 | 8 | 10 | 15.9 | 20 | NOTE 2 | UL3/DL1 direct-hit |
| n8 | n41 | 5 | 15 | 8 | 100 | 6.5 | 9.5 | NOTE 2 | UL3/DL1 direct-hit |
| n8 | n79 | 5 | 15 | 5 | 10 | 14.9 | 19 | NOTE 2 | UL5/DL1 direct-hit |
| n8 | n79 | 5 | 15 | 5 | 100 | 6.2 | 8.4 | NOTE 2 | UL5/DL1 direct-hit |
| n25 | n77 | 5 | 15 | 12 | 10 | 26.9 | 31.9 | NOTE 2 | UL2/DL1 direct-hit |
| n25 | n77 | 10 | 15 | 12 | 100 | 16.8 | 20.8 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 5 | 15 | 12 | 10 | 26.9 | 31.9 | NOTE 2 | UL2/DL1 direct-hit |
| n66 | n77 | 20 | 15 | 12 | 100 | 16.7 | 20.8 | NOTE 2 | UL2/DL1 direct-hit |
| n71 | n2510,11 | 5 | 15 | 8 | 5 | 9.4 | 12.9 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n2510,11 | 5 | 15 | 8 | 40 | 0.7 | 1.1 | NOTE 3 | UL3/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 6 | 10 | 13.6 | 17.8 | NOTE 4 | UL4/DL1 direct-hit |
| n71 | n41 | 5 | 15 | 6 | 100 | 5.1 | 6.1 | NOTE 4 | UL4/DL1 direct-hit |
| n71 | n77 | 5 | 15 | 5 | 10 | 13.2 | 17.4 | NOTE 5 | UL5/DL1 direct-hit |
| NOTE 1: Void NOTE 2: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 3: The requirements should be verified for UL NR ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with the carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the low band. NOTE 4: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 5: The requirements should be verified for UL NR-ARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 6: The near-miss requirements are only applicable when direct-hit requirements do not apply. These requirements should be verified for downlink channel bandwidths no larger than 10 MHz and with a carrier frequency at ±10+BWChannelHB/2 MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 7: Applicable to UEs supporting PC2 with single Tx. NOTE 8: Applicable to UEs supporting PC2 with 2Tx. NOTE 9: These requirements apply when the upper edge frequency of the 5 MHz uplink channel in Band n71 is located at 668 MHz and the downlink channel in Band n25 is located with its upper edge at 1995 MHz. NOTE 10: The direct-hit requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd / 3rd / 4th / 5th transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band. The requirements should be verified using RBstart = floor((NRB-LCRB)/2), where floor(x) is the greatest integer less than or equal to x, and where the UL parameters NRB and LCRB are respectively, the transmission bandwidth configuration and the number of RB’s for the specified UL band channel bandwidth and the UL band subcarrier spacing. NOTE 11: Void. |  |  |  |  |  |  |  |  |  |

<The end of the changes>