3GPP TSG-RAN WG4 Meeting #113	   R4-2417531

Orlando, US, 18 - 22 November, 2024

Title:	Reply LS on carrierBandwidth configuration for less-than-5MHz carriers

Response to:	R2-2409409

Release:	Rel-18

Work Item:	NR_FR1_lessthan_5MHz_BW

Source:	RAN4

To:	RAN2

Cc: 	RAN1

Contact Person:

Name:	Man Hung Ng

Email:		man_hung.ng AT nokia DOT com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

1. Overall Description:

RAN4 would like to thank RAN2 for their LS R2-2409409.

RAN4 would like to reply RAN2 that from RAN4 perspective it is feasible to configure carrierBandwidth to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB) case respectively in SIB1 and in RRC dedicated signaling. This is because RAN4 has specified that for band n100:

- PCell or SCell with 12 PRB transmission bandwidth configuration within 3 MHz channel can be configured only with SS Block frequency positions same as defined in TS 38.101-1 Table 5.4.3.1-3.

- PCell or SCell with 20 PRB transmission bandwidth configuration within 5 MHz channel can be configured only with SS Block frequency positions same as defined in TS 38.101-1 Table 5.4.3.1-3.

Therefore, UE can determine 12 or 20 PRB transmission bandwidth configuration should be used instead of15 or 25 PRB transmission bandwidth configuration, respectively, even though carrierBandwidth is configured to 15 PRB (for the 12 PRB case) and 25 PRB (for the 20 PRB) case respectively in SIB1 and in RRC dedicated signaling.

2. Actions:

To RAN2:

ACTION: RAN4 respectfully asks RAN4 to take above into account.

3. Date of Next RAN4 Meetings:

RAN2#114	17th – 21st February 2025				Athens, Greece