3GPP TSG-RAN WG4 Meeting # 113	R4-2417536

Orlando, US, 18th – 22nd November 2024

Agenda Item:	7.2.4.1

Source: 	Nokia

Title: 	Urban macro coexistence simulation results for 14800 to 15350 MHz frequency range with 1 UE per slot

Document for:	Information

# 1.	Introduction

The SI on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz was approved at TSG RAN#103 [1]. One of the objectives of this SI is to study the IMT parameters relevant for sharing and compatibility for 14800 to 15350 MHz frequency range.

This study item aims as answering requests from ITU-R WP5D regarding NR in these frequencies for IMT [2]. Two set of parameters requested by ITU-R WP5D are the ACLR and ACS of BS and UE for 14800 to 15350 MHz frequency range. Currently, different sets of ACLR and ACS are specified in RAN4 specifications [3, 4, 5] for NR BS and UE in FR1 and FR2 based on coexistence studies as recorded in TR 38.803 [6]. Therefore, coexistence study will need to be carried out to provide answers to ITU-R WP5D on these parameters with sound technical justifications from coexistence perspective.

The text proposal on the revised simulation assumptions for the coexistence study was agreed in RAN4#112 [7]. This contribution provides the urban macro coexistence simulation results for 14800 to 15350 MHz frequency range according to the revised assumptions.

# 2.	Discussion

2.1	Downlink Results

The urban macro simulation results of the downlink SINR of the victim UE at 15 GHz carrier frequency with 450 m inter-site distance, 20% indoor UE ratio, 200 MHz channel bandwidth (previous coexistence studies have shown simulation results are similar with different channel bandwidths) and 1 UE per slot are provided in Figure 1 below using 16x4x24 BS and different UE antenna configurations. Here the downlink ACIR is obtained using 45 dB BS ACLR and 33 dB UE ACS (currently defined for BS and UE in FR1). It can be seen from the figures that in all simulated cases enough downlink SINR (up to 70 dB and 78 dB for FR1 and FR2 like UE, respectively) can be provided using 46 dBm BS conducted output power (as agreed in [7]) together with AAS BS beamforming.

(a) 64x24 BS, FR1 like UE					(b) 64x24 BS, FR2 like UE

Figure 1: Urban Macro DL SINR of victim UE

The urban macro simulation results of the downlink throughput losses of the victim UE with different ACIR offsets (compared to the ACIR using 45 dB BS ACLR and 33 dB UE ACS) using un-coordinated operation with FR1 like UE and coordinated operation with FR2 like UE (the simulation results in [8] show that these are more stringent cases for throughput losses) are provided, respectively, in Tables 1 and 2 below. It can be seen from the tables that the average and 5%-tile downlink throughput losses of the victim UE in all simulated cases are limited to around 5% with downlink ACIR offset of -12 dB. The larger downlink ACIR offset compared to those at 7 GHz and 10 GHz carrier frequencies in TR 38.921 [9] can be explained by the fact that the larger BS antenna configurations at 15 GHz can achieve narrower beamwidths and thus lower interference from the interfering BS to the victim UE.

| ACIR offset X [dB] | 0 | -3 | -6 | -9 | -12 | -15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (%) | 0.17 | 0.28 | 0.45 | 0.70 | 1.06 | 1.55 |
| 5%-tile throughput loss (%) | 8.8485e-04 | 0.18 | 0.35 | 1.00 | 1.38 | 3.53 |

Table 1: Urban Macro DL throughput loss of victim FR1 like UE

| ACIR offset X [dB] | 0 | -3 | -6 | -9 | -12 | -15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (%) | 8.7157e-02 | 0.15 | 0.25 | 0.39 | 0.56 | 0.78 |
| 5%-tile throughput loss (%) | 0.70 | 0.70 | 0.70 | 2.97 | 3.83 | 5.26 |

Table 2: Urban Macro DL throughput loss of victim FR2 like UE

Comparing the results between Tables 1 and 2, the average throughput losses for FR1 like UE are higher than those for FR2 like UE with the use of beamforming at the FR2 like UE, while the 5%-tile throughput losses for FR1 like UE are lower than those for FR2 like UE. This can be explained by the fact that the 5%-tile victim UE at the cell edge have lower SNR and thus are more affected by the ACI from the interfering BS, and the FR2 like UE beam is directed towards the interfering BS and thus increases the ACI from the interfering BS transmitting 3 beams.

2.2	Uplink Results

The urban macro simulation results of the victim and interfering UE transmit power at 15 GHz carrier frequency with 450 m inter-site distance, 20% indoor UE ratio, 200 MHz channel bandwidth, 3 UE per slot and 23 dBm UE maximum TX power are provided in Figure 2 below using 16x4x24 BS and different UE antenna configurations. Here the uplink ACIR is obtained using 30 dB UE ACLR and 45 dB BS ACS (currently defined for BS and UE in FR1). It can be seen from the figure that around 20% of the FR1 like UE and 13% of the FR2 like UE are transmitting at maximum power.

(a) 64x24 BS, FR1 like UE					(b) 64x24 BS, FR2 like UE

Figure 2: Urban Macro UE transmit power

The urban macro simulation results of the victim UE UL SINR are provided in Figure 3 below. Here the uplink ACIR is obtained using 30 dB UE ACLR and 45 dB BS ACS (currently defined for BS and UE in FR1). It can be seen from the figure that the target UL SINR of 15 dB is achieved by around 80% of the FR1 like UE and 85% of the FR2 like UE.

(a) 64x24 BS, FR1 like UE					(b) 64x24 BS, FR2 like UE

Figure 3: Urban Macro UE UL SINR

The urban macro simulation results of the victim BS received blocking signal power at the TAB connector (i.e. including the antenna array gain) with un-coordinated and coordinated operations are provided, respectively, in Figure 4 below. The 99.99%-tile received blocking signal power levels are less than -48 dBm and -46 dBm, respectively, in un-coordinated operation with FR1 like UE and coordinated operation with FR2 like UE.

(a) 64x24 BS, FR1 like UE					(b) 64x24 BS, FR2 like UE

Figure 4: Urban Macro received blocking signal power

The urban macro simulation results of the uplink throughput losses of the victim UE with different ACLR offsets (compared to the 30 dB UE ACLR) using un-coordinated operation with FR1 like UE and coordinated operation with FR2 like UE are provided, respectively, in Tables 3 and 4 below. It can be seen from the tables that the average and 5%-tile uplink throughput losses in all simulated cases are limited to around 5% with an uplink ACLR offset of -9 dB.

| ACIR offset X [dB] | 0 | -3 | -6 | -9 | -12 | -15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (a) | 7.6453e-02 | 0.12 | 0.20 | 0.30 | 0.46 | 0.68 |
| 5%-tile throughput loss (a) | 1.14 | 2.21 | 3.52 | 3.55 | 3.61 | 3.73 |

Table 3: Urban Macro UL throughput loss of victim FR1 like UE

| ACIR offset X [dB] | 0 | -3 | -6 | -9 | -12 | -15 |
| --- | --- | --- | --- | --- | --- | --- |
| Average throughput loss (a) | 9.3114e-02 | 0.14 | 0.21 | 0.31 | 0.44 | 0.61 |
| 5%-tile throughput loss (a) | 5.9654e-06 | 1.1720e-05 | 2.14 | 2.31 | 5.28 | 6.53 |

Table 4: Urban Macro UL throughput loss of victim FR2 like UE

Comparing the results between Tables 3 and 4, the average throughput losses for FR1 like UE are similar to those for FR2 like UE with the use of beamforming at the FR2 like UE, while the 5%-tile throughput losses for FR1 like UE are lower than those for FR2 like UE at larger ACIR offsets. This can be explained by the fact that the 5%-tile victim UE at the cell edge have lower SNR and thus are more affected by the ACI from the interfering BS, especially at larger ACIR offsets, and the FR2 like UE beam is directed towards the interfering BS and thus increases the ACI from the interfering BS transmitting 3 beams.

# 3.	Conclusion

This contribution has provided the urban macro coexistence simulation results for 14800 to 15350 MHz frequency range with 1 UE per slot according to the agreed assumptions.

Observations from simulation results:

1) The average and 5%-tile downlink throughput losses of the victim UE in all simulated cases are limited to around 5% with downlink ACIR offset of -12 dB.

2) The average throughput losses for FR1 like UE are higher than those for FR2 like UE with the use of beamforming at the FR2 like UE, while the 5%-tile throughput losses for FR1 like UE are lower than those for FR2 like UE.

3) The average and 5%-tile uplink throughput losses in all simulated cases are limited to around 5% with an uplink ACLR offset of -9 dB.

4) The average throughput losses for FR1 like UE are similar to those for FR2 like UE with the use of beamforming at the FR2 like UE, while the 5%-tile throughput losses for FR1 like UE are lower than those for FR2 like UE at larger ACIR offsets.

5) The 99.99%-tile received blocking signal power levels are less than -48 dBm and -46 dBm, respectively, in un-coordinated operation with FR1 like UE and coordinated operation with FR2 like UE.

# References

[1]	RP-240787, “New SI proposal: Study on IMT parameters for 4400 to 4800 MHz, 7125 to 8400 MHz and 14800 to 15350 MHz”, Ericsson.

[2]	R4-2400333, “Parameters of terrestrial component of IMT for sharing and compatibility studies in the frequency bands 4 400-4 800 MHz, 7 125-8 400 MHz and 14.8-15.35 GHz”, ITU-R WP5D.

[3]	3GPP TS 38.101-1 v18.5.0, “NR; User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone”.

[4]	3GPP TS 38.101-2 v18.5.0, “NR; User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”.

[5]	3GPP TS 38.104 v18.5.0, “NR; Base Station (BS) radio transmission and reception”.

[6]	3GPP TR 38.803 v14.2.0, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”.

[7]	R4-2417105, “TP to TR 38.922: Revisions of system level simulation assumptions for study on IMT parameters for 14800 to 15350 MHz frequency range”, Nokia.

[8]	R4-2412589, “Urban macro coexistence simulation results for 14800 to 15350 MHz frequency range”, Nokia.

[9]	3GPP TR 38.921 v18.0.1, “NR; Study on International Mobile Telecommunications (IMT) parameters for 6.425-7.025GHz, 7.025-7.125GHz and 10.0-10.5 GHz”.