3GPP TSG-RAN WG4 Meeting # 113	R4-2417543

Hefei, China, 14th – 18th October 2024

Agenda Item:	7.23.3

Source: 	Nokia

Title: 	BS RF requirements for low-power wake-up signal for NR

Document for:	Approval

# 1.	Introduction

The revised WI for low-power wake-up signal and receiver for NR was approved at TSG RAN#105 [1]. One of the objectives of this WI is to:

- Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline

This topic was discussed at TSG RAN4#112bis, and the Way Forward proposals were agreed [2].

This contribution provides proposals to progress the issues that were FFS in the agreed WF.

# 2.	Discussion

2.1	Concept of LP-WUS power boosting

It is stated in the agreed WF as follows:

- Proposals - Proposal 1: Since EPRE ratio has the merit that power boosting declaration for LP-WUS does not vary with different CBWs, RAN4 to introduce and specify power boosting represented by EPRE ratio between LP-WUS and NR REs excluding LP-WUS. (CATT, Huawei, [vivo]) - The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs (the NR carrier excluding the LP-WUS REs). - Proposal 2: Adopt following definition rather than introduce EPRE ratio. (Ericsson, ZTE, Nokia) - The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs). - WF - Compare the Pros and Cons of the two definitions - At least the following aspects should be considered - Convenience for manufacturer’s declaration - Compatibility with other issues relevant to power boosting, e.g. the possible upper bound, applicable lowest CBW, etc. - Cases of power boosting with different CBW and LP-WUS / NR RE allocation should be provided - Comparison of power boosting with NB-IoT with the same definition should be provided - Test aspects of power boosting should be considered - Other aspects are not precluded - Try to define the power boosting concept next RAN4 meeting with one definition as baseline but allow flexibility to manufacturer declaration of boosting level for the supported CBW

The Pros and Cons of Proposal 1 compared to Proposal 2 are provided in Table 1 below:

Table 1: Pros and Cons of the two definitions

| Proposal 1 compared to Proposal 2 | Pros | Cons |
| --- | --- | --- |
| Convenience for manufacturer’s declaration | The same power boosting level may be declared for all supported CBW. | Different power boosting levels may not be declared for different supported CBW. |
| Compatibility with other issues relevant to power boosting, e.g. the possible upper bound, applicable lowest CBW, etc. | The same upper bound may be applied to all supported CBW. | Different upper bounds may not be declared for different supported CBW. |
| Cases of power boosting with different CBW and LP-WUS / NR RE allocation should be provided | The power boosting level will be the same irrespective of the different CBW and LP-WUS / NR RE allocation. | Definition may not be used for lower CBW including 5MHz when all available RBs are allocated for LP-WUS and blanked guard RBs resulting in no NR REs allocation. |
| Comparison of power boosting with NB-IoT with the same definition should be provided | The same power boosting level may be declared for all LP-WUS allocation within all supported CBW. | Different power boosting levels may not be declared for different LP-WUS allocation within different supported CBW as used with NB-IoT. |
| Test aspects of power boosting should be considered | New test configuration and model can be defined similar to NB-IoT power boosting. | There must be NR REs allocation in the new test configuration and model, otherwise the definition cannot be used as the average power of NR REs is zero. |

To summarize, Proposal 1 have some advantages compared to Proposal 2, but it has one major issue that it cannot be used for lower CBW including 5MHz when there is no NR REs allocation. Therefore, it is proposed to modify Proposal 1 to handle the case when there is no NR REs allocation.

2.2	On minimum power boosting level for LP-WUS

It is stated in the agreed WF as follows:

- Proposals - Need to have it. - If power boosting will be defined as EPRE ratio. - Proposal 1: Use 3dB as minimum requirement. (CATT, Huawei, [vivo]) - If power boosting will not be defined as EPRE ratio. - Proposal 2: Use 3dB as minimum requirement. (Ericsson, [vivo]) - Proposal 3: The core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0dB. (Nokia) - Proposal 4: No matter how to define power boosting, the requirement should consider the assumption in RAN1 of 3dB or 6dB means the EPRE ratio. The requirement can be different for different CBW. (vivo) - Proposal 5: No need to have it. (CMCC, ZTE, Nokia) - WF - FFS

Currently, the RE power control dynamic range requirement for NR BS is specified in TS 38.104 [3] as follows:

### 6.3.2 RE power control dynamic range #### 6.3.2.1 General The RE power control dynamic range is the difference between the power of an RE and the average RE power for a BS at maximum output power (Pmax,c,AC or Pmax,c,TABC) for a specified reference condition. For BS type 1-C this requirement shall apply at the antenna connector supporting transmission in the operating band. For BS type 1-H this requirement shall apply at each TAB connector supporting transmission in the operating band. #### 6.3.2.2 Minimum requirement for BS type 1-C and BS type 1-H RE power control dynamic range: Table 6.3.2.2-1: RE power control dynamic range Modulation scheme used; RE power control dynamic range (dB). on the RE; (down); (up). QPSK (PDCCH); -6; +4. QPSK (PDSCH); -6; +3. 16QAM (PDSCH); -3; +3. 64QAM (PDSCH); 0; 0. 256QAM (PDSCH); 0; 0. 1024QAM (PDSCH); 0; 0. NOTE: The output power per carrier shall always be less or equal to the maximum output power of the base station..

In table 6.3.2.2-1 above, the RE power control dynamic range requirement is specified depending on the modulation schemes and physical channels. The RE power up and down range for each modulation scheme was derived by considering the relation between RE power boosting/de-boosting and other RF requirements like UEM, ACLR and EVM, as explained in TR 38.817-02 [4].

Since LP-WUS/LP-SS is a new physical channel using On-Off Keying (OOK) modulation, a new core requirement shall be added for the dynamic range for LP-WUS/LP-SS. In table 6.3.2.2-1 above, the RE power up and down range for modulation order higher than 16QAM is 0 dB considering the impact on PA with the higher PAPR of higher order modulation. It is shown in Figure 1 below that the PAPR of LP-WUS/LP-SS is more like 64QAM than 16QAM. Therefore, the core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB.

(a) LP-WUS (bit sequence 1101) using ZC sequence with M=1

(b) LP-WUS (bit sequence 1101) using ZC sequence with M=2

(c) LP-WUS (bit sequence 1101) using ZC sequence with M=4

(d) LP-WUS (bit sequence 1101) using M sequence with M=1

(e) LP-WUS (bit sequence 1101) using M sequence with M=2

(f) LP-WUS (bit sequence 1101) using M sequence with M=4

(g) Typical QAM signals

Figure 1: PAPR comparison of LP-WUS and typical QAM signals

On the other hand, it was agreed at RAN1#117 to support 11 PRBs for LP-WUS and LP-SS with SCS 30kHz [5]. The power degradation of RBs other than LP-WUS signal for this setting with different LP-WUS power boosting levels are provided in Table 2 below, which shows that the power reduction for other RBs because of LP-WUS power boosting varies between 0.1 dB to 1.1 dB depending on the power boosting level and channel bandwidth.

Table 2: Impact of LP-WUS power boosting

| LP-WUS power boosting (dB) | SCS (KHz) | Number of RBs | LP-WUS RBs | Rated carrier output power (dBm) | Channel bandwidth (MHz) | Power reduction for other RBs (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | 30 | 162 | 11 | 46 | 60 | -0.32687277 |
| 3 | 30 | 189 | 11 | 46 | 70 | -0.27568025 |
| 3 | 30 | 217 | 11 | 46 | 80 | -0.237165576 |
| 3 | 30 | 245 | 11 | 46 | 90 | -0.208095127 |
| 3 | 30 | 273 | 11 | 46 | 100 | -0.185374077 |
| 4 | 30 | 162 | 11 | 46 | 60 | -0.506770566 |
| 4 | 30 | 189 | 11 | 46 | 70 | -0.425992238 |
| 4 | 30 | 217 | 11 | 46 | 80 | -0.365577629 |
| 4 | 30 | 245 | 11 | 46 | 90 | -0.32017752 |
| 4 | 30 | 273 | 11 | 46 | 100 | -0.284811572 |
| 6 | 30 | 162 | 11 | 46 | 60 | -1.063293018 |
| 6 | 30 | 189 | 11 | 46 | 70 | -0.884288268 |
| 6 | 30 | 217 | 11 | 46 | 80 | -0.752987437 |
| 6 | 30 | 245 | 11 | 46 | 90 | -0.655701573 |
| 6 | 30 | 273 | 11 | 46 | 100 | -0.580711194 |

Moreover, support of higher levels of LP-WUS power boosting will increase BS implementation complexity, like NB-IoT power boosting in TS 36.141 [6] and TS 38.141-1 [7], BS manufacturers should be allowed to declare the supported LP-WUS power boosting level in the manufacturer declarations for BS supporting LP-WUS operation to differentiate their product performance.

2.3	Whether a cap for LP-WUS power boosting should be considered

It is stated in the agreed WF as follows:

- Proposals - Yes, because impact on NR signal available transmission power should be considered. - Proposal 1: 3dB. (Ericsson) - Proposal 1: 6dB. (Huawei) - Proposal 2: Not to introduce maximum value for LP-WUS power boosting. (ZTE, Nokia, CATT, [vivo]) - WF - [Leave the maximum power dynamic range to manufacturer’s declaration]

As discussed in section 2.2 above, the core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB, and BS manufacturers should be allowed to declare the supported LP-WUS power boosting level, as such there is no need to cap the LP-WUS power boosting if BS manufacturers declare to support higher levels of power boosting with LP-WUS operation.

2.4	Whether to preclude small CBW for consideration of LP-WUS power boosting

It is stated in the agreed WF as follows:

- Proposals - Preclusion on small CBW respective to different power boosting level and/or power degradation on NR signal can be considered for LP-WUS power boosting. (Ericsson) - Proposal 1: Also introduce the maximum allowed power degradation as one of the core requirements for LP-WUS power boosting. (CATT) - Proposal 2: Operator inputs on the tolerable power degradation can be considered for the determination on the CBW to be precluded. (Huawei) - Proposal 3: For 30kHz SCS LP-WUS operation, power boosting within 5MHz and 10MHz CBW can be precluded. For 15kHz SCS LP-WUS operation, no CBW should be precluded. (vivo) - No need to preclude any CBW. (CMCC, Nokia) - Proposal 4: Only if the LP-WUS power boosting can be defined as dynamic range where the level is up to manufacturer declaration and 0dB as minimum core requirement. (Nokia) - WF - Consider exclusion of small CBW together with LP-WUS power boosting definition, manufacturer’s declaration as a package for further discussion

On the other hand, it was agreed at RAN1#118-bis as follows [8]:

Agreement From RAN1 perspective, support X=11 PRBs for LP-WUS and LP-SS with SCS 15kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5 MHz

Therefore, RAN1 support LP-SS and LP-WUS with a channel bandwidth equal or larger than 5MHz. Besides, as discussed in section 2.1 above, the core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB, and BS manufacturers should be allowed to declare the supported LP-WUS power boosting level, as such there is no need to preclude small CBW for LP-WUS power boosting if BS manufacturers declare to support power boosting with small CBW for LP-WUS operation.

2.5	Whether to define the LP-WUS requirements only for supported BS types

It is stated in the agreed WF as follows:

- Proposals - Proposal 1: RAN4 to only define conductive based BS RF requirements for LP-WUS in Release-19. (CATT) - Proposal 2: Only conductive based BS type 1-C and 1-H requirements are considered for LP-WUS. (Huawei) - WF - FFS.

As it was agreed in the WF that ‘Non-AAS, AAS could be considered for LP-WUS feature’, both conducted and radiated BS RF requirements should be defined for LP-WUS, otherwise AAS BS that do not have antenna connectors cannot be tested for 3GPP compliance.

2.6	General consideration on other requirements

It is stated in the agreed WF as follows:

- Proposals - Proposal 1: RAN4 to first determine whether a full set or simplified set of BS Tx requirements should be specified for LP-WUS. (Huawei) - WF - FFS.

Considering the time constraint, it is proposed to first focus on the power boosting and transmit signal quality requirements that clearly need to be defined, then work on other Tx requirements when their need become clear.

2.7	Transmitted signal quality

It is stated in the WF as follows:

- Proposals - Transmitted signal quality requirements should be defined for LP-WUS, but depending on further RAN1 progress on waveform design. (Huawei, Nokia, CATT) - Proposal 1: FFS whether the modulation depth can be used as transmitted signal quality for LP-WUS. (ZTE) - Proposal 2: OOK-based EVM can be defined for LP-WUS. How to map OOK on-off performance to OOK-based EVM is FFS. (vivo) - No need to introduce transmitted signal quality requirement for LP-WUS. (Ericsson) - Recommended WF - FFS.

Currently, the EVM requirement for NR BS is specified in TS 38.104 [3] as follows:

### 6.5.2 Modulation quality #### 6.5.2.1 General Modulation quality is defined by the difference between the measured carrier signal and an ideal signal. Modulation quality can e.g. be expressed as Error Vector Magnitude (EVM). The Error Vector Magnitude is a measure of the difference between the ideal symbols and the measured symbols after the equalization. This difference is called the error vector. Details about how the EVM is determined are specified in Annex B. For BS type 1-C this requirement shall be applied at the antenna connector supporting transmission in the operating band. For BS type 1-H this requirement shall be applied at each TAB connector supporting transmission in the operating band. #### 6.5.2.2 Minimum Requirement for BS type 1-C and BS type 1-H For BS type 1-C and 1-H, the EVM levels of each NR carrier for different modulation schemes on PDSCH outlined in table 6.5.2.2-1 shall be met using the frame structure described in clause 6.5.2.3. Table 6.5.2.2-1: EVM requirements for BS type 1-C and BS type 1-H carrier Modulation scheme for PDSCH; Required EVM. QPSK; 17.5 %. 16QAM; 12.5 %. 64QAM; 8 %. 256QAM; 3.5 %. 1024QAM; 2.5 %1 2.8 %2. Note1: This requirement is applicable for frequencies equal to or below 4.2 GHz. Note 2: This requirement is applicable for frequencies above 4.2 GHz..

In table 6.5.2.2-1 above, the EVM requirement is specified depending on the modulation schemes and physical channels. The EVM for each modulation scheme was derived by considering the EVM impacts on overall link performance, as explained in TR 38.817-02 [4].

Since LP-WUS/LP-SS is a new physical channel using On-Off Keying (OOK) modulation, a new core requirement shall be added for the modulation quality for LP-WUS/LP-SS. The EVM requirement for LP-WUS/LP-SS should be derived by considering the EVM impacts on overall link performance according to RAN1 waveform design.

On the other hand, it was agreed at RAN1#117 as follows [7]:

For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR = NF of MR+ 2dB - Note 1: The NF of MR is assumed as 7dB

The equation between EVM and SNR is as follows:

SNR=-[PAPR+20logEVM100]							(1)

It has been shown in section 2.2 that the PAPR of LP-WUS/LP-SS is more like 64QAM than 16QAM, and thus the PAPR of 64QAM (equal to 3.7) could be used in equation (1) to obtain the required EVM when the SNR is decided in RAN1.

# 3.	Conclusion

This contribution has provided proposals to progress the issues that were FFS in the agreed WF in last RAN4 meeting.

Proposal 1: Modify Proposal 1 to handle the case when there is no NR REs allocation.

Proposal 2: The core requirement for the dynamic range for LP-WUS/LP-SS shall be specified the same as 64QAM, i.e., 0 dB, and BS manufacturers should be allowed to declare the supported LP-WUS power boosting level.

Proposal 3: There is no need to cap the LP-WUS power boosting.

Proposal 4: There is no need to preclude small CBW for LP-WUS power boosting.

Proposal 5: Both conducted and radiated BS RF requirements should be defined for LP-WUS.

Proposal 6: First focus on the power boosting and transmit signal quality requirements that clearly need to be defined, then work on other Tx requirements when their need become clear.

Proposal 7: A new EVM requirement shall be added for LP-WUS/LP-SS and it should be derived by considering the EVM impacts on overall link performance according to RAN1 waveform design.

# References

[1]	RP-241824, “Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO.

[2]	R4-2416589, “Way Forward for [112bis][308] NR_LPWUS”, Huawei.

[3]	3GPP TS 38.104 v18.7.0: “NR; Base Station (BS) radio transmission and reception”.

[4]	3GPP TR 38.817-02 v15.11.0: “NR; General aspects for Base Station (BS) Radio Frequency (RF) for NR”.

[5]	RP-241039, “Status report for WI: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo.

[6]	3GPP TS 36.141 v18.4.0: “Evolved Universal Terrestrial Radio Access (E-UTRA); Base Station (BS) conformance testing”.

[7]	3GPP TS 38.141-1 v18.7.0: “NR; Base Station (BS) conformance testing Part 1: Conducted conformance testing”.

[8]	R1-2409089, “Summary #1 of discussions on LP-WUS and LP-SS design”, vivo.