3GPP TSG-RAN WG4 Meeting # 113						    			R4-2417544

Orlando, USA, 18th – 22nd November 2024

Title: Discussions on VSAT antenna patterns

Agenda Item: 7.9.2

Source: Sharp

Document for: Discussion

# 1	Introduction

In RAN4#112bis, VSAT antenna pattern was studied and discussed. It is kept open for further study as follows in the ad hoc meeting minutes [1].

- Sub-topic 2-8: VSAT antenna pattern - Proposals: - Proposal 1 [Huawei, HiSIlicon, Thales] section 6.4.1 of 38.811 - Proposal 2 [Huawei, HiSIlicon] section 8.1.1 of 38.921 - Moderator Recommendation: - Discussion required. 38.811 is parabolic reflector,38.921 is phased array Phase array dominant use case. Proposal: Adopt proposal 2 as the baseline then - Develop the parameters from Sharp’s pattern in R4-2415276 - Adapt 38.921 for horizontal assumption with equivalent VSAT beamwidth Ericsson: Prefer to go for parabolic from Ka first Decision, keep it open for now.

In this contribution we provide our views on antenna pattern.

# 2	Discussions

There are two existing antenna patterns defined in 3GPP technical reports. One is for reflector antennas defined in TR38.811 and the other is for phased arrays defined in TR38.921. Considering that in our view, phased arrays will be a promising solution for the Ku band, we propose reusing TR38.921 as a starting point.

Proposal 1: Reuse antenna patterns defined in TR38.921 as a starting point.

The antenna pattern in TR38.921 is defined as follows.

Table 8.1.1-2: Array antenna model details

| Description | Equation | Unit |
| --- | --- | --- |
| Peak normalized element radiation pattern | Aθ,φ=-min--min12φφ3dB2,Am-min12θ-90θ3dB2,SLAv ,Am | dB |
| Peak gain normalized element radiation pattern | AEθ,φ=GE,max+Aθ,φ | dBi |
| Composite array radiation pattern | AAθ,φ=AEθ,φ+10log10m=1Mn=1Nwm,nvm,n2 , where vm,n=expj2πm-1dvλcosθ+n-1dhλsinθsinφ wm,n=1MNexpj2πm-1dvλsinθetilt-n-1dhλcosθetiltsinφescan | dBi |

We further present our views on simulation parameters in Table 1. Values for the parameters related to antenna elements (i.e., Am, SLAv, ϕ3dB, θ3dB) in TR38.921 can be reused for this study. However, GE,max requires adjustment to fit into the agreed max Tx/Rx gain of 36.6 [2]. Considering that the agreed VSAT equivalent antenna aperture is 60 cm[2], a 45 x 45 = 2025 element antenna can be implemented given lambda/2 element spacing at 14Ghz in UL. Then, the phased array gain is 10*log10(2045) = 33.1069. We propose GE,max=36.6-33.1=3.5.

Proposal 2: GE,max=36.6-33.1=3.5.

Based on the discussion above, we propose the following parameters for study.

Proposal 3: The parameter set in Table 1 is considered for further study.

Table 1: Antenna parameters for coexistence simulation

| Parameter | Symbol | Unit | Value |
| --- | --- | --- | --- |
| Front to back ratio | Am | dB | 30 |
| Side lobe suppression | SLAv | dB | 30 |
| Horizontal half power band width | ϕ3dB | Degrees | 90 |
| Vertical half power band width | θ3dB | Degrees | 90 |
| Array element peak gain | GE,max | dBi | 3.5 |
| Number of radiating elements rows and columns | (M, N) | Integer | (45, 45) |
| Horizontal element separation | dh | m | 0.5 𝜆 |
| Vertical element separation | dv | m | 0.5 𝜆 |

# 6	Summary

Proposal 1: Reuse antenna patterns defined in TR38.921 as a starting point.

Proposal 2: GE,max=36.6-33.1=3.5

Proposal 3: The parameter set in Table 1 is considered for further study.

# 7	References

R4-2416568, Ad hoc meeting minutes for [112bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#112bis meeting, October, 2024

R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#112bis meeting, October, 2024