3GPP TSG-RAN WG4 Meeting #113	R4-2417559

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	Discussion on MIMO layer evaluation for 6Rx UE

Agenda item:	7.1.1.3.2

Document for:	Discussion

Introduction

During RAN#104 the work item on UE RF enhancements in Rel-19 was revised [2].

The performance part of this work item has not commenced yet, however discussions during RAN4#110-bis and RAN4#111 on the number of MIMO layers to support 6Rx has led to an Agenda Item to be introduced during RAN4#112 to specifically discuss MIMO layers for this Work Item with regards to 6Rx.

Specifically looking at the following aspects of the WID:

6Rx for handheld and FWA UE [RAN4, RAN1] - Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario - Example bands: n41, n77/n78, n79, n104 - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers - Specify the Rx requirements including reference sensitivity requirements for support 6Rx - Note: the specified requirements can be applicable to both handheld UE and FWA devices - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, depending on UE capability - RAN1 to study and specify, if necessary, SRS antenna switching to support 3T6R depending on UE capability. - Note: RAN1 discussion starts from RAN1#118bis (Oct. 2024) - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution.

In this document we present views on the open issues which were identified as part of the Way Forward from RAN4#112-bis [1].

Discussion

General considerations for MIMO layer evaluation for 6Rx UE

Tx EVM assumptions for 6-Layer Performance Evaluation

In RAN4#112-bis the topic of TxEVM was discussed, with the following described in the Way Forward [1] :

Issue 3-1-1: Tx EVM assumptions for 6-layer performance evaluation Way Forward: RAN4 to further discuss the assumption of Tx EVM for 6-layer performance evaluation and decide during performance part of work.

Current in RAN4 discussions on TxEVM have ranged from having values of 6%, 3.5%, 2.5% or none, as has been captured in the WF documentation [1]. Nokia would like to add some clarification to this discussion and in particular would like to focus this with the ‘lens’ of ‘feasibility’.

Firstly, the definition of ‘feasibility’ from the “Shorter Oxford English Dictionary”[14] as defined in TR 21.801 [12] (3GPP Specification drafting rules) is defined as such:

Capability of being done; practicability.

Therefore, for 6-layer MIMO to be feasible then it must have the ‘capability of being done’ in at least one scenario.

For 6-layer MIMO to be feasible then it must have the ‘capability of being done’ in at least one scenario.

This key observation will be repeated throughout this contribution as it is key to the work being discussed.

From a network perspective, a UE must be capable to work with a wide variety of RAN vendors in various configurations; but to define feasibility then if one of those configurations is viable for the UE to support 6 layers then the feature should be deemed ‘capable of being done’.

If there is any configuration of the RAN from any vendor whereby a 6-Layer UE is capable of supporting 6 Layer MIMO, then by definition 6-Layers is feasible.

With TxEVM, we have a wide range of values that can be used, for example in small cell deployments, with multiple beams being produced collectively adding to TxEVM the value can reach close to that seen in TS 38.104 [13] of 3.5% or in some cases higher (noting that 256 QAM would not be supported in those specific deployments)

However, in scenarios (such as those employed by 6Rx devices) with low mobility, or even for Fixed Wireless Access, the RAN can be designed to support 6 layers with 8 Tx ports and minimize both TxEVM and BS antenna correlation; ensuring that a high SNR can be achieved in practice with minimal correlation across BS side antenna elements.

RAN deployments and Products that specifically support deployments that a 6Rx device may be seen within will be designed for low TxEVM.

RAN4 should use a TxEVM of either 1% or 0% for feasibility

Equally a RAN deployment such as that which may be expected with 6Rx devices could be designed in such a manner that the correlation between BS Tx ports is close to 0, such that for simulations and all practical purposes we have no correlation.

RAN4 shall use no correlation on BS Tx Ports.

Full details of the simulation campaign aligned with the way forward can be found in our companion simulation TDoc.

6-Layer Performance Evaluation Assumptions

In RAN4#112-bis the topic of 6-layer performance evaluation assumptions was discussed, with the following described in the Way Forward [1] :

Issue 3-1-1: Tx EVM assumptions for 6-layer performance evaluation Way Forward: RAN4 to further discuss and align the 6-Layer MIMO performance evaluation assumptions considering realistic antenna correlation assumptions and deployment scenarios considering all proposals from RAN4#112bis and proposals from previous meetings and decide during performance part of work.

The proposed 6-layer MIMO configuration would utilize two codewords, a feasible approach that should be acknowledged. For performance evaluation, it is crucial to assess this configuration in a rank-deficient channel. This would allow for the observation of distinct performance differences among the MIMO layers, mirroring the conditions encountered in realistic channels.

In order to fully assess the performance of 6Rx a spatial channel model must be used.

Feasibility evaluation

The results for Nokia’s simulation campaign can be observed in our companion TDoc for RAN4#113 [15], within this contribution we can see that 6 Layer MIMO is indeed feasible and track well the performance of 4 Layer, offering higher performance at higher SNRs in a truly spatial channel such as CDL.

6-Layer performance is comparable to 4-layer performance

6-Layer MIMO is shown to outperform 4-layer in spatially diverse channel

6-Layer MIMO is capable of being done

Since feasibility is defined as the capability of being done; these simulations show that unequivocally 6 Layer MIMO is indeed capable of being done, thus 6 Layer MIMO should be deemed feasible.

RAN4 to conclued 6 Rx 6 Layer is feasible.

Way forward

Since the performance aspect of this work item has not yet commenced, it is advisable to defer this discussion until the performance evaluation phase begins. The output of the 'Spatial Channel Model' study item can then provide valuable insights into establishing appropriate performance requirements.

The performance part of this work item has not yet commenced

There is an ongoing study item on introduction of a spatial channel model for RAN4 performance requirements.

RAN4 shall discuss the performance requirements for 6Rx once the performance part commences.

6-layer Support

6-layer Support

In RAN4#112-bis it was discussed if 6-layer shall be supported, with the following described in the Way Forward [1] :

Issue 3-2-1: 6-layer Support Way Forward: RAN4 to further discuss the following options. - Option 1: 6-layer support is feasible for handheld and FWA based on some company evaluations considering realistic antenna correlation assumptions and deployment scenarios. - Option 2: 6-layer support is feasible for FWA devices. - Option 3: Companies to align on the feasibility criteria and simulation assumptions to evaluate 6-Layer feasibility using the following as a starting point. - Feasibility criteria - Consider 4-layer vs 6-layer throughput performance. - Simulation assumptions - Only consider link-level simulation for performance evaluations. - Simulation assumptions provided in Annex A as a starting point for discussions on alignment at RAN4#113 and can be used by companies to provide simulation results at RAN4#113 for discussion. Companies to clarify the settings used in the simulation assumptions for BS correlation matrix and Tx EVM. Representative BS Tx EVM values for the feasibility study are requested from interested BS vendors for RAN4#113. - Option 4: 6-layer support is not feasible for handheld UEs.

During RAN4#112 several offlines were held whereby delegates from different companies were unable to come to agreement on what ‘feasible’ means.

During RAN4#112-bis RAN4 was unable to agree what is determined by Feasibility.

Nokia noted that several companies, including Qualcomm [9], Nokia [11], Oppo [8] had identified that indeed 6-layer MIMO was feasible on devices that were able to accommodate 6Rx, and some companies such as Huawei [10], showed that it was feasible, even if in their simulations they showed it had no gain over 4-layers in SNR performance.

Several individual companies showed that in their opinion 6-layer MIMO was indeed feasible during RAN4#112.

Finally, during RAN4#112 and RAN4#112-bis a series of simulation assumptions were agreed upon, for which Nokia was provided some simulations in [15].

As discussed above and in proposal 4, these results show that 6 Layer MIMO is indeed feasible according to the definition of feasibility as ‘Capability of being done’.

Conclusion

The paper discusses the topic of 6 Layer MIMO evaluation in RAN4, and presents some of the Nokia views on this topic for discussion during RAN4#112.

The following Observations and Proposals were made:

Observation 1: For 6-layer MIMO to be feasible then it must have the ‘capability of being done’ in at least one scenario.

Observation 2: If there is any configuration of the RAN from any vendor whereby a 6-Layer UE is capable of supporting 6 Layer MIMO, then by definition 6-Layers is feasible.

Observation 3: RAN deployments and Products that specifically support deployments that a 6Rx device may be seen within will be designed for low TxEVM.

Proposal 1: RAN4 should use a TxEVM of either 1% or 0% for feasibility

Proposal 2: RAN4 shall use no correlation on BS Tx Ports.

Observation 4: In order to fully assess the performance of 6Rx a spatial channel model must be used.

Observation 5: 6-Layer performance is comparable to 4-layer performance

Observation 6: 6-Layer MIMO is shown to outperform 4-layer in spatially diverse channel

Observation 7: 6-Layer MIMO is capable of being done

Proposal 3: RAN4 to conclued 6 Rx 6 Layer is feasible.

Observation 8: The performance part of this work item has not yet commenced

Observation 9: There is an ongoing study item on introduction of a spatial channel model for RAN4 performance requirements.

Proposal 4: RAN4 shall discuss the performance requirements for 6Rx once the performance part commences.

Observation 10: During RAN4#112-bis RAN4 was unable to agree what is determined by Feasibility.

Observation 11: Several individual companies showed that in their opinion 6-layer MIMO was indeed feasible during RAN4#112.

- References

- R4-2417095, “WF on requirements for 6Rx”, AT&T, RAN4#112-bis, Hefei, October  2024

- RP-241656,  “New WID: UE RF enhancements for NR FR1/FR2 and EN-DC, Phase 4”, Huawei, RAN#104 Shanghai, China, June 2024

- TS 38.822 v17.1.0 “NR; User Equipment (UE) feature list”, June 2023

- R4-072223, “TP for 36.803 for MIMO correlation matrices for LTE”, Motorola, Ericsson, Nokia, NEC, NXP, InterDigitial, Agilent, RAN4#45, Jeju, Korea, November 2007

- R4-080620, “High Dimension E-UTRA MIMO Correlation matrixes”, Ericsson, RAN4#46-bis, Shenzhen, China, April 2008

- R4-1708533, “Discussion on SI for 8Rx”, Huawei, RAN4#84, Berlin, Germany, Aug 2017

- R4-2311072, “Discussion on PDSCH Demodulation Requirements for 8Rx”, Nokia, Nokia Shanghai Bell, RAN4#108, Aug 2023

- R4-2412352, “R19 Simulation results of 6Layer performance”, OPPO, RAN4#112, Aug 2024

- R4-2411680, “Discussion on the demodulation performance requirements for 6Rx devices”, Qualcomm, RAN4#112, Aug 2024

- R4-2412572, “On MIMO layer evaluation for 6Rx UE”, Huawei, HiSilicon, RAN4#112, Aug 2024

- R4-2411525, “Discussion on MIMO layer evaluation for 6Rx UE, Nokia, RAN4#112, Aug 2024

- TR 21.801 v18.0.0 “Specification drafting rules”, March 2024

- TS 38.104 v18.7.0 “NR; Base Station (BS) radio transmission and reception”, Sept 2024

- Oxford English Dictionary, 2024 (https://www.oed.com/dictionary/feasibility_n)

- R4-2418011, “Simulation Results for on MIMO layer evaluation for 6Rx UE”, Nokia, RAN4#113, Orlando, US, Nov 24