3GPP TSG RAN WG4 Meeting #113 	R4-2417569

Orlando, USA, November 18 - 22, 2024

Title:	Discussion on IOT-NTN TDD mode in L-band (1616 – 1626.5 MHz) and UE RF Requirements

Source: 	Iridium, CCL

Type:	Discussion

Document for:	Discussion

Agenda Item:	7.28.1

# Introduction

In RAN#105 new WID on introduction of IoT NTN TDD mode operating in L-band (MSS band 1616 -1626.5 MHz) was approved in [1]. This contribution shares our view on the new IoT-NTN TDD RF band numbering, EARFCN mapping and UE RF characteristics.

The Objectives of the WID, as per [1]:

The work item aims to specify enhancements for NB-IoT NTN to enable NTN operation with a NB-IoT TDD mode leveraging commonalities with half-duplex NB-IoT FDD NTN, by defining a new NB-IoT TDD mode for NTN based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures for the NB-IoT operation in the targeted unpaired MSS allocated band (TN deployment is not expected in this band). The feature is not intended to be applicable to existing 3GPP bands. The study and work objectives assume the following: - LEO @600 km and @1200 km orbit respectively, with set-1 satellite parameters as reference scenarios (See 3GPP TR 36.763) - Target the 1616-1626.5 MHz MSS allocated band - Standalone deployment with anchor and non-anchor carriers (i.e. operating in carrier(s) used only for NB-IoT) - Operate with Earth fixed Tracking area, with either Earth fixed cells or Earth moving cells for NGSO - The new NB-IoT NTN TDD mode allows configuring the usage of radio resources in the targeted MSS allocated band with a periodic subset of the UL and DL subframes in N radio frames. The periodic pattern should consist of non-overlapping set of usable contiguous UL subframes and set of usable contiguous DL subframes, and guard periods, which is periodic every N radio frames, with N=9 as baseline. No blind detection is assumed at the UE side. The value of N and the configuration of the periodic pattern are fixed per band. This work item includes the following objectives: - Study the impact due to the periodic pattern, at least on UE downlink synchronization and other aspects (if identified) [RAN1, RAN4] - Checkpoint in RAN#106 for the completion of the study phase. RAN1 start from Oct’24. RAN4 start from Nov’24 - Specify a new NB-IoT TDD NTN mode based on minimum necessary changes to the NB-IoT NTN FDD frame structure and procedures, based on the outcome of the study, including: - Definition, configuration (if needed) and signaling (if needed) of the periodic pattern including confirming the value of N, and associated UE procedures [RAN1, RAN2] - Other necessary impacts on higher layers [RAN2] - RRM and RF core requirements [RAN4] - Specify a new NB-IoT TDD operating NTN band for the MSS allocation spanning 1616-1626.5 MHz for DL and UL, based on the outcome of the study, to be used as example band for this WI [RAN4]. - Specify band numbering - Specify SAN and UE RF characteristics - Specify DL and UL channelization. - Specify channel bandwidth as 200 kHz - Note1: No NTN-NTN coexistence study needed. - Note2: Leverage existing work as much as possible for TN-NTN coexistence of adjacent bands Note3: Reuse the existing RAN4 requirements as much as possible (no intention to have any relaxation of the RF emissions requirements).

# Discussion

## Operating Band

As per [3] section 5.2, satellite bands are numbered in descending order from 256. In Rel-18, the last allocated satellite band number is 253 for extended L-band. In [4] and [5] next available band number 252 is proposed for the S-band. Following this rule next free band number is 251 which can be allocated to IoT-NTN TDD band.

Table 2.1-1: E-UTRA operating bands for satellite access

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| [251] | 1616 MHz | – | 1626.5 MHz | 1616 MHz | – | 1626.5 MHz | TDD |

Proposal 1: The IoT-NTN TDD MSS band 1616-1626.5 MHz can take the number 251.

Figure 1 Adjacent bands to the proposed Band 251

## Channel Bandwidth

The current specification [3] specifies channel bandwidth for category M1 and category NB1/NB2 in a band agnostic way, therefore, the addition of an IoT-NTN L-band (MSS band 1616-1626.5 MHz UL and DL) does not introduce any change on the channel bandwidth sub-clauses.

Proposal 2: The IoT-NTN TDD L-band (MSS band 1616-1626.5 MHz) does not have any impact on the channel bandwidth of category M1 or NB1/NB2

## Channel raster, carrier frequency and EARFCN

The global frequency raster is defined for all frequencies. The granularity of the global frequency raster is 100 kHz, which means that the carrier center frequency must be an integer multiple of 100 kHz. For each operating band, a subset of frequencies from the global frequency raster is applicable and forms a channel raster with a granularity ΔFRaster..

In the EARFCN allocation for the terrestrial EUTRA bands in [6], there is an EARFCN gap between the FDD and TDD bands. Following the same convention for NTN TDD bands, we can introduce a gap in the EARFCN allocation for NTN-TDD and NTN-FDD bands. This gap should be sufficient for any future NTN TDD and NTN FDD band allocations.

Figure 2 EUTRA bands EARFCN value allocation

As illustrated in Figure 2, the EARFCN values allocated to the NTN bands decrease from 218 – 1, whereas for terrestrial bands, they are allocated in ascending order, with band 106 as the last defined band. The newly proposed Band [251] is the first NTN TDD band. The proposed EARFCN range for the Band [251] has sufficient gaps to accommodate any future EUTRA NTN FDD bands.

| E-UTRA Operating Band | ΔFRaster (kHz) | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FDL_low (MHz) | NOffs-DL | Range of NDL (First – <Step size> – Last) | FUL_low (MHz) | NOffs-UL | Range of NUL (First – <Step size> – Last) |
| [251] | 100 | 1616 | 198196 | 198196-<1>-198300 | 1616 | 198196 | 198196-<1>-198300 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7 channel numbers at the lower operating band edge and the last 6 channel numbers at the upper operating band edge shall not be used for channel bandwidth of 1.4 MHz. |  |  |  |  |  |  |  |

Table 1 EUTRA channel number for IoT-NTN TDD L-band

Proposal 3: For IoT-NTN TDD L-band, the channel raster, carrier frequency and EARFCN can be defined as per Table 1.

## TX-RX Frequency Separation

The default E-UTRA TX channel (carrier center frequency) to RX channel (carrier center frequency) separation is only specified for FDD mode of operation in [3]. Even though the proposed NTN TDD mode is using IoT-NTN FDD frame structure, TDD mode in the band definition is sufficient to indicate that there is no TX-RX frequence separation.

Proposal 4: For IoT-NTN TDD L-band, there is no need to explicitly state that TX-RX frequency separation is zero.

## UE Transmitter Characteristics

We have distinguished band agnostic and band dependent UE RF requirements for NB-IoT and eMTC UEs and provide our view on which parameters can be re-used from existing bands.

| 36.102 Clause: Requirements | Band Agnostic | Re-use for TDD band [251] | Comments |
| --- | --- | --- | --- |
| 6.2A.1/6.2B.1: UE maximum output power | Yes | Yes | - Support UE power class 3 (23 dBm) - Support UE power class 5 (20 dBm) |
| 6.2A.1/6.2B.1: UE maximum output power tolerance | Yes | Yes |  |
| 6.2A.2/6.2B.2: UE maximum output power reduction | Yes | Yes |  |
| 6.2A.3/6.2B.3: UE additional maximum output power reduction | No | Yes | No additional maximum power reduction is required. Once Band [251] is added to the table 5.2-1 in 36.102, no additional change is required |
| 6.2.A.4/ 6.2.B.4: Configured transmit power | Yes | Yes |  |
| 6.3: Output power dynamics | Yes | Yes |  |
| 6.4: Transmit signal quality | Yes | Yes |  |
| 6.5A.3.2/6.5B.3.2: Spectrum emission mask | Yes | Yes |  |
| 6.5A.3.4/6.5B.3.4: Adjacent Channel Leakage Ratio | Yes | Yes |  |
| 6.5A.4.2/6.5B.4.2: General spurious emissions | Yes | Yes |  |
| 6.5A.4.3/6.5B.4.3: Spurious emissions for UE co-existence | No |  | Band [251] needs to be added in Table 6.5A.4.3-1 Spurious emission band UE co-existence. See comments in section 2.6 |
| 6.5A.4.4/6.5B.4.4: Additional spurious emissions | No |  | Additional spurious emissions requirement for B254 can be reused for the new IoT-NTN TDD L-band. UL frequency range of B254 is superset of B251. See Proposal 6. |
| 6.6: Transmit intermodulation for category NB1 and NB2 | Yes | Yes |  |

Table 2 UE RF transmitter requirements

Proposal 5: For IoT-NTN TDD L-band, all band agnostic UE transmitter requirements can be reused.

Proposal 6: For IoT-NTN TDD L-band, UE transmitter requirement for additional spurious emission from Band [254] can be reused.

## Spurious emission band UE co-existence

Here we have shared our view on the requirement for EUTRA satellite band IoT-NTN Band [251] coexistence with protected bands. In 36.102, table 6.5A.4.3-1 should be updated to include Band [251] contents as per Table 3. Here we have re-used protected band list of EUTRA band 254 whose uplink frequency range overlaps with EUTRA IoT-NTN Band [251].

|  | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA Band | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 251 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 31, 41, 48, 54, 66, 70, 71, 72, 85, 87, 88, 103 NR Band n1, n3, n7, n8, n18, n20, n28, n34, n38, n39, n40, n50, n51, n53, n65, n67, n74, n75, n76, n77, n78, n90, n91, n92, n93, n94, n105, n106, n109 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.5A.4.2-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |

Table 3 Requirements for spurious emissions for UE co-existence

Proposal 7: Table 6.5A.4.3-1 in 36.102 should be updated to add new Band [251] with parameters as specified in Table 3.

## UE Receiver Characteristics

We can distinguish band agnostic UE RF requirements for NB-IoT and eMTC UEs and provide our view on which parameters can be re-used.

| 36.102 Clause: Requirements | Band Agnostic | Re-use for IoT-NTN TDD L-band | Comments |
| --- | --- | --- | --- |
| 7.2: Diversity characteristics | Yes | Yes |  |
| 7.3: Reference sensitivity power level | No | No | To be studied for IoT-NTN TDD mode. |
| 7.4: Maximum input level | Yes | Yes |  |
| 7.5: Adjacent Channel Selectivity | Yes | Yes |  |
| 7.6A.2/7.6B.2: In-band blocking | No | Yes | Different band group may have different In-band blocking tables for Cat-M1 and NB1/NB2 separately. Currently, bands 255,254, and 253 apply the same IBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. B251 can re-use IBB requirements from B255,254 and 253 |
| 7.6A.3/7.6B.3: Out-of-band blocking | No |  | Different Out-of-band requirements for IoT NTN bands. Currently, bands 255,254, and 253 apply the same OBB requirements. Only for B256 lower frequency range is modified to accommodate specific implementation. B251 can re-use OBB requirements from B255, 254 and 253 |
| 7.6A.4/7.6B.4: Narrow band blocking | No | Yes | Only applicable for M1 category, can re-use existing requirements from Table 7.6A.4-1 of [3] |
| 7.7: Spurious response | Yes | Yes |  |
| 7.8: Intermodulation characteristics | Yes | Yes |  |
| 7.9: RX spurious emission | No | Yes | Re-use spurious emission requirements from Table 7.9.1-1 of 36.101 |

Table 4 UE RF receiver requirements

Proposal 8: The IoT-NTN TDD Band [251] can re-use in-band blocking requirements from Band 255, 254 and 253.

Proposal 9: The IoT-NTN TDD Band [251] can re-use out-of-band blocking requirements from Band 255, 254 and 253.

Proposal 10: The IoT-NTN TDD Band [251] can re-use existing UE receiver requirements for narrow band blocking from Table 7.6A.4-1 of 36.102

Proposal 11: The IoT-NTN TDD Band [251] can re-use existing UE receiver requirements for receiving spurious emissions from Table 7.9.1-1 of 36.101

Proposal 12: The IoT-NTN TDD Band [251], new reference measurement channel may be required to consider discontinuous transmissions and evaluate new reference sensitivity power level (REFSENS) for category M1 and NB-1/NB-2.

Proposal 13: For IoT-NTN TDD L-band, all band agnostic UE receiver requirements can be reused.

# Conclusion

In this contribution, we have suggested introducing new RF band for IoT-NTN TDD mode and shared our view on UE transmitter and receiver characteristics for the new band. Our observations and proposals are listed below:

Proposal 1: The IoT-NTN TDD MSS band 1616-1626.5 MHz could be numbered as 251.

Proposal 2: The IoT-NTN TDD band L-band (MSS band 1616-1626.5 MHz) does not have any impact on the channel bandwidth of category M1 or NB1/NB2.

Proposal 3: For IoT-NTN TDD L-band, the channel raster, carrier frequency and EARFCN can be defined as per Table 1.

Proposal 4: For IoT-NTN TDD L-band, there is no need to explicitly state that TX-RX frequency separation is zero.

Proposal 5: For IoT-NTN TDD L-band, all band agnostic UE transmitter requirements can be reused.

Proposal 6: For IoT-NTN TDD L-band, UE transmitter requirement for additional spurious emission from Band [254] can be reused.

Proposal 7: Table 6.5A.4.3-1 in 36.102 should be updated to add new Band [251] with parameters as specified in Table 3.

Proposal 8: The IoT-NTN TDD Band [251] can re-use in-band blocking requirements from Band 255, 254 and 253.

Proposal 9: The IoT-NTN TDD Band [251] can re-use out-of-band blocking requirements from Band 255, 254 and 253.

Proposal 10: The IoT-NTN TDD Band [251] can re-use existing UE receiver requirements for narrow band blocking from Table 7.6A.4-1 of 36.102

Proposal 11: The IoT-NTN TDD Band [251] can re-use existing UE receiver requirements for receiving spurious emissions from Table 7.9.1-1 of 36.101

Proposal 12: The IoT-NTN TDD Band [251], new reference measurement channel may be required to consider discontinuous transmission and evaluate new reference sensitivity power level (REFSENS) for category M1 and NB-1/NB-2.

Proposal 13: For IoT-NTN TDD L-band, all band agnostic UE receiver requirements can be reused.

# References

[1]	RP-242415, “New WID on introduction of IoT-NTN TDD mode”, 3GPP TSG RAN Meeting #105, Melbourne, Australia, September 9-12, 2024

[2] 	3GPP TR 36.763, Study on Narrow-Band Internet of Things (NB-IoT) / enhanced Machine Type Communication (eMTC) support for Non-Terrestrial Networks (NTN) (Release 17)

[3]     3GPP TS 36.102, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception for satellite access (Release 18)

[4]     R4-2411548, Discussion on an IoT-NTN S-band (MSS band 2000-2020 MHz UL and 2180-2200 MHz DL) UE RF requirements, Maastricht, Netherlands, August 19-23, 2024

[5]     R4-2411845, Discussion on system parameters and UE RF requirements for IoT-NTN S-band, Maastricht, Netherlands, August 19-23, 2024

[6]     3GPP TS 36.101, Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception (Release 18)