3GPP TSG-RAN WG4 Meeting #113												R4-2417589

Orlando, USA, 18 – 22 November, 2024

Agenda item:			7.12.3.1

Source:	Huawei, HiSilicon

Title:	on TRP TRS OTA tests

Document for:

# Introduction

The WF [1] in the last meeting captures the following issues. This contribution discusses them further.

Issue 4-2-1: How to specify PDA hand related TRP/TRS requirements (assume no large scale measurement campaign, current requirement is a basis)

Agreements:

  - RAN4 can initiate a preliminary (not large scale) measurement campaign for size 2 UEs, FFS how to down select between measurement-based approach and the offset-based approach.

  - Encourage volunteer test labs to perform the above measurement campaign, Rel-17 and/or Rel-18 aligned test lab is preferred.

  - Encourage companies to provide seize 2 UEs (small phone with PDA hand) for preliminary measurements.

  - Note: Size 1 (wide, width >72mm and ≤92mm), Size 2 (narrow, width ≥56mm and ≤72mm);

Issue 4-2-2: PDA hand related TRP/TRS requirements

Agreements:

  - RAN4 further discuss how to specify PDA hand related TRP/TRS requirements (Size 2 UEs).

Issue 4-1-3: Whether there is MU difference for 2Tx test system

Agreements:

  - RAN4 can further discuss this issue to check whether there is MU impact.

# Discussion

Current options for determining size 2 TRP and TRS performance are

- Measurement campaign for size 2 devices

- Offset relative to size 1 TRP and TRS performance

Option (b) is difficult to carry out because performance measurement needs to be taken for devices with width of 72mm exactly. Such devices could be limited on the commercial markets. A search on GSMarena.com [2] shows 27 smartphones came to market since 2022 with widths between 72mm and 73mm. However only two of them, viz. Google Pixel 9, Motorola Edge, is at 72mm exactly. GSMArena.com lists most, if not all, smartphone brands on the market. Even if some smartphones are not listed on GSMArena.com, the chance of finding additional smartphones with exact width of 72mm is slim. Measurement of two or three smartphones cannot be regarded as statistically significant. In other words, option (b) does not seem to be viable. This naturally leads to option (a).

Proposal 1: measurement campaign should be used to determine the OTA performance for size 2 devices as the number of smartphones with width of 72mm is too few to determine performance offset between size 1 and 2.

Huawei OTA lab would like to participate in R19 TRP and TRS measurement campaign, but not MIMO OTA measurement campaign due to resource constraints.

Observation 1: Huawei OTA lab would like to participate in R19 TRP and TRS measurement campaign as a volunteer lab.

##### Annex of TS 38.561 [3] has the follwoing text on MU term due to offset of phase centre from axis of rotation.

##### A.4.2.7.1.1	Offset of DUT phase centre from axis(es) of rotation

In all the DUT measurements (Stage 2) defined in this test procedure, the DUT and head phantom combination is rotated about the ear reference point of SAM phantom, which is also assumed to be the location of the phase centre in both angular directions of the measurements.

In the context of 2Tx devices, the two transmit antennas are likely to be located away from each other as much as possible. For example, one at the top of a smartphone, the other at the bottom. In this case, the offset from axis of rotation for 2Tx may be different from the one for 1Tx and could therefore impact MU term under measurement distance.

Proposal 2: to ascertain if offset from axis of rotation for 2Tx is different or not from the one for 1Tx in order to determine potential impact of MU term under measurement distance.

# Conclusion

Proposal 1: measurement campaign should be used to determine the OTA performance for size 2 devices as the number of smartphones with width of 72mm is too few to determine performance offset between size 1 and 2.

Observation 1: Huawei OTA lab would like to participate in R19 TRP and TRS measurement campaign as a volunteer lab.

Proposal 2: to ascertain if offset from axis of rotation for 2Tx is different or not from the one for 1Tx in order to determine potential impact of MU term under measurement distance.

# References

- R4-2416590 WF [112bis][323] TRP_TRS_Ph3

- GSM Arena phones with width between 72mm and 73mm: https://www.gsmarena.com/results.php3?nYearMin=2022&nWidthMin=72&nWidthMax=73&s5Gs=0

- 38.561 v18.0 UE TRP (Total Radiated Power) and TRS (Total Radiated Sensitivity) requirements and test methodologies for FR1 (NR SA and EN-DC)