3GPP TSG RAN WG4 #113															     R4-2417606

Orlando, US, November 18th – 22nd 2024

Agenda Item:		7.25.2

Source:				InterDigital Europe

Title:		Discussion on NR_XR_Ph3 RRM core requirements

Document for:		Discussion and decision

# Background

During RAN1 Meeting #117, an LS to RAN4 [1] related to the UE assistance information on skipping RRM measurement occasions for NR XR has been approved. Further, in RAN1 Meeting #118 an agreement was made on the NW scheduling indication for a skipping occasion [2].

In the last RAN4 meeting #112bis, the discussions on this topic progressed. As a result, a WF has been agreed in the meeting in [3].

# Discussion

In this contribution, we share our analysis on gap skipping measurements occasions for NR XR case and the NW scheduling mechanism. For convenience we are listing below the RAN1 LS text [2]:

Working Assumption For solutions based on triggering/enabling by network signaling to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements select the following option: - Alt. 1: Dynamic indication to enable Tx/Rx in particular gap/restriction that are caused by RRM measurements. - Alt 1-1: Explicit indication by DCI to skip a particular gap/restriction; - Indication is included as part of scheduling DCI: - Bit-field size is one bit; - The bit in the DCI is used to indicate whether to skip the first gap/restriction occasion after a minimum time offset required between the last symbol of the PDCCH carrying the DCI format and the start of corresponding skipped gap/restriction occasion indicated by the DCI. Send an LS to RAN4 to inform them of the above working assumption and ask them if there is any issue with it. Agreement If Alt. 1 from RAN1#117 agreement is supported, minimum time offset(s) X between indication to skip and skipped measurement occasion is up to RAN4 to discuss and decide on particular value(s). To RAN4 ACTION: RAN1 kindly asks RAN4 to consider the above information into account and decide whether or not to introduce any UE assistance information related to measurement occasions.

In RAN4 meeting #112bis, the agreed WF in [3] treats a variety of topics that we will address in the following sections.

# NR deployment scenarios and MEASUREMENT types

In the last meeting, the following agreements and were made:

## Issue 1-1: Deployment scenarios - Frequency ranges - Agree on FR1 and FR2-1, and FFS FR2-2 - Scenarios - Start the discussion for - NR-SA - Carrier aggregation - Further Discuss - NR-DC, EN-DC, NE-DC

One of the FFS points was about the dual connectivity support for this feature. In our view, dual connectivity scenarios are more complex due to the involvement of two scheduling entities, that may be uncorrelated when considering the UE measurements gap skipping through DCI signaling. The scheduling in DC deployments may lead to colliding DCI gap skipping orders, and thus more difficult to address. Even more complex than NR-DC is EN-DC or NE-DC where LTE and NR schedulers are involved.

Observation 1: NR-DC, EN-DC and NE-DC deployments involve different schedulers that may lead to colliding DCI gap skipping orders.

Thus, we propose to deprioritize discussions for NR-DC, EN-DC and NE-DC and focus on NR-SA and CA scenarios.

Proposal 1: Deprioritize discussions for NR-DC, EN-DC and NE-DC.

## Issue 1-2: Types of measurements to be considered <Way forward>: - Option 1: Agree at least on the following and further discuss other scenario - Intra-frequency - Inter-Frequency - LTE inter-RAT - Option 2: Agree at least on the following and further discuss other scenario - Intra-frequency - Inter-Frequency - LTE inter-RAT - XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC - Option 3: include all related scenarios

For the types of measurements to be considered we prefer Option 2 since 2G and 3G chipsets are not supposed to be affected in terms of measurement timelines and of course following our proposal to deprioritize DC EN-DC and NE-DC scenarios.

Observation 2: Supporting IRAT gap skipping for 2G and 3G chipsets may affect their measurement timelines.

Proposal 2: Support Option 2 for further discussions:

  - Option 2: Agree at least on the following and further discuss other scenario

    - Intra-frequency

    - Inter-Frequency

    - LTE inter-RAT

    - XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC

## Issue 1-3: L3 Measurements without gaps with scheduling restrictions <Way forward>: - Option 1: Support skipping of measurements without gaps - Option 2: Measurement skipping does not apply for measurements without gaps

In our opinion, the WI targets the L3 scheduled measurements gaps, and thus we propose to continue discussions considering Option 2 from the above list.

Proposal 3: Support Option 2: Measurement skipping does not apply for measurements without gaps.

## Issue 1-4: L1 measurements with scheduling restrictions and procedures <Way forward>: - Option 1: Support skipping of L1 measurements and procedures such as RLM, BFD, L1-RSRP etc - Option 2: Measurement skipping does not apply for L1 measurements and procedures

In our view, the measurement gaps are not supposed to affect the intra-frequency operations and procedures. Thus, we are proposing supporting Option 2 for further work.

Proposal 4: Support Option 2: Measurement skipping does not apply for L1 measurements and procedures.

## Issue 1-5: L1/L2 triggered mobility measurements and procedures <Way forward>: - Option 1: support skipping of L1 measurements and procedures for LTM within and outside gap - Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM

LTM is supposed to resolve the latency problem in mobility and reduce the HO failure. Thus, related measurements, triggering conditions and delay shall not be affected by this feature. We also supported to treat in RAN4 L3 measurements, a RAN1 decision.

Proposal 5: Support Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM.

.

# Measurement with gaps

The WF from last meeting describe a few options on what gaps type to consider moving forward.

Issue 2-1 and Issue 2-2 are to be discussed again:

## Issue 2-1: General approach for defining requirements for measurements with gaps <Way forward>: - Option 1: RAN4 to deprioritize the discussion on applicable type of MGs at early stage of the WI, and focus on the solutions/impacts on RRM requirements. Unless, it is found that a particular type of MG cannot be supported or need special treatment, RAN4 can discuss whether the feature can apply to this MG type. - Option 2: As a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS.

The following issue reiterates the assumed definitions to be used in discussions and ask for further input.

## Issue 2-2: Types of measurement gaps <Way forward>: - In this issue we discuss the types of measurement gaps to be considered in this WID. In order to help the discussion, we reuse the definition of Type-1 and Type-2 measurement gaps as agreed in Rel-18 NR_MG-enh2: - Type-1 MG: Gap(s) configured via GapConfig without suffix - Type-2 MG: Gap(s) configured via GapConfig-r17 without preConfigInd-r17 or ncsgInd-r17 - Rel-18 Concurrent Gap: gap with assigned priority which can be also NCSG or Pre-MG.

In our view, Type-1 should be the priority, as they are supported by all UEs, while Type-2 and/or concurrent gaps should be a second priority.

Proposal 6: Support Option 2: As a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS.

# XR skipping requirements impact

In this section, we assume that L3 measurements are considered. That means average is used over the measurement samples. If the accuracy is to be maintained, then the number of samples should be preserved. Our preference is to maintain the measurements accuracy.

## Issue 3-1: How to capture impact of measurement skipping <Way forward>: - Option 1: - As minimum requirements, cell detection, index reading, and measurement period is extended. - FFS how to extend - Reuse existing measurement accuracy requirements. - Option 2: - For the cases where measurement impact is needed - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended. - FFS how to extend. - Reuse existing measurement accuracy requirements. - For the cases where measurement impact is not needed - FFS the feasible conditions and necessary UE assistance information.

At this stage, we support Option 2 which offer more flexibility.

Proposal 7: Support Option 2:

- For the cases where measurement impact is needed

    - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended.

      - FFS how to extend.

    - Reuse existing measurement accuracy requirements.

  - For the cases where measurement impact is not needed

    - FFS the feasible conditions and necessary UE assistance information.

## Issue 3-2: Principles for measurement delay calculation <Way forward>: - Option 1: Follow the scaling factor (Ntotal/Navailable) from Concurrent MG spec to extend the total measurement delay. - Option 2: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period. - Other options are not precluded

In our view, if the delay calculation is required, the simplest way is to follow Option 2 which offers a direct mode of calculation/evaluation.

Proposal 8: Support Option 2: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period.

# Time offset for measurement skipping processing

In the last meeting different proposals have been made and all were preserved in the WF for further discussion:

## Issue 4-1: Time offset requirement for measurement gap skipping <Way forward>: - Option 1: 7 ms - Option 2: 5 ms - Option 3: DCI decoding time defined in TS 38.214 + [5]ms - Option 4: assume subclause 5.3.1 of TS38.214 as a baseline for X offset. - Option 5: Fraction of a slot - Option 6: besides the DCI decoding delay, no additional processing delay is needed. - Option 7: N2 symbols, which is to follow the same approach for Rel-17 positioning about the time offset between UE determination of the presence of DL channels and DL PRS processing window (N2 is about UE PUSCH preparation procedure time, which is defined in Clause 6.4, TS38.214)

According to the LS sent by RAN1, a working assumption was made to support Alt. 1-1 (explicit indication by DCI to skip a particular gap/restriction). In our opinion, the time gap of X slots before the gap occasion can be covered by a variety of scheduling DCI format types as suggested during RAN1 discussions on the subject [7]. Since it is important to consider the timing for both DL and UL scheduling upon skipping the gap, we believe that the legacy behavior corresponding to the application delay of the minimum scheduling restrictions mentioned in subclause 5.3.1 in TS 38.214 can be considered as a baseline for the time gap. For convenience, we are outlining the text of the subclause below:

### 5.3.1 Application delay of the minimum scheduling offset restriction When the UE is scheduled with DCI format 0_1, 0_3, 1_1 or 1_3 with a 'Minimum applicable scheduling offset indicator' field in slot n, it shall determine the K0min and K2min values, if configured respectively, to be applied, while the previously applied K0min and/or K2min values are applied until the new values take effect. If the DCI in slot n also indicates an active DL (UL) BWP change for a serving cell, the indicated K0min (K2min) value in the new active DL (UL) BWP, if configured, is applied from the slot indicated by the slot offset value of the time domain resource assignment field in the DCI. Otherwise, change of applied minimum scheduling offset restriction indication carried by DCI in slot n, shall be applied in slot n+X of the scheduling cell. The UE does not expect to be scheduled with DCI format 0_1, 0_3, 1_1 or 1_3 with 'Minimum applicable scheduling offset indicator' field indicating another change to K0min or K2min for the same active BWP of the scheduled cell before slot n+X of the scheduling cell. When the DCI format 0_1, 0_3, 1_1 or 1_3 with 'Minimum applicable scheduling offset indicator' field indicating a change to the applied K0min or K2min is contained within the first three symbols of slot n, the value of application delay X is determined by, X=maxK0minOld∙2μPDCCH2μPDSCH,Zμ where K0minOld is the currently applied K0min value of the active DL BWP in the scheduled cell and is zero, if minimumSchedulingOffsetK0 is not configured for the active DL BWP in the scheduled cell, Zµ is determined by the subcarrier spacing of the active DL BWP in the scheduling cell in slot n, and given in Table 5.3.1-1, and µPDCCH and µPDSCH are the sub-carrier spacing configurations for PDCCH of the active DL BWP in the scheduling cell and PDSCH of the active DL BWP in the scheduled cell, respectively, in slot n. After indication of a change to the applied K0min or K2min of the scheduled cell in slot n of the scheduling cell, if there is an active DL BWP change in the scheduling cell before slot n+X, the new K0min and/or K2min values are applied from the first slot no earlier than the start of slot n+X based on the sub-carrier spacing configuration of the active DL BWP in the scheduling cell in slot n. When the DCI format 0_1, 0_3, 1_1 or 1_3 with 'Minimum applicable scheduling offset indicator' field is received outside the first three symbols of the slot, value of Zµ from Table 5.3.1-1 is incremented by one before determining the application delay X. When the PDCCH reception includes two PDCCH candidates from two respective search space sets, as described in clause 10.1 of [6, TS 38.213], for the purpose of determining 'Minimum applicable scheduling offset indicator' field is received outside the first three symbols of the slot, the PDCCH candidate that ends later in time is used. Table 5.3.1-1: Definition of Zµ µ; Zµ. 0; 1. 1; 1. 2; 2. 3; 2. 5; 8. 6; 16.

Thus, we still believe that our proposal (Option 4) can be considered for the time offset determination.

Proposal 9: Support Option 4: assume subclause 5.3.1 of TS38.214 as a baseline for X offset.

# Need/feasibility of UE assistance information

The WF mention the following options:

## Issue 5-1: General on UAI <Way forward>: - Companies are tasked to discuss the need and feasibility of UAI, based on the following options - Option 1: UAI is specified - Companies supporting this option are encouraged to bring contribution with exact proposals on what information is to be included in UAI. - Option 2: UAI is not specified

If the UAI is to be supported, then it shall contain a bitmap, or a skipping rule based on the RRC configured gaps. The UE gets configured with a L3 gaps by network. Then UE, based on the configuration received replies to the network with a bitmap or a sort of gaps skipping rule.

If the UAI is not supported, then the network will make its own decisions when to skip a gap based on pure buffer status and channel conditions.

We don’t have a strong opinion. However, for UAI case we prefer to leave the gap skipping rules for UE implementation.

Proposal 10: If UAI mechanism is agreed, the rules for UAI gap skipping shall be left for UE implementation.

# additional skipping schemes

In the last meeting, in addition to the dynamic schemes, semi-static schemes were brought back into discussion even though RAN1 already discussed and excluded them one by one.

Here are the proposals retained in the WF:

## Issue 6-1: Semi-static schemes <Way forward>: - Option 1: In addition to the dynamic indication scheme to deactivate MGs as agreed by RAN1, support semi-static schemes such as RRC and MAC-CE based signaling as well. Send LS to RAN2 and RAN1. - Option 2: RAN4 also suggest RAN1/2 to introduce the semi-static approach (Alt.3) that NW indicate the indicate occasions where to skip gaps/restrictions, and UE can have a stable measurement pattern/period. - Option 3: To guarantee mobility performance, one potential improvement on dynamic indication by DCI is configuring gaps to be skipped during a time period by RRC in advance and gaps indicated by DCI should be subset of gaps configured by RRC - Option 4: Follow RAN1 working assumption and consider DCI only

In our opinion, RAN4 should focus on the dynamic scheme that was agreed in RAN1. Thus, we are strongly support Option 4 from the above list.

Proposal 11: Support Option 4: Follow RAN1 working assumption and consider DCI only.

# Reply LS to RAN1

As we support pursuing RAN1 working assumption considering dynamic scheme using DCI only, we propose to respond strictly on the feasibility of the Alt. 1-1 working assumption and possibly mention that the delay time offset determination work is ongoing.

Proposal 12: Respond to RAN1 LS:

- RAN4 confirms that working assumption in Al. 1-1 is feasible

- The work for the time offset delay X determination is ongoing and further updates will be sent to RAN1 in the future.

# Conclusions

This contribution discussed our analysis related to measurements occasions skipping for NR XR case. Based on the discussion, the following observations and proposals are made:

Observation 1: NR-DC, EN-DC and NE-DC deployments involve different schedulers that may lead to colliding DCI gap skipping orders.

Proposal 1: Deprioritize discussions for NR-DC, EN-DC and NE-DC.

Observation 2: Supporting IRAT gap skipping for 2G and 3G chipsets may affect their measurement timelines.

Proposal 2: Support Option 2 for further discussions:

  - Option 2: Agree at least on the following and further discuss other scenario

    - Intra-frequency

    - Inter-Frequency

    - LTE inter-RAT

    - XR requirement do not apply for UE configured to measure UTRAN FDD for SRVCC or 2G/3G for EN-DC/NE-DC

Proposal 3: Support Option 2: Measurement skipping does not apply for measurements without gaps.

Proposal 4: Support Option 2: Measurement skipping does not apply for L1 measurements and procedures

Proposal 5: Support Option 2: measurement skipping do not apply for L1 measurements and procedures for LTM.

Proposal 6: Support Option 2: As a baseline RAN4 to define requirements due to measurement skipping based on Type 1 measurement gap. Other gaps are FFS.

Proposal 7: Support Option 2:

For the cases where measurement impact is needed

    - As minimum requirements, cell detection, index reading, and measurement period for NR cells is extended.

      - FFS how to extend.

    - Reuse existing measurement accuracy requirements.

  - For the cases where measurement impact is not needed

  - FFS the feasible conditions and necessary UE assistance information.

Proposal 8: Support Option 2: Follow NR-U like approach to extend the delays by the number of MGs skipped during the measurement period.

Proposal 9: Support Option 4: assume subclause 5.3.1 of TS38.214 as a baseline for X offset.

Proposal 10: If UAI mechanism is agreed, the rules for UAI gap skipping shall be left for UE implementation.

Proposal 11: Support Option 4: Follow RAN1 working assumption and consider DCI only.

Proposal 12: Respond to RAN1 LS:

- RAN4 confirms that working assumption in Al. 1-1 is feasible

- The work for the time offset delay X determination is ongoing and further updates will be sent to RAN1 in the future.

# References

- R1-2405736, “LS on UE assistance information”, RAN1 Meeting #117, Fukuoka, Japan, May 2024

- R1-2407512, “LS on gaps/restrictions that are caused by RRM measurements”, RAN1 Meeting #118, Maastricht, NL, August 2024

- R4-2416864 “WF on RRM requirements for XR_Ph3, RAN4 Meeting #112bis, Hefei, CN, October 2024

- TS 38.133, Requirements for support of radio resource management, Rel-18

- TS 38.214, Physical layer procedures for data, v18.3.0, June 2024