3GPP TSG-RAN WG4 Meeting #113				R4-2417607

Orlando, USA, November, 2024

Agenda item:			7.11.3

Source:	Spark NZ

Title:	TP for the Proof of mean EIRP equations and a step wise method of measurement

Document for:	Discussion and Approval

# 1.0 Introduction

The EIRP measurement and compliance can be done either in radians or in degrees. Some RAN4 experts have expressed an understanding that the WRC-23 Res COM 4/7 Annex 1 procedure must be completed in radians and that its computation in degrees is not allowed, however this is not the case.

In fact, the units of angles in azimuth and elevation are given in degrees in the notes to the EIRP compliance table in Res Com 4/7. Furthermore, the units of θL and θH are also clearly given in degrees.

There are two aspects to this contribution:

- A proof that the equations in Annex 1 can be either computed in degrees or in radians and that the answers should be the same via both routes.

- Text proposals for EIRP measurement.

2.0 Proof That In The Evaluation of Mean EIRP Equations The Angles Can Be Either in Radians or in Degrees and Both Give the Same Answer

Take the equation below as a start point:

EIRP = 12πsinθH- sinθL θLθH-ππEIRPθ,φcosθ dφ dθ ……………….(1)

This can be written in a summation form as:

EIRP= 12πsinθH- sinθL n=1n=Nm=1m=MEIRP (θn ,   , ∅m  ) cos(θn    )∆∅∆θ……(2)

EIRP= 12πsinθH- sinθL n=1n=Nm=1m=MEIRP (θn ,   , ∅m  ) cos(θn    ) 2πM (θH  - θLN)…………………..(3)

Assume all the above angles are in radians. Convert to degrees:

Let

t = 180π θ    and

u=  180π ∅

where t and u are in degrees. Therefore,

d∅ = π180 du  and,

dθ= π180 dt

substituting the above in equation (1)

EIRP = 12πsintH- sintL tLtH-180180EIRPπt180,πu180 cos ( πt180)  π180 du  π180 dt ….. (4)

Expressing the above in summation form

EIRP= 12πsintH- sintL  π21802n=1n=Nm=1m=MEIRP (πtn180 ,   ,  πum180) cos (πtn180) 360M  (tH  - tLN) ………(5)

Comparing with equation (3)

EIRP= 12πsintH- sintL  n=1n=Nm=1m=MEIRP (πtn180 ,   ,  πum180) cos (πtn180) 360M π180 (tH  - tLN) π180………(6)

The shaded text is the same as in equation (3) if it is noted that the angles are in degrees.

Therefore, the angles can either be in degrees or in radians and will return the same answer. Specifically:

- If the start point of the angles are in degrees the conversion of the integrals to summation mode should be based on all quantities in degrees

- If the start point of the angles are in radians, the conversion of the integrals to summations mode should be based on all quantities in radians

In both cases the answers are the same. It would be very odd and flawed if that was not the case.

The EIRP compliance table in the WRC Resolution specifically gives the units of all angles in degrees.  There is absolutely no confusion and therefore all measurements may be done in degrees. Furthermore, as the angles involved are small values their conversion to radians and processing such small numbers will result in many rounding errors as the conversion involves multiplying degrees with pi/180 and pi is.an irrational number and the accuracy will depend upon the numbers of decimal digits to use.  Take for example the above the horizon angles in the first bin is 0- 5 degrees. The corresponding radians for the 5 degrees are 5 *pi/180. Even taking the first two digits of pi the corresponding radians value is 0.087222. So what level of accuracy is required?. Why take this route all?

Proposals:

- There is NO difference between the evaluation of mean EIRP equations in integral form either in radians or in degrees.

- All compliance measurements should be completed in degrees. The corresponding angles in radians have very small values and any calculations using these numbers are especially prone to rounding errors.

3.0 Expressing the Mean EIRP integral equations in a Summation Form

In order to do conformance measurements on the EIRP, the integral equations must be converted to summation form. For each horizontal elevation bin:

EIRP per bin per beam= 12πsinθH- sinθL n=1n=Nm=1m=MEIRP (θn ,   , ∅m  ) cos(θn    ) 2πM (θH  - θLN) (7)

Here all angles are in degrees, M and N are the number of intervals for the azimuth and above horizontal angles respectively.

The values of angles θH  and θL are according to the upper and lower limits of the corresponding bin ie 5 and 0 degrees respectively for the first bin.  The above equation is simple and is not cluttered with further indices of each bin that could lend to errors.

The value of cosθn should be computed at the mid point of the nth interval ie at 4.5 degrees for the 4-5 degree interval.  Computing it in the centre of the first bin ie at 2.5 degrees and deemed to apply over the whole bin will not be accurate when the Cos function is rapidly changing at higher values of the above horizontal angles.

Proposals

- The mean EIRP per bin is given by equation (7) where the angles are in degrees and there is no need to write this equation for all bins as it remains the same.

- The value of cosθn should be computed at the mid point of the nth interval in a bin instead of computing over the central angle of the whole bin.

- Step Wise Measurement Procedure.

It is assumed that the angles θ and φ are uniform random variables in degrees within the steering range. It is assumed that test beam locations of all the N beams have been agreed.

Pick a beam with coordinates α1, β1  .

Record values of P(α1, β1)  by using a step size of say 1 degrees  for both the elevation and azimuth angles over the radiation range respectively.  This means that P( α1, β1)  is a  361 x 181 matrix (361 rows and 181 columns). This is shown below in Fig(1). Note the actual number of azimuth locations are 360 and + π  will overlap with – π.

Fig(1) : Storing measured data in a  matrix

Proposal 5 : Step size must be agreed for both domains. Here it is assumed that the step size is equal and for both domains and is 1 degree. Noting that a one-degree step size will only result in 5 points for averaging in a bin of width 5 degrees.

- Repeat the above for all the N test beams ( Note the N here is the number of beams and is different from the N in equation 7).

- Now all the data required to evaluate the mean EIRP is available and rest, of the procedure is analysis of the data. Summing the N matrices and dividing the sum by N as shown in Fig(3)

Fig(2) : Summing and averaging the N beams

- Proposal 6 : The weighting factor of the beams must be agreed. It is 1/N in Fig (3) as it is assumed that all beams are equally likely.

In order to compute the confidence intervals, individual means are required. In this case summing and averaging over the N beams is not required but the procedure given below must be applied to each test beam separately to compute the individual mean.

Computing the summations in equation (7).  The summand is a function of  θ and φ.   It should be summed over the limits of   θ and φ  and the result evaluated between the limits  θL and θH  per bin. There are two alternatives:

Alternative 1: For each bin, compute an average for a given azimuth angle and then average these averages (there are 361 such averages) . Example of bin 1, is shown below:

Fig(3) : computing integrals in eqn (2)- Alternative 1

This is further explained below:

For Bin1, θL =0°, θH = 4° , There are 5 values of Pave(θ)  per azimuth cut. There are 361 azimuth cuts. Sum at all the 361 azimuth cuts:

- θL=0θH=4Pave(θ,φ)φ=-180o + …………+θL=0θH=4Pave(θ,φ)φ=+180o

Each summation contains 5 elements. Scale each sum by Cos (θ)as below, by the center of the angle. Divide the scaled sum by (5 ×361) to get the expected EIRP of the first bin

Scale this again by dividing with  12π(SinθH-SinθL) where  θH  and  θL  bin 1 are 4 and zero degrees respectively.

Alternative 2: The other alternative is averaging over the azimuth and then averaging over the bin as shown below for bin 1:

Fig(4); Computing integrals in step 2, Alternative 2

Each of the cells in a bin are summed over the 361 azimuth cuts. This sum is scaled by Cos (θ)  as shown below. This is similar to Alternative (1). All the five cells in the above are of bin(1) are summed and the sum is divided by (5 ×361)  to  get EIRP of first bin.

Scale this again by dividing the above by   12π(SinθH-SinθL)

Both alternatives will yield the same result.

Proposal 7 :

RAN4 should leave it to individual IMs to choose amongst Alternative 1 or 2

5.0 Text Proposals

## Expressing Mean EIRP equations in summation

In order to do conformance tests the EIRP  the integral equations must be converted to summation form. For each horizontal elevation bin:

EIRP per bin= 12πsinθH- sinθL n=1n=Nm=1m=MEIRP (θn ,   , ∅m  ) cos(θn    ) 2πM (θH  - θLN)…(7)

Here all angles are in degrees, M and N are the number of intervals for the azimuth and above horizontal angles respectively.  The above equation is the same for all bins except for the appropriate values of θH and  θL respectively.

The values of angles θH  and θL are according to the upper and lower limits of the corresponding bin ie 5 and 0 degrees respectively for the first bin.

The value of cosθn should be computed at the midpoint of the nth interval ie at 4.5 degrees for the 4-5 degree interval.  Computing it in the centre of the first bin ie at 2.5 degrees and deemed to apply over the whole bin will not be accurate when the Cos function is rapidly changing at higher values of the above horizontal angles.

## Step Wise Procedure

- It is assumed that the angles θ and φ are uniform random variables in degrees within the steering range.

- It is assumed that test beam locations of all the N beams have been agreed ( Note N here is different from the N in eqn 7).

- Pick  a beam with co ordinates α1, β1  .

- Record values of P( α1, β1)  by using a step size of say 1 degrees  for both the elevation and azimuth angles over the radiation range respectively.  This means that P( α1, β1)  is a  361 x 181 matrix ( 361 rows and 181 columns).This is shown below in Fig(1). Note the actual number of azimuth locations are 360 and + π  will overlap with – π.

Fig(1) : Storing measured data in a  matrix

- Repeat the above for all the N test beams

- Now all the data required to evaluate the mean EIRP is available and rest, of the procedure is analysis of the data. Summing the N matrices and dividing the sum by N as shown in Fig(3)

Fig(2) : Summing and averaging the N beams

- In order to compute the confidence intervals, individual means are required. In this case summing and averaging over the N beams is not required and shown above for completeness . The procedure given below must be applied to each test beam separately to compute the individual mean.

- Computing the summations in eqn (7).  The summand is a function of  θ andφ.   It should be summed over the limits of   θ and φ  and the result evaluated between the limits  θL and θH  per bin. There are two alternatives:

Alternative 1: For each bin, compute an average for a given azimuth angle and then average these averages (there are 361 such averages) . Example of bin 1, is shown below:

Fig(3) : computing integrals in eqn (2)- Alternative 1

This is further explained below:

For Bin1, θL =0°, θH = 4° , There are 5 values of Pave(θ)  per azimuth cut. There are 361 azimuth cuts. Sum at all the 361 azimuth cuts:

- θL=0θH=4Pave(θ,φ)φ=-180o + …………+θL=0θH=4Pave(θ,φ)φ=+180o

Each summation contains 5 elements. Scale each sum by Cos (θ)as below, by the center of the angle. Divide the scaled sum by (5 ×361) to get the expected EIRP of the first bin

Scale this again by dividing with  12π(SinθH-SinθL) where  θH  and  θL  bin 1 are 4 and zero degrees respectively.

Alternative 2: The other alternative is averaging over the azimuth and then averaging over the bin as shown below for bin 1:

Fig(4); Computing integrals in step 2, Alternative 2

Each of the cells in a bin are summed over the 361 azimuth cuts. This sum is scaled by Cos (θ)  as shown below. This is similar to Alternative (1). All the five cells in the above are of bin(1) are summed and the sum is divided by (5 ×361)  to  get EIRP of first bin.

Scale this again by dividing the above by   12π(SinθH-SinθL)

Both alternatives will yield the same result.