3GPP TSG-RAN WG4 Meeting #113	R4-2417612

Orlando, US, 18th – 22th November, 2024

Agenda Item:	7.17.1

Source:	CAICT

Title:	Discussion on general aspects of AI/ML for NR air interface

Document for:	Discussion

1	Introduction

In this contribution, we provide discussion on following issue:

- Issue 1-4: AI/ML performance

2	Discussion

2.1 Issue 1-4: AI/ML performance

In last RAN4 meeting, we had following agreement on this issue.

Agreement (RAN4 #112bis): - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

- For the performance of CSI prediction, two bullets are left for discussion. Regarding to the first one, In our understanding, the relevant feature in specification is non-AI based CSI prediction scheme, a.k.a, TypeII-Doppler-r18 codebook, which has also been taken as baseline in RAN1’s evaluation. The Demod and PMI reporting requirements defined for TypeII-Doppler-r18 codebook could be considered as starting point. This part was discussed in NR_MIMO_evo_DL_UL item. Some agreements are excerpted below for reference.

RAN4 #109 Issue 1-1-1: clarify criteria of feasibility for ‘typeII-Doppler-r18’ codebook Agreement: - Define PMI reporting requirements with ‘typeII-Doppler-r18’ using option 2 if both option 1 and option 2 could be fulfilled. Otherwise, if only option 1 is fulfilled, further discuss if feasible to define PMI reporting requirement using option 1 only. - Option 1: UE throughput with ‘typeII-Doppler-r18’ codebook could outperform Rel-16 Type II codebook with the same CSI-RS configurations and medium/high UE speed. - Option 2: UE throughput with ‘typeII-Doppler-r18’ codebook could outperform random precoding based on Single Panel Type I codebook with the same CSI-RS configurations and medium/high UE speed. Issue 1-1-2: clarify test metric for PMI reporting requirements with ‘typeII-Doppler-r18’ codebook Agreement: - Test metric defined as γ_1=t_(typeII-doppler)/t_rnd as a starting point, where t_(typeII-doppler) is X % (e.g. X=90) of the maximum throughput obtained at SNR_(typeII-doppler) using the typeII-Doppler-r18 precoder configured according to the UE reports, and t_rnd is the throughput measured at SNR_(typeII-doppler) with random precoding based on Type I Single Panel codebook. RAN4 #110bis Issue 1-1-3: X% of the maximum throughput in Test metric Agreement: - 90% of the maximum throughput in Test metric RAN4 #111 Issue 1-1-2: Test requirements of TypeII-Doppler-r18 codebook Agreement: - Set 2.5 as the test requirement for both 2Rx and 4Rx FR1 TDD cases for TypeII-Doppler-r18 codebook. Issue 1-1-3: Test setup for FR1 FDD case of TypeII-Doppler-r18 codebook Agreement: - Define FR1 FDD test requirement for TypeII-Doppler-r18 codebook based on current simulation results with current configuration agreed in RAN4#110bis meeting. Issue 1-1-2: Test requirements of TypeII-Doppler-r18 codebook Agreement: - Set 1.8 as the test requirement for both 2Rx and 4Rx FR1 FDD cases for TypeII-Doppler-r18 codebook.

Regarding to the second FFS, “whether AI/ML based performance should be better than the legacy”, we think this could be decided based on further evaluation in RAN4. The test setup adopted for TypeII-Doppler-r18 codebook could be taken as starting point, shown as following.

RAN4 #110bis Issue 1-1-5: Test setup for FR1 FDD case of TypeII-Doppler-r18 codebook Agreement: - For FR1 FDD 15kHz SCS case, test setup is as below descriptions: - Non-CSI slots: scheduled in mod(i, 10) = {1,3,5,7,8,9}, i={0,1,2,…,19} - CSI-RS is scheduled in mod(i, 10) = {0,2,4,6}, i={0,1,2,…,19} - SSB slots: i=0 - TRS slots: i=10 and 11. - CSI report is transmitted in slot#10n+10 to estimate precoder in slot#10n+11 (i.e., δ=1) - Test equipment apply the estimated precoder in slot#10n+{15, 17, 18, 19, 21, 23} Issue 1-1-6: Test setup for FR1 TDD case of TypeII-Doppler-r18 codebook Agreement: - For FR1 TDD 30kHz SCS case, test setup is as below descriptions for the simulation of following Rel18 Doppler codebook vs random Type I single panel codebook: - Scheduling pattern for TDD SCS=30kHz with the periodicity of 20ms (40 slots): - Non-CSI slots: scheduled in mod(i, 10) = {5,6}, i={0,1,2,…,39} - CSI slots: scheduled in mod(i, 10) = {0,1,2,3}, i={0,1,2,…,39} - SSB slots: i=0 - TRS slots: i=20 and 21. - CSI report is transmitted in slot#10n+9 to estimate precoder in slot#10n+10 (i.e., δ=1) - Test equipment apply the estimated precoder in slot#10n+{15, 16}

Proposal 1: The Demod and PMI reporting requirements defined for TypeII-Doppler-r18 codebook could be considered as starting point for AI-based CSI prediction.

Proposal 2: Whether to define better requirements for AI-based CSI prediction shall be decided based on further evaluations. The test setup adopted for TypeII-Doppler-r18 codebook could be taken as starting point.

- To address the performance requirements for AI/ML based beam management, we understand that there are two separate aspects shall be considered. First, regarding to the accuracy requirements, whether it should be better than or no worse than the legacy depends on the results of subsequent evaluations (impact of measurement error on prediction accuracy).

Second, regarding to signaling, procedures and delay requirements related to beam measurements, we need to first identify which specific requirements are impacted by the introduction of beam prediction behavior (related discussion please refer to our companion contribution [1]). Subsequently, we can explore whether the existing legacy requirements can still be satisfied with reduced measurement resources, or if the legacy requirements can be improved upon maintaining the same resource overhead for beam measurements.

Proposal 3: For AI/ML based beam management, comparison to legacy requirements could be separately discussed for accuracy requirements and signaling/procedure/delay requirements.

Proposal 4: RAN4 to identify the impact of reduced resource configuration (only for beams in Set B) on existing RAN4 requirements with L1-RSRP related measurements.

Conclusion

In this contribution, the issue of AI/ML performance is discussed with following proposals:

Proposal 1: The Demod and PMI reporting requirements defined for TypeII-Doppler-r18 codebook could be considered as starting point for AI-based CSI prediction.

Proposal 2: Whether to define better requirements for AI-based CSI prediction shall be decided based on further evaluations. The test setup adopted for TypeII-Doppler-r18 codebook could be taken as starting point.

Proposal 3: For AI/ML based beam management, comparison to legacy requirements could be separately discussed for accuracy requirements and signaling/procedure/delay requirements.

Proposal 4: RAN4 to identify the impact of reduced resource configuration (only for beams in Set B) on existing RAN4 requirements with L1-RSRP related measurements.

Reference

[1] R4-24xxxxx, Discussion on RRM core requirements for beam management, CAICT