# 3GPP TSG-RAN WG4 Meeting #113	R4-2417620

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.17.4.2

Source: 	OPPO

Title: 	Testability and interoperability issues for beam management

Document for:	Approval

- Discussion

Regarding the testability issues on BM, from our understanding, following aspects need to be considered.

- The number of FR2 beams that RAN4 tests can support remains unclear. It is necessary to clarify the limitations regarding the FR2 beams or probes that TE vendors could support. Based on this limitation(if any), RAN4 can further consider how to reach a consensus on designing/conducting tests for BM use cases and how to set/form beam set A and B of different sizes for different BM cases.

- A potential test setup could be (as shown in Figure 1)

- Assuming that TE supports X probes in a chamber

- By utilizing X probes, X different angles of arrival (AOA) can be achieved and can be mapped to a TX beam transmission with X paths(clusters)

- Different TX beams in BM set A or set B can be represented by setting different pathloss for the different AOA paths, to mimic the beam sweeping procedure

- Maximum number of set B Tx beams that test system should be able to emulate: 8

- Maximum number of set A Tx beams that test system should be able to emulate:  64

- RAN4#111 agreed “maximum number of set B Tx beams that test system should be able to emulate: [8-16]” and “maximum number of set A Tx beams that test system should be able to emulate:  [64-128]”

- In R18 TR 38.843, BS Antenna Configuration and UE Antenna Configuration are defined as below,

| Parameter | Value |
| --- | --- |
| BS Antenna Configuration | Number of BS beams: 32 or 64 downlink Tx beams (max number of available beams) at NW side. Other values, e.g., 256 not precluded. |
| UE Antenna Configuration | Number of UE beams: 4 or 8 downlink Rx beams (max number of available beams) per UE panel at UE side. Other values, e.g., 16 not precluded. |

- According to above agreements in both RAN4 and R18 TR, we can choose 8 beam for set B and 64 						beam for set A in RAN4 tests.

- FFS the number of probes that could be supported by TE vendors

- FFS the number of AoAs that RAN4 needs to represent a beam

- FFS how to map the X AoAs and corresponding X pathloss to different beams in BM set A/B

Figure 1 A potential test setup for BM tests

- Besides, it's also crucial to further consider that, as a data/scenario-driven solution, the chamber and beam patterns provided by different TEs may impact the test results of BM (e.g., due to variations in beam settings, different considerations on multipath, and other implementations). Besides, even for a same TE, different DUT hardware implementations may also lead to variations in receiving behavior and beam measurement results. Attention must be paid to ensure that the BM model constructed on the DUT side(e.g. by corresponding training dataset) can match(or approximate match) and be utilized in the chamber environment on the TE side. Otherwise, it becomes challenging for a DUT to pass the test under unknown chamber conditions/scenarios. This may require a close collaboration between DUT and TE to ensure that the model under test is compatible with the testing environment.

In RAN4 #112 meeting, it has been agreed to further discuss and down-select the following options considering TE feasibility：

- TDL

- Simplified CDL: CDL channel model with a small number of clusters. CDL simplification needs to be further discussed

Following issues should be taken into account when identifying the BM test settings:

- to simulate differences in beam measurements caused by beam sweeping in FR2 within the chamber

- to model correlation between the measurements of different beams to ensure that the beams remain predictable

- the predictability of the beams must not be too strong, as an extreme case where there is no distinction between different beams would undermine the necessity for BM testing

- the test should not be overly complex and should optimally utilize the existing test environment.

Taking these issues into account, based on our understanding, simplified CDL should be used as the channel assumption.

Proposal 1: For BM testability, RAN4 need to:

- Clarify the limitations regarding the FR2 beams or probes that TE vendors could support

- To determine the test setup for BM, a potential approach could be:

- Assuming that TE supports X probes in a chamber

- By utilizing X probes, X different angles of arrival (AOA) can be achieved and can be mapped to a TX beam transmission with X paths(clusters)

- Different TX beams in BM set A or set B can be represented by setting different pathloss for the different AOA paths, to mimic the beam sweeping procedure

- FFS the number of probes that could be supported by TE vendors

- FFS the number of AoAs that RAN4 needs to represent a beam

- FFS how to map the X AoAs and corresponding X pathloss to different beams in BM set A/B

Proposal 2:

- Maximum number of set B Tx beams that test system should be able to emulate: 8

- Maximum number of set A Tx beams that test system should be able to emulate:  64

Proposal 3: Further consider how to ensure that the BM model constructed on the DUT side can match(or approximate match)  and be utilized in the testing environment on the TE side.

Proposal 4: The simplified CDL should be used as the channel assumption.

- Conclusions

In this contribution, we have discussed the testability and interoperability issue on AI/ML based BM and got following proposals:

Proposal 1: For BM testability, RAN4 need to:

- Clarify the limitations regarding the FR2 beams or probes that TE vendors could support

- To determine the test setup for BM, a potential approach could be:

- Assuming that TE supports X probes in a chamber

- By utilizing X probes, X different angles of arrival (AOA) can be achieved and can be mapped to a TX beam transmission with X paths(clusters)

- Different TX beams in BM set A or set B can be represented by setting different pathloss for the different AOA paths, to mimic the beam sweeping procedure

- FFS the number of probes that could be supported by TE vendors

- FFS the number of AoAs that RAN4 needs to represent a beam

- FFS how to map the X AoAs and corresponding X pathloss to different beams in BM set A/B

Proposal 2:

- Maximum number of set B Tx beams that test system should be able to emulate: 8

- Maximum number of set A Tx beams that test system should be able to emulate:  64

Proposal 3: Further consider how to ensure that the BM model constructed on the DUT side can match(or approximate match)  and be utilized in the testing environment on the TE side.

Proposal 4: The simplified CDL should be used as the channel assumption.

- References

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface