3GPP TSG-RAN WG4 Meeting #113										   R4-2417624

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.8.3.4

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for less than 5MHz over NTN

WID/SID:        NR_IoT_NTN_req_test_enh

Document for:	Discussion

# 1 Introduction

In RAN#104 meeting, the “Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN” has been approved in [1]. This paper presents our perspective on RRM requirement, focusing on the unresolved issues identified in RAN4#112-bis WF [1].

# 2 Impact from Less than 5MHz for NR NTN on RRM core requirements

The RRM requirements specified in R18 TN NR_FR1_lessthan_5MHz_BW can be leveraged to update the requirements for FR1 NR NTN for less than 5MHz. The potential impact on RRM requirement is analyzed below, with considerations that it introduces smaller BW for <5MHz (e.g. 12/15/20 PRBs PDCCH transmission parameters), and it only impacts on SSB/PBCH-related procedures while CSI-RS has not impacted.

| The potential RRM impact from “less than 5MHz” | Agreements | Discussion & Proposals |
| --- | --- | --- |
| IDLE/INACTIVE | Agreement in 2024.08 No core part impact. | n/a. |
| CONN Mobility (Handover) & CONN Control | Agreement in 2024.10 T∆ in HO requirement for NTN in exiting TS38.133 clause 6.1C.1.2. shall be extended to [3 Trs] | According to the current TS 38.133, HO requirements for 12 PRB SSB BW, it can confirm T∆ = 3*Trs. 6.1.1.2 NR FR1 - NR FR1 Handover 6.1.1.2.2 Interruption time …. T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW. Proposal 1: T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW. |
| Timing | Agreement in 2024.10 No core part impact. | n/a. |
| Signaling characteristics (RLM) (CONN), BFD/CBD | Agreement in 2024.08 Prioritize SSB based RLM and L1 measurement for NTN less than 5MHz Agreements in 2024.10 The hypothetical PDCCH transmission parameter for RLM/BFD in R18 for TN less than 5MHz can also be reused, i.e., the one defined for 3MHz CBW & 15 PRBs. | n/a. |
| Meas. Procedure in CONN | Agreements Define new requirements for time index detection is needed. Agreements/Wayforward The number of SSB samples for Rel-18 NTN measurement requirements (Tidentify_xxx_with_index) will be extended by X and Y more samples for intra-frequency and inter-frequency cases respectively. For X and Y, the values from TN less than 5MHz as starting point. | The distinction between without and with an index is that the latter requires the additional consideration of TSSB_time_index_intra/inter (as exemplified in section 9.2.5.1 as below), which has been extended for <5MHz by 4 samples (resulting in a total of 7) for intra-frequency measurement and 3 samples (resulting in a total of 6) for inter-frequency measurement. These adjustments are detailed in the Appendix. #### 9.2.5.1 Intrafrequency cell identification Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra) ms Tidentify_intra_with_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra + TSSB_time_index_intra) ms Proposal 2: For NTN <5MHz, the number of SSB samples for Rel-18 NTN time index detection requirements will be extended by X=4 and Y=3 more samples for intra-frequency and inter-frequency cases respectively. |
| SSB L1-RSRP | Agreements in 2024.10 For L1-RSRP measurement based on SSB there is no RRM impacts. | n/a. |

# 3 Conclusion

Proposal 1: T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.

Proposal 2: For NTN <5MHz, the number of SSB samples for Rel-18 NTN time index detection requirements will be extended by X=4 and Y=3 more samples for intra-frequency and inter-frequency cases respectively.

# References

- R4-2416851, WF on RRM requirements for [112bis][207] NR_IoT_NTN_req_test_enh, Oct. 2024.

# Appendix

// intra-frequency

Table 9.2.5.1-23: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, ceil(7 x Kp) x SMTC period) |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7 x Kp) x max(SMTC period,DRX cycle)) |
| DRX cycle>320ms | ceil(7 x Kp) x DRX cycle |
| NOTE 1: FFS When highSpeedMeasInterFreq-r17 |  |

Table 9.2.6.1-12: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1) (Frequency range FR1)

| DRX cycle | TSSB_time_index_intra_less_than_5Mhz |
| --- | --- |
| No DRX | max(120ms, 7 x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320ms | max(120ms, ceil(1.5 x 7) x max(MGRP, SMTC period,DRX cycle) x CSSFintra) |
| DRX cycle>320ms | 7 x max(MGRP, DRX cycle) x CSSFintra |
| NOTE 1: Void NOTE 2: FFS When highSpeedMeasInterFreq-r17 |  |

// inter-frequency

Table 9.3.4-11: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (Frequency range FR1)

| Condition NOTE1 | TSSB_time_index_inter_less_than_5Mhz |
| --- | --- |
| No DRX | Max(120ms, Ceil(6 * Kgap) Max(MGRP , SMTC period)) CSSFinter |
| DRX cycle ≤ 320ms | Max(120ms, Ceil(6 1.5 * Kgap) Max(MGRP, SMTC period, DRX cycle)) CSSFinter |
| DRX cycle > 320ms | Ceil(6* Kgap) DRX cycle CSSFinter |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 NOTE 2: FFS When highSpeedMeasInterFreq-r17 |  |

Table 9.3.9.1-5: Time period for time index detection for a UE operating on a target cell with 12 PRB SSB (FR1)

| DRX cycle | TSSB_time_index_inter_less_than_5MHz |
| --- | --- |
| No DRX | max(120ms, ceil( 6 x Kp ) x SMTC period)Note 1 x CSSFinter |
| DRX cycle≤ 320ms | max(120ms, ceil (6 x Kp) x max(SMTC period,DRX cycle)) x CSSFinter |
| DRX cycle>320ms | Ceil( 6 x Kp) x DRX cycle x CSSFinter |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified NOTE 2: FFS When highSpeedMeasInterFreq-r17 |  |