3GPP TSG-RAN WG4 Meeting #113											R4-2417626

Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.22.2.2

Source:	MediaTek Inc.

Title:	Discussion on RRM requirements for adaptation of common signal/channel transmission

WID/SID:        Netw_Energy_NR_enh-Core

Document for:	Discussion

# 1  Introduction

In RAN#104 meeting, the “Revised WID: Enhancements of network energy savings for NR” has been approved in [1].

The RRM requirement for adaptation of common signal/channel transmission was discussed in WF [3].

# 2 Time domain adaptation of L1/L3 SSB measurement in CONN

By following the WF [3] as below, the RRM requirements for L1/L3 SSB periodicity adaptation in CONNECTED mode will be discussed.

Issue 1-1-1: Scenario clarification based on RAN1 progress

Agreement:

  - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion:

    - CONNECTED mode, and

    - SSB adaptation for SCell, and

    - SSB adaptation for NCD-SSB, and

    - SSB periodicity adaptation in time-domain

Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism)

Issue 1-2-1: Requirements need updating

Agreement:

  - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption:

    - L1-RSRP for serving cell

    - L1-SINR

    - BFD

    - CBD

    - SCell activation

    - TCI state switching

    - NR intra-frequency L3 measurement

    - NR inter-frequency L3 measurement

Note: Above list could be further updated based on RAN4 discussion.

The concern scenario is that the transitions measurement behavior when the SSB is updated from the longer periodicity to the shorter periodicity. However, it can be analogous the existing minimum requirement at transitions, as specified in 9.1.6, as copied below, where when the UE transitions when DRX changes / CSSF updates, the cell identification and measurement period requirements apply based on the longer delay before or after the transition.

### 9.1.6	Minimum requirement at transitions

When the measurement on one intra-frequency measurement object transitions from measurements performed outside gaps to measurements performed within gaps or vice versa during one measurement period, the cell identification and measurement period requirements with the longer delay apply.

When the measurement on one intra-frequency measurement object transitions from measurements performed outside gaps to measurements performed within NCSG or vice versa during one measurement period, the cell identification and measurement period requirements with the longer delay apply.

The carrier-specific scaling factor specified in clause 9.1.5 that applies to the other impacted measurement objects will also apply based on the longer measurement or cell identification delay before or after the transition.

When the UE transitions between DRX and non-DRX or when DRX cycle periodicity changes, the cell identification and measurement period requirements apply based on the longer delay before or after the transition.

Subsequent to this measurement period, the cell identification and measurement period requirements on each measurement object are corresponding to the second mode after transition.

For UE supporting ncd-SSB-BWP-Wor-r18, when the intra-frequency measurement transitions from measurements performed on CD-SSB to measurements performed on NCD-SSB or vice versa due to BWP switching or SCG activation/deactivation during one cell identification period or measurement period, the UE shall use a cell identification period that is the maximum of the cell identification periods or measurement periods corresponding to the first SSB type and the second SSB type after the BWP switching or SCG activation/deactivation.

Subsequent to the above-mentioned measurement periods and/or cell identification periods, the cell identification and measurement period requirements on each measurement object are based on the SSB type after the BWP switch or SCG activation/deactivation.

Besides, for L3 measurements, which is performed based on MO and SMTCs, therefore it is probable that the MO and SMTC will be reconfigured accordingly, and it appears not necessary to define new requirements if it has been reconfigured.

Proposal 1: When the UE transitions when SSB periodicity changes, the L1/L3 measurement requirements apply based on the longer delay before or after the transition.

# Conclusion

Proposal 1: When the UE transitions when SSB periodicity changes, the L1/L3 measurement requirements apply based on the longer delay before or after the transition.

# References

- RP-241650, Revised WID: Enhancements of network energy savings for NR

- Chair notes RAN1_117 eom1

- R4-2413885, WF on Netw_Energy_NR_enh, Ericsson, Aug. 2024.

- R4-2416860, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei, Oct. 2024.