3GPP TSG-RAN WG4 Meeting #113	R4-2417631

Orlando, FL, US, Nov. 2024

Source	:	Qualcomm Incorporated

Title	:	On test method for LPWUS

Agenda item	:	7.23.2.4

Document for	:	Approval

# 1.	Introduction

In this contribution we first share our views on testing LPWUS. There is no recent precedent in UE RF RAN4 for detection-based sensitivity, legacy requirements are largely BLER based. While testing for successful detection, it is important to discriminate the test outcome between false detection and intended detection.

# 2. 	Discussion

## 2.1	Test method pre-requisite

In the case of the LPWUS detector, the core requirement criterion is agreed to be a 1% MDR (missed detection rate). While there is no explicit core requirement on FAR (false alarm rate), it is necessary to ensure that any test method never counts a false alarm as a successful detection. A false alarm can be generated by either a noisy LPWUR, or by mis-detecting a sequence not intended for the UE. It is therefore necessary to test with 3 different stimuli: intended WUS sequence, WUS sequence not intended for UE under test, and missing WUS during monitoring occasion.

Proposal 1: The LPWUS core requirement test method must ensure that the detection outcomes of the UE under test are separately counted for 3 different stimuli: intended WUS sequence, WUS sequence not intended for UE under test, and missing WUS during monitoring occasion.

A robust way to ensure this level of discrimination is to just rely on the natural process of waking up the receiver to schedule a message in UL that conveys an acknowledgement. This way the TE is in control of maintaining the count for each of the 3 bins identified above.

## 2.2	Test mode

As the requirement evolves, test duration and other challenges may identify the need for test mode for verification. Such a determination should come from RAN5, not RAN4. Not only is such determination in their purview, RAN4 are not well suited to identifying all the detail requirements of any future test mode. RAN4 could instead communicate core requirement details and surrounding RAN4 assumptions with RAN5 via LS.

Proposal 2: Discussion on whether to define and specify a test mode is left to RAN5. RAN4 side condition assumptions can be communicated to RAN5 via LS if needed.

# 3.	Conclusion

Proposal 1: The LPWUS core requirement test method must ensure that the detection outcomes of the UE under test are separately counted for 3 different stimuli: intended WUS sequence, WUS sequence not intended for UE under test, and missing WUS during monitoring occasion.

Proposal 2: Discussion on whether to define and specify a test mode is left to RAN5. RAN4 side condition assumptions can be communicated to RAN5 via LS if needed.

# 4.	References

[1] 	R4-2408032, ‘On LPWUR system considerations’, Qualcomm, RAN4#111, May 2024

[2] 	R4-2417112, ‘WF on UE RF requirements for LP-WUS’, vivo, RAN4#112-Bis, Oct. 2024