3GPP RAN WG4 Meeting #113	R4-2417695

Orlando, US, November 18th – November 22nd, 2024

Title: 	Discussion on Performance requirements of CPP

Source: 	CATT

Agenda item:	5.22.2.6

Document for:	Discussion

Introduction

In RAN4#112bis meeting, fruitful progress regarding the performance requirements was achieved which was captured in the WF [1]. And there are only one issue left which needs further discussion. In this contribution, we will discuss it and present our views.

Discussion

In last meeting, it was agreed to remove [] in CPP accuracy requirements for BWs larger than 24 PRBs and further discuss how to handle accuracy requirements for BW equal to 24 PRBs and those set as TBD in the accuracy tables [2].

First, it is stated in RRM spec that the measurement accuracy requirements for both DL RSCPD and DL RSCP are defined based on single measurement sample and same set of symbols, and considering the purpose of CPP is to achieve higher positioning accuracy, the scenario using small bandwidth, i.e., 24 PRBs, for positioning is not a general case, therefore the corresponding accuracy requirements can be removed, which also can mitigate the impacts of carrier frequency offset (CFO) on positioning accuracy and avoid the discussion on margin values due to CFO for accuracy requirements corresponding to 24 PRBs.

After removing accuracy requirements for 24 PRBs, a new issue is there will not be any accuracy requirements for SCS 60 kHz in FR1. Our view is to define corresponding accuracy requirements based on simulation results submitted in RAN4#113 meeting, considering companies are encouraged to submit results in WF [1] and actually some results are already captured in simulation summary [3]. To facilitate the completion of CPP performance requirements, we did some simulations for the PRS configurations for which the accuracy requirements are set as TBD, and the results are captured in Annex A.

Based on above discussions, we have the following proposal for CPP accuracy requirements

Proposal 1: RAN4 to remove the CPP accuracy requirements for 24 PRBs and define the accuracy requirements which are set as TBDs.

A CR is also submitted to reflect corresponding changes [4].

Conclusions

This paper discussed the performance requirements for carrier phase positioning and provided the following proposal:

Proposal 1: RAN4 to remove the CPP accuracy requirements for 24 PRBs and define the accuracy requirements which are set as TBDs.

References

[1] R4-2416870, WF on RRM requirements for NR_pos_enh2_Part2, CATT, RAN4#112bis.

[2] R4-2416892, Draft big CR to 38133 for RRM performance part for expanded and improved NR positioning, Ericsson, RAN4#112bis.

[3] R4-2416906, Updated summary of simulation results for CPP accuracy requirements, CATT, RAN4#112bis.

[4] R4-2417693, Corrections to performance parts for CPP, CATT, RAN4#113.

- Annex A: CPP simulation results

In this appendix, we provide simulation results for the PRS configurations for which the accuracy requirements are set as TBD in the performance bigCR [2].

Table A.1 Simulation results for DL RSCPD

| FR1 | SCS (kHz) | BW (PRBs) | (-6, 13) | (-3, -6) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | AWGN | AWGN | Two-tap |
|  |  |  | max(5%, 95%) |  |  |
|  | 30 | 132 | 5.19 | 1.84 | 1.83 |
|  | 60 | 64 | 7.35 | 2.48 | 2.45 |
|  |  | 132 | 5.09 | 1.74 | 1.73 |
| FR2 | 120 | 128 | 5.56 | 1.79 | 1.79 |

Table A.2 Simulation results for DL RSCP

| FR1 | SCS (kHz) | BW (PRBs) | (-3, 13) | (0, -6) |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | AWGN | AWGN | Two-tap |
|  |  |  | max(5%, 95%) |  |  |
|  | 30 | 132 | 4.65 | 1.04 | 1.03 |
|  | 60 | 64 | 6.43 | 1.41 | 1.39 |
|  |  | 132 | 4.41 | 1 | 0.99 |
| FR2 | 120 | 128 | 4.75 | 1.04 | 1.04 |