3GPP RAN WG4 Meeting #113	R4-2417696

Orlando, US, November 18th – November 22nd, 2024

Title: 	Discussion on UE to UE CLI handling

Source: 	CATT

Agenda item:	7.20.3.1

Document for:	Discussion

Introduction

In RAN4#112bis meeting, RAN4 continued the discussions on the UE to UE CLI requirements of R19 SBFD WI. Some agreements were achieved and captured in way forward [1]. In this contribution, we will further discuss the UE to UE CLI handling.

Discussion

Measurement methods

Regarding measurement methods, some proposals were brought up and a recommended way forward was provided:

RAN4#112bis: - Recommended WF - RAN4 shall specify L1 CLI measurement requirements at least for below methods: - Method#1: UE measures RSSI within DL subband - Method#2: UE measures RSRP of aggressor UE within UL subband - Additional methods can be considered based on further RAN1 agreements. - Differentiation on requirements based on different methods should be considered, based on further RAN1 conclusions.

First, it is reasonable to specify L1 CLI measurement requirements for Method#1 and Method#2 which correspond to L1 CLI-RSSI measurement requirements and L1 CLI SRS-RSRP measurement requirements respectively. In regards to other measurement methods, there is no further consensus in RAN1 meeting. Therefore RAN4 can focus on defining requirements for both methods and discussions on requirements for new methods can be triggered if additional methods are agreed in RAN1.

On defining L1 CLI-RSSI measurement requirements and L1 CLI SRS-RSRP measurement requirements, RAN4 has agreed the simulation assumption [2] in last RAN4 meeting, which facilitates the work of defining L1 CLI SRS-RSRP measurement requirements. But, no discussion on defining L1 CLI-RSSI measurement requirements is initiated. In RAN1#118bis meeting, it was agreed to support that a single wideband RSSI measurement report for one CLI-RSSI measurement resource across two DL subbands [3].

Agreement For Method#1 (RSSI measurement within DL subband), the frequency resources of CLI-RSSI measurement resource type#2 (One CLI-RSSI measurement resource across two DL subbands) is derived by excluding the frequency resources outside DL usable PRBs. - A single wideband RSSI measurement report is supported - FFS: two per-DL-subband CLI-RSSI measurements reports with wideband report in each DL-subband.

Considering the legacy CLI-RSSI measurements are performed on the total configured bandwidth, while L1 CLI-RSSI measurements will be performed on two discontinuous bandwidths and the frequency resources outside DL usable PRBs should be excluded, RAN4 needs to evaluate whether the legacy CLI-RSSI measurement accuracy requirements are still applicable for L1 CLI-RSSI measurements.

Proposal 1: RAN4 to evaluate whether the legacy CLI-RSSI measurement accuracy requirements are applicable for L1 CLI-RSSI measurements, when one CLI-RSSI resource occupies two discontinuous DL subbands.

Rx beam

In regards to Rx beam, there was some progress in RAN1, which is very helpful to RAN4 discussion.

Agreement For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH, support the following in the IE CSI-AperiodicTriggerStateList: - In CSI-AssociatedReportConfigInfo a list of TCI state(s) with qcl-Type set to 'typeD' is optionally configured where the TCI state(s) correspond to the resource(s) in the set of CLI measurement resources indicated by CSI-AssociatedReportConfigInfo - The list of TCI states can be configured (present) only if unifiedTCI-StateType is configured and the CLI measurement resource(s) are aperiodic - If the list of TCI states is not configured (absent), the UE assumes the following for each CLI measurement resource: - If unifiedTCI-StateType is not configured - Same assumption as for L3-based CLI measurement as in 38.133: “For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET” - If unifiedTCI-StateType is configured - “Indicated” DL only/joint TCI state as specified in TS 38.214 - Note: Above default rules apply to aperiodic reporting based on aperiodic, semi-persistent, and periodic CLI measurement resources FFS: details of configuration of TCI state(s) for aperiodic reporting based on periodic and semi-persistent CLI measurement resources

According to RAN1’s agreement, the Rx beam assumptions for L1 CLI measurements will be quite clear for different configurations:

- UE follows the ‘typeD’ TCI state(s) configured in CSI-AssociatedReportConfigInfo for L1 CLI measurements, if it is configured.

- If ‘typeD’ TCI state(s) is not configured but unifiedTCI-StateType is configured, UE follows the rules specified in TS 38.214.

- Otherwise, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET.

Proposal 2: For performing L1 CLI measurements in FR2,

- UE follows the ‘typeD’ TCI state(s) configured in CSI-AssociatedReportConfigInfo for L1 CLI measurements, if it is configured.

- If ‘typeD’ TCI state(s) is not configured but unifiedTCI-StateType is configured, UE follows the TCI state indication rules specified in TS 38.214.

- Otherwise, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET.

Measurement period

A main issue for defining L1 CLI SRS-RSRP measurement period is to select the value of measurement sample. In our opinion, RAN4 needs to consider the NW configuration as well. In CSI-ReportConfig, network may configure the IE, timeRestrictionForChannelMeasurements, to restrict the measurement duration. As defined in L1-RSRP measurement requirements, number of measurement samples can be 1 if the mentioned IE is configured as ‘configured’ or it equals to 3. This description can be reused for L1 CLI SRS-RSRP.

Proposal 3: The number of measurement samples equals to 1, if timeRestrictionForChannelMeasurement is ‘configured’. Otherwise it equals to 3.

Side condition

Lots of effects were spent on discussion on side conditions in last meeting and to better understanding the impacts of each side condition, e.g., timing offset, SRS Es/Iot, on measurement requirements, a simulation assumption was agreed [2]. Based on the assumptions, we did some simulation and the results are provided in Annex A. As we discussed the number of measurement samples in section 2.3, both 1 and 3 can be considered and the specific value depends on NW configuration. For SRS bandwidth, the legacy value in R16 CLI measurement requirements is 48 PRBs. However, this value may not be reasonable for some UL subband configurations. Based on RF agreements, minimum UE bandwidth in FR1 is 40M, i.e., 51 PRBs with SCS 60 kHz, for which the 48 PRBs of UL subband assumption is not applicable any more. Similar situations may exist in FR2 as well. At the same time, RF group is discussing the restrictions on subband size of DL/UL subbands. Hence, RAN4 RRM can use 24 PRBs as a baseline and wait for more progress in RF group. The baseline can be revisited if new agreements in RAN1/RF are made. Regarding the number of SRS symbols and SRS Es/Iot, RAN4 can follow the R16 assumptions.

Proposal 4: RAN4 to use the side conditions below for L1 CLI SRS-RSRP measurement requirements:

- Measurement sample: 1 and 3.

- SRS BW: 24 PRBs as baseline and wait for more progress in RF group.

- Number of SRS symbols: 1.

- SRS Es/Iot = 1 dB.

Measurement reporting

Only aperiodic reporting is supported till now and no new reporting type is supported by RAN1. Therefore RAN4 can focus on aperiodic reporting and, in our understanding, L1-RSRP measurement reporting requirements can be reused as baseline.

Proposal 5: L1-RSRP measurement reporting requirements can be reused as baseline for CLI SRS-RSRP aperiodic reporting.

Scheduling restriction

Many proposals were brought up and a recommended way forward was summarized:

- Recommended WF - RAN4 to define scheduling and measurement restriction for L1-SRS-RSRP and L1-CLI-RSSI measurement. Details are FFS and following aspects are to be considered: - Measurement timing including timing offset between DL timing and SRS arrival timing - Rx beam assumption - Further RAN1 conclusions are to be considered.

Both aspects that can be considered are under discussions in other issues and their impacts can be considered later. In regards to the framework of scheduling restrictions, in our opinion, the R16 CLI related requirements can be used as baseline. For measurement restriction requirements, no related requirements are defined for R16 CLI measurements. Whether to define them for R19 L1 CLI measurement needs further discussions.

Proposal 6: R16 CLI scheduling restriction requirements can be used as baseline and further discuss the whether to define requirements for measurement restrictions.

Report mapping

In our understanding, there is no huge performance difference between R16 CLI and R19 CLI. Therefore the range of R16 report mapping requirements probably can cover R19 CLI measurement results. Therefore RAN4 can reuse the legacy mapping tables. Some proposals suggested not discussing this issue unless it is triggered by RAN1 and the reason is UCI under RAN1 discussion may affect the reporting range. However, no related RAN1 agreements are observed till RAN1 October meeting. Hence, RAN4 can assume that there is no impact from other WGs on reporting range and the R16 reporting mapping table can be reused. This assumption can be revisited if impacts are observed based on agreements in future meetings, if any.

Proposal 7: RAN4 to reuse the R16 report mapping tables for R19 CLI measurements.

Requirement applicability

Some proposals discussed the requirement applicability and they are reproduced below for reference:

- Proposals - Proposal 1 (LGE): - L1 based UE-to-UE CLI measurement for SBFD are only applicable for RRC_CONNECTED intra-frequency: - SRS-RSRP measurement resource is fully confined within BW of UL sub-band - CLI-RSSI measurement resource is configured within DL sub-band - Proposal 2 (QC): - The L1-CLI-RSSI and L1-SRS-RSRP measurement resources must be confined to the DL active BWP of the UE.

Based on our understanding on RAN1 definitions of L1 CLI-RSSI and L1 CLI SRS-RSRP measurements, we think the corresponding requirements should apply for RRC_CONNECTED intra-frequency when L1 SRS-RSRP measurement resources are confined in UL subband and L1 CLI-RSSI measurement resources are confined in DL subband(s).

Proposal 8: L1 CLI measurement requirements apply for RRC_CONNECTED intra-frequency when L1 SRS-RSRP measurement resources are confined in UL subband and L1 CLI-RSSI measurement resources are confined in DL subband(s).

Conclusions

This contribution discussed the impacts of L1 CLI on RRM requirements and some proposals are provided:

Proposal 1: RAN4 to evaluate whether the legacy CLI-RSSI measurement accuracy requirements are applicable for L1 CLI-RSSI measurements, when one CLI-RSSI resource occupies two discontinuous DL subbands.

Proposal 2: For performing L1 CLI measurements in FR2,

- UE follows the ‘typeD’ TCI state(s) configured in CSI-AssociatedReportConfigInfo for L1 CLI measurements, if it is configured.

- If ‘typeD’ TCI state(s) is not configured but unifiedTCI-StateType is configured, UE follows the TCI state indication rules specified in TS 38.214.

- Otherwise, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET.

Proposal 3: The number of measurement samples equals to 1, if timeRestrictionForChannelMeasurement is ‘configured’. Otherwise it equals to 3.

Proposal 4: RAN4 to use the side conditions below for L1 CLI SRS-RSRP measurement requirements:

- Measurement sample: 1 and 3.

- SRS BW: 24 PRBs as baseline and wait for more progress in RF group.

- Number of SRS symbols: 1.

- SRS Es/Iot = 1 dB.

Proposal 5: L1-RSRP measurement reporting requirements can be reused as baseline for CLI SRS-RSRP aperiodic reporting.

Proposal 6: R16 CLI scheduling restriction requirements can be used as baseline and further discuss the whether to define requirements for measurement restrictions.

Proposal 7: RAN4 to reuse the R16 report mapping tables for R19 CLI measurements.

Proposal 8: L1 CLI measurement requirements apply for RRC_CONNECTED intra-frequency when L1 SRS-RSRP measurement resources are confined in UL subband and L1 CLI-RSSI measurement resources are confined in DL subband(s).

References

[1] R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, RAN4#112bis.

[2] R4-2416910, Simulation assumption for evaluate L1-SRS-RSRP measurement requirements, Ericsson, RAN4#112bis.

[3] Chair notes RAN1#118bis, RAN1#118bis.

- Annex A: Simulation results for CLI SRS-RSRP

In this section, the simulation results for CLI SRS-RSRP based on simulation assumption [2] are summarized. Results for all combinations are provided except for those under the fading channel.

Table A.1 CLI SRS-RSRP difference under AWGN channel with 1 SRS symbol

| FR | SCS (kHz) | SRS BW (PRBs) | Sample | RSRP difference (dB) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1.67us for FR1 and 0.67us for FR2 |  |  |  |  |  | 4.67us for FR1 and 3.67us for FR2 |  |  |  |  |  |
|  |  |  |  | SRS SNR = 1 dB |  | 2 dB |  | 3 dB |  | 1 dB |  | 2 dB |  | 3 dB |  |
|  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| FR1 | 15 | 24 | 1 | -0.61 | 0.71 | -0.56 | 0.59 | -0.52 | 0.5 | -0.98 | 0.41 | -0.93 | 0.3 | -0.88 | 0.21 |
|  |  |  | 2 | -0.41 | 0.54 | -0.39 | 0.46 | -0.36 | 0.38 | -0.77 | 0.2 | -0.73 | 0.11 | -0.71 | 0.04 |
|  |  |  | 3 | -0.35 | 0.48 | -0.32 | 0.39 | -0.31 | 0.32 | -0.71 | 0.11 | -0.69 | 0.03 | -0.68 | -0.03 |
|  |  | 48 | 1 | -0.5 | 0.48 | -0.47 | 0.4 | -0.43 | 0.33 | -0.83 | 0.13 | -0.79 | 0.06 | -0.76 | -0.01 |
|  |  |  | 2 | -0.36 | 0.33 | -0.34 | 0.27 | -0.32 | 0.21 | -0.7 | -0.01 | -0.68 | -0.08 | -0.66 | -0.13 |
|  |  |  | 3 | -0.29 | 0.26 | -0.29 | 0.2 | -0.28 | 0.16 | -0.63 | -0.06 | -0.63 | -0.12 | -0.62 | -0.16 |
|  | 30 | 24 | 1 | -0.81 | 0.57 | -0.76 | 0.46 | -0.72 | 0.36 | -1.58 | -0.07 | -1.52 | -0.18 | -1.47 | -0.29 |
|  |  |  | 2 | -0.61 | 0.34 | -0.59 | 0.26 | -0.58 | 0.19 | -1.38 | -0.26 | -1.35 | -0.36 | -1.33 | -0.44 |
|  |  |  | 3 | -0.55 | 0.3 | -0.53 | 0.22 | -0.51 | 0.14 | -1.27 | -0.37 | -1.25 | -0.47 | -1.25 | -0.55 |
|  |  | 48 | 1 | -0.67 | 0.3 | -0.64 | 0.22 | -0.61 | 0.15 | -1.46 | -0.35 | -1.41 | -0.45 | -1.37 | -0.53 |
|  |  |  | 2 | -0.54 | 0.16 | -0.52 | 0.1 | -0.51 | 0.04 | -1.29 | -0.52 | -1.27 | -0.6 | -1.25 | -0.66 |
|  |  |  | 3 | -0.48 | 0.1 | -0.47 | 0.04 | -0.46 | -0.01 | -1.23 | -0.59 | -1.21 | -0.65 | -1.2 | -0.71 |
|  | 60 | 24 | 1 | -1.28 | 0.21 | -1.22 | 0.09 | -1.16 | -0.02 | -3.12 | -1.27 | -3.03 | -1.43 | -2.95 | -1.55 |
|  |  |  | 2 | -1.03 | 0 | -1 | -0.1 | -0.97 | -0.18 | -2.82 | -1.53 | -2.77 | -1.65 | -2.74 | -1.75 |
|  |  |  | 3 | -0.93 | -0.1 | -0.92 | -0.18 | -0.9 | -0.25 | -2.66 | -1.62 | -2.65 | -1.74 | -2.63 | -1.82 |
|  |  | 48 | 1 | -1.09 | -0.09 | -1.05 | -0.17 | -1.01 | -0.24 | -2.9 | -1.58 | -2.84 | -1.69 | -2.79 | -1.79 |
|  |  |  | 2 | -0.96 | -0.23 | -0.94 | -0.3 | -0.92 | -0.36 | -2.68 | -1.77 | -2.66 | -1.86 | -2.63 | -1.93 |
|  |  |  | 3 | -0.89 | -0.3 | -0.87 | -0.35 | -0.86 | -0.4 | -2.61 | -1.85 | -2.59 | -1.93 | -2.57 | -2 |
| FR2 | 60 | 24 | 1 | -0.71 | 0.63 | -0.57 | 0.29 | -0.63 | 0.43 | -2.36 | -0.66 | -2.29 | -0.81 | -2.23 | -0.93 |
|  |  |  | 2 | -0.5 | 0.43 | -0.45 | 0.16 | -0.46 | 0.27 | -2.07 | -0.93 | -2.05 | -1.06 | -2.01 | -1.15 |
|  |  |  | 3 | -0.44 | 0.36 | -0.38 | 0.12 | -0.42 | 0.21 | -1.98 | -1.04 | -1.97 | -1.14 | -1.96 | -1.23 |
|  |  | 48 | 1 | -0.6 | 0.38 | -0.67 | 0.52 | -0.54 | 0.22 | -2.21 | -1.04 | -2.15 | -1.14 | -2.11 | -1.22 |
|  |  |  | 2 | -0.47 | 0.23 | -0.48 | 0.35 | -0.43 | 0.1 | -2.04 | -1.2 | -2.01 | -1.28 | -1.99 | -1.35 |
|  |  |  | 3 | -0.39 | 0.18 | -0.43 | 0.29 | -0.38 | 0.07 | -1.97 | -1.27 | -1.95 | -1.34 | -1.93 | -1.4 |
|  | 120 | 24 | 1 | -1.07 | 0.35 | -1 | 0.24 | -0.97 | 0.14 | -5.58 | -3.12 | -5.47 | -3.33 | -5.38 | -3.51 |
|  |  |  | 2 | -0.86 | 0.16 | -0.83 | 0.07 | -0.8 | -0.02 | -5.22 | -3.48 | -5.13 | -3.64 | -5.09 | -3.79 |
|  |  |  | 3 | -0.76 | 0.07 | -0.75 | -0.02 | -0.73 | -0.1 | -5.06 | -3.62 | -5 | -3.78 | -4.97 | -3.9 |
|  |  | 48 | 1 | -0.93 | 0.09 | -0.89 | 0 | -0.86 | -0.07 | -5.37 | -3.56 | -5.29 | -3.72 | -5.23 | -3.86 |
|  |  |  | 2 | -0.77 | -0.07 | -0.75 | -0.14 | -0.73 | -0.19 | -5.07 | -3.82 | -5.05 | -3.95 | -5.02 | -4.06 |
|  |  |  | 3 | -0.73 | -0.15 | -0.71 | -0.21 | -0.7 | -0.26 | -4.96 | -3.92 | -4.93 | -4.03 | -4.9 | -4.12 |

Table A.2 CLI SRS-RSRP difference under AWGN channel with 2 SRS symbols

| FR | SCS (kHz) | SRS BW (PRBs) | Sample | RSRP difference (dB) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1.67us for FR1 and 0.67us for FR2 |  |  |  |  |  | 4.67us for FR1 and 3.67us for FR2 |  |  |  |  |  |
|  |  |  |  | SRS SNR = 1 dB |  | 2 dB |  | 3 dB |  | 1 dB |  | 2 dB |  | 3 dB |  |
|  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| FR1 | 15 | 24 | 1 | -0.44 | 0.5 | -0.41 | 0.42 | -0.38 | 0.35 | -0.84 | 0.17 | -0.8 | 0.08 | -0.77 | 0 |
|  |  |  | 2 | -0.3 | 0.35 | -0.29 | 0.29 | -0.27 | 0.23 | -0.7 | 0.02 | -0.68 | -0.05 | -0.66 | -0.11 |
|  |  |  | 3 | -0.24 | 0.3 | -0.23 | 0.24 | -0.23 | 0.2 | -0.64 | -0.03 | -0.63 | -0.1 | -0.61 | -0.15 |
|  |  | 48 | 1 | -0.41 | 0.28 | -0.39 | 0.22 | -0.37 | 0.17 | -0.69 | 0.01 | -0.67 | -0.04 | -0.64 | -0.09 |
|  |  |  | 2 | -0.31 | 0.18 | -0.29 | 0.14 | -0.28 | 0.09 | -0.6 | -0.1 | -0.58 | -0.14 | -0.57 | -0.18 |
|  |  |  | 3 | -0.26 | 0.14 | -0.25 | 0.1 | -0.25 | 0.07 | -0.54 | -0.14 | -0.53 | -0.18 | -0.52 | -0.21 |
|  | 30 | 24 | 1 | -0.7 | 0.31 | -0.67 | 0.22 | -0.64 | 0.15 | -1.35 | -0.28 | -1.31 | -0.37 | -1.28 | -0.44 |
|  |  |  | 2 | -0.58 | 0.14 | -0.56 | 0.08 | -0.54 | 0.02 | -1.17 | -0.43 | -1.14 | -0.51 | -1.13 | -0.57 |
|  |  |  | 3 | -0.49 | 0.09 | -0.47 | 0.03 | -0.47 | -0.02 | -1.11 | -0.53 | -1.11 | -0.59 | -1.1 | -0.64 |
|  |  | 48 | 1 | -0.56 | 0.14 | -0.53 | 0.08 | -0.51 | 0.03 | -1.23 | -0.46 | -1.19 | -0.52 | -1.17 | -0.57 |
|  |  |  | 2 | -0.45 | 0.04 | -0.44 | 0 | -0.43 | -0.04 | -1.1 | -0.55 | -1.09 | -0.61 | -1.08 | -0.65 |
|  |  |  | 3 | -0.42 | -0.02 | -0.41 | -0.06 | -0.4 | -0.09 | -1.07 | -0.61 | -1.06 | -0.65 | -1.04 | -0.69 |
|  | 60 | 24 | 1 | -1.02 | 0.02 | -0.98 | -0.06 | -0.94 | -0.14 | -2.55 | -1.33 | -2.5 | -1.43 | -2.46 | -1.52 |
|  |  |  | 2 | -0.87 | -0.17 | -0.85 | -0.23 | -0.83 | -0.28 | -2.37 | -1.5 | -2.35 | -1.58 | -2.32 | -1.65 |
|  |  |  | 3 | -0.82 | -0.21 | -0.8 | -0.28 | -0.79 | -0.33 | -2.27 | -1.56 | -2.26 | -1.64 | -2.24 | -1.7 |
|  |  | 48 | 1 | -0.94 | -0.21 | -0.91 | -0.27 | -0.88 | -0.33 | -2.38 | -1.51 | -2.36 | -1.59 | -2.33 | -1.66 |
|  |  |  | 2 | -0.83 | -0.32 | -0.81 | -0.37 | -0.8 | -0.41 | -2.24 | -1.63 | -2.23 | -1.71 | -2.22 | -1.76 |
|  |  |  | 3 | -0.78 | 0.37 | -0.77 | -0.41 | -0.76 | -0.44 | -2.19 | -1.71 | -2.18 | -1.77 | -2.18 | -1.81 |
| FR2 | 60 | 24 | 1 | -0.59 | 0.39 | -0.56 | 0.3 | -0.53 | 0.23 | -1.9 | -0.78 | -1.86 | -0.88 | -1.82 | -0.97 |
|  |  |  | 2 | -0.43 | 0.24 | -0.41 | 0.18 | -0.4 | 0.12 | -1.74 | -0.94 | -1.72 | -1.02 | -1.7 | -1.09 |
|  |  |  | 3 | -0.39 | 0.17 | -0.38 | 0.11 | -0.38 | 0.06 | -1.66 | -1 | -1.66 | -1.07 | -1.65 | -1.13 |
|  |  | 48 | 1 | -0.48 | 0.17 | -0.46 | 0.12 | -0.45 | 0.07 | -1.86 | -1.03 | -1.83 | -1.1 | -1.8 | -1.17 |
|  |  |  | 2 | -0.39 | 0.08 | -0.37 | 0.03 | -0.37 | -0.01 | -1.74 | -1.16 | -1.73 | -1.21 | -1.71 | -1.26 |
|  |  |  | 3 | -0.34 | 0.03 | -0.34 | -0.02 | -0.33 | -0.05 | -1.67 | -1.21 | -1.66 | -1.26 | -1.66 | -1.3 |
|  | 120 | 24 | 1 | -0.89 | 0.1 | -0.86 | 0.02 | -0.83 | -0.05 | -4.43 | -2.86 | -4.38 | -3 | -4.32 | -3.12 |
|  |  |  | 2 | -0.74 | -0.04 | -0.72 | -0.11 | -0.71 | -0.17 | -4.17 | -3.11 | -4.14 | -3.22 | -4.11 | -3.32 |
|  |  |  | 3 | -0.69 | -0.07 | -0.68 | -0.13 | -0.67 | -0.19 | -4.09 | -3.21 | -4.06 | -3.31 | -4.05 | -3.39 |
|  |  | 48 | 1 | -0.79 | -0.07 | -0.75 | -0.13 | -0.73 | -0.18 | -3.09 | -4.2 | -4.15 | -3.2 | -4.11 | -3.29 |
|  |  |  | 2 | -0.67 | -0.17 | -0.65 | -0.22 | -0.64 | -0.26 | -4.03 | -3.26 | -4.01 | -3.35 | -4 | -3.42 |
|  |  |  | 3 | -0.62 | -0.24 | -0.62 | -0.28 | -0.62 | -0.31 | -3.96 | -3.33 | -3.95 | -3.4 | -3.93 | -3.46 |

Table A.3 CLI SRS-RSRP difference under AWGN channel with 3 SRS symbols

| FR | SCS (kHz) | SRS BW (PRBs) | Sample | RSRP difference (dB) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1.67us for FR1 and 0.67us for FR2 |  |  |  |  |  | 4.67us for FR1 and 3.67us for FR2 |  |  |  |  |  |
|  |  |  |  | SRS SNR = 1 dB |  | 2 dB |  | 3 dB |  | 1 dB |  | 2 dB |  | 3 dB |  |
|  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| FR1 | 15 | 24 | 1 | -0.37 | 0.39 | -0.34 | 0.33 | -0.32 | 0.27 | -0.8 | 0.06 | -0.77 | -0.01 | -0.74 | -0.08 |
|  |  |  | 2 | -0.27 | 0.28 | -0.26 | 0.23 | -0.25 | 0.18 | -0.67 | -0.06 | -0.65 | -0.12 | -0.64 | -0.17 |
|  |  |  | 3 | -0.21 | 0.23 | -0.2 | 0.18 | -0.2 | 0.14 | -0.62 | -0.13 | -0.62 | -0.18 | -0.61 | -0.23 |
|  |  | 48 | 1 | -0.36 | 0.21 | -0.34 | 0.16 | -0.32 | 0.12 | -0.65 | -0.06 | -0.62 | -0.1 | -0.6 | -0.15 |
|  |  |  | 2 | -0.27 | 0.11 | -0.26 | 0.07 | -0.25 | 0.04 | -0.56 | -0.15 | -0.54 | -0.18 | -0.53 | -0.21 |
|  |  |  | 3 | -0.25 | 0.07 | -0.24 | 0.04 | -0.23 | 0.01 | -0.51 | -0.18 | -0.51 | -0.21 | -0.5 | -0.24 |
|  | 30 | 24 | 1 | -0.64 | 0.18 | -0.61 | -0.11 | -0.58 | -0.05 | -1.26 | -0.39 | -1.23 | -0.47 | -1.2 | -0.54 |
|  |  |  | 2 | -0.55 | 0.05 | -0.52 | -0.01 | -0.51 | -0.05 | -1.12 | -0.53 | -1.11 | -0.59 | -1.09 | -0.64 |
|  |  |  | 3 | -0.49 | 0 | -0.48 | -0.05 | -0.47 | -0.09 | -1.07 | -0.61 | -1.06 | -0.65 | -1.05 | -0.7 |
|  |  | 48 | 1 | -0.5 | 0.06 | -0.47 | 0.02 | -0.45 | -0.03 | -1.13 | -0.52 | -1.1 | -0.57 | -1.09 | -0.61 |
|  |  |  | 2 | -0.41 | -0.03 | -0.41 | -0.07 | -0.4 | -0.1 | -1.05 | -0.61 | -1.03 | -0.65 | -1.02 | -0.68 |
|  |  |  | 3 | -0.39 | -0.06 | -0.38 | -0.1 | -0.37 | -0.12 | -0.99 | -0.64 | -0.98 | -0.68 | -0.98 | -0.71 |
|  | 60 | 24 | 1 | -0.92 | -0.08 | -0.88 | -0.15 | -0.86 | -0.21 | -2.36 | -1.38 | -2.32 | -1.46 | -2.29 | -1.54 |
|  |  |  | 2 | -0.79 | -0.21 | -0.78 | -0.26 | -0.76 | -0.31 | -2.23 | -1.5 | -2.2 | -1.58 | -2.18 | -1.64 |
|  |  |  | 3 | -0.75 | -0.27 | -0.73 | -0.31 | -0.73 | -0.35 | -2.17 | -1.58 | -2.16 | -1.64 | -2.14 | -1.69 |
|  |  | 48 | 1 | -0.86 | -0.27 | -0.84 | -0.32 | -0.82 | -0.37 | -2.21 | -1.5 | -2.18 | -1.56 | -2.16 | -1.62 |
|  |  |  | 2 | -0.78 | -0.36 | -0.76 | -0.4 | -0.75 | -0.43 | -2.1 | -1.61 | -2.09 | -1.67 | -2.08 | -1.71 |
|  |  |  | 3 | -0.74 | -0.4 | -0.73 | -0.43 | -0.73 | -0.46 | -2.01 | -1.67 | -2.06 | -1.71 | -2.05 | -1.75 |
| FR2 | 60 | 24 | 1 | -0.53 | 0.24 | -0.5 | 0.17 | -0.48 | 0.12 | -1.76 | -0.83 | -1.72 | -0.91 | -1.69 | -0.98 |
|  |  |  | 2 | -0.42 | 0.13 | -0.41 | 0.07 | -0.39 | 0.02 | -1.63 | -0.98 | -1.61 | -1.05 | -1.6 | -1.1 |
|  |  |  | 3 | -0.36 | 0.08 | -0.35 | 0.03 | -0.35 | -0.01 | -1.57 | -1.03 | -1.56 | -1.1 | -1.55 | -1.14 |
|  |  | 48 | 1 | -0.45 | 0.1 | -0.43 | 0.05 | -0.41 | 0 | -1.74 | -1.06 | -1.71 | -1.12 | -1.68 | -1.17 |
|  |  |  | 2 | -0.37 | 0.01 | -0.36 | -0.02 | -0.36 | -0.06 | -1.63 | -1.16 | -1.62 | -1.21 | -1.61 | -1.25 |
|  |  |  | 3 | -0.35 | -0.03 | -0.34 | -0.06 | -0.33 | -0.09 | -1.59 | -1.2 | -1.58 | -1.25 | -1.57 | -1.28 |
|  | 120 | 24 | 1 | -0.83 | 0 | -0.79 | -0.06 | -0.77 | -0.13 | -4.04 | -2.8 | -3.98 | -2.92 | -3.95 | -3.01 |
|  |  |  | 2 | -0.7 | -0.12 | -0.69 | -0.18 | -0.68 | -0.23 | -3.86 | -3 | -3.83 | -3.09 | -3.8 | -3.16 |
|  |  |  | 3 | -0.66 | -0.17 | -0.65 | -0.21 | -0.64 | -0.26 | -3.77 | -3.04 | -3.77 | -3.13 | -3.76 | -3.2 |
|  |  | 48 | 1 | -0.72 | -0.14 | -0.7 | -0.19 | -0.68 | -0.23 | -3.84 | -2.97 | -3.8 | -3.06 | -3.77 | -3.13 |
|  |  |  | 2 | -0.64 | -0.22 | -0.63 | -0.26 | -0.62 | -0.29 | -3.71 | -3.1 | -3.69 | -3.17 | -3.68 | -3.23 |
|  |  |  | 3 | -0.6 | -0.26 | -0.59 | -0.29 | -0.59 | -0.33 | -3.65 | -3.14 | -3.63 | -3.2 | -3.63 | -3.25 |

Table A.4 CLI SRS-RSRP difference under AWGN channel with 4 SRS symbols

| FR | SCS (kHz) | SRS BW (PRBs) | Sample | RSRP difference (dB) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 1.67us for FR1 and 0.67us for FR2 |  |  |  |  |  | 4.67us for FR1 and 3.67us for FR2 |  |  |  |  |  |
|  |  |  |  | SRS SNR = 1 dB |  | 2 dB |  | 3 dB |  | 1 dB |  | 2 dB |  | 3 dB |  |
|  |  |  |  | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% | 5% | 95% |
| FR1 | 15 | 24 | 1 | -0.34 | 0.32 | -0.32 | 0.27 | -0.3 | 0.22 | -0.76 | -0.02 | -0.73 | -0.09 | -0.7 | -0.14 |
|  |  |  | 2 | -0.26 | 0.22 | -0.24 | 0.18 | -0.23 | 0.14 | -0.64 | -0.13 | -0.63 | -0.17 | -0.62 | -0.22 |
|  |  |  | 3 | -0.21 | 0.17 | -0.2 | 0.13 | -0.19 | 0.1 | -0.6 | -0.19 | -0.6 | -0.23 | -0.59 | -0.27 |
|  |  | 48 | 1 | -0.34 | 0.16 | -0.32 | 0.11 | -0.31 | 0.07 | -0.6 | -0.1 | -0.58 | -0.14 | -0.57 | -0.18 |
|  |  |  | 2 | -0.28 | 0.07 | -0.27 | 0.04 | -0.26 | 0.01 | -0.53 | -0.18 | -0.52 | -0.21 | -0.51 | -0.24 |
|  |  |  | 3 | -0.24 | 0.04 | -0.23 | 0.01 | -0.23 | -0.02 | -0.5 | -0.21 | -0.5 | -0.24 | -0.49 | -0.27 |
|  | 30 | 24 | 1 | -0.59 | 0.11 | -0.57 | 0.05 | -0.55 | -0.01 | -1.21 | -0.46 | -1.18 | -0.53 | -1.16 | -0.59 |
|  |  |  | 2 | -0.5 | 0.01 | -0.49 | -0.05 | -0.48 | -0.09 | -1.09 | -0.58 | -1.08 | -0.63 | -1.07 | -0.67 |
|  |  |  | 3 | -0.46 | -0.05 | -0.46 | -0.09 | -0.44 | -0.12 | -1.04 | -0.63 | -1.03 | -0.68 | -1.03 | -0.72 |
|  |  | 48 | 1 | -0.47 | 0.02 | -0.45 | -0.03 | -0.43 | -0.06 | -1.07 | -0.53 | -1.05 | -0.5 | -1.03 | -0.62 |
|  |  |  | 2 | -0.4 | -0.05 | -0.39 | -0.08 | -0.38 | -0.11 | -1 | -0.62 | -0.98 | -0.65 | -0.97 | -0.69 |
|  |  |  | 3 | -0.37 | -0.09 | -0.36 | -0.12 | -0.36 | -0.14 | -0.96 | -0.66 | -0.96 | -0.69 | -0.95 | -0.71 |
|  | 60 | 24 | 1 | -0.87 | -0.13 | -0.84 | -0.2 | -0.81 | -0.25 | -2.26 | -1.39 | -2.22 | -1.47 | -2.2 | -1.53 |
|  |  |  | 2 | -0.75 | -0.24 | -0.74 | -0.29 | -0.72 | -0.33 | -2.13 | -1.52 | -2.12 | -1.59 | -2.11 | -1.64 |
|  |  |  | 3 | -0.72 | -0.29 | -0.71 | -0.33 | -0.7 | -0.37 | -2.05 | -1.57 | -2.05 | -1.63 | -2.05 | -1.67 |
|  |  | 48 | 1 | -0.82 | -0.32 | -0.8 | -0.36 | -0.79 | -0.4 | -2.11 | -1.52 | -2.09 | -1.57 | -2.08 | -1.62 |
|  |  |  | 2 | -0.75 | -0.39 | -0.74 | -0.43 | -0.73 | -0.46 | -2.04 | -1.61 | -2.02 | -1.65 | -2.01 | -1.69 |
|  |  |  | 3 | -0.71 | -0.42 | -0.7 | -0.45 | -0.7 | -0.47 | -1.97 | -1.65 | -1.97 | -1.68 | -1.97 | -1.72 |
| FR2 | 60 | 24 | 1 | -0.49 | 0.16 | -0.47 | 0.11 | -0.45 | 0.05 | -1.69 | -0.88 | -1.65 | -0.95 | -1.63 | -1.01 |
|  |  |  | 2 | -0.41 | 0.07 | -0.39 | 0.02 | -0.38 | -0.02 | -1.57 | -0.99 | -1.56 | -1.04 | -1.54 | -1.09 |
|  |  |  | 3 | -0.36 | 0.02 | -0.36 | -0.02 | -0.35 | -0.05 | -1.51 | -1.05 | -1.5 | -1.1 | -1.5 | -1.15 |
|  |  | 48 | 1 | -0.42 | 0.05 | -0.41 | 0 | -0.4 | -0.03 | -1.65 | -1.1 | -1.63 | -1.14 | -1.61 | -1.19 |
|  |  |  | 2 | -0.36 | -0.02 | -0.36 | -0.06 | -0.35 | -0.08 | -1.56 | -1.17 | -1.56 | -1.21 | -1.55 | -1.24 |
|  |  |  | 3 | -0.33 | -0.05 | -0.33 | -0.08 | -0.32 | -0.11 | -1.54 | -1.21 | -1.53 | -1.24 | -1.52 | -1.27 |
|  | 120 | 24 | 1 | -0.8 | -0.08 | -0.77 | -0.14 | -0.75 | -0.2 | -3.85 | -2.8 | -3.81 | -2.89 | -3.78 | -2.98 |
|  |  |  | 2 | -0.69 | -0.18 | -0.68 | -0.24 | -0.67 | -0.28 | -3.68 | -3.97 | -3.67 | -3.04 | -3.65 | -3.1 |
|  |  |  | 3 | -0.66 | -0.23 | -0.64 | -0.27 | -0.64 | -0.31 | -3.62 | -3.02 | -3.61 | -3.08 | -3.61 | -3.13 |
|  |  | 48 | 1 | -0.68 | -0.18 | -0.67 | -0.22 | -0.65 | -0.26 | -3.65 | -2.92 | -3.62 | -2.98 | -3.59 | -3.04 |
|  |  |  | 2 | -0.62 | -0.26 | -0.61 | -0.29 | -0.6 | -0.32 | -3.55 | -3.02 | -3.53 | -3.08 | -3.52 | -3.13 |
|  |  |  | 3 | -0.58 | -0.29 | -0.58 | -0.32 | -0.57 | -0.34 | -3.5 | -3.06 | -3.49 | -3.11 | -3.48 | -3.16 |