3GPP RAN WG4 Meeting #113	R4-2417697

Orlando, US, November 18th – November 22nd, 2024

Title: 	Discussion on impacts of SBFD on RRM requirements

Source: 	CATT

Agenda item:	7.20.3.2

Document for:	Discussion

Introduction

In RAN4#112bis meeting, RAN4 continued the discussions on the RRM requirements of R19 SBFD WI. Some agreements were achieved and captured in way forward [1]. In this contribution, we will further discuss the impacts of SBFD on RRM requirements.

Discussion

Requirements for SSB-based measurements

In last RAN4 meeting, requirements for SSB-based measurements were discussed and the following agreements were achieved.

- Agreements (from online session) - Down-select one of the two options: - Option 1: - As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission. - This can be revisited if it proves to be conflicting with further RAN1 agreements. - Option 2: - For inter-frequency measurement without gap, UL transmission is prioritized. - Follow option 1 in other cases.

According to the agreement, SSB based measurements are prioritized for most of cases except that one controversial case. Some companies argued that UL transmission should be prioritized for inter-frequency measurement without gap. In RAN1#118bis meeting, a working assumption was made which is shown below for reference [2]:

Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

Based on our understanding of the above working assumption, SSB based measurements are prioritized when SSB symbols collide with SBFD symbols. Therefore SSB based measurements should also be prioritized for inter-frequency measurement without gap.

Proposal 1: For SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission, including inter-frequency measurement without gap case.

Requirements for CSI-RS-based measurements

Requirements for CSI-RS based measurements were discussed in last meeting and agreements are copied below:

- Agreements (from online session) - RAN4 to discuss whether and how L1 CSI-RS measurement requirements would be impacted by the following SBFD operations. - CSI-RS colliding with dynamically scheduled UL transmission - CSI-RS puncturing in frequency domain in SBFD symbols - CSI-RS distributed over SBFD and non-SBFD symbols - Align the understanding whether above SBFD operations are applicable for CSI-RS for RLM, BFR and L1-RSRP, or they are only applicable for CSI-RS for CSI. - RAN4 to discuss whether and how L3 CSI-RS measurement requirements would be impacted by SBFD operations, and further RAN1/2 conclusions can be considered.

Further discussions are required to study the impacts of SBFD operation on CSI-RS measurement requirements. For the collision between CSI-RS resources and dynamically scheduled UL transmission, RAN1 has specified rules for possible collision cases in past meetings and they can be adapted for CSI-RS based measurements as follows:

- For CSI-RS-based serving cell measurements,

- UL transmission is cancelled if cancellation timeline is met when semi-statically configured UL transmission collides with dynamically scheduled DL reception (i.e., aperiodic CSI-RS transmissions).

- UE does not receive DL channel/signal when dynamically scheduled UL transmissions collide with semi-statically configured DL reception (SP CSI-RS and periodic CSI-RS transmissions).

Furthermore, from RAN1 perspective, it is an error case when dynamically scheduled UL transmissions collide with dynamically scheduled DL receptions.

For CSI-RS-based neighbor cell measurements, it is clearer to be discussed in terms of intra-frequency measurements and inter-frequency measurements with two R18 prerequisites, i.e., MG is not needed for intra-frequency measurements and MG is required for inter-frequency measurements.

For measurements without MG, the legacy scheduling restrictions defined for measurements in TDD bands can be reused, e.g., specified in clause 9.10.2.6.1, while for measurements with MG, there are no requirement impacts since communication transmissions are not allowed during MG occasions.

Proposal 2: Requirements for CSI-RS-based measurements are defined based on the following principles:

- For CSI-RS-based serving cell measurements,

  - UL transmission is cancelled if cancellation timeline is met when semi-statically configured UL transmission collides with dynamically scheduled DL reception (i.e., aperiodic CSI-RS transmissions).

  - UE does not receive DL channel/signal when dynamically scheduled UL transmissions collide with semi-statically configured DL reception (SP CSI-RS and periodic CSI-RS transmissions).

  - It is an error case when dynamically scheduled UL transmissions collide with dynamically scheduled DL receptions.

- For CSI-RS based neighbor cell measurements without MG, RAN4 reuses legacy scheduling restrictions for measurements in TDD bands, e.g., specified in clause 9.10.2.6.1.

- For CSI-RS based neighbor cell measurements with MG, there are no impacts on RRM requirements.

Requirements for scheduling restrictions

Issues related to requirements for scheduling restrictions were discussed in last meeting without agreements. Two aspects are provided for further discussions:

- Recommended WF - RAN4 to discuss whether scheduling restriction requirements are needed for DL/UL transition, e.g. after SSB measurement, or they can be covered by RAN1 spec. - RAN4 to discuss scheduling or measurement restriction when SSB/CSI-RS for L3 measurement is not completely contained in active DL sub-bands on SBFD symbols/slots.

For scheduling restriction requirements for DL/UL transition, RAN1 is discussing it and RAN4 can be triggered by RAN1 to define corresponding requirements. If no LS are received from RAN1, RAN4 can assume that the impacts will be defined in RAN1 spec.

Proposal 3: RAN4 can be triggered by RAN1 to defined scheduling restriction requirements for DL/UL transition. Otherwise, RAN4 assumes the impacts will be covered in RAN1 spec.

For scenarios where SSB/CSI-RS for L3 measurement is not completely contained in active DL sub-bands on SBFD symbols/slots, according to our understanding on RAN1/RF discussions and agreements, SSB will be confined in DL subbands by RAN1/RF design and CSI-RS is mapped to the REs only inside DL subbands. Therefore neither of the cases will happen in the field and RAN4 does not need to define corresponding measurement restriction requirements.

Proposal 4: RAN4 does not define measurement restriction requirements for SSB/CSI-RS not fully contained in DL subbands.

Conclusions

This contribution discussed the impacts of SBFD on RRM requirements and some proposals are provided:

Proposal 1: For SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission, including inter-frequency measurement without gap case.

Proposal 2: Requirements for CSI-RS-based measurements are defined based on the following principles:

- For CSI-RS-based serving cell measurements,

  - UL transmission is cancelled if cancellation timeline is met when semi-statically configured UL transmission collides with dynamically scheduled DL reception (i.e., aperiodic CSI-RS transmissions).

  - UE does not receive DL channel/signal when dynamically scheduled UL transmissions collide with semi-statically configured DL reception (SP CSI-RS and periodic CSI-RS transmissions).

  - It is an error case when dynamically scheduled UL transmissions collide with dynamically scheduled DL receptions.

- For CSI-RS based neighbor cell measurements without MG, RAN4 reuses legacy scheduling restrictions for measurements in TDD bands, e.g., specified in clause 9.10.2.6.1.

- For CSI-RS based neighbor cell measurements with MG, there are no impacts on RRM requirements.

Proposal 3: RAN4 can be triggered by RAN1 to defined scheduling restriction requirements for DL/UL transition. Otherwise, RAN4 assumes the impacts will be covered in RAN1 spec.

Proposal 4: RAN4 does not define measurement restriction requirements for SSB/CSI-RS not fully contained in DL subbands.

References

[1] R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, RAN4#112bis.

[2] Chair notes RAN1#118bis, RAN1#118bis.