3GPP RAN WG4 Meeting #113	R4-2417698

Orlando, US, November 18th – November 22nd, 2024

Title: 	Discussion on impacts of AIML mobility on RRM requirements

Source: 	CATT

Agenda item:	7.18.2

Document for:	Discussion

Introduction

In RAN#105, RAN4 scope of AIML mobility was clarified and one aspect is to study the impacts of AI/ML mobility on RRM requirements [1]. The details are reproduced below:

- Evaluate testability, interoperability, and impacts on RRM requirements and performance [RAN4] - Study the impacts on requirements based on RAN2 assumptions, and coordinate with RAN2 if needed - Study the testability and interoperability based on RAN2 framework (e.g., number of cells to measure, beams etc.) - NOTE 4: Leverage the work from “AI/ML for NR air interface” led by RAN1 and avoid the duplicate study for testability and interoperability. - NOTE 5: Avoid the overlaps with RAN2 work for evaluation - NOTE 1: RAN1/3 work can be triggered via LS NOTE 2: To avoid duplicate study with “AI/ML for NG-RAN” led by RAN3 NOTE 3: Two-sided model is not included

And in RAN4#112bis meeting, RAN4 started the first discussion on this SI and the agreements were captured in [2]. In this contribution, we will present our views on impacts of AI/ML mobility on RRM requirements.

Discussion

In last meeting, many aspects related to RRM spec were brought up and summarized. The list is reproduced below.

- Performance metrics

- Impacts of measurement errors

- Impacts on core requirements

- Prediction definition

- LCM related requirements

- Training and testing dataset

With no doubt, each aspect has an impact on RRM spec. However, considering this is SI phase and many key parts of AI/ML mobility are not clearly defined in RAN2 yet, it is premature to discuss the details of impacts on core spec. In addition, impacts of measurement errors and LCM related requirements are under discussions in AI/ML for NR air WI. Hence, we prefer that RAN4 defers the related discussions and focuses on the performance metrics.

Proposal 1: RAN4 prioritizes the discussion on performance metrics and defers the discussions on the following aspects in SI phase until RAN2’s design on AI/ML mobility is clear:

- Impacts of measurement errors

- Impacts on core requirements

- LCM related requirements

Regarding the prediction definition and datasets, they need to be discussed in a case-by-case manner.

Performance metrics

This issue is related to the test goals discussed in our companion contribution [3]. To avoid redundant discussion, in this contribution we would like to discuss the reference points for which RAN4 needs to define accuracy requirements, as the figure below illustrated.

Figure 1 Measurement model

In RAN2, the L3 cell-level output is point C, while the legacy RRM measurement accuracy requirements are generally defined for Point B. Some proposed to define accuracy requirements for point C, which is straightforward. The issue is coefficients of L3 filtering need to be taken into account if RAN4 defines accuracy requirements for point C and the coefficients become a side condition or applicability condition for corresponding requirements, which narrows down the application scope. In other hand, what level of results is outputted depends on the training datasets and it is implementation-dependent. Hence, even though the output of AI/ML models is filtered L3 cell-level RSRP in RAN2 simulation assumptions, it does not mean RAN4 has to define accuracy requirements for point C. And testing goal also affects the final decision as we discussed in [3]. Similarly, RAN4 does not have to define new accuracy requirements for point E. Besides, whether RAN2 will support beam-level predictions is not known yet. Hence, RAN4 does not need to make a decision at the beginning of SI phase.

Observation 1: What level of results is outputted by AI/ML models depends on the training datasets and it is implementation-dependent.

Observation 2: Defining accuracy requirements with filtering coefficients being a side condition may narrow down the applicable scope of requirements.

Observation 3: Test goals also affect the determination of performance metrics.

Proposal 2: RAN4 does not define accuracy requirements for point C and evaluate if the existing accuracy requirements can be reused for measurement reduction cases.

Conclusions

This contribution discussed the impacts of AI/ML mobility on RRM requirements and some proposals are provided:

Proposal 1: RAN4 prioritizes the discussion on performance metrics and defers the discussions on the following aspects in SI phase until RAN2’s design on AI/ML mobility is clear:

- Impacts of measurement errors

- Impacts on core requirements

- LCM related requirements

Observation 1: What level of results is outputted by AI/ML models depends on the training datasets and it is implementation-dependent.

Observation 2: Defining accuracy requirements with filtering coefficients being a side condition may narrow down the applicable scope of requirements.

Observation 3: Test goals also affect the determination of performance metrics.

Proposal 2: RAN4 does not define accuracy requirements for point C and evaluate if the existing accuracy requirements can be reused for measurement reduction cases.

References

[1] RP-242393, Revised SID on AIML for mobility in NR, OPPO, RAN#105.

[2] R4-2416855, WF on AI/ML Mobility, Nokia, RAN4#112bis.

[3] R4-2417699, Discussion on testability and interoperability issues for AIML mobility, CATT, RAN4#113.