3GPP RAN WG4 Meeting #113	R4-2417705

Orlando, USA, November 18th – November 22nd, 2024

Title: 	Discussion on RRM core requirements for positioning

Source: 	CATT

Agenda item:	7.17.5.1

Document for:	Discussion

Introduction

Impacts of AI/ML based positioning on RRM requirements were discussed in last meeting and agreements were captured in [1]. In this contribution, we will continue to discuss the potential impacts on RRM core requirements and the rest of open issues summarized in [2].

Discussion

Requirements for case 1

In case 1, UE can be requested to report its inferred locations in some scenarios and RAN2 has agreed to reuse LPP signallings for at least case 1. Furthermore, a step-by-step procedure for requesting UE to perform AI/M based positioning was provided in RAN2#127bis meeting [3].

Agreements: 1: The following procedures for LCM for UE sided model for AI positioning case 1 is the baseline: Step 1: LMF may request the UE to report the supported functionalities at the UE side by LPP request capabilities message. Step 2: UE sends LPP provide capabilities message to LMF with the supported functionalities at the UE side. Step 3: LMF sends the LPP provide assistance data message (which may contain network side additional condition). Step 4: UE reports the applicable functionality to the LMF by the LPP provide capabilities message. Step 5: The LMF requests the inferred location information using the LPP request location information message. Step 6: UE reports the inferred location using LPP provide location information message.

However, the performance of AI/ML functionality cannot be guaranteed if no corresponding performance requirements are defined in current framework. Therefore RAN4 should try its best to define requirements for case 1. Regarding the methodology for defining requirements, RAN4 needs to determine the metric. In our opinion, the 3-D distance between ground truth and the inferred location would be a reasonable candidate and the ground truth can be set by TE before the test starts. Then RAN4 needs to do some evaluations to determine the relationships between side conditions, e.g., BW of PRS/SRS, and the 3-D distance with a given SINR. The selection of side conditions also depends on inputs to AI/ML models which may need more progress in RAN1. And if a reasonable relationship is found, i.e., larger BW of PRS leading to shorter 3-D distance, then RAN4 can define corresponding performance requirements.

Proposal 1: RAN4 evaluate the applicability of defining performance requirements for case 1 with following methodology:

- Agree on test metric first: 3-D distance between ground truth and inferred location can be considered.

- Evaluate relationship between side conditions and 3-D distance based on simulations. Potential side conditions include bandwidth of PRS, SINR, etc.

Except for the performance requirements, some core requirements need to be defined as well, e.g., measurement period requirements. But more inputs from RAN1/2 are required before RAN4 can make concrete progress. Hence, we prefer to postpone this issue.

Proposal 2: Postpone the discussion on core requirements until more RAN1/2 inputs are available.

Requirements for case 3a/3b

In case 3a/3b, gNB measures SRS resources and reports the measurement results to LMF for positioning. Considering that RRM requirements, including core and performance requirements, are mainly defined for user equipment, RAN4 does not need to define requirements for case 3a/3b.

Proposal 3: RAN4 does not define RRM requirements for case 3a/3b.

Conclusions

This contribution discussed the potential impacts of AI/ML enabled positioning accuracy enhancement on RRM core requirements and following proposals are provided:

Proposal 1: RAN4 evaluate the applicability of defining performance requirements for case 1 with following methodology:

- Agree on test metric first: 3-D distance between ground truth and inferred location can be considered.

- Evaluate relationship between side conditions and 3-D distance based on simulations. Potential side conditions include bandwidth of PRS, SINR, etc.

Proposal 2: Postpone the discussion on core requirements until more RAN1/2 inputs are available.

Proposal 3: RAN4 does not define RRM requirements for case 3a/3b.

References

[1] R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated, RAN4#112bis.

[2] R4-2415231, Topic summary for [112-Bis][127] NR_AI/ML_air, Qualcomm Incorporated, RAN4#112bis.

[3] R2_127bis_ChairNotes_24-10-18_15-45_eom_clean, RAN2#127bis.