3GPP TSG-RAN WG4 Meeting #113	R4-2417720

# Orlando , US, Nov 18 – 22, 2024

Title: 	Discussion on (e)RedCap RRM requirements for Rel-19 NTN phase3

Source: 	CATT

Agenda item:	7.26.4.1

Document for:	Discussion

- Introduction

A new RAN4 leading work item for Non-Terrestrial Networks (NTN) for NR Phase 3 was approved in RAN#102, and the WID have been revised in RAN#105[1]. This document will discuss (e)RedCap RRM requirements for Rel-19 NTN phase 3 and give our understanding and proposals.

- Discussion

  - General consideration

- 2.2.1 What RRM requirements are defined for (e)RedCap UE with FR1-NTN

Further discussion on cell reselection requirements

In the last meeting, RAN4 further discussed cell reselection requirements for (e)RedCap UE with FR1-NTN bands and reached the following agreements, but there was still FFS for the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement [2]:

<Agreement>: - The existing cell reselection requirements in RRC_IDLE/RRC_INACTIVE state defined for Rel-17 and Rel-18 (e)NTN can be the baseline for (e)RedCap 2Rx UE with FR1-NTN bands. - Both time-based and location-based cell reselection measurement criteria will be defined. - For both earth fixed and earth moving scenario - Both NTN-NTN and NTN-TN (from NTN to TN and from TN to NTN) cell reselection requirements will be defined. - FFS for the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

In our view, since the measurement requirements for inter-RAT (NR NTN to E-UTRA TN) measurement has been defined in NTN for normal UE, it is reasonable to define them for (e)RedCap UE with FR1-NTN bands, and there is no much differences between normal UE and RedCap regarding this aspect.

Proposal 1: The measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement can be defined for (e)RedCap UE with FR1-NTN bands.

Further discussion on L3 measurements for (e)RedCap UE with FR1-NTN

In the last meeting, the WF of Further discussion on L3 measurements for (e)RedCap UE with FR1-NTN is copied as follows [2]:

<Way Forward>: - Proposal 1 (CATT): RAN4 to define the requirements of the following types of L3 measurements for (e)RedCap UEs with FR1-NTN: - Intra-frequency measurements without measurement gaps - Intra-frequency measurements with measurement gaps - Inter-frequency measurement without measurement gaps - Inter-frequency measurement with measurement gaps

For L3 measurements, since intra-frequency measurements with/without gaps and inter-frequency measurements with/without gaps were supported for both existing NTN requirements and (e)RedCap UEs with TN, so we believe that the same supported type of L3 measurements can be considered and the related measurement requirements will be defined for (e)RedCap UEs with FR1-NTN.

Proposal 2: RAN4 to define the requirements of the following types of L3 measurements for (e)RedCap UEs with FR1-NTN:

- Intra-frequency measurements without measurement gaps

- Intra-frequency measurements with measurement gaps

- Inter-frequency measurement without measurement gaps

- Inter-frequency measurement with measurement gaps

The Requirements to be further determined

In the last meeting, RAN4 discussed the requirements should be further determined for (e)RedCap UE with FR1-NTN and agreed to define NR Rel-17 and Rel-18 Conditional Handover requirements, RACH-less Handover requirements and  SDT requirements (including CG-SDT and RA-SDT) in RRC_INACTIVE state. However, TA validation in SDT requirements and MDT requirements were still FFS, and the agreements and FFS are copied as below [3]:

<Agreement>: - Define NR Rel-17 and Rel-18 Conditional Handover requirements. - Include location-based CHO and time-based CHO. - Support earth moving cell scenario - Define RACH-less Handover requirements. - Define SDT requirements (including CG-SDT and RA-SDT) in RRC_INACTIVE state. - FFS TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN. - FFS: MDT requirements in RRC_IDLE and RRC_INACTIVE state.

SDT requirements

Regarding CG-SDT requirements, TA validation requirements has been introduced for RedCap UE with TN and the TA validation was based on the RSRP change criterion. The RSRP change was used to reflect the change of distance between UE and BS. When the change of distance exceeds the threshold, TA will be considered invalid.

But for NTN, the change of distance has already been compensated by UE in UE specific TA, so RSRP change is not a good indicator for TA verification. Therefore, we support to remove the TA validation based on the RSRP change criterion and the requirements may refer to the PUR requirements for IoT for NTN.

Observation 1: TA validation in CG-SDT requirements has been introduced for RedCap UE with TN and the TA validation was based on the RSRP change criterion.

Observation 2: In NTN scenario, the change of distance has already been compensated by UE in UE specific TA, so RSRP change is not a good indicator for TA verification.

Proposal 3: For (e)RedCap UE with FR1-NTN, TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN.

MDT requirements

In our view, the purpose of MDT is for network optimization. Although MDT requirements are important for NTN scenario as proponent said, if drive tests are needed in NTN scenarios, it is more resonable to use normal UE or even dedicated terminals for this task and we don’t think a RedCap UE will be used for drive tests unless in private network. From the perspective of practical use and scenario, the necessity of defining MDT requirements for (e)RedCap UE with FR1-NTN is not strong, and MDT requirements for normal UE is enough. Therefore, we support to deprioritize to define MDT requirements for (e)RedCap UE with FR1-NTN bands

Observation 3: The purpose of MDT is for network optimization.

Observation 4: Although MDT requirements are important for NTN scenario, if drive tests are needed in NTN scenarios, it is more resonable to use normal UE or even dedicated terminals for this task and a RedCap UE will not be used for drive tests unless in private network.

Proposal 4: Deprioritize to define MDT requirements for (e)RedCap UE with FR1-NTN bands.

- 2.2.2 Features to be considered for (e)RedCap UE with FR1-NTN

Relaxed measurements

For R17 relaxed measurements, they were introduced for R17 RedCap UEs in TN network, including stationary/ low mobility/not-at-cell-edge criteria and their combinations in RRC_IDLE/RRC_INACTIVE mode. In current NTN requirements, only R16 TN relaxed measurements were introduced in RRC_IDLE/RRC_INACTIVE mode, whether to consider these R16/R17 relaxed measurements for (e)RedCap UEs with FR1-NTN bands should be discussed.

In the last meeting, RAN4 discussed the related issues, but there were still FFS for the following [2]:

<Agreement>: - FFS: R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for normal UE are applicable for (e)RedCap UE with FR1-NTN bands. - FFS: RAN4 to define Rel-17 RedCap relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands, and the corresponding requirements defined for RedCap UE can be the baseline. - If defined, the requirements apply provided that target cell is GSO

For Rel-17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxations), the relaxed measurement criteria include low mobility criterion and not-at-cell edge criterion, and the corresponding criteria was evaluated based on RSRP by UE.

Observation 5: Rel-17 NR NTN relaxed criteria in GSO (corresponding to the TN R16 relaxations) include low mobility criterion and not-at-cell edge criterion, and the corresponding criteria was evaluated based on RSRP by UE.

For Rel-17 RedCap relaxations, the relaxed measurement criteria not only include low mobility criterion and not-at-cell edge criterion but also include stationary criterion for further restriction on mobility. For stationary criterion, the framework is the same as low mobility criterion and just stricter judgment thresholds was introduced. And for not-at-cell edge criterion defined in R17 RedCap, it was configured separately with TN R16 not-at-cell edge criterion, and only different thresholds were used. Therefore, R17 RedCap relaxations greatly reused the framework of TN R16 relaxations. In addition, stationary criterion is mandatory and not-at-cell edge criterion is configurable.

Observation 6: Rel-17 RedCap relaxed criteria not only include low mobility criterion and not-at-cell edge criterion but also include stationary criterion for further restriction on mobility.

Observation 7: For stationary criterion defined in Rel-17 RedCap, the framework is the same as low mobility criterion and just stricter judgment thresholds was introduced.

Observation 8: For not-at-cell edge criterion defined in R17 RedCap, it was configured separately with TN R16 not-at-cell edge criterion, and only different thresholds were used.

In TN network, if RedCap UE is configured with both R16 relaxations and Rel-17 RedCap relaxations at the same time, RedCap UE can select relaxed measurement criteria based on its implementation, so the relaxed measurement requirements didn’t consider the case of UE configured with both R16 and R17 relaxations from RRM requirements perspective.

Observation 9: In TN network, if RedCap UE is configured with both R16 relaxations and Rel-17 RedCap relaxations at the same time, RedCap UE can select relaxed measurement criteria based on its implementation, and the relaxed measurement requirements didn’t consider the case of UE configured with both R16 and R17 relaxations.

For (e)RedCap UE with FR1-NTN bands, since TN R16 relaxations have been introduced in NTN for normal UE, Rel-17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxations) can be naturally supported. For Rel-17 RedCap relaxations, the whole framework is the same as TN R16 relaxations, and the new introduced stationary criterion only used stricter judgment thresholds compared to low mobility criterion. Considering that stationary criterion and low mobility criterion were evaluated based on RSRP, and the change of RSRP is not obvious in NTN scenario, at least the stationary criterion and the low mobility criterion cannot be well distinguished.

Therefore, we believe that the low mobility criterion and not-at-cell edge criterion are enough for (e)RedCap UE with FR1-NTN. In our view, a better way is not to distinguish TN R16 relaxations or R17 RedCap relaxations and just to configure one set of relaxation since the frameworks of R16 and R17 RedCap relaxations are the same, and RAN4 can focus on the supported specific criteria. For the thresholds in supported criteria, RAN4 can further discuss whether to reuse R16 relaxation thresholds or further to relax.

Observation 10: The stationary criterion and low mobility criterion were evaluated based on RSRP, and the change of RSRP is not obvious in NTN scenario, at least the stationary criterion and the low mobility criterion cannot be well distinguished.

Proposal 5: A better way is not to distinguish TN R16 relaxations or R17 RedCap relaxations and just consider one set of relaxation, and RAN4 can focus on the supported specific criteria.

Proposal 6: The low mobility criterion and not-at-cell edge criterion can be supported for (e)RedCap UE with FR1-NTN since they have been introduced in NTN.

Proposal 7: For the thresholds in supported criteria, RAN4 can further discuss whether to reuse R16 relaxation thresholds or further to relax.

eDRX configuration

The eDRX enhancement has been introduced for (e)RedCap UEs in TN. However, the requirements related to eDRX wasn’t been defined in NTN, so whether and how to introduce the eDRX for (e)RedCap UEs with FR1-NTN bands should be discussed.

In the last meeting, RAN4 discussed the related issues and some agreements were reached for GSO, but there was no consensus for LEO among the companies. The agreement are copied as below [2]:

<Agreement>: - For GSO: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - For GSO, All DRX and eDRX cycle can be supported, and the DRX and eDRX related requirements for RedCap UE in TN are re-used. - Further discuss for LEO.

In Rel-17 RedCap WI, requirements were introduced for eDRX_IDLE up to 10485.76s and eDRX_INACTIVE cycles up to 10.24s, and the minimum value of eDRX cycle length can be 2.56s. In Rel-18 eRedCap WI, enhanced requirements were introduced for eDRX_INACTIVE up to 10485.76s [4].

In our view, for LEO deployment, we should discuss case by case. For earth-moving LEO deployment, since the serving time of a satellite is too short and 2.56s DRX cycle length has been excluded for earth-moving LEO deployment [4], considering that 2.56s is the minimum value of eDRX cycle length, there is no need to introduce eDRX for (e)RedCap UE with FR1-NTN.

Proposal 8: For earth-moving LEO deployment, the requirements are not applicable for eDRX cycle for (e)RedCap UE with FR1-NTN.

For quasi-earth fixed cell LEO deployment, as far as we know, the serving time of satellite is about several minutes (about 300s ~ 500s). According to Table 4.2B.2.3-2 in TS 38.133 [4], for cell Re-selection in IDLE mode, the existing Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap defined for RedCap UE configured with eDRX_IDLE cycle are copied as follows:

Table 4.2B.2.4-2: Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap for UE configured with eDRX_IDLE cycle (Frequency range FR1)

| eDRX_IDLE cycle length [s] | DRX cycle length [s] | PTW length [s] (number of 1.28s periods) | Tdetect,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tmeasure,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) | Tevaluate,NR_Inter_RedCap [s] (number of DRX cycles or eDRX cycles Note 3) |
| --- | --- | --- | --- | --- | --- |
| 2.56 | - | - | 58.88 (23) | 2.56 (1) | 7.68 (3) |
| 5.12 | - | - | 117.76 (23) | 5.12 (1) | 10.24 (2) |
| 10.24 | - | - | 235.52 (23) | 10.24 (1) | 20.48 (2) |
| 20.48 ≤ eDRX_IDLE cycle length ≤10485.76 | 0.32 | ≥1.28 (1) | eDRX_cycle_length×23PTW/DRX_cycle_length (23) | 0.32 x 1.5 (1 x 1.5) | 0.64 x 1.5 (2 x 1.5) |
|  | 0.64 | ≥1.28 (1) |  | 0.64 (1) | 1.28 (2) |
|  | 1.28 | ≥2.56 (2) |  | 1.28 (1) | 2.56 (2) |
|  | 2.56 | ≥5.12 (4) |  | 2.56 (1) | 5.12 (2) |
| NOTE 1: The number of DRX cycles in this table corresponds to the DRX cycles within PTWs, when PTW is configured. NOTE 2: The eDRX_IDLE cycle lengths are as specified in Section 10.5.5.32 of TS 24.008 [42]. NOTE 3: Number of eDRX cycles when eDRX_IDLE cycle length equals 2.56s, 5.12s and 10.24s. Otherwise, number of DRX cycles. NOTE 4: The lower bound of PTW length is derived based on Tevaluate,NR_Inter_RedCap*DRX_cycle1.28*1.28. |  |  |  |  |  |

Technically speaking, according to the values of Tdetect,NR_Inter_RedCap, Tmeasure,NR_Inter_RedCap and Tevaluate,NR_Inter_RedCap defined for RedCap UE, the serving time of a satellite can cover the time for UE to perform cell reselection for up to 10.24s eDRX cycle. However, for inter-frequency NR cells, the number of NR inter-frequency carriers also needed to be considered, so some margin of measurement time is needed. Therefore, we beilieve that at least 2.56s eDRX cycle length can be supported and 10.24s eDRX cycle length will be excluded. For 5.12s eDRX cycle length, it can be used in some cases such as intra-frequency measurement, so it can be introduced and whether it is configured depends on the network.

Observation 11:For quasi-earth fixed cell LEO deployment, for inter-frequency NR cells measurement, the number of NR inter-frequency carriers also needed to be considered, so some margin of time is needed.

Proposal 9: For quasi-earth fixed cell LEO deployment,

- 2.56s eDRX cycle length can be supported.

- 10.24s eDRX cycle length will be excluded.

- 5.12s eDRX cycle length can be used in some cases and whether it is configured depends on NW.

Multiple SMTCs

In the last meeting, the WF of multiple SMTCs related issue is copied as follows [2]:

<Way Forward>: - Proposal 1 (HW): RAN4 to discuss support of multiple SMTCs for (e)Redcap UE with FR1-NTN bands. - Option 1: make it optional for RedCap UE to support 2 SMTC per MO. - Option 2: change the requirements so that measurement for different SMTCs are not parallel. - Proposal 2 (ZTE): Multiple SMTCs shall be reused for RedCap UEs in NTN scenario. - Proposal 2a (Ericsson): RedCap for NTN supports (at the least) 2 SMTC. - Proposal 3 (OPPO): RAN4 to not consider the support of multiple SMTCs for (e)RedCap UE with FR1-NTN bands. - Proposal 4 (QC): NTN RRM requirements defined for simultaneous measurements on multiple satellites are not applicable for RedCap NTN UE.

In existing NTN requirements, normal UE is mandatorily required to support 2 SMTCs in parallel per MO, and supporting more SMTCs is an optional UE capability. However, we think that to support parallel measurement for the two or more SMTCs may be challenging for RedCap UE with reduced complexity. If introduced, we only support to make it optional for RedCap UE to support 2 SMTC per MO and do not support more SMTCs.

Proposal 10: Make it optional for RedCap UE to support 2 SMTC per MO and not to introduce more SMTCs.

Concurrent gap

In the last meeting, the WF of concurrent gap related issue is copied as follows [2]:

<Way Forward>: - Proposal 1 (CMCC, HW): Consider concurrent gap for Redcap over NTN. - Proposal 1a (HW): Concurrent gaps are supported for (e)Redcap UE with FR1-NTN bands, and existing requirements for normal UE in NTN are re-used as baseline. - Proposal 2 (CATT, QC): NTN RRM requirements defined for concurrent measurement gaps are not applicable for RedCap NTN UE. - Proposal 3 (Ericsson): RedCap for NTN supports (at the least) 1 MG.

For (e)RedCap UEs in TN network, only both per-UE and per-FR gap related requirements were defiined, and other enhanced gaps were not supported, such as NeedForGap, NCSG, concurrent gaps and so on. In current NTN requirements for normal UE, both per-UE and per-FR gap related requirements were defiined, and concurrent gaps was also introduced.

For (e)RedCap UE with FR1-NTN bands, we believe that the supported types of gaps should consider the types of UEs and UE capabilities, and introducing enhanced gap will bring more complexity for (e)RedCap UE with FR1-NTN. In addition, the reason why RAN4 introduce concurrent gap in Rel-17 was not for special NTN feature consideration, just because Rel-17 has already introduced concurrent gap in Gap related WI. Therefore, we slightly prefer not to introduce any enhanced gaps compared to the gap defined for (e)RedCap UEs in TN.

However, since concurrent gaps related requirements have been defined in NTN, so if concurrent gap related requirements can be reused directly, from workload perspective, option 1 is also ok for us, and it should be an optional UE capability.

Observation 12: The supported types of gaps are related to UE capibility, and introducing concurrent gaps will bring more complexity for (e)Redcap UE with FR1-NTN.

Observation 13: The reason why RAN4 introduce concurrent gap in Rel-17 was not for special NTN feature consideration, just because Rel-17 has already introduced concurrent gap in Gap related WI.

Proposal 11: For (e)Redcap UE with FR1-NTN, if concurrent gap related requirements can be reused directly, from workload perspective, option 1 is also ok, and it should be an optional UE capability.

The number of searchers and CSSF

In the last meeting, the WF of the number of searchers and CSSF is copied as follows [2]:

<Way Forward>: - Proposal 1 (CMCC, CATT): Single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

Regarding the searcher assumption and CSSF for (e)RedCap UEs in TN network, 1Rx and 2Rx (e)RedCap UEs use only a single cell search and measurement engine, with no CA in mind. The single searcher was shared by all of intra- and inter-frequency layers. For CSSF, its values are categorized into CSSFoutside_gap,i and CSSFwithin_gap,i. The legacy design of CSSFwithin gap were reused for (e)RedCap UEs in TN, and CSSFoutside_gap,I were defined as follows:

For UE in SA operation mode, the carrier-specific scaling factor CSSFoutside_gap_RedCap,i for intra-frequency SSB-based measurements, inter-frequency SSB-based measurements performed outside measurements gaps, will be specified as follows: CSSFoutside_gap,i =1, if only one measurement object is configured to be measured outside of MG for RedCap. CSSFoutside_gap,i =2 for intra-frequency measurement, and CSSFoutside_gap,i = 2*Y for inter-frequency measurement with no measurement gap, Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG. If intra-frequency measurement is with measurement gap, CSSFoutside_gap,i = Y for inter-frequency measurement with no measurement gap, Y is the number of configured inter-frequency MOs without MG that are being measured outside of MG.

For NTN requirements, the legacy searcher assumption and CSSF in TN were reused for normal UE. In our view, a single searcher and CSSF defined for (e)RedCap UEs in TN will be reused for (e)Redcap UE with FR1-NTN since they are related to the UE capability to process and depend on the type of UEs.

Observation 14: The legacy searcher assumption and CSSF are related to the capability of the UE to process and depend on the type of UEs.

Proposal 12: Single searcher assumption and CSSF defined for (e)RedCap UEs in TN will be reused for (e)Redcap UE with FR1-NTN.

FH in PRS measurement for NW verified location requirements

In the last meeting, the WF of FH in PRS measurement for NW verified location requirements is copied as follows [2]:

<Way Forward>: - Proposal 1 (HW): RAN4 to discuss whether to support FH in the requirements for NW verified location for (e)RedCap UE with FR1-NTN bands. - Proposal 2 (Ericsson, OPPO): Not support FH in PRS measurement for RedCap for NTN.

In our view, using frequency hopping can effectively combat system interference in traditional larger bandwidths and FH was introduced in PRS measurement for RedCap in TN.

However, the configurable bandwidth of NTN in FR1 is only 20MHz, so the gain of FH is very small, so we support not to introduce FH in PRS measurement for (e)RedCap UE with FR1-NTN.

Observation 15: The configurable bandwidth of NTN in FR1 is only 20MHz, so the gain of FH in PRS measurement is very small.

Proposal 13: Not support FH in PRS measurement for (e)RedCap UE with FR1-NTN.

NCD-SSB

In RAN4#112 meeting, RAN4 has already reached the following agreements [3]:

<Agreement> For (e)RedCap UEs with FR1-NTN, there is no need to define NCD-SSB specific measurement requirements.

In our view, even though NCD-SSB can bring benefits to RedCap UE in NTN, but considering that the NTN bandwidth is too small since the configurable bandwidth of NTN is only 20MHz, so the benefit of NCD-SSB is not obvious, so we support to still keep the above agreements.

Another point we suggest to clarifying is the meaning of ‘no need to define NCD-SSB specific measurement requirements’ in the agreement. To facilitate the writing of CR, RAN4 to only use ‘CD-SSB’ to define the requirements or use ‘SSB’?  For this issue, the WF in the last meeting is copied as follows [3]:

<Way Forward>: - Option 1: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause on this aspect (i.e, the requirements are only applicable to CD-SSB). - Option 2: Use the terminology of CD-SSB explicitly for (e)RedCap with FR1-NTN requirements. - Note: Other options are not precluded.

We support to use the terminology of SSB without differentiating CD-SSB/NCD-SSB in the spec and we think that the types of SSB need to be clarified. Considering that the network configuration can’t be restricted, so it is better to add a clarification of applicability in a separate clause on this aspect (such as clause 3 in TS 38.133[4]), and the wording can be: the requirements defined for (e)RedCap UEs with FR1-NTN are only applicable to CD-SSB.

Proposal 14: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause on this aspect (such as clause 3 in TS 38.133), and the wording can be:

- The requirements defined for (e)RedCap UEs with FR1-NTN are only applicable to CD-SSB.

  - Reduction in the number of UE Rx branches

RAN4 has agreed to follow the same principle for (e)RedCap UE in TN network as baseline [3], so we suggest that the following requirements need to be defined for 1Rx and 2Rx (e)RedCap UEs respectively with FR1-NTN bands, and other requirements not mentioned can be defined as a common set of RRM requirements for 1Rx and 2 Rx (e)RedCap UE.

Table 1 RRM Requirements need to be defined for 1Rx and 2Rx (e)RedCap UEs respectively

SA: RRC_IDLE/ RRC_INACTIVE state mobility

- Cell Selection in RRC_IDLE state - Cell Re-selection in RRC_IDLE/ RRC_INACTIVE state - Configured Grant based Small Data Transmissions (CG-SDT) in RRC_INACTIVE state - Random access based Small Data Transmissions in RRC_INACTIVE state

SA: RRC_CONNECTED state mobility

- NR Handover - NR SAN Handover - NR SAN FR1 – NR SAN FR1 conditional handover - NR SAN Conditional Handover - NR SAN FR1 – NR SAN FR1 conditional handover - NR SAN FR1 – NR SAN FR1 conditional handover without L3 measurement criteria - NR SAN Satellite switching with re-synchronization - RRC Connection Mobility Control - SA: RRC Re-establishment - Random access - SA: RRC Connection Release with Redirection

Signalling characteristics

- Radio Link Monitoring - Link Recovery Procedures

Measurement Procedure

- NR intra-frequency measurements - NR inter-frequency measurements - L1-RSRP measurements for Reporting - NR measurements for positioning (Network verified UE location)

Note: Other requirements not listed will be not distinguished between 1Rx and 2 Rx (e)RedCap UE

- 2.3.1 Mobility requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

1. RRC_IDLE/RRC_INACTIVE mode mobility

RRC_IDLE/RRC_INACTIVE mode mobility

For the mobility requirements in RRC_IDLE / RRC_INACTIVE state, RAN4 can reuse the legacy requirements in FR1-NTN defined for normal UEs for 2Rx (e)RedCap UEs with FR1-NTN bands.

Proposal 15: For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy mobility requirements in RRC_IDLE / RRC_INACTIVE state defined for normal UEs.

For 1Rx (e)RedCap UEs with FR1-NTN bands, firstly RAN4 need to consider the offset for the relative cell-specific RSRP thresholds. In TN network, -1 dB offset was used for the thresholds  of Qrxlevmin and Qqualmin. We support to reuse the same offset for 1Rx(e)RedCap UEs with FR1-NTN bands.

Proposal 16: RAN4 to consider the offset for the reletive cell-specific RSRP thresholds for 1Rx (e)RedCap UEs with FR1-NTN bands, including Qrxlevmin and Qqualmin:

- -1 dB offset can be reused as a baseline.

For the UE measurement capability for 1 Rx (e)RedCap UEs with FR1-NTN, the number of NR inter-frequency carriers that UE shall at least be capable of monitoring should be reduced. RAN4 can refer to the capability for 1 Rx (e)RedCap UEs in TN and 6 carriers can be reused.

Proposal 17: RAN4 to reduce the number of NR inter-frequency carriers that UE shall at least be capable of monitoring for 1Rx (e)RedCap UEs with FR1-NTN bands.

- 6 NR inter-frequency carriers can be reused.

For the measurement and evaluation of serving cell for (e)RedCap with FR1-NTN, Nserv_RedCap will be not distinguished between 1Rx and 2 Rx (e)Redcap UEs.

For the measurements of intra- and inter-frequency NR cells for (e)RedCap with FR1-NTN, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap will be not distinguished between 1Rx and 2 Rx (e)Redcap UE. For the threshold of absThreshSS-BlocksConsolidation, +1 dB offset was used for the 1Rx (e)RedCap UEs in TN network, and we support to reuse the same offset for 1Rx (e)RedCap UEs with FR1-NTN bands. For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset was used for better cell ranked for 1Rx (e)RedCap UEs in TN network, and the same offset can be reused for 1Rx (e)RedCap UEs with FR1-NTN bands.

Proposal 18: The paremeters of Nserv_RedCap, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap defined for cell re-selection requirements will be not distinguished between 1Rx and 2 Rx (e)Redcap UE.

Proposal 19: For the measurements of intra- and inter-frequency NR cells,

- +1 dB offset on the threshold of absThreshSS-BlocksConsolidation can be reused as a baseline for 1Rx(e)RedCap UEs with FR1-NTN bands.

- For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset can be reused for better cell ranked for 1Rx (e)RedCap UEs with FR1-NTN bands.

For measurements of inter-frequency NR cells in RRC_IDLE / RRC_INACTIVE state for (e)RedCap with FR1-NTN, the same +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria, which are as follows [4]:

For 1 Rx RedCap by a margin of at least - 6 dB in FR1 for reselections based on ranking or - 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or - 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

The above margin can also be reused for 1Rx (e)RedCap with FR1-NTN.

Proposal 20: In RRC_IDLE / RRC_INACTIVE state, +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria for 1Rx (e)RedCap with FR1-NTN:

- For 1 Rx RedCap by a margin of at least

- 6 dB in FR1 for reselections based on ranking or

- 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or

- 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

2. RRC_CONNECTED mode mobility

2.1 Handover

NR HO requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

For NR FR1 – NR FR1 handover requirements, handover delay extension has been considered for 1 Rx RedCap UE in TN, and the relaxations on Tsearch for unknown intra/inter- frequency cell were introduced in HO requirements in TS 38.133 are as following [4]:

If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = 1*Trs ms if UE is capable of 2 Rx antennas; Tsearch = 2*Trs ms if UE is only required to support 1 Rx antenna.

- the target cell is an unknown inter-frequency cell. In this case, if the target cell Es/Iot≥-2 dB, then Tsearch = 3* Trs ms if the UE is operating with 2 Rx antennas; Tsearch = 5*Trs ms if UE is operating with only 1 Rx antenna.

Since the values of Tsearch in NR SAN FR1 – NR SAN FR1 handover requirements are the same as those in TN network for normal UE, so we believe that the same relaxation on Tsearch for unknown intra/inter-frequency cell in HO requirements can be reused as baseline for (e)RedCap with FR1-NTN.

Proposal 21: For NR RACH-based HO requirements,

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RACH-based HO requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, RACH-based handover delays should be extended compared to those defined for 2Rx (e)RedCap UEs.

- Define the relaxation on Tsearch for unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

- Relax by 1 sample: Tsearch = 2*Trs for intra-frequency HO

- Relax by 2 samples: Tsearch = 5* Trs for inter-frequency HO

2.2 RRC Connection Mobility Control

RRC Re-establishment requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

For RRC Re-establishment requirements, UE re-establishment delay (TUE_re-establish_delay) extension has been considered for 1 Rx RedCap UE in TN, and the relaxations on Tidentify_intra_NR and Tidentify_inter_NR, i in TS 38.133 are as following [4]:

The requirements in clause 6.2.1 shall apply when RedCap UE is capable of 2 Rx. When UE is only required to support 1 Rx antenna, the requirements defined in clause 6.2.1 shall apply except that: - Tidentify_intra_NR as specified in Table 6.2.1B.2-1. - Tidentify_inter_NR, i as specified in Table 6.2.1B.2-2. Table 6.2.1B.2-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC); MAX (800 ms, [11] x TSMTC). < -8; FR1; N/A; 800Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Table 6.2.1B.2-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 7 x TSMTC, i); MAX (800 ms, [14] x TSMTC, i). < -8; FR1; N/A; 800Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB..

Since the values of Tidentify_intra_NR and Tidentify_inter_NR, i in UE Re-establishment delay requirements in NTN network are the same as those in TN network for normal UE, so we believe that the same relaxation on Tidentify_intra_NR and Tidentify_inter_NR, i for known and unknown intra-/inter-frequency NR cell in re-establishment delay requirements can be reused as baseline, that is, RAN4 to relax the Tidentify for (e)RedCap UEs with FR1-NTN by 1 more samples for FR1.

Proposal 22:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RRC re-establishment delay requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, UE RRC re-establishment delay (TUE_re-establish_delay) should be extended compared to those defined for 2Rx (e)RedCap UEs.

- Define the relaxation on Tidentify for known and unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

- Relax the Tidentify_intra_NR and Tidentify_inter_NR, i by 1 more samples for FR1 known and unknown intra/inter-frequency cell.

RA requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

For random access requirements, the offsets for the following relative cell-specific RSRP thresholds are defined in TN network in TS 38.133 as below [4]:

The 1 Rx RedCap UE for performing the random access procedure defined in clause 5.1 [7] applies: - rsrp-ThresholdSSB as the signaled value of rsrp-ThresholdSSB [2] + 1 dB. - msgA-RSRP-ThresholdSSB as the signaled value of msgA-RSRP-ThresholdSSB [2] + 1 dB. - msgA-RSRP-Threshold as the signaled value of msgA-RSRP-Threshold [2] + 1 dB. - rsrp-ThresholdMsg3 as the signaled value of rsrp-ThresholdMsg3 [2] - 1 dB.

We believe that the same consideration and same offset can be reused as a baseline for 1Rx(e)RedCap UEs with FR1-NTN bands.

Proposal 23:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy random access requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to consider the offset for the following relative cell-specific RSRP thresholds for random access requirements

- +1 dB offset can be reused as a baseline for rsrp-ThresholdSSB, msgA-RSRP-ThresholdSSB and msgA-RSRP-Threshold.

- -1 dB offset can be reused as a baseline for rsrp-ThresholdMsg3.

RRC Connection Release with redirection requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

For RRC connection release with redirection to NR requirements, the time delay (Tconnection_release_redirect_NR) extension has been considered for 1 Rx RedCap UE in TN, and the relaxation on Tidentify-NR in TS 38.133 is as following [4]:

The requirements in clause 6.2.3.2.1 shall apply when RedCap UE is capable of 2 Rx. When UE is only required to support 1 Rx antenna, the requirements defined in clause 6.2.3.2.1 shall apply except that: - Tidentify-NR as specified in Table 6.2.3A.2.1-1. Table 6.2.3A.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, [12] x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell..

Since the value of Tidentify-NR in RRC connection release with redirection to NR requirements in NTN network is the same as those in TN network for normal UE, so we believe that the same relaxation on Tidentify-NR for FR1 can be reused as baseline, that is, RAN4 to relax the Tidentify-NR for (e)RedCap UEs with FR1-NTN by 1 more samples for FR1.

Proposal 24:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RRC Connection Release with redirection requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, the delay of RRC connection release with redirection to NR (Tconnection_release_redirect_NR) should be extended compared to those defined for 2Rx (e)RedCap UEs.

- Define the relaxation on Tidentify-NR for FR1, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

- Relax the Tidentify-NR by 1 more samples for FR1.

- 2.3.3 Timing

Timing requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

In TN network, timing requirements are not distinguished between 1Rx and 2 Rx (e)RedCap UE, so there is also no need to distinguish the timing requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands.

Proposal 25:

- The following existing timing requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands:

- UE transmit timing

- UE timer accuracy

- Timing Advance

- The timing requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

- 2.3.4 Signalling characteristics

RLM/BFD/CBD requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

For SSB and CSI-RS based RLM/BFD requirements, some PDCCH transmission parameters for out-of-sync/in-sync evaluation and beam failure instance were defined for 1Rx and 2Rx RedCap UE respectively in TN network, including aggregation level (CCE) and bandwidth (PRBs). In detail, 16 was used for 1Rx RedCap UE for aggregation level (CCE), and 48 was used for 1Rx RedCap UE for bandwidth (PRBs).

Since the values of aggregation level (CCE) and bandwidth (PRBs) in RLM/BFD hypothetical PDCCH parameters in NTN are the same as those in TN for normal UE, so we believe that the same values of aggregation level (CCE) and bandwidth (PRBs) can be reused, that is, RAN4 to support aggregation level (CCE) of 16 and bandwidth (PRBs) of 48 for 1Rx (e)RedCap UEs with FR1-NTN.

In addition, considering that the impact on bandwidth for SSB based and CSI-RS based RLM/BFD, SCS=60kHz was not applicable for FR1 for RedCap UE in TN network, so the same applicability should be apply for (e)RedCap UEs with FR1-NTN.

Proposal 26: For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RLM/BFD/CBD requirements for normal UEs.

Proposal 27: For 1Rx (e)RedCap UEs with FR1-NTN bands,

- For SSB based and CSI-RS based RLM/BFD requirements, some of hypothetical PDCCH parameters including aggregation level (CCE) and bandwidth (PRBs) are defined as follows:

- Aggregation level (CCE) of 16.

- Bandwidth (PRBs) of 48.

- For SSB based and CSI-RS based RLM/BFD requirements, SCS=60kHz is not applicable for (e)RedCap UEs with FR1-NTN.

In additon, RLM OOS and BFD evaluation periods extension has been considered for 1 Rx RedCap UE in TN, and the relaxations on TEvaluate_out/in_SSB,RedCap, TEvaluate_out/in_CSI-RS,RedCap,TEvaluate_BFD_SSB _Redcap and TEvaluate_BFD_CSI-RS_Redcap in TS 38.133 are as following [4]:

Table 8.1B.2.2-3: Evaluation period TEvaluate_out_SSB,RedCap and TEvaluate_in_SSB,RedCap for FR1 for 1Rx RedCap UE Configuration; TEvaluate_out_SSB,RedCap (ms); TEvaluate_in_SSB,RedCap (ms). no DRX; Max(400, Ceil(20 P) TSSB); Max(100, Ceil(5 P) TSSB). DRX cycle≤320ms; Max(400, Ceil(30 P) Max(TDRX,TSSB)); Max(100, Ceil(7.5 P) Max(TDRX,TSSB)). DRX cycle>320ms; Ceil(20 P) TDRX; Ceil(5 P) TDRX. NOTE: TSSB is the periodicity of the SSB configured for RLM. TDRX is the DRX cycle length.. Table 8.1B.3.2-3: Evaluation period TEvaluate_out_CSI-RS,RedCap and TEvaluate_in_CSI-RS,RedCap for FR1 for 1Rx RedCap UE Configuration; TEvaluate_out_CSI-RS,RedCap (ms); TEvaluate_in_CSI-RS,RedCap (ms). no DRX; Max(400, Ceil(Mout,RedCap×P)×TCSI-RS); Max(100, Ceil(Min,RedCap×P) × TCSI-RS). DRX ≤ 320ms; Max(400, Ceil(1.5×Mout,RedCap ×P)× Max(TDRX, TCSI-RS)); Max(100, Ceil(1.5×Min,RedCap×P)× Max(TDRX, TCSI-RS)). DRX > 320ms; Ceil(Mout,RedCap ×P) × TDRX; Ceil(Min,RedCap ×P) × TDRX. NOTE: TCSI-RS is the periodicity of the CSI-RS resource configured for RLM. The requirements in this table apply for TCSI-RS equal to 5 ms, 10ms, 20 ms or 40 ms. TDRX is the DRX cycle length.. The values of Mout,RedCap and Min,RedCap used in Table 8.1B.3.2-1 and Table 8.1B.3.2-2 for UE with 2Rx are defined as: - Mout,RedCap = 20 and Min,RedCap = 10, if the CSI-RS resource configured for RLM is transmitted with higher layer CSI-RS parameter density [6, clause 7.4.1] set to 3 and over the bandwidth ≥ 24 PRBs. The values of Mout,RedCap and Min,RedCap used in Table 8.1B.3.2-3 for RedCap UE with 1Rx are defined as: - Mout,RedCap = 40 and Min,RedCap = 10, if the CSI-RS resource configured for RLM is transmitted with higher layer CSI-RS parameter density [6, clause 7.4.1] set to 3 and over the bandwidth ≥ 24 PRBs. Table 8.5B.2.2-1: Evaluation period TEvaluate_BFD_SSB_Redcap for FR1 Configuration; TEvaluate_BFD_SSB_Redcap (ms) for Redcap UE with 2 Rx; TEvaluate_BFD_SSB_Redcap (ms) for 1 Rx Redcap. no DRX; Max(50, Ceil(5 P) TSSB); [ Max(50, Ceil(10 P) TSSB)]. DRX cycle ≤ 320ms; Max(50, Ceil(7.5 P) Max(TDRX,TSSB)); [ Max(50, Ceil(15 P) Max(TDRX,TSSB)) ]. DRX cycle > 320ms; Ceil(5 P) TDRX; [ Ceil(10 P) TDRX]. Note: TSSB is the periodicity of SSB in the set . TDRX is the DRX cycle length.. Table 8.5B.3.2-1: Evaluation period TEvaluate_BFD_CSI-RS_Redcap for FR1 Configuration; TEvaluate_BFD_CSI-RS_Redcap (ms) for Redcap UE with 2 Rx; TEvaluate_BFD_CSI-RS_Redcap (ms) for 1 Rx Redcap. no DRX; Max(50, Ceil(MBFD P PBFD) TCSI-RS); Max(50, Ceil(2MBFD P PBFD) TCSI-RS). DRX cycle ≤ 320ms; Max(50, Ceil(1.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)); Max(50, Ceil(21.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)). DRX cycle > 320ms; Ceil(MBFD P PBFD) TDRX; Ceil(2MBFD P PBFD) TDRX. Note: TCSI-RS is the periodicity of CSI-RS resource in the set . TDRX is the DRX cycle length..

Since the values of TEvaluate_out/in_SSB, TEvaluate_out/in_CSI-RS,TEvaluate_BFD_SSB and TEvaluate_BFD_CSI-RS in RLM/BFD requirements in NTN network are the same as TN network for normal UE, so we believe that the same extension for SSB and CSI-RS based RLM OOS and BFD evaluation periods can be reused as baseline for 1Rx (e)RedCap UEs with FR1-NTN, that is,

- For RLM evaluation periods,

- RAN4 not to extend the TEvaluate_in_SSB/CSI-RS for FR1.

- RAN4 to extend the TEvaluate_out_SSB/CSI-RS by factor 2 for FR1.

- For BFD evaluation periods,

- RAN4 to extend the TEvaluate_BFD_SSB/CSI-RS by factor 2 for FR1.

Proposal 28: The SSB and CSI-RS based RLM OOS and BFD evaluation periods should be extended for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- For RLM evaluation periods,

- RAN4 not to extend the TEvaluate_in_SSB/CSI-RS for FR1.

- RAN4 to extend the TEvaluate_out_SSB/CSI-RS by factor 2 for FR1.

- For BFD evaluation periods,

- RAN4 to extend the TEvaluate_BFD_SSB/CSI-RS by factor 2 for FR1.

Other requirements in signaling characteristics for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

In TN network, the following requirements were not distinguished between 1Rx and 2 Rx (e)Redcap UE:

- CBD evaluation period TEvaluate_CBD_SSB_Redcap

- Active BWP switch delay

- Active TCI state switching delay

- UE-specific CBW change

Therefore, there is also no need to distinguish these requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands, and reuse these requirements defined in NTN for (e)RedCap UEs with FR1-NTN.

Proposal 29:

- The existing following requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands:

- Active BWP switch delay,

- active TCI state switch delay

- UE specific CBW change requirements

- Pathloss reference signal switching delay

- The above requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

- 2.3.5 Measurement Procedure

NR intra/inter-frequency L3 measurements

For the measurement requirements, the relaxation on the time period for PSS/SSS detection, SSB index identification (PBCH decoding) and measurement period have been considered for 1Rx (e)RedCap UEs compared to those defined for 2Rx (e)RedCap UEs in TN network, According to TS 38.133 [4], the specific relaxation principles for 1Rx (e)RedCap UEs compared to 2Rx (e)RedCap UEs in TN are listed as following:

|  | Intra-f without measurement gaps | Intra-f with measurement gaps | Inter-f without measurement gaps | Inter-f with measurement gaps |
| --- | --- | --- | --- | --- |
| Time period for PSS/SSS detection | Relax for 1Rx - Increase number of attempts by 2 SSB samples: from ‘5’ to ‘7’ - Not to extend the lower boundary ‘600ms’ | Relax for 1Rx - Increase number of attempts by 2 SSB samples: from ‘5’ to ‘7’ - Not to extend the lower boundary ‘600ms’ | Relax for 1Rx - Increase number of attempts by 2 SSB samples: from ‘5’ to ‘7’ - Not to extend the lower boundary ‘600ms’ | Relax for 1Rx - Increase number of attempts by 2 SSB samples: from ‘8’ to ‘10’ - Not to extend the lower boundary ‘600ms’ |
| Time period for SSB time index detection | Relax for 1Rx - Extend the lower boundary from ‘120 ms’ to ‘160ms’ - Increase number of attempts by 3 SSB samples: from ‘3’ to ‘6’ | Relax for 1Rx - Extend the lower boundary from ‘120 ms’ to ‘160ms’ - Increase number of attempts by 3 SSB samples: from ‘3’ to ‘6’ | Relax for 1Rx - Extend the lower boundary from ‘120 ms’ to ‘160ms’ - Increase number of attempts by 3 SSB samples: from ‘3’ to ‘6’ | Relax for 1Rx - Extend the lower boundary from ‘120 ms’ to ‘160ms’ - Increase number of attempts by 3 SSB samples: from ‘3’ to ‘6’ |
| Measurement period | No relaxation | No relaxation | No relaxation | Relax for 1Rx - Extend the lower boundary from ‘200 ms’ to ‘400ms’ - Keep the same number of samples. |

According to the above, we believe that the same relaxation can be reused as baseline for 1Rx (e)RedCap UEs with FR1-NTN.

Proposal 30: For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN measurement requirements for normal UEs.

Proposal 31: The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter.

- Not to extend the lower boundary.

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600ms, ceil( 7 x Kp) x SMTC period )Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(600ms, ceil(1.5 x 7 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | ceil(7 x Kp) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

| Condition NOTE1 | TPSS/SSS_sync_inter |
| --- | --- |
| No DRX | Max(600ms, 10 Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(600ms, Ceil(10 *1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | 10 DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

Proposal 32: The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter.

- Extend the lower boundary from ‘120 ms’ to ‘160ms’.

| DRX cycle | TSSB_time_index_intra |
| --- | --- |
| No DRX | max(160ms, ceil( 6 x Kp ) x SMTC period)Note 1 x CSSFintra_RedCap |
| DRX cycle≤ 320ms | max(160ms, ceil (1.5 x 6 x Kp) x max(SMTC period,DRX cycle)) x CSSFintra_RedCap |
| DRX cycle>320ms | Ceil(6 x Kp) x DRX cycle x CSSFintra_RedCap |
| NOTE 1: If different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified |  |

| Condition NOTE1 | TSSB_time_index_inter |
| --- | --- |
| No DRX | Max(160ms, 6 Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(160ms, Ceil(6 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | 6 DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

Proposal 33: The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Extend the lower boundary from ‘200 ms’ to ‘400ms’

- Keep the same number of samples for T SSB_measurement_period_inter.

| Condition NOTE1 | T SSB_measurement_period_inter |
| --- | --- |
| No DRX | Max(400ms, 8 Max(MGRP, SMTC period)) CSSFinter_RedCap |
| DRX cycle ≤ 320ms | Max(400ms, ceil(1.5 8) Max(MGRP, SMTC period, DRX cycle)) CSSFinter_RedCap |
| DRX cycle > 320ms | 8 DRX cycle CSSFinter_RedCap |
| NOTE 1: DRX or non DRX requirements apply according to the conditions described in clause 3.6.1 |  |

Proposal 34: The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

L1-RSRP measurements for Reporting

For SSB and CSI-RS based L1-RSRP measurement requirements, there were no extensions on the measurement period TL1-RSRP_Measurement_Period_SSB_RedCap and TL1-RSRP_Measurement_Period_CSI-RS_RedCap for FR1 for 1 Rx RedCap UE in TN network [4]. Therefore, we believe that RAN4 doesn’t need to extend the measurement period TL1-RSRP_Measurement_Period_SSB and TL1-RSRP_Measurement_Period_CSI-RS for 1Rx (e)RedCap UEs with FR1-NTN bands, that is, reuse those defined for 2Rx.

Proposal 35: The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

- 2.3.6 Measurement accuracy requirements

Measurement accuracy relaxation for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

The specific measurement accuracy requirements were defined in clauses 10.1A and 10.2A in TS 38.133[4] for (e)RedCap UEs in TN, including NR measurements for RedCap and E-UTRAN measurements for RedCap. For 2Rx (e)RedCap UEs in TN network, the legacy accuracy requirements were reused. For 1Rx (e)RedCap UEs in TN network, 1 dB relaxation on SS-RSRP/ SS-RSRQ/ SS-SINR accuracy requirements was considered for L3 intra- and inter-frequency measurement in FR1, and 3dB relaxation on L1-RSRP accuracy requirements was considered for SSB based and CSI-RS based L1 measurement in FR1.

In current NTN requirements, the specific measurement accuracy requirements were also defined with the suffix C in TS 38.133[4]. SS-RSRP/ SS-RSRQ/ SS-SINR accuracy requirements were considered for L3 intra- and inter-frequency measurement in FR1, and SSB based L1-RSRP accuracy requirements was considered in FR1. All of the values of accuracy are the same as the legacy.

Therefore, we believe that RAN4 can reuse the accuracy requirements defined in NTN for 2Rx (e)RedCap UEs with FR1-NTN bands, and consider the same relaxation as TN for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

Proposal 36:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the measurement accuracy requirements defined in NTN.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, support to relax the measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

- The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

- SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

- L1-RSRP: relax by 3dB.

For SSB_RP and SSB Ês/Iot, we believe that RAN4 can reuse the conditions defined in NTN for a corresponding Band. For 1Rx (e)RedCap UEs and 2Rx (e)RedCap UEs, the values for SSB_RP and SSB Ês/Iot are the same.

Proposal 37: RAN4 to reuse the conditions defined in NTN for a corresponding Band, and the values for SSB_RP and SSB Ês/Iot are the same for 1Rx (e)RedCap UEs and 2Rx (e)RedCap UEs with FR1-NTN bands.

  - The impact of HD-FDD

RAN4 has agreed that the TN HD-FDD related requirements and applicable conditions defined for (e)RedCap UE can be used as a starting point [3]. According to the RedCap WI, the following  RRM requirements are affected by HD-FDD for (e)RedCap UE in TN network, so the same requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN.

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

Proposal 38: At least the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

For the specific impact on above RRM requirements, RAN4 should consider the requirement applicability for HD-FDD (e)RedCap UE with FR1-NTN bands as legacy, and the legacy applicability can be used as baseline. Since RAN1 is still discussing the enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB, and there is no much conclusions, the HD-FDD related enhancement on legacy requirement applicability should wait for more RAN1 progress.

Observation 16: RAN1 is still discussing the enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB, and there is no much conclusions.

Proposal 39: Whether the legacy HD-FDD related enhancement on legacy requirement applicability can be reused in NTN directly should wait for more RAN1 progress.

  - Others

Specification structure

To facilitate the writing of CR, the specification structure also should be discussed. In the last meeting, the candidate proposals about specification structure are copied as below [3]:

<Way Forward>: - Proposal 1 (CATT, HW): - Use new sections and section number with new suffix X in the specification to capture requirements for (e)RedCap UE with FR1-NTN bands. - Utilize the reference method for the requirements that can be reused. - Proposal 2 (Nokia): - Adopt RedCap subclauses under NTN sections (under Suffix C).

We support to define the requirements for (e)RedCap UEs with FR1-NTN in the new sections of section number with new suffix X in the specification, like current way used for NTN requirements and (e)RedCap requirements. It is more clear and simple way. In addition, RAN4 will utilize the reference method for the requirements that can be reused, thereby reducing the redundancy of the spec to a certain extent.

For the method of introducing new sub clauses in current NTN section, one question is that both current NTN requirements and (e)RedCap requirements already had their own suffixes [4], so a new suffix after NTN suffix may need to be added, and we’re not sure it is a permitted and general methods.

Observation 17: For introducing new subclauses in current NTN section, one question is that both current NTN requirements and (e)RedCap requirements already had their own suffixes, so a new suffix after NTN suffix may need to be added.

Proposal 40: To use the following principles to define the RRM requirements for (e)RedCap UEs with FR1-NTN bands:

- Define them in the new sections of section number with new suffix X in the specification.

- Utilize the reference method for the requirements that can be reused.

- Conclusion

This document discussed the issues for (e)RedCap RRM requirements for R19 NR NTN and presented following observations and proposals:

Proposal 1: The measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement can be defined for (e)RedCap UE with FR1-NTN bands.

Proposal 2: RAN4 to define the requirements of the following types of L3 measurements for (e)RedCap UEs with FR1-NTN:

- Intra-frequency measurements without measurement gaps

- Intra-frequency measurements with measurement gaps

- Inter-frequency measurement without measurement gaps

- Inter-frequency measurement with measurement gaps

Observation 1: TA validation in CG-SDT requirements has been introduced for RedCap UE with TN and the TA validation was based on the RSRP change criterion.

Observation 2: In NTN scenario, the change of distance has already been compensated by UE in UE specific TA, so RSRP change is not a good indicator for TA verification.

Proposal 3: For (e)RedCap UE with FR1-NTN, TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN.

Observation 3: The purpose of MDT is for network optimization.

Observation 4: Although MDT requirements are important for NTN scenario, if drive tests are needed in NTN scenarios, it is more resonable to use normal UE or even dedicated terminals for this task and a RedCap UE will not be used for drive tests unless in private network.

Proposal 4: Deprioritize to define MDT requirements for (e)RedCap UE with FR1-NTN bands.

Observation 5: Rel-17 NR NTN relaxed criteria in GSO (corresponding to the TN R16 relaxations) include low mobility criterion and not-at-cell edge criterion, and the corresponding criteria was evaluated based on RSRP by UE.

Observation 6: Rel-17 RedCap relaxed criteria not only include low mobility criterion and not-at-cell edge criterion but also include stationary criterion for further restriction on mobility.

Observation 7: For stationary criterion defined in Rel-17 RedCap, the framework is the same as low mobility criterion and just stricter judgment thresholds was introduced.

Observation 8: For not-at-cell edge criterion defined in R17 RedCap, it was configured separately with TN R16 not-at-cell edge criterion, and only different thresholds were used.

Observation 9: In TN network, if RedCap UE is configured with both R16 relaxations and Rel-17 RedCap relaxations at the same time, RedCap UE can select relaxed measurement criteria based on its implementation, and the relaxed measurement requirements didn’t consider the case of UE configured with both R16 and R17 relaxations.

Observation 10: The stationary criterion and low mobility criterion were evaluated based on RSRP, and the change of RSRP is not obvious in NTN scenario, at least the stationary criterion and the low mobility criterion cannot be well distinguished.

Proposal 5: A better way is not to distinguish TN R16 relaxations or R17 RedCap relaxations and just consider one set of relaxation, and RAN4 can focus on the supported specific criteria.

Proposal 6: The low mobility criterion and not-at-cell edge criterion can be supported for (e)RedCap UE with FR1-NTN since they have been introduced in NTN.

Proposal 7: For the thresholds in supported criteria, RAN4 can further discuss whether to reuse R16 relaxation thresholds or further to relax.

Proposal 8: For earth-moving LEO deployment, the requirements are not applicable for eDRX cycle for (e)RedCap UE with FR1-NTN.

Observation 11:For quasi-earth fixed cell LEO deployment, for inter-frequency NR cells measurement, the number of NR inter-frequency carriers also needed to be considered, so some margin of time is needed.

Proposal 9: For quasi-earth fixed cell LEO deployment,

- 2.56s eDRX cycle length can be supported.

- 10.24s eDRX cycle length will be excluded.

- 5.12s eDRX cycle length can be used in some cases and whether it is configured depends on NW.

Proposal 10: Make it optional for RedCap UE to support 2 SMTC per MO and not to introduce more SMTCs.

Observation 12: The supported types of gaps are related to UE capibility, and introducing concurrent gaps will bring more complexity for (e)Redcap UE with FR1-NTN.

Observation 13: The reason why RAN4 introduce concurrent gap in Rel-17 was not for special NTN feature consideration, just because Rel-17 has already introduced concurrent gap in Gap related WI.

Proposal 11: For (e)Redcap UE with FR1-NTN, if concurrent gap related requirements can be reused directly, from workload perspective, option 1 is also ok, and it should be an optional UE capability.

Observation 14: The legacy searcher assumption and CSSF are related to the capability of the UE to process and depend on the type of UEs.

Proposal 12: Single searcher assumption and CSSF defined for (e)RedCap UEs in TN will be reused for (e)Redcap UE with FR1-NTN.

Observation 15: The configurable bandwidth of NTN in FR1 is only 20MHz, so the gain of FH in PRS measurement is very small.

Proposal 13: Not support FH in PRS measurement for (e)RedCap UE with FR1-NTN.

Proposal 14: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause on this aspect (such as clause 3 in TS 38.133), and the wording can be:

- The requirements defined for (e)RedCap UEs with FR1-NTN are only applicable to CD-SSB.

Proposal 15: For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy mobility requirements in RRC_IDLE / RRC_INACTIVE state defined for normal UEs.

Proposal 16: RAN4 to consider the offset for the reletive cell-specific RSRP thresholds for 1Rx (e)RedCap UEs with FR1-NTN bands, including Qrxlevmin and Qqualmin:

- -1 dB offset can be reused as a baseline.

Proposal 17: RAN4 to reduce the number of NR inter-frequency carriers that UE shall at least be capable of monitoring for 1Rx (e)RedCap UEs with FR1-NTN bands.

- 6 NR inter-frequency carriers can be reused.

Proposal 18: The paremeters of Nserv_RedCap, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap defined for cell re-selection requirements will be not distinguished between 1Rx and 2 Rx (e)Redcap UE.

Proposal 19: For the measurements of intra- and inter-frequency NR cells,

- +1 dB offset on the threshold of absThreshSS-BlocksConsolidation can be reused as a baseline for 1Rx(e)RedCap UEs with FR1-NTN bands.

- For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset can be reused for better cell ranked for 1Rx (e)RedCap UEs with FR1-NTN bands.

Proposal 20: In RRC_IDLE / RRC_INACTIVE state, +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria for 1Rx (e)RedCap with FR1-NTN:

- For 1 Rx RedCap by a margin of at least

- 6 dB in FR1 for reselections based on ranking or

- 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or

- 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

Proposal 21: For NR RACH-based HO requirements,

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RACH-based HO requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, RACH-based handover delays should be extended compared to those defined for 2Rx (e)RedCap UEs.

- Define the relaxation on Tsearch for unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

- Relax by 1 sample: Tsearch = 2*Trs for intra-frequency HO

- Relax by 2 samples: Tsearch = 5* Trs for inter-frequency HO

Proposal 22:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RRC re-establishment delay requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, UE RRC re-establishment delay (TUE_re-establish_delay) should be extended compared to those defined for 2Rx (e)RedCap UEs.

- Define the relaxation on Tidentify for known and unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

- Relax the Tidentify_intra_NR and Tidentify_inter_NR, i by 1 more samples for FR1 known and unknown intra/inter-frequency cell.

Proposal 23:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy random access requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to consider the offset for the following relative cell-specific RSRP thresholds for random access requirements

- +1 dB offset can be reused as a baseline for rsrp-ThresholdSSB, msgA-RSRP-ThresholdSSB and msgA-RSRP-Threshold.

- -1 dB offset can be reused as a baseline for rsrp-ThresholdMsg3.

Proposal 24:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RRC Connection Release with redirection requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, the delay of RRC connection release with redirection to NR (Tconnection_release_redirect_NR) should be extended compared to those defined for 2Rx (e)RedCap UEs.

- Define the relaxation on Tidentify-NR for FR1, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

- Relax the Tidentify-NR by 1 more samples for FR1.

Proposal 25:

- The following existing timing requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands:

- UE transmit timing

- UE timer accuracy

- Timing Advance

- The timing requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

Proposal 26: For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RLM/BFD/CBD requirements for normal UEs.

Proposal 27: For 1Rx (e)RedCap UEs with FR1-NTN bands,

- For SSB based and CSI-RS based RLM/BFD requirements, some of hypothetical PDCCH parameters including aggregation level (CCE) and bandwidth (PRBs) are defined as follows:

- Aggregation level (CCE) of 16.

- Bandwidth (PRBs) of 48.

- For SSB based and CSI-RS based RLM/BFD requirements, SCS=60kHz is not applicable for (e)RedCap UEs with FR1-NTN.

Proposal 28: The SSB and CSI-RS based RLM OOS and BFD evaluation periods should be extended for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- For RLM evaluation periods,

- RAN4 not to extend the TEvaluate_in_SSB/CSI-RS for FR1.

- RAN4 to extend the TEvaluate_out_SSB/CSI-RS by factor 2 for FR1.

- For BFD evaluation periods,

- RAN4 to extend the TEvaluate_BFD_SSB/CSI-RS by factor 2 for FR1.

Proposal 29:

- The existing following requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands:

- Active BWP switch delay,

- active TCI state switch delay

- UE specific CBW change requirements

- Pathloss reference signal switching delay

- The above requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

Proposal 30: For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN measurement requirements for normal UEs.

Proposal 31: The time period for PSS/SSS detection defined for intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter.

- Not to extend the lower boundary.

Proposal 32: The time period for SSB index identification (PBCH decoding) defined in intra/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter.

- Extend the lower boundary from ‘120 ms’ to ‘160ms’.

Proposal 33: The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

- Reuse the same extensions for 1Rx RedCap UE in TN network.

- Extend the lower boundary from ‘200 ms’ to ‘400ms’

- Keep the same number of samples for T SSB_measurement_period_inter.

Proposal 34: The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

Proposal 35: The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

Proposal 36:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the measurement accuracy requirements defined in NTN.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, support to relax the measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

- The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

- SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

- L1-RSRP: relax by 3dB.

Proposal 37: RAN4 to reuse the conditions defined in NTN for a corresponding Band, and the values for SSB_RP and SSB Ês/Iot are the same for 1Rx (e)RedCap UEs and 2Rx (e)RedCap UEs with FR1-NTN bands.

Proposal 38: At least the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

Observation 16: RAN1 is still discussing the enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB, and there is no much conclusions.

Proposal 39: Whether the legacy HD-FDD related enhancement on legacy requirement applicability can be reused in NTN directly should wait for more RAN1 progress.

Observation 17: For introducing new subclauses in current NTN section, one question is that both current NTN requirements and (e)RedCap requirements already had their own suffixes, so a new suffix after NTN suffix may need to be added.

Proposal 40: To use the following principles to define the RRM requirements for (e)RedCap UEs with FR1-NTN bands:

- Define them in the new sections of section number with new suffix X in the specification.

- Utilize the reference method for the requirements that can be reused.

- Reference

- RP-241789 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- R4-2416865, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)

- R4-2414037 The WF on RRM requirements for Rel-19 NR NTN phase3, Moderator (CATT)

- 3GPP TS 38.133, NR Requirements for support of radio resource management