3GPP TSG-RAN WG4 Meeting #113				                                               R4-2417726

Orlando, US, 18th – 22nd November, 2024

Title:	Discussion on RRM requirements for on-demand SSB Scell operation

Source:	CATT

Agenda Item:	7.22.2.1

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the RRM requirements for Rel-19 NES enhancement were discussed. The conclusions for OD-SSB SCell operation were captured in the approved WF [1] and the open issues can be found in the summary [2]. In this paper, we further discuss the potential RRM impact for on-demand SSB SCell operation and give our views.

2 Discussion

General

Based on the following RAN1 agreements, the RRC based OD-SSB SCell operation is supported for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration.

RAN1#118: Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. - Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

We understand this case means the direct SCell activation at SCell addition and the OD-SSB indication is provided when UE receives SCell activation command which is scenario #2A. With this agreement, we think RAN4 can discuss and define the SCell activation delay requirements for this case.

Proposal 1: RAN4 to define OD-SSB based SCell activation delay requirements for direct SCell activation at SCell addition for scenario #2A.

Meanwhile RAN2 has the following agreement in RAN2#127bis meeting which means RRC based OD-SSB indication is also supported.

Agreements on OD-SSB SCell

- No need to restrict the OD-SSB activation/deactivation state indication in RRC to initial configuration. No special specification effort is required.

According to the above RAN2 agreements, RRC based OD-SSB indication can also be considered in deactivated SCell measurement requirements and SCell activation delay requirements.

Proposal 2: RRC based OD-SSB indication can also be considered in deactivated SCell measurement requirements and SCell activation delay requirements.

Deactivated SCell L3 measurement part

For deactivated SCell L3 measurement for case 1, the following agreements were reached in last meeting:

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1) Agreements: Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

It was agreed that UE will perform the OD-SSB based fast L3 measurement for deactivated SCell within a time window and perform with a slow mode after the time window. The FFS parts are the definition of fast/slow measurement and time window.

For OD-SSB based fast deactivated SCell measurement, we think the measurement can be based on OD-SSB periodicity ignoring the DRX and measCycleSCell configuration since the OD-SSB will be stopped after certain duration and the measurement cycle may be too long for OD-SSB transmission which may lead to that UE does not finish one measurement before the OD-SSB is stopped. It makes the OD-SSB useless for the SCell. On the other hand, since we have agreed that this fast SCell measurement is only within a time window, the duration for fast measurement will not last too long which should be acceptable to UE.

For the slow mode after time window, if the OD-SSB still exists, it should be measured with legacy framework, i.e., based on measCycleSCell configuration.

For the duration of time window, the starting point has been agreed as the time when UE receives OD-SSB activation indication. To guarantee the deactivated SCell measurement performance and make the best use of OD-SSB, we think the duration should be able to accommodate one measurement period of deactivated SCell measurement which is based on OD-SSB periodicity, i.e., the ending point can be the time when UE firstly reports the target OD-SSB based deactivated SCell measurement results after receiving the OD-SSB indication.

Proposal 3: For the deactivated SCell L3 measurement for case 1:

- The fast L3 measurement requirements within time window are defined based on OD-SSB periodicity ignoring the DRX and measCycleSCell configuration.

- The slow L3 measurement requirements after time window are defined as same as legacy, i.e., based on DRX and measCycleSCell configuration.

- The ending point of time window can be the time when UE firstly reports the target OD-SSB based deactivated SCell measurement results after receiving the OD-SSB indication.

For case #2 where always-on SSB is periodically transmitted on the cell, the UE behaviors before OD-SSB is activated and after OD-SSB is deactivated have been agreed as below in last meeting:

Issue 1-3-2: Deactivated SCell measurement before OD-SSB transmission activation Agreement: - The UE is assumed to perform deactivated SCell measurement based on legacy SSB (measCycleSCell) before UE receiving OD-SSB transmission indication. Issue 1-3-3: Deactivated SCell measurement after OD-SSB transmission deactivation Agreement: - The UE is assumed to perform deactivated SCell measurement based on legacy SSB (measCycleSCell) after the OD-SSB transmission deactivation.

For the UE behavior when OD-SSB is activated, the following options are proposed:

Issue 1-3-4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) Agreements: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) - Option 1: RAN4 requirement is defined based on OD-SSB periodicity.(CATT, Samsung, CMCC, ZTE) - Option 2: RAN4 requirement is defined based on legacy measCycleSCell (Apple, Xiaomi, vivo, OPPO) - Option 3: RAN4 requirement is defined based on combining the always-on SSB and OD-SSB. (Ericsson, LG, ZTE) - Option 4: RAN4 for further discussion (Nokia, LG, CMCC, ZTE, Qualcomm) - Option 5: RAN4 requirement is defined based on the indicated measurement periodicity (either OD-SSB periodicity or measCycleSCell for always-on SSB) (vivo)

In our understanding, although UE have different SSB configurations, to maximize the NES gain, similar as case #1, we think UE can also follow OD-SSB periodicity to perform the measurements regardless the DRX and measCycleSCell configuration. But we can also understand the concern from UE side for power saving, so we think the approach in case 1 can also be used to limit the fast SCell measurement within a time window. On the other hand, since we have sent the LS to RAN1 to clarify the relation of always-on SSB and OD-SSB, we can also postpone the decision after receiving the reply LS.

Proposal 4: For the deactivated SCell L3 measurement for case 2, the following options can be considered:

- Option 1: The L3 measurement requirements when OD-SSB is activated are defined based on OD-SSB periodicity ignoring the DRX and measCycleSCell configuration.

- Option 2: Use the same principle as case 1 to limit the fast L3 measurement within a time window.

- Option 3: Postpone the decision after receiving the RAN1 reply LS on the relation between always-on SSB and OD-SSB.

Meanwhile, we noticed that RAN2 is also discussing the L3 measurement framework and the SSB to be used in case 2. To avoid the duplicated discussion and possible misalignment between different groups, we think we can send LS to RAN2 to inform our agreements.

3. L3 RRM Framework - Option1: Based on different measurement configuration when OD-SSB is transmitted - Option2: Based on OD-SSB pattern ignoring SMTC when OD-SSB is transmitted 4. Whether always-on SSB and/or OD-SSB are measured in case 2? Proposal 1: For case #2, RAN2 to discuss which option is followed by UE for RRM measurements of SCell configured with on demand SSB. - Option1: UE uses on-demand SSB for RRM measurements when they are transmitted, otherwise UE uses periodic SSB - Option2: UE uses on-demand SSB for RRM measurements when they are transmitted in addition to periodic SSB for RRM measurements.

Proposal 5: Send LS to RAN2 to inform RAN4 agreements on the measurement framework including:

- Framework for neighbour cell measurement.

- UE measurement behavior on the deactivated SCell on the selected SSB when/before/after OD-SSB indication is received for case 1 and case 2.

SCell activation part

In last meeting, it was agreed that RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A. And the known cell condition principle was also agreed as below:

Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded.

In our understanding, the legacy conditions can be reused without differentiating always-on SSB or OD-SSB which means the conditions can be applied to both case 1 and case 2. But since the measurement before SCell activation is to be used during SCell activation procedure, the SSB used in known condition and SCell activation procedure should be the same or have same frequency and spatial relation. We should discuss how to define such applicability into the specification.

Proposal 6: It should be clarified that the SSB used in known cell condition and SCell activation procedure should be the same or have same frequency and spatial relation.

After defining the known cell condition, the existing requirements framework can be reused by clarifying the signaling processing time, TFirstSSB/TFirstSSB_MAX and the SSB to be used in the SCell activation procedure.

For the SSB to be used in the SCell activation, it was agreed as below in last meeting:

Issue 1-4-5: Which type of SSB is used during SCell activation Agreement: - In Case 1, UE is required to always use OD-SSB for Scell activation. - In Case 2, when both OD-SSB and always-on SSB are transmitted, - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline - The agreement can be revisited if different with other WG

For case 2, we have agreed to assume UE use OD-SSB as baseline but can be revisited if different with other WGs. We have noticed the following agreement in RAN1 which is discussing the association between CSI report configuration and SSB:

Agreement For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB. - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB - FFS: Whether OD-SSB and always on SSB have same beam or not

We understand the discussion above would impact the SSB used for L1 measurement during SCell activation. So we should wait for further progress to decide whether to update our conclusions about case 2. And the above discussion can also be applied to the L1 measurement after SCell is activated. RAN4 should discuss the OD-SSB based L1 measurement requirements for activated SCell if RAN1 made the agreements on above issue.

Proposal 7: RAN4 to discuss whether any updates are needed for case 2 SCell activation based on RAN1 progress on the L1 measurement and the relation between always-on SSB and OD-SSB.

Proposal 8: RAN4 to define OD-SSB based L1 measurement requirements based on RAN1 progress.

For the definition of TFirstSSB/TFirstSSB_MAX which is the time to the end of the first complete SSB burst after SCell activation command, it depends on the discussion on SSB type used in SCell activation. If OD-SSB is used, it can be updated to the end of first actual transmitted SSB to reflect time uncertainty between OD-SSB indication and actual OD-SSB transmission.

Proposal 9: Update the definition of TFirstSSB and TFirstSSB_MAX to the time to the end of first actual transmitted SSB. Whether it can be always-on SSB or OD-SSB depends on the discussion on proposal 7.

For the signalling processing time, the main issue is for scenario #2A. For scenario #2A, the initial understanding is that the OD-SSB indication and SCell activation command are provided by a single MAC CE. But RAN2 has the following agreements in last meeting for scenario #2A which means the two MAC CE signalling can be separate.

RAN2#127bis Agreements on OD-SSB SCell

- Don’t introduce further new MAC CE that combines SCell activation/deactivation and OD-SSB indication for scenario 2A.

Since there is no limitation on the order and time gap between the MAC CE for OD-SSB indication and SCell activation, RAN4 need to discuss whether such additional MAC CE would impact the SCell activation delay requirements and whether to define requirements applicability regarding the time gap between the MAC CE for OD-SSB indication and SCell activation for scenario #2A.

Proposal 10: RAN4 to discuss the RRM impact of separate MAC CE for SCell activation and on-demand SSB indication, e.g., whether separate MAC CE processing time is needed in the activation delay requirements or it can be covered by the waiting time for the first available SSB.

Proposal 11: RAN4 to discuss whether any restriction or requirements applicability are needed for scenario #2A regarding the time gap between the MAC CE for OD-SSB indication and SCell activation.

RAN1 LS on timeline for On-demand SSB operation on SCell

In last meeting, we received the LS from RAN1 on timeline for on-demand SSB operation on SCell. RAN1 discussed the timeline of OD-SSB transmission and defined the starting point of actual OD-SSB transmission, i.e., time instance A, as below. Based on the agreements, RAN1 sent the LS [3] to ask RAN4 to confirm the minimum value of T.

Agreement The previous RAN1 agreement made in RAN1#117 is revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] of on-demand SSB burst [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is at least T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min= where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and is as defined in current specification. - RAN4 to confirm that T_min can be equal to - FFS: Whether the value of T is predefined or indicated/configured by gNB - (Working assumption) T=T_min - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

We understand the value should at least include the time for MAC CE signaling processing and the time waiting for uplink resources which is same as legacy. And according to the RAN1 LS, both aspects have been considered. So we understand it is reasonable to define T_min as m+3Nslotsubframe,  μ+1 and can reply to RAN1 to confirm the understanding.

In last meeting, some companies mentioned that additional UE preparation time for measurements is needed. But we think this time can be parallel with the time for waiting for uplink resources of HARQ-QCK information. And on the other hand, the time instance discussed in RAN1 is the time for network to transmit on-demand SSB rather than the time that UE receives on-demand SSB, so we think as long as network receives the feedback information from UE which means UE have acknowledge of on-demand SSB configuration, it should be the time to transmit the on-demand SSB. Even if more preparation time is needed for UE, it should be considered in the measurement or SCell activation delay requirements in RAN4 rather than to delay the on-demand SSB transmission from network side. Based on the understanding above, we think it can be confirm to RAN1 that T_min is equal to m+3Nslotsubframe,  μ+1 and a draft LS is provided in the Annex.

Proposal 12: Reply LS to RAN1 to confirm that T_min is equal to m+3Nslotsubframe,  μ+1.

3 Summary

In this paper, we have some further discussions on on-demand SSB SCell operation, and the following proposals are given:

Proposal 1: RAN4 to define OD-SSB based SCell activation delay requirements for direct SCell activation at SCell addition for scenario #2A.

Proposal 2: RRC based OD-SSB indication can also be considered in deactivated SCell measurement requirements and SCell activation delay requirements.

Proposal 3: For the deactivated SCell L3 measurement for case 1:

- The fast L3 measurement requirements within time window are defined based on OD-SSB periodicity ignoring the DRX and measCycleSCell configuration.

- The slow L3 measurement requirements after time window are defined as same as legacy, i.e., based on DRX and measCycleSCell configuration.

- The ending point of time window can be the time when UE firstly reports the target OD-SSB based deactivated SCell measurement results after receiving the OD-SSB indication.

Proposal 4: For the deactivated SCell L3 measurement for case 2, the following options can be considered:

- Option 1: The L3 measurement requirements when OD-SSB is activated are defined based on OD-SSB periodicity ignoring the DRX and measCycleSCell configuration.

- Option 2: Use the same principle as case 1 to limit the fast L3 measurement within a time window.

- Option 3: Postpone the decision after receiving the RAN1 reply LS on the relation between always-on SSB and OD-SSB.

Proposal 5: Send LS to RAN2 to inform RAN4 agreements on the measurement framework including:

- Framework for neighbour cell measurement.

- UE measurement behavior on the deactivated SCell on the selected SSB when/before/after OD-SSB indication is received for case 1 and case 2.

Proposal 6: It should be clarified that the SSB used in known cell condition and SCell activation procedure should be the same or have same frequency and spatial relation.

Proposal 7: RAN4 to discuss whether any updates are needed for case 2 SCell activation based on RAN1 progress on the L1 measurement and the relation between always-on SSB and OD-SSB.

Proposal 8: RAN4 to define OD-SSB based L1 measurement requirements based on RAN1 progress.

Proposal 9: Update the definition of TFirstSSB and TFirstSSB_MAX to the time to the end of first actual transmitted SSB. Whether it can be always-on SSB or OD-SSB depends on the discussion on proposal 7.

Proposal 10: RAN4 to discuss the RRM impact of separate MAC CE for SCell activation and on-demand SSB indication, e.g., whether separate MAC CE processing time is needed in the activation delay requirements or it can be covered by the waiting time for the first available SSB.

Proposal 11: RAN4 to discuss whether any restriction or requirements applicability are needed for scenario #2A regarding the time gap between the MAC CE for OD-SSB indication and SCell activation.

Proposal 12: Reply LS to RAN1 to confirm that T_min is equal to m+3Nslotsubframe,  μ+1.

4 References

- R4-2416916, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN4#112bis

- R4-2415662, Topic summary for [112bis][215] Netw_Energy_NR_enh_Part1, Ericsson, RAN4#112bis

- R1-2407565, LS on timeline for On-demand SSB operation on SCell, Apple (RAN1), RAN1#118

- R4-2416913, LS on SSB relation in OD-SSB and SSB adaptation on Scell, Ericsson, RAN4#112bis

Annex draft reply LS

3GPP TSG-RAN WG4 Meeting #113				                                               R4-24xxxxx

Orlando, US, 18th – 22nd November, 2024

Title:	Reply LS on timeline for On-demand SSB operation on SCell

Release:	Release 19

Work Item:	Netw_Energy_NR_enh

Source:	RAN4

To:	RAN1

Cc:

Contact Person:

Name:	Qiuge Guo

E-mail Address: guoqiuge@CATT.CN

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

1. Overall Description:

RAN4 thanks RAN1 for the LS R4-2414911 (R1-2407565) on timeline for On-demand SSB operation on SCell. RAN4 discussed the issue and has the following conclusion:

- RAN4 confirms that T_min is equal to m+3Nslotsubframe,  μ+1.

RAN4 kindly asks RAN1 to take the above answer into account.

2. Actions:

To RAN1:

RAN4 respectfully asks RAN1 to take the above answer into account.

3. Date of Next TSG-RAN4 Meetings:

TSG-RAN4 #114			17 Feb. – 21 Feb. 2025	Athens, Greece

TSG-RAN4 #114bis 	      7 Apr. – 11 Apr. 2025	     China