3GPP TSG-RAN WG4 Meeting #113				                                               R4-2417727

Orlando, US, 18th – 22nd November, 2024

Title:	Discussion on RRM requirements for adaptation of common signal/channel transmission

Source:	CATT

Agenda Item:	7.22.2.2

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the RRM requirements for Rel-19 NES enhancement were discussed. The conclusions for adaptation of common signal/channel transmission were captured in the approved WF [1] and the open issues can be found in the summary [2]. In this paper, we further discuss the potential RRM impact for adaptation of common signal/channel transmission and give our views.

2 Discussion

It was agreed in last RAN4 meeting as below to discuss potential impacts on the following requirements due to SSB adaptation impact.

Issue 1-2-1: Requirements need updating Agreement: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement Note: Above list could be further updated based on RAN4 discussion.

RAN4 has agreed to discuss SSB periodicity adaptation in time domain as starting point according to RAN1 agreements. According to the existing L1/L3 measurement requirements in TS 38.133, we can see that the requirements are based on SSB periodicity or SMTC periodicity. If network only indicate the periodicity adaption in time-domain, the measurement requirements definition would not be impacted. The only potential difference is that this may lead to SMTC reconfiguration which may impact L3 measurement requirements. But since we have sent LS [3] to RAN2 to check the SMTC impact, we can wait for the reply to further discuss the L3 measurement impact.

Proposal 1: RAN4 to discuss the L3 measurement requirements impact due to SSB adaptation after receiving RAN2 reply on the SMTC configuration.

Proposal 2: RAN4 to discuss minimum requirement at transitions for L3 measurement due to SSB adaptation after receiving RAN2 reply on the SMTC configuration.

Proposal 3: For SSB periodicity adaptation in time-domain, the existing L1 measurement requirements can still apply.

The other impact of the SSB periodicity change is that it may introduce new transition requirements for L1/L3 measurement. In the existing specification, the minimum requirement at transitions is defined for L1 and L3 measurement in clause 8.1.4 and 9.1.6 respectively. For L1 measurement, we understand the following transition requirements between different configurations of RLM resource can already accommodate the impact of SSB periodicity adaptation.

### 8.1.4 Minimum requirement at transitions When the UE transitions between DRX and no DRX or when DRX cycle periodicity changes, for each RLM-RS resource, for a duration of time equal to the evaluation period corresponding to the second mode after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation period corresponding to the first mode and the second mode. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second mode for each RLM-RS resource. This requirement shall be applied to both out-of-sync evaluation and in-sync evaluation of the monitored cell. When the UE transitions from a first configuration of RLM resources to a second configuration of RLM resources that is different from the first configuration, for each RLM resource present in the second configuration, for a duration of time equal to the evaluation period corresponding to the second configuration after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation periods corresponding to the first configuration and the second configuration. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second configuration for each RLM resource present in the second configuration. This requirement shall be applied to both out-of-sync evaluation and in-sync evaluation of the monitored cell. When the UE transitions from a first configuration of active TCI state of the CORESET to a second configuration of active TCI state of the CORESET, for each CSI-RS for RLM present in the second configuration, the UE shall use an evaluation period corresponding to the second configuration from the time of transition. This requirement shall be applied to both out-of-sync evaluation and in-sync evaluation of the monitored cell. For UE supporting ncd-SSB-BWP-Wor-r18, when the UE transitions between RLM CD-SSB resource and RLM NCD-SSB resource due to BWP switching or due to SCG activation or deactivation during one evaluation period, the UE shall use an evaluation period that is the maximum of the evaluation periods corresponding to the first SSB type and the second SSB type after the BWP switching or the SCG activation or deactivation. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second SSB type for each RLM-RS resource.

But since the existing transition requirements are defined as no less than the minimum of evaluation periods corresponding to the first configuration and the second configuration which means no requirements, we think it is better to define a new section for SSB adaptation to define the requirements as maximum of evaluation periods corresponding to the first configuration and the second configuration.

Proposal 4: For SSB periodicity adaptation in time-domain, define the minimum requirement at transitions for L1 measurement:

- When SSB periodicity changes upon SSB periodicity adaptation, the UE shall use an evaluation period that is the maximum of the evaluation periods corresponding to the SSB periodicity before and after the adaptation. Subsequent to this duration, the UE shall use an evaluation period corresponding to the SSB periodicity after adaptation.

For SCell activation and TCI state switching, we understand it is not typical case to indicate SSB adaptation during the activation or switching procedure. So no impact due to SSB adaptation is expected for these requirements.

Proposal 5: No impact is expected for SCell activation and TCI state switching due to SSB adaptation.

In RAN1#118 meeting, RAN1 has the following agreements for SSB adaptation:

Agreement - For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’)

Based on the above agreement, the SSB periodicity adaptation on SCell is supported while the adaptation on PCell is FFS. From RAN4 perspective, this would only impact the requirements applicability and RAN4 can wait for more progress to check if the defined requirements apply to PCell.

Proposal 6: The defined requirements due to SSB periodicity adaptation apply to SCell. And RAN4 wait for RAN1 progress on whether the requirements apply to PCell.

3 Summary

In this paper, we have some further discussions on adaptation of common signal/channel transmission, and the following proposals are given:

Proposal 1: RAN4 to discuss the L3 measurement requirements impact due to SSB adaptation after receiving RAN2 reply on the SMTC configuration.

Proposal 2: RAN4 to discuss minimum requirement at transitions for L3 measurement due to SSB adaptation after receiving RAN2 reply on the SMTC configuration.

Proposal 3: For SSB periodicity adaptation in time-domain, the existing L1 measurement requirements can still apply.

Proposal 4: For SSB periodicity adaptation in time-domain, define the minimum requirement at transitions for L1 measurement:

- When SSB periodicity changes upon SSB periodicity adaptation, the UE shall use an evaluation period that is the maximum of the evaluation periods corresponding to the SSB periodicity before and after the adaptation. Subsequent to this duration, the UE shall use an evaluation period corresponding to the SSB periodicity after adaptation.

Proposal 5: No impact is expected for SCell activation and TCI state switching due to SSB adaptation.

Proposal 6: The defined requirements due to SSB periodicity adaptation apply to SCell. And RAN4 wait for RAN1 progress on whether the requirements apply to PCell.

4 References

- R4-2416860, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei, RAN4#112bis

- R4-2415663, Topic summary for [112bis][216] Netw_Energy_NR_enh_Part2, Huawei, RAN4#112bis

- R4-2416911, LS on SSB adaptation, Apple, RAN4#112bis