3GPP TSG-RAN WG4 Meeting #113	                                   R4-2417728

Orlando, US, 18th – 22nd November, 2024

Title: 	Discussion on RRM requirements for LP-WUS/WUR

Source: 	CATT

Agenda Item:	7.23.4.2

Document for:	Discussion

1. Introduction

In last RAN4 meeting, the RRM requirements for R19 LP-WUS/WUR were further discussed. The conclusions were captured in the approved WF [1] and the detailed issue list can be found in the summary [2].

In this contribution, we have some further discussions on RRM requirements for LP-WUS/WUR and give our proposals.

2. Discussion

Criteria (entry/exit conditions)

The discussion of criteria of entry/exit conditions includes the conditions for LP-WUS monitoring, LP-WUR measurement and MR RRM measurement relaxation. Firstly, RAN4 need to align the understanding on the three procedures and our understandings on the entry of three procedures are as below:

- LP-WUR measurement: LR starts serving cell measurement and meets the measurement requirements regardless MR status.

- MR measurement relaxation: MR performs relaxed measurement and meets relaxed requirements when LR is turned on, i.e., the case #3 above.

- LP-WUS monitoring: LR starts monitoring LP-WUS. MR may stop the legacy PO monitoring before LR receives LP-WUS indicating wake-up.

For the entry/exit condition of LP-WUS monitoring, RAN1 and RAN2 had the following conclusions:

RAN1#116bis meeting: Working Assumption - From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion. - RAN2#126 meeting: - Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. - Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS. - RAN2#127 meeting: - Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS. - Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. - The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. - The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ. RAN2#127bis meeting: - If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s). - The LPWUS monitoring exit condition does not include MR measurements.

And for RRM relaxation and offloading in RRC_IDLE/INACTIVE, RAN2 had the following agreements in RAN2#126 and RAN2#127 meeting:

- RAN2#126 meeting: - For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. - RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. - RAN2 will further discuss the neighbour cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’. - RAN2#127 meeting: - RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results.

We can see that RAN1/2 already had initial agreements on the criteria for LP-WUS monitoring, RRM measurement relaxation and fully offloading. And from the previous experience, the criteria discussion is usually within RAN2 scope. So we think we can follow the RAN1/2 conclusions and focus on the requirements discussion. Any further discussion can be triggerred by RAN2 LS.

For the entry/exit conditions for LP-WUR measurement which is not discussed in RAN1/2, we think when to turn on the LR and start LP-WUR measurement is UE implementation, and what we should consider is when UE can relax or stop the MR measurements. And even when the measurement is performed by MR, UE is not mandatory to turn off LR and stop LR measurement. Based on such understanding, we think we do not need to define the entry/exit condition for LP-WUR measurement.

Proposal 1: Do not define entry/exit conditions for LP-WUR measurement.

Proposal 2: The criteria discussion for LP-WUS monitoring, RRM measurement relaxation and fully offloading  can be left to RAN2, and RAN4 to focus on the LP-WUR measurement requirements and MR measurement relaxation requirements.

LP-WUR measurement requirements in RRC_IDLE/INACTIVE state

According to the existing cell reselection requirements, the UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell within a certain cycle. But for fully offloading case, the MR measurements are stopped and no neighbour cell measurement is performed. In such case, whether the evaluation of cell selection criterion is still needed should be discussed. In our understanding, although only LR measurement is performed in such case, the serving cell measurement should be still guaranteed and the cell selection criterion S should be evaluated. With this assumption, we think the similar evaluation requirements can be defined based on LP-WUR measurement. And the existing evaluation requirements can be the baseline.

Proposal 3: RAN4 to define evaluation requirements for cell selection criterion S based on LR measurement for fully offloading case, and the existing MR based evaluation requirements can be the baseline.

Besides the above requirements, we need to define measurement period requirements for LR. In previous meeting, it was agreed both LP-SS and SSS based serving cell measurement requirements need to be defined.

For LP-SS based measurement, the following agreements were reached in last meeting.

Issue 1-2-5: Nubmer of Rx for LP-WUR requirements Agreement Single Rx is the baseline for LR-WUR RRM measurement. Issue 1-2-6: LP-SS based RRM requirements periodicity and filtering Agreement: Measurement delay requirements for LP-WUR based on LP-SS in IDLE/Inactive mode shall be defined based on LP-SS periodicity.

Based on the agreements above, the existing SSB based measurement requirements can be used as baseline except that DRX cycle can be replaced by LP-SS periodicity and 1Rx impact can be considered. For 1Rx impact, the principle in existing RedCap measurement requirements can be reused.

For the SSS based measurement, we think the existing intra-frequency cell reselection requirements can be used as baseline. But since the DRX configuration is not applied to LP-WUR, we think a fixed value or network configured measurement cycle can be used to define the measurement requirements. If LP-SS is configured simultaneously, it should be also fine to use LP-SS periodicity. And the side condition can be further discussed following RAN1 conclusions on the SNR and the RAN4 evaluation on accuracy requirements.

Proposal 4: For LP-SS based LP-WUR measurement:

- The existing intra-frequency cell reselection requirements can be used as baseline with the DRX cycle replaced by LP-SS periodicity.

- The principle for 1Rx impact in existing RedCap measurement requirements can be used as baseline.

- The side condition can be further discussed based on the RAN4 evaluation on accuracy requirements.

Proposal 5: For SSS based LP-WUR measurement,

- The existing intra-frequency cell reselection requirements can be used as baseline with the DRX cycle replaced by a fixed or network configured measurement cycle.

- The principle for 1Rx impact in existing RedCap measurement requirements can be used as baseline.

- The side condition can be further discussed based on the RAN4 evaluation on accuracy requirements.

And considering UE need to evaluate the entry/exit condition for LP-WUS monitoring, MR measurement relaxation and/or fully offloading, the evaluation period requirements may also be needed. We understand the evaluation period requirements can be the same as the measurement requirements and further discuss whether to define measurement filtering (number of samples and separation between samples).

Proposal 6: RAN4 to define requirements for entry/exit criteria evaluation for LP-WUS monitoring, MR RRM relaxation and fully offloading based on LP-WUR measurement period, and further discuss whether to define measurement filtering (e.g., number of samples).

In last meeting, there was also proposal to define interruption requirements due to the time gap between LP-WUS reception and MR to start PDCCH monitoring. For this time gap, RAN1 has the following agreements:

- RAN1#118bis - Agreement - From RAN1 perspective, when a UE is monitoring LP-WUS (based on the entry/exit condition for LP-WUS), a UE is not required to monitor a PO if - it does not detect a LP-WUS on the monitored LO - or the LP-WUS does not indicate a wake-up indication for the UE’s corresponding subgroup - [RAN1#118]: - Agreement - For the wake-up delay, consider the following options: - Option 2: UE reports one value from X candidate values for the wake-up delay via UE capability reporting. - FFS: X is 2, 3, 4 - Above applies for IDLE/INACTIVE mode. - Definition of wake-up delay: Minimum gap time between LP-WUS reception and MR to start PDCCH monitoring - [RAN1#118bis]: - Agreement - Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state. - FFS the values - Note: Ultra-deep sleep state and deep sleep state are mentioned above solely for the purpose of RAN1 discussions and it is not intended to be captured in specifications from RAN1 perspective

According to the agreements above, the time gap is based on UE report but the value is still FFS. As to whether to define interruption requirements due to the time gap, we prefer to wait for RAN1 progress on the candidate values and the possible components.

Proposal 7: Regarding whether to define interruption requirements due to the time gap between LP-WUS reception and MR to start PDCCH monitoring, wait for RAN1 further progress on the candidate values and the possible components.

MR RRM relaxation for serving cell/neighbour cell

For MR RRM relaxation requirements, it was agreed to define relaxation requirements for both serving cell and neighbour cell measurements. But the detailed requirements are still open.

Issue 1-1-3: Core requirements to be specified for MR RRM relaxation Agreement RAN4 specifies relaxed UE MR measurement requirements for both serving cell and neighbour cell measurements at idle/inactive mode for UE supporting LP-WUR

For neighbour cell measurement relaxation, we have defined some relaxed requirements in Rel-16 and Rel-17 power saving, and there are proposals in previous meeting to further relax the requirements comparing to legacy relaxation factor. But we think it depends on the criteria of neighbour cell measurement relaxation and RAN2 has the following working assumption in RAN2#127bis meeting:

- Working assumption For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

RAN2 only agreed to use not at cell edge criterion, and the low mobility criterion and any enhancements are FFS. In our understanding, if we only reuse the legacy criteria and do not include any condition of LR measurement, the effectiveness of LP-WUR cannot be guaranteed and it is not reasonable to further relax the neighbour cell measurement requirements without any new enhancements. And even if LR measurement is included, based on the design of LP-WUR, it can only be used for serving cell measurements, so it is not clear how this new LR measurement can further relax the neighbour cell measurement. But if new criteria considering the LR measurement are designed or new thresholds are introduced, to maximize the power saving gain, the relaxation factor for neighbour cell measurement can be further increased ( e.g., 8). Based on the discussion, we think it is better to wait for RAN2 further progress on MR measurement relaxation criteria to define the requirements.

For the serving cell measurement, there are no relaxed measurement requirements before, but we think the principle when defining neighbor cell measurement relaxation can be reused, i.e., to introduce a relaxation factor based on legacy requirements. Regarding whether to use same relaxation factor with neighbour cell, we think it also depends on the relaxation criteria design. If the same criteria are used as neighbour cell measurement, i.e., when UE meets certain conditions, both serving cell and neighbour cell measurement can be relaxed, to maximum the power saving gain and simplify the requirements, we are fine to use the same relaxation factor.

Proposal 8: For MR neighbor cell measurement relaxation, the existing relaxed requirements in 4.2.2 can be used as baseline. Whether to further relax the requirements can be discussed after the criteria are defined by RAN2.

Proposal 9: For MR serving cell measurement relaxation, a scaling factor can be introduced on existing serving cell measurement requirements in 4.2.2. Wait for RAN2 progress on relaxation criteria to decide whether to use same scaling factor as neighbour cell measurement.

LP-WUS operation in RRC_CONNECTED state

Based on the WID as copied below, no LP-WUR measurement is requested and the UE RRM/RLM/BFD/CSI measurements are performed by MR in for CONNECTED mode. So no LP-WUR measurement requirements are needed and no RAN4 impact is expected.

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

RAN1 has discussed LP-WUS operation in RRC_CONNECTED state, and has the following agreements:

- RAN1#116: - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance - Minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following： - LP-WUS processing time - MR transition time for ramp up - Time/frequency synchronization of MR - RAN1#117: - For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring. - RAN1#118bis: - Support Alt 1: For RRC CONNECTED mode, UE reports one value for each SCS from X candidate values for the determination of the minimum time gap between LP-WUS reception and MR to start PDCCH monitoring via UE capability reporting - FFS: X - FFS: definition of reported candidate value - UE can indicate its preference for configured time offset between LP-WUS and PDCCH reception using UAI mechanism. FFS details. UAI is optional. - FFS: A single time offset is configured for a UE at a given time

From the agreements, we can see that the only requirements aspect is the time gap between LP-WUS reception and PDCCH monitoring. And this has been defined as UE capability in RAN1. RAN1 will decide the candidate values and no RAN4 impact is expected so far.

Proposal 10: For LP-WUS operation in RRC_CONNECTED state, no RAN4 impact is expected so far.

RAN1 LS

In last meeting, RAN4 received one LS [3] from RAN1 to check the applicability and specification support needed for LP-WUS reception.

In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

RAN4 discussed the LS and had the following agreement:

Issue 1-1-8: LP-WUR operating carrier frequency Agreement: - RAN4 will further discuss whether any technical issues regarding the LS from RAN1. - From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2. Issue for further discussion based on contribution driven: - For WUS monitoring assumption, further discuss whether or not the same carrier frequency is assumed for LR and MR.]

It was agreed that RAN4 assumed LR and MR are operating on the same carrier frequency as baseline from Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective. For the other cases where MR and LR work on the different carrier frequencies, we think there is no need to consider in Rel-19.

For the WUS monitoring assumption raised in last meeting, some companies mentioned that it may be possible to perform WUS monitoring on one carrier to indicate the UE behavior on multiple other CCs if no RRM measurement is needed. We understand this may be possible for CONNECTED mode e.g., CA mode, but for IDLE/INACTIVE mode, UE can only have at most one serving cell and different carriers mean different neighbour cells. We understand it may not reasonable to indicate UE behaviour on neighbour cells by only WUS monitoring without any measurements. So we think we do not need to consider this case in Rel-19, i.e., only consider the case when MR and LR are operating on the same carrier frequency. The corresponding conclusion can be informed to RAN1 by sending the response LS.

Proposal 11: RAN4 only consider the case when MR and LR are operating on the same carrier frequency in Rel-19 and send response LS to RAN1 to inform the conclusion.

3. Conclusion

In this paper, we provide our views on RRM core requirements for R19 LP-WUS/WUR. From this discussion we have derived the following observations and proposals:

Proposal 1: Do not define entry/exit conditions for LP-WUR measurement.

Proposal 2: The criteria discussion for LP-WUS monitoring, RRM measurement relaxation and fully offloading  can be left to RAN2, and RAN4 to focus on the LP-WUR measurement requirements and MR measurement relaxation requirements.

Proposal 3: RAN4 to define evaluation requirements for cell selection criterion S based on LR measurement for fully offloading case, and the existing MR based evaluation requirements can be the baseline.

Proposal 4: For LP-SS based LP-WUR measurement:

- The existing intra-frequency cell reselection requirements can be used as baseline with the DRX cycle replaced by LP-SS periodicity.

- The principle for 1Rx impact in existing RedCap measurement requirements can be used as baseline.

- The side condition can be further discussed based on the RAN4 evaluation on accuracy requirements.

Proposal 5: For SSS based LP-WUR measurement,

- The existing intra-frequency cell reselection requirements can be used as baseline with the DRX cycle replaced by a fixed or network configured measurement cycle.

- The principle for 1Rx impact in existing RedCap measurement requirements can be used as baseline.

- The side condition can be further discussed based on the RAN4 evaluation on accuracy requirements.

Proposal 6: RAN4 to define requirements for entry/exit criteria evaluation for LP-WUS monitoring, MR RRM relaxation and fully offloading based on LP-WUR measurement period, and further discuss whether to define measurement filtering (e.g., number of samples).

Proposal 7: Regarding whether to define interruption requirements due to the time gap between LP-WUS reception and MR to start PDCCH monitoring, wait for RAN1 further progress on the candidate values and the possible components.

Proposal 8: For MR neighbor cell measurement relaxation, the existing relaxed requirements in 4.2.2 can be used as baseline. Whether to further relax the requirements can be discussed after the criteria are defined by RAN2.

Proposal 9: For MR serving cell measurement relaxation, a scaling factor can be introduced on existing serving cell measurement requirements in 4.2.2. Wait for RAN2 progress on relaxation criteria to decide whether to use same scaling factor as neighbour cell measurement.

Proposal 10: For LP-WUS operation in RRC_CONNECTED state, no RAN4 impact is expected so far.

Proposal 11: RAN4 only consider the case when MR and LR are operating on the same carrier frequency in Rel-19 and send response LS to RAN1 to inform the conclusion.

4. Reference

- R4-2416861, WF on RRM requirements for NR_LPWUS, vivo, RAN4#112bis

- R4-2415664, Topic summary for [112bis][217] NR_LPWUS, vivo, RAN4#112bis

- R4-2414909 (R1-2407559), LS on LP-WUS operation in IDLE/INACTIVE mode, Apple (RAN1), RAN4#112bis

Annex draft reply LS

3GPP TSG-RAN WG4 Meeting #113	                         R4-24xxxxx

Orlando, US, 18th – 22nd November, 2024

Title:	Reply LS on LP-WUS operation in IDLE/INACTIVE mode

Release:	Release 19

Work Item:	NR_LPWUS

Source:	RAN4

To:	RAN1, RAN2

Cc:

Contact Person:

Name:	Qiuge Guo

E-mail Address: guoqiuge@CATT.CN

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments: -

1. Overall Description:

RAN4 thanks RAN1 for the LS R4-2414909 (R1-2407559) on LP-WUS operation in IDLE/INACTIVE mode. RAN4 discussed the issue and has the following conclusion:

- RAN4 only consider the case when MR and LR are operating on the same carrier frequency in Rel-19.

RAN4 kindly asks RAN1 to take the above information into account.

2. Actions:

To RAN1:

RAN4 respectfully asks RAN1 to take the above information into account.

3. Date of Next TSG-RAN4 Meetings:

TSG-RAN4 #114			17 Feb. – 21 Feb. 2025	    Athens, Greece

TSG-RAN4 #114bis 	      7 Apr. – 11 Apr. 2025	     China