3GPP RAN WG4 Meeting #113																	R4-2417749

Orlando, USA, 18th – 22nd November 2024

Agenda item:	7.8.3.2

Source:	Apple

Title:	On UE RF requirements for the 3MHz channel

WI/SI:	NR_IoT_NTN_req_test_enh

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications. And since that time the smallest channel bandwidth was 5MHz, all NTN bands have had only 5MHz as the smallest bandwidth.

After the RAN#104 meeting a new WI was agreed [2], one of the aims of which is to enable the 3MHz channel for the NTN bands. Based on the WF document from the previous RAN4#112bis meeting [3] there are several open issues for the UE Rx and Tx requirements, for which this discussion paper presents more technical considerations.

# 2	UE RF requirements for the 3MHz NTN channel

## 2.1	Rx requirements

One of the open Rx issues is which REFSENS values to adopt for the 3MHz channel in the existing NTN bands: n254, n255 and n256.  As summarised in Table 2.1-1 below, the NTN bands n256 and n254 belong to the same frequency range and thus can have the same REFSENS, as in fact was already agreed for the 5MHz channel in these NTN bands. And the value of -101.2dBm is chosen so that it is aligned with existing LTE band 65 as defined in TS 36.101. As for the NTN band n255, since its DL resides in the lower frequency ranges, it may have better REFSENS as it is already the case for the 5MHz channel.

Table 2.1-1: Proposed REFSENS values for the existing NTN bands.

| NTN band | REFSENS value | Comment |
| --- | --- | --- |
| n256 | -101.2dBm | Aligned with LTE band 65 from TS 36.101 |
| n254 | -101.2dBm | Aligned with the NTN band n256 because of the same DL frequency range |
| n255 | -101.7dBm |  |

Proposal 1a:	REFSENS values for the 3MHz channel in existing NTN bands n254, n255 and n256 are -101.2dBm, -101.7dBm and -101.2dBm, respectively.

It is worth noting that in Rel-19 new NTN bands will be standardised, where the 3MHz channel can be also deployed. If so, the REFSENS values for those bands can be as the ones presented in Table 2.1-2 below.

Table 2.1-2: Proposed REFSENS values for the new NTN bands.

| NTN band | REFSENS value | Comment |
| --- | --- | --- |
| n252 | -101.2dBm | Aligned with the NTN band n256 |
| n251 | -101.7dBm | Aligned with the NTN band n255 as these bands share the same DL frequency range |
| n250 |  |  |

Proposal 1b:	REFSENS values for the 3MHz channel in new NTN bands n252, n250/n251 are -101.2dBm and -101.2dBm, respectively.

## 2.2	Tx requirements

Last meeting it was discussed which spurious emission requirements should be adopted for the 3MHz channel. Our general understanding is that the baseline is the existing spurious emission requirements because they come from the internation regulations, such as ITU-R SM.329-13. There, of course, can exist additional band specific spurious emission requirements, e.g. ETSI requirements for the NTN band n254. In this case additional spurious emission requirements (which will also apply to the 3MHz channel) can be associated with the existing NS flags.

Proposal 2a:	Existing general spurious emission requirements are adopted for the 3MHz channel.

Proposal 2b:	Additional band specific emission requirements, if any, can be associated through the existing NS flags.

Another open issue for the UE Tx requirements was the potential UE power back-off and A-MPR values. In general, there should be band-by-band analysis on whether A-MPR is needed for the 3MHz channel in the given NTN band. At least for the NTN band n254 our preliminary analysis in [4] indicates that some A-MPR is needed for at the lower and upper edges of the band.  Similarly, power back-off analysis for the NTN band n256 also indicate that band n256 needs A-MPR for the 3MHz channel [5].

Proposal 2c:	Whether A-MPR is needed for the 3MHz channel depends on the given NTN band and should be analysed accordingly.

Proposal 2d:	The 3MHz channel needs A-MPR at least in case of band n254 and n256.

# 3	Conclusions

In this discussion paper we have provided our input of the UE RF requirements for the 3MHz NTN channel. For the Rx requirements, we propose REFSENS values for the existing NTN bands – n254, n255 and n256 – whereupon the proposed values are aligned with the existing requirements. And since new NTN bands, such as n250/n251 and n252, share the same frequency range as the existing bands, the corresponding values can be also considered for new bands. As for the Tx requirements, existing spurious emission requirements can be adopted with band specific requirements, if any, enabled through the corresponding NS flags. As for A-MPR, at least band n254 and n256 need some A-MPR also for the 3MHz channel.

Proposal 1a:	REFSENS values for the 3MHz channel in existing NTN bands n254, n255 and n256 are -101.2dBm, -101.7dBm and -101.2dBm, respectively.

Proposal 1b:	REFSENS values for the 3MHz channel in new NTN bands n252, n250/n251 are -101.2dBm and -101.2dBm, respectively.

Proposal 2a:	Existing general spurious emission requirements are adopted for the 3MHz channel.

Proposal 2b:	Additional band specific emission requirements, if any, can be associated through the existing NS flags.

Proposal 2c:	Whether A-MPR is needed for the 3MHz channel depends on the given NTN band and should be analysed accordingly.

Proposal 2d:	The 3MHz channel needs A-MPR at least in case of band n254 and n256.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"

- R4-2416587, "Way Forward for [112bis][311] NR_IoT_NTN_less_than_5MHz_UERF ", Xiaomi

- R4-2417750, "Power back-off values for the 3MHz channel in band n254", Apple, Globalstar

- R4-2417751, "Power back-off values for the 3MHz channel in band n256", Apple