3GPP RAN4 Meeting #113		R4-2417758

Orlando, USA, November 18-22, 2024

Agenda item:	7.12.3.1

Source:	Apple

Title:		On Rel-19 TRP TRS performance aspects

WI/SI:	TRP_TRS_MIMO_OTA_Ph3-Perf

Release:	Rel-19

Document for:	Discussion

# 1	Introduction

During the RAN #103 meeting a new Work Item on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3 was approved with the following scope related to the performance part of the work focused on TRP TRS:

Objectives for this performance part work item are as follows - Specify TRP and TRS requirements: - Specify TRP and TRS requirements for new bands for UE based on operators demand - Handheld UEs with 1Tx at bands n3, n5, n7, n8, n77, n79 are first priority - Handheld UEs with 2Tx (non-coherent UL MIMO) at n41 and n78 depending on measurement data availability are first priority. Additionally, n77 and n79 are second priority. - Headworn XR OTA requirements will not be considered until development of suitable head phantom and sufficient number of commercial devices become available - Coherent UL MIMO UE OTA requirements will not be considered until sufficient number of commercial devices become available

The specific objectives for the TRP TRS Performance part as of RAN4#113 were also clearly laid out in [3]

This contribution provides our views related to the objectives for TRP TRS performance part listed above.

# 2	Discussion

## 2.1	Inputs to Rel-19 lab alignment campaign

Observation 1:	The RAN4#113 meeting is anticipated expected to trigger further refinement of the lab alignment work plan for Release 19.

The way forward in [3] from RAN4#112-bis provides the working procedure for Rel-19 AC 2Tx lab alignment campaign. However, some items were still left open and need to be closed.

Proposal 1: For the 2Tx (non-coherent UL-MIMO) lab alignment portion the following open items can be closed

- The Rel-19 WID clearly calls for n41 and n78 as bands of focus and therefore these can be scope of the lab alignment campaign for non-coherent UEs.

- Use Scenario: As with previous lab alignment campaigns the focus can be only on one use scenario – Talk Mode (if aligning with requirements creation) OR Browse Mode (to exercise both hand and head phantoms).

## Size 1 UE Requirements: Band coverage for 1Tx and 2Tx Performance Requirements in Rel-19

#### 2.2.1	Band coverage for 1Tx and 2Tx Performance Requirements in Rel-19

During the work item scoping discussion in RAN #103, considerations of RAN4 workload drove a significant reduction in the overall work scope, when compared to the RAN4 work package proposals reviewed in RAN #102.  However, this principle was not reflected well in the scope of work for the performance phase of TRP/TRS.  For FR1 TRP/TRS (1Tx) a total of 6 bands (n3, n5, n7, n8, n77, and n79) are listed as priority bands.  Considering that RAN4 specified TRP/TRS requirements for only 2 bands in Rel-17 and for only 4 bands in Rel-18, 6 bands is clearly too many bands.  Furthermore, the Rel-19 performance work scope also includes 2 bands for 2Tx (non-coherent UL MIMO) radiated power:  these are bands n41, n78 as the priority.  With the understanding that 2Tx performance work will include lab alignment procedures for the new metric not previously tested by 3GPP, the overall time needed for the performance phase of 1Tx (6 bands) plus 2Tx (2 bands, including lab alignment procedures) is clearly much more than is available in Rel-19. This is further complicated by Size 2 (device width <72mm) UEs not precluded from the performance campaign as per the LS response to GCF in [4]. A total number of 1Tx and 2Tx bands that can be feasibly covered in the performance campaign should be highlighted at this stage so that planning for performance campaign can proceed.

Proposal 2:	Further reduction in the number of bands for Size1 UE TRP/TRS performance part of the work is needed (for example: 2-3 total bands for 1Tx along with the 2 bands for 2Tx).  The outcome should be captured in a revision of the WID.

Observation 2:	The actual bands of interest for Rel-19 performance campaign is to be finalized is based on operator and other company inputs on the priority list that is included in the WID.

#### 2.2.2	Browse-Mode only focus for 2Tx Performance Requirements in Rel-19

It would be critical to perform a detailed evaluation of the use cases associated with 2Tx/Uplink-MIMO scenarios for which performance requirements will be created in Release 19. Rather than extend the test points from 1Tx cases (which at a high-level cover both Browsing Mode and Talk Mode scenarios), the focus should be on specific test requirements which reflect UE usage in the field and real-life.

Uplink MIMO is generally more suited and tailored for high data rate applications rather than traditional voice calls. There are some reasons, including but not limited to, listed below:

High Data Rate Requirements

Voice Calls (VoNR or VoLTE): Voice calls have relatively low data rate requirements, even with advanced voice codecs. The channel candwidth is quite minimal and achieved using a single data stream. On the other hand, high data rate applications such as video/calls/streaming, internet browsing, gaming specifically benefit from the increased throughput provided by uplink MIMO.

QoS factors

For voice services, the QOS requirements are mainly focused on latency and reliability rather than high data rates. UL-MIMO improves capacity and throughput and does not directly contribute to better voice call quality for traditional voice-based services

Real network Conditions

UL-MIMO requires good network conditions to enable multiple spatial streams effectively which is characterized by high SNR and minimal interferences and more aligned with data service usage. Voice services on the other hand are latency sensitive.

Network Efficiency

Allocating network resources (bandwidth, capacity) via uplink MIMO is highly inefficient for low bandwidth voice calls. The network generally reserves uplink MIMO for applications where it helps enable an enhanced user experience such as boosting data rates for applications that demand it.

In addition to the above, any upcoming high definition or ultra-HD voice which requires more bandwidth compared to standard voice codecs is probably associated with applications more aligned with browsing mode. Or related to XR devices which is a separate category and still focused on applications where voice and data are concurrently enabled and therefore more aligned with browsing mode.

It is therefore clear from the above use case analysis that there is no value in testing a non-coherent UL-MIMO enabled UE in talk mode. It would therefore be more prudent to align the testing and requirments creation with real-world scenarios and focus on browsing mode requirements only.

Proposal 3:	Focus only Hand-Only/Browsing Mode for 2Tx/non-coherent Uplink MIMO performance requirements.

## Size 2 UE Requirements: Limited Performance Campaign

The WF in [4] provided guidance on the possible approaches to creating requirements for Size2 UEs (PDA hand)

With reference to the offset-based approach, there is no clear procedure or approach that has been identified to pursue. Based on previous discussion the key data point would be drawn from the limited measurement campaign. In which case, the measurement campaign might as well be utilized to come up with absolute TRP TRS requirements rather than a derivative offset which might be confusing to the ecosystem. Therefore, we propose to focus the energy on the limited measurement campaign

Proposal 4:	Focus on the limited measurement campaign to define Size 2 UE 1Tx requirements.

The parameters of the limited performance campaign can be defined in order to proceed with the same. To provide time for volunteer labs to procure Size2 UEs a reasonable timeframe can be defined (keeping in mind the workplan for Rel-19 OTA WI allows measurement campaign to proceed and conclude RAN4#118 – February 2026).

Proposal 5:	Use the following parameters for the limited measurement campaign for Size 2 UE 1Tx requirements

-	At least 15 devices

-	Head and Hand Phantom/Talk Mode

-	1 Band to Test: n78

- 	Device Year of Production: 2022-2025

-    Encourage volunteer test labs to perform the above measurement campaign, Rel-17 and/or Rel-18 aligned test lab is preferred (Already Agreed)

-	Start campaign after RAN4#113 and provide all data by RAN4#116 (August 2025). Interim data can be shared earlier.

-	Discuss further on whether existing approach of volunteer labs sharing device information only with RAN4 secretary can be re-used for this Size2 UE limited campaign.

(Note for reference: Workplan for Rel-19 OTA WI allows 1Tx Size1 measurement campaign to proceed and conclude RAN4#118 – February 2026)

# 3	Conclusions

This contribution provides our views on the topic of TRP/TRS performance work scope.  The following observation and proposal are made:

Observation 1:	The RAN4#113 meeting is anticipated expected to trigger further refinement of the lab alignment work plan for Release 19.

Observation 2:	The actual bands of interest for Rel-19 performance campaign is to be finalized is based on operator and other company inputs on the priority list that is included in the WID.

Proposal 1: For the 2Tx (non-coherent UL-MIMO) lab alignment portion the following open items can be closed

	The Rel-19 WID clearly calls for n41 and n78 as bands of focus and therefore these can be scope of the lab alignment campaign for non-coherent UEs.

	Use Scenario: As with previous lab alignment campaigns the focus can be only on one use scenario – Talk Mode (if aligning with requirements creation) OR Browse Mode (to exercise both hand and head phantoms).

Proposal 2:	Further reduction in the number of bands for Size1 UE TRP/TRS performance part of the work is needed (for example: 2-3 total bands for 1Tx along with the 2 bands for 2Tx).  The outcome should be captured in a revision of the WID.

Proposal 3:	Focus only Hand-Only/Browsing Mode for 2Tx/non-coherent Uplink MIMO performance requirements.

Proposal 4:	Focus on the limited measurement campaign to define Size 2 UE 1Tx requirements.

Proposal 5:	Use the following parameters for the limited measurement campaign for Size 2 UE 1Tx requirements

-	At least 15 devices

-	Head and Hand Phantom/Talk Mode

-	1 Band to Test: n78

- 	Device Year of Production: 2022-2025

-    Encourage volunteer test labs to perform the above measurement campaign, Rel-17 and/or Rel-18 aligned test lab is preferred (Already Agreed)

-	Start campaign after RAN4#113 and provide all data by RAN4#116 (August 2025). Interim data can be shared earlier.

-	Discuss further on whether existing approach of volunteer labs sharing device information only with RAN4 secretary can be re-used for this Size2 UE limited campaign.

(Note for reference: Workplan for Rel-19 OTA WI allows 1Tx Size1 measurement campaign to proceed and conclude RAN4#118 – February 2026).

# 4	References

RP-240841, “New WID: WI on TRP (Total Radiated Power), TRS (Total Radiated Sensitivity) and MIMO OTA (Over the Air) testing enhancement Phase 3,” Qualcomm, 3GPP RAN #103, March 2024

RP-2406086, “Way Forward for [110bis][338] TRP_TRS_MIMO_OTA”, Vivo, 3GPP RAN #110-bis, April 2024

R4-2406084, “Workplan of Rel-19 OTA WI”, Vivo; 3GPP RAN #112-bis, October 2024

R4-2416590, “Way Forward for [112bis][323] TRP_TRS_Ph3”, Vivo, CAICT; 3GPP RAN #110-bis, April 2024

R4-2413537, “Reply LS on 3GPP NR TRP TRS OTA requirements”, Vivo, Apple, 3GPP RAN #112, August 2024