3GPP RAN WG4 Meeting #113		R4-2417766

Orlando, USA, November 18th – 22nd, 2024

Agenda item:	8.2

Source:	Apple

Title:	On FR2-NTN inclusion to specifications

WI/SI:	NR_NTN_enh-Core

Release:	Rel-18

Document for:	Approval

# 1	Introduction

RAN4 received an LS from RAN1 on “FR2-NTN inclusion to specifications” [1] to seek for RAN4’s clarifications on the following two questions:

- RAN1 identified that some RAN1 specifications are still unclear regarding whether FR2-1 or FR2-2 procedure should apply to the FR2-NTN, as the current RAN1 specifications define different behaviour between FR2-1 and FR2-2.

- Furthermore, RAN1 understands that NOTE 2 in Table 5.1-1 of 38.101-5 states that FR2-NTN are regarded as FR2 bands, likely to minimize the impact when referencing RAN1 and RAN2 specifications. RAN1 would like to ask RAN4 to consider whether NOTE 2 may need to be updated.

In last RAN4 meeting, though it was agreed to confirm to RAN1 that FR2-NTN follows FR2-1 procedure, there was no consensus on the handling of NOTE 2 in Table 5.1-1 of 38.101-5 where a few options were considered which are captured in the approved “WF on FR2-NTN inclusion to specifications” [2]. In this contribution, we share our view and preference on the handling of NOTE 1 and NOTE 2 in Table 5.1-1 of 38.101-5.

# Discussion

In response to RAN1’s second question on whether NOTE 2 in Table 5.1-1 of 38.101-5 as recaptured in Table 2-1 may need to be updated, RAN4 in general agreed some update is needed and the following options had been considered in last RAN4 meeting [2].

| Frequency range designation | Corresponding frequency range |
| --- | --- |
| FR1-NTN1 | 410 MHz – 7125 MHz |
| FR2-NTN2 | 17300 MHz – 30000 MHz |
| NOTE 1: [NTN bands within this frequency range are regarded as a FR1 band when references from other specifications.] NOTE 2: [NTN bands within this frequency range are regarded as a FR2 band when references from other specifications.] |  |

Table 2-1 Definition of NTN frequency ranges

- Option 1: Remove both Note 1 and Note 2

- Option 2: Keep Note 1 and remove Note 2

- Option 3a: Revise Note 2 (FR2 -> FR2-1)

- Option 3b: Update NOTE 2 to “NTN bands within this frequency range are to follow FR2-1 band procedure when referenced from other specifications.”

- Option 3c: Update NOTE 2 to:	In other specifications, when FR2-NTN is not explicitly defined, NTN bands within this frequency range are regarded as FR2, or FR2-1 when there is a differentiation between FR2-1 and FR2-2.”

- Option 3d: Update NOTE 2 to “NTN bands within this frequency range are regarded as a FR2 band when referenced from other specifications, if applicable.”

In our view, though FR1-NTN frequency range and system parameters are well aligned with the terrestrial network FR1 specifications, the FR2-NTN frequency range however does not fully correlate with the terrestrial network FR2 nor FR2-1 range, therefore, having NOTE 2 referencing to either FR2 or FR2-1 band may cause some confusion. If FR2-NTN frequency range can be confirmed to follow the FR2-1 procedure to RAN1 and RAN2, then NOTE 2 may be removed to avoid the potential confusion with FR2 or FR2-1 (Option 1). Or another alternative is to update the note to as “NTN bands within this frequency range are to follow FR2-1 band procedure when referenced from other specifications.” (Option 3b)

Observation: FR2-NTN frequency range does not fully correlate with the terrestrial network FR2 nor FR2-1 range, having NOTE 2 referencing to either FR2 or FR2-1 band may cause some confusion.

Proposal: Option 1 or Option 3b

Option 1: Remove both Note 1 and Note 2.

Option 3b: Update NOTE 2 to “NTN bands within this frequency range are to follow FR2-1 band procedure when referenced from other specifications.”

# 3	Conclusion

In this contribution, we share our view and preference on the handling of NOTE 1 and NOTE 2 in Table 5.1-1 of 38.101-5.

Observation: FR2-NTN frequency range does not fully correlate with the terrestrial network FR2 nor FR2-1 range, having NOTE 2 referencing to either FR2 or FR2-1 band may cause some confusion.

Proposal: Option 1 or Option 3b

Option 1: Remove both Note 1 and Note 2.

Option 3b: Update NOTE 2 to “NTN bands within this frequency range are to follow FR2-1 band procedure when referenced from other specifications.”

# 4	References

R1-2407406 “LS on FR2-NTN inclusion to specifications”, RAN1, 3GPP TSG-RAN WG1 Meeting #118, Maastricht, Netherlands, August 19th – 23rd, 2024

R4-2417185 “WF on FR2-NTN inclusion to specifications”, Apple, 3GPP TSG RAN WG4 Meeting #112bis, Hefei, China, October 14th – 18th, 2024