3GPP TSG-RAN WG4 Meeting #113	                                 R4-2417784

Orlando, USA, November 18 – November 22, 2024

Source: 	Murata Manufacturing Co Ltd.

Title: 	Further discussion on HPUE MSD rules

Agenda item:	7.1.1.1.1

Document for:	Discussion

Introduction

In the last RAN4#112 meeting, the general MSD rules for HPUE was discussed and the WF [1] was approved. One way-forward option is that RAN4 specifies or provides equations/LUT/simplified guidelines identifying the HPUE MSD either as normative or informative requirements. However, the detailed rules and methods need further discussion and consensus between interested companies from now on.

In this contribution	we evaluate the HPUE MSD calculation method based on the existing PC3 MSD. From the analysis results, we make some proposals to make the discussion move forward.

Discussion

One UL HPUE MSD calculation

It is recommended that the existing specified 1Tx PC3 MSD should be used as a reference when deriving HPUE MSD [2]-[5].

In general, MSD is calculated using the MRC equation with primary receiver and secondary receiver noise composed of thermal and interferer noise. But it should be noted that the primary receiver noise and the secondary receiver noise are not necessarily same and there might be interferer power imbalance between them. This is because the RF front end architecture is not always symmetrical for both primary and secondary receiver path. So, we need to consider this interferer power imbalance for the MSD calculation of HPUE.

Table 1 shows the MSD calculation parameters for PC3. From the existing specification, we know PC3 MSD and reference sensitivity level (RefSens). However, the interferer power imbalance (Imb) between PRX and DRX is difficult to know for every PC3 MSD because of diverse RF architecture and band combinations. Therefore, the interferer power imbalance (Imb) should be fixed before the interferer noise at PRX Ant (A) can be determined.

The interferer power at PRX Ant according to PC3 MSD is plotted in Appendix for -100dBm Refsens with 7dB and 9dB interferer power imbalance as an example.

Table 1 MSD calculation parameters for PC3

Note that “Log[N1 + N2]” means noise power summation using 10*log10(10^(N1/10) + 10^(N2/10)).

Table 2, Table 3 and Table 4 show how we can derive PC2 1Tx MSD, PC2 2Tx MSD and PC1.5 2Tx MSD respectively based on the existing PC3 MSD parameters in Table 1.

PC2 1Tx MSD (Table 2) can be derived by simply adding increased interferer noise to PC3 interferer noise at PRX which is A in Table 1.

Table 2 MSD calculation parameters for PC2 1Tx

However, PC2 2Tx(23+23) MSD (Table 3) and PC1.5 2Tx(26+26) MSD (Table 4) have different calculation approach because there is an additional Tx chain.

In other words, both PRX and DRX see the same interferer power as “G” and “J” in Table 3 and Table 4. It should be noted that the interferer imbalance between PRX and DRX is same here but the interferer (E, F, H, I) is based on 1Tx interferer (A and B from PC3, C and D from PC2) which is dependent on interferer imbalance.

Another point we need to consider is that there might be interferer noise increase (Rtx) due to reverse Tx signal in case of 2Tx similarly for MPR/A-MPR calculation as shown in Table 3 (E,F) and Table 4 (H, I). This impact is hard to simulate so some experiment or assumption might be needed.

Table 3 MSD calculation parameters for PC2 2Tx

Table 4 MSD calculation parameters for PC1.5 2Tx

Figure 1 shows the procedure to derive HPUD MSD based on the existing PC3 MSD as discussed above. It should be noted that the two parameter values; 1) interferer power imbalance at PC3, 2) additional interferer noise due to reverse Tx impact for 2Tx should be agreed within RAN4 first for further progress. The interferer increase is different according to the MSD types and power class, which will be discussed in the following sections.

Figure 1. Flow chart for HPUE MSD derivation based on the PC3 MSD

Proposal 1: RAN4 to use the flow chart in Figure 1 to derive the HPUE MSD from the existing PC3 MSD.

Proposal 2: RAN4 to firstly agree interferer imbalance and additional noise due to reverse Tx impact for 2Tx for further progress.

Harmonic, harmonic mixing MSD

There have been measurements showing that the harmonic interferer increase from PC3 to PC2 is about 3dB regardless of harmonic order because PC2 PA has better linearity than PC3. So, it is proposed to use 3dB for harmonic interferer increase for PC2 1Tx MSD.

Figure 2(a) shows Delta MSD between PC2 1Tx and PC3 (Y1-X). There is almost no difference between different interferer imbalance (7dB and 9dB).

Figure 2(b) shows Delta MSD between PC2 2Tx and PC3 (Y2-X) and Figure 2(c) shows Delta MSD between PC1.5 2Tx and PC3 (Z-X) respectively. On the other hand, it can be shown that the Delta MSD is highly dependent on the interferer imbalance and reverse Tx impact. MSD is the minimum requirement for any type of UE, so we propose to use 9dB imbalance and 1dB additional noise due to reverse (Rtx) Tx for the HPUE MSD derivation rules.

Proposal 3: Propose to use [9]dB imbalance and [1]dB additional noise due to reverse Tx (Rtx) for the HPUE MSD derivation rules.

Figure 2 (a). Delta MSD between PC2 1Tx and PC3

Figure 2(b). Delta MSD between PC2 2Tx and PC3

Figure 2(c). Delta MSD between PC1.5 2Tx and PC3

Crossband isolation MSD

Unlike the harmonic interfere increase of 3dB, measurements or simulations are needed to evaluate interferer increase for PC2 1Tx accurately. This is because the ACLR 1 and ACLR 2 might not show flat interferer spectrum over the region and depend on the offset frequency which is determined by CA combination. Also there have not been enough measurement results of noise increase of HPUE for >ACLR2 region.

Proposal 4: RAN4 to study the interferer increase for HPUE MSD due to crossband isolation considering CA combination and DL frequency bandwidth and offset from UL.

After agreeing the interferer increase, the same procedure as in Figure 1 can be applied. Please note that the PC2 2Tx MSD can be derived regardless of the interferer increase because it refers to PC3 interfere noise.

Two UL HPUE MSD calculation

The same procedure as 1 UL case can be applied to derive HPUE MSD due to IMD as shown in Figure 3.

Figure 3. Flow chart for HPUE MSD (due to IMD) derivation based on the PC3 MSD

There are different PA configurations for PC2 and PC1.5 [1]. But 2 Tx configuration can be a starting point for the initial discussion. So, we consider three cases as below. Moreover, the main purpose of HPUE MSD rules is to simplify the process and lower the burden on the HPUE MSD study.

- PC2: PC3 PA(23dBm) + PC3 PA(23dBm)

- PC2: PC2 PA(23dBm) + PC2 PA(23dBm)

- PC1.5: PC2 PA(26dBm) + PC2 PA(26dBm)

However, it is not straightforward to determine the interferer IMD increase (PC3  PC2 and PC3   PC1.5) because this is highly dependent on the PA configuration (PC3 PA or PC2 PA) as well as how much PA IMD is dominant over other components’ IMD.

This is depicted in Figure 4.

For PC2 operation with two PC3 PAs, the interferer IMD increase is constant for each IMD order – 6/9/12/15 dB for IMD2/3/4/5. However, if two PC2 PAs are used for PC2 operation, the total IMD increase would vary according to difference between PA IMD and non-PA IMD contribution. Non-PA component IIPn remains same. On the other hand, the linearity of PC2 PA is better than PC3 PA which results in only 3dB IMD interferer increase regardless of IMD order like harmonic interferer increase. For example, the IMD2 interferer increase is 3dB when PA IMD is dominant whereas the IMD2 interferer increase would be 6dB when non-PA IMD is dominant.

For PC1.5 operation with two PC2 PAs, the interferer increase will converge to 9/12/15/18dB for IMC2/3/4/5 when PA IMD is dominant (+3dB + 3xIMD order). On the other hand, the IMD interferer increase will approach to 12/18/24/30dB (6xIMD order) when non-PA IMD is dominant.

Observation 1: For PC2 with two PC3 PA, the IMD interferer is 6/9/12/15 dB for IMD2/3/4/5.

Observation 2: For PC2 with two PC2 PA, the IMD interferer is 3dB when PA IMD is dominant regardless of IMD order and approaches to 6/9/12/15dB as non-PA IMD is dominant for IMD2/3/4/5.

Observation 3: For PC1.5 with two PC2 PA, the IMD interferer is 9/12/15/18 dB when PA IMD is dominant and approaches to 12/18/24/30dB as non-PA IMD is dominant for IMD2/3/4/5.

- IMD2                                   (b) IMD3

(c) IMD4                                   (d) IMD5

Figure 4. Interferer IMD increase for different PA configuration

Although most of the IMD comes from the PA nonlinearity, there might be cases where LNA nonlinearity are dominant for certain CA combinations or test points as in [6] and [7].

So further investigation would be needed to determine the interferer IMD increase for 2UL HPUE MSD.

Proposal 5: RAN4 to investigate and determine the interferer increase for HPUE MSD due to IMD considering PA configuration and IMD budget between PA and non-PA components.

Note that we don’t need to consider the reverse Tx impact. Therefore, the only parameter to be considered is the interferer increase from PC3 IMD.

Table 5 shows the summary of HPUE MSD rules discussed in this contribution.

Table 5 MSD calculation parameters for PC2/PC1.5 due to IMD

| MSD type | Parameter | PC3 | 1Tx PC2 | 2Tx PC2 | PC1.5 |
| --- | --- | --- | --- | --- | --- |
| Harmonic/Harmonic mixing | Interferer imbalance | [9]dB | [9]dB |  |  |
|  | Interferer increase |  | 3dB | PC3 ref | 1Tx PC2 ref |
|  | Reverse Tx impact |  |  | [1]dB | [1]dB |
| Crossband isolation | Interferer imbalance | [9]dB | [9]dB |  |  |
|  | Interferer increase |  | FFS | PC3 ref | 1Tx PC2 ref |
|  | Reverse Tx impact |  |  | [1]dB | [1]dB |
| IMD | Interferer imbalance | [9]dB | [9]dB |  | [9]dB |
|  | Interferer increase |  | FFS |  | FFS |
|  | Reverse Tx impact |  |  |  |  |

Conclusion

In this contribution, we analyzed and provided our view on the HPUE MSD derivation rules based on the existing PC3 MSD. With this analysis result, we make the following proposals.

Proposal 1: RAN4 to use the flow chart in Figure 1 to derive the HPUE MSD from the existing PC3 MSD.

Proposal 2: RAN4 to firstly agree interferer imbalance and additional noise due to reverse Tx impact for 2Tx for further progress.

Proposal 3: Propose to use [9]dB imbalance and [1]dB additional noise due to reverse Tx (Rtx) for the HPUE MSD derivation rules.

Proposal 4: RAN4 to study the interferer increase for HPUE MSD due to crossband isolation considering CA combination and DL frequency bandwidth and offset from UL.

Observation 1: For PC2 with two PC3 PA, the IMD interferer is 6/9/12/15 dB for IMD2/3/4/5.

Observation 2: For PC2 with two PC2 PA, the IMD interferer is 3dB when PA IMD is dominant regardless of IMD order and approaches to 6/9/12/15dB as non-PA IMD is dominant for IMD2/3/4/5.

Observation 3: For PC1.5 with two PC2 PA, the IMD interferer is 9/12/15/18 dB when PA IMD is dominant and approaches to 12/18/24/30dB as non-PA IMD is dominant for IMD2/3/4/5.

Proposal 5: RAN4 to investigate and determine the interferer increase for HPUE MSD due to IMD considering PA configuration and IMD budget between PA and non-PA components.

References

[1]	R4-2417098, “WF on HPUE RF requirements”, Samsung, Skyworks, Nokia, 3GPP TSG RAN WG4 Meeting #112-bis, Hefei, China, October 14th – 18th, 2024

[2]	 R4-2407374, “Simplified MSD calculation guidelines for PC2 FDD”, Murata Manufacturing Co Ltd., 3GPP TSG RAN WG4 Meeting #111, Fukuoka, Japan, May 20th – 24th, 2024

[3]	R4-2407156, “Guidelines for FDD PC2 Harmonic MSD Analysis”, Skyworks Solutions, Inc., 3GPP TSG RAN WG4 Meeting #111, Fukuoka, Japan, May 20th – 24th, 2024

[4]	R4-2410605, “WF on HPUE MSD Simplification”, Skyworks Solutions, Inc., Murata, Qualcomm, Samsung, Nokia, 3GPP TSG RAN WG4 Meeting #111, Fukuoka, Japan, May 20th – 24th, 2024

[5]	R4-2416263, “MSD for HPUE”, Qualcomm France, 3GPP TSG RAN WG4 Meeting #112-bis, Hefei, China, October 14th – 18th, 2024

[6]	R4-2215525, “Triple beat MSD for DC_3C_n41C”, Skyworks Solutions, Inc., 3GPP TSG RAN WG4 Meeting #104-bis-e, Electronic Meeting, October 10th – 19th, 2024

[7]	R4-2215528, “Triple beat MSD for DC_3C_n28A”, Skyworks Solutions, Inc., 3GPP TSG RAN WG4 Meeting #104-bis-e, Electronic Meeting, October 10th – 19th, 2024

- Appendix