# 3GPP TSG-RAN WG4 Meeting #113	                                R4-2417788

Orlando, US, 18th – 22nd November, 2024

Source:                Tejas Networks Pvt. Ltd.

Title:                     Discussion on existing Tx requirements for SBFD operation

Agenda Item:       7.20.2.2

Document for:     Discussion

# Introduction

In this contribution, we present overview of impact on existing BS RF TX requirements for unwanted emissions.

# Discussion

## Unwanted emissions

In the RAN4 #112 bis meeting, RAN4 agreed to further study on ACLR and OBUE with following agreement and WF [1].

FFS on the followings: - Whether to define additional ACLR and/or OBUE requirement to ensure feasible coexistence with SBFD-capable gNB firstly? - Option 1: Yes, since legacy ACLR and OBUE requirement might not be sufficient to ensure feasible coexistence. - Option 2: No, since specific co-existence requirements can be covered by regional requirements or customer specific proprietary requirements. - How to define the additional co-existence requirements: - Option 1: based on CL assumption for BS2BS CLI interference and BS DESENS level. - [1dB DESENS in the victim SBFD receiver] - [typical CL values based on typical SBFD-capable gNB implementation] - [frequency offset for the additional OBUE requirement is declared] - Option 2: by improving ACLR1 and ACLR2 requirements - Option 3: others

### ACLR/OBUE

We would like to present our views on FR1 macro deployments when coexisting with a new SBFD system in an adjacent channel, RAN4, to define additional ACLR and/or OBUE requirements to ensure feasible coexistence with SBFD-capable gNB. In [2], the feasibility analysis of SBFD BS co-location has been discussed, where for FR1 aggressor WA BS, the output power is 49 dBm, the bandwidth of the channel is 100 MHz, and if we assume the degradation of victim BS sensitivity is 1 dB, the required total isolation between the two BSs is 144 dB, that is, the sum of ACLR, couple loss of co-location, and the rejection of RF BPF used after PA must be larger than 144 dB. For FR1 macro-BS with the output power of 49 dBm and a 100 MHz bandwidth of channel, the adjacent +1 ACLR of 55 dB can be easily achieved with the help of the PA linearization technique. Moreover, due to the advancement in the Digital Predistortion (DPD) techniques for PA linearization, it is possible to achieve better than 55 dB of ACLR in adjacent +1 channels.

In the case of location, to achieve this value, we additionally propose the gNB-gNB CLI handling scheme to be introduced. Therefore, we revisit the feasibility analysis of SBFD BS co-location as discussed in [2].

Table 1: Feasibility analysis of SBFD BS co-location

| Parameters | Value |
| --- | --- |
| Output power(dBm) | 49 |
| BW(MHz) | 100 |
| Degradation of victim BS sensitivity(dB) | 1 |
| Required total isolation(dB) | 144 |
| Adjacent +1 (2nd) ACLR (dB) | 55 |
| Coupling loss of co-location (dB) | 30 |
| Rejection of RF BPF used after PA (dB) | 59 |

Building on the above OBUE discussion, the minimum ACLR requirement, as outlined in TR 38.858 [3], specifies that the ACLR should be defined across the entire carrier, rather than just within a sub-band, for DL symbols or slots. Specifically, the ACLR is calculated as the ratio of the total transmitted power within the full carrier bandwidth to the power leakage into adjacent carriers. Additionally, the manufacturer will indicate whether their equipment supports the co-location requirements for SBFD symbols or slots. This declaration will confirm whether the equipment can meet the necessary interference management criteria when co-located with other systems in adjacent frequency bands.

Observation 1: The current legacy ACLR and OBUE requirements may not be sufficient to guarantee reliable coexistence with FR1 macro base stations (BS).

Observation 2: Improving the ACLR1 and ACLR2 requirements can enhance the rejection of RF Bandpass Filters (BPF), leading to better interference management.

Proposal 1: Introduce additional ACLR and/or OBUE requirements to ensure reliable coexistence with SBFD-capable gNBs.

Proposal 2: We propose tightening the minimum ACLR1 and ACLR2 requirements to improve the rejection performance of RF BPFs used after the power amplifier (PA), enabling more effective interference mitigation.

Proposal 3: Set the minimum colocation ACLR to -50 dBc for a 100 MHz channel, considering the potential application of the CLI scheme in adjacent channels or deployment constraints.

# Conclusion

In this contribution, we would like to present our perspectives on the RF TX requirements for SBFD BS, outlined as follows.

Observation 1: The current legacy ACLR and OBUE requirements may not be sufficient to guarantee reliable coexistence with FR1 macro base stations (BS).

Observation 2: Improving the ACLR1 and ACLR2 requirements can enhance the rejection of RF Bandpass Filters (BPF), leading to better interference management.

Proposal 1: Introduce additional ACLR and/or OBUE requirements to ensure reliable coexistence with SBFD-capable gNBs.

Proposal 2: We propose tightening the minimum ACLR1 and ACLR2 requirements to improve the rejection performance of RF BPFs used after the power amplifier (PA), enabling more effective interference mitigation.

Proposal 3: Set the minimum colocation ACLR to -50 dBc for a 100 MHz channel, considering the potential application of the CLI scheme in adjacent channels or deployment constraints.

# Reference

- R4-2416585 Way Forward for [112bis][307] NR_duplex_evo_BSRF

- R4-2411080, “Discussion on modification of existing Tx requirements for SBFD operation”, CATT

- 3GPP TR 38.858: “Study on Evolution of NR Duplex Evolution”.