3GPP TSG-RAN WG4 Meeting#113							         		R4-2417792

Orlando, Florida, USA, 18 – 22 November 2024

Title: 	MSD test points for CA_n40A-n41C and CA_n41C-n79A with n41C UL

Source: 	Skyworks Solutions, Inc.

Agenda Item:	6.3.3	UE RF requirements for NR inter-band CA/DC configurations including inter band CA for 2 DL with up to 2UL (NR_CADC_R19_2BDL_xBUL)	[NR_CADC_SUL_R19-Core]

Document for:	Approval

# 1	Background

In RAN#112bis, we discussed how to introduce a second test point for MSD due to contiguous intra-band ULCA IMDs in [1], and way forward [2] captured the new test points and the options to be further discussed and is , based on input in [1, 3, 4, 5]. In this contribution, we provide our MSD input for the new test points and discuss how the two test points are captured in the 38.101-1.

# Discussion

## 2.1 Way Forward and Revised SID inputs

The way forward [2] captured agreements on the scope in italic below.

Proposal 1: In Rel-19, to keep as-is in spec as most of cases are covered and some history is built since Rel-17.

Proposal 2: For the specific cases DL_n40A-n41C_UL_n41C and DL_n41C-n79A_UL_n41C, the following test configurations for MSD requirements can be introduced in next meeting.

- Test case 1: Discrete UL RB allocation, e.g. 1RB + 1RB UL RB allocation in UL intra-band CA.

- Test case 2: fully UL RB allocation with maximum aggregated BW.

NOTE: In the next meeting, RAN4 need to discuss how to capture these two test cases into the spec considering some consistencies.

Proposal 3: For the MSD value DL_n40A-n41C_UL_n41C, the following inputs from companies can be further checked considering the assumption of filter performance and PA interference.

- 3.1) 1RB+1RB test case 1:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 42.5 | TDD | IMD3x |
|  | n41 | 2555 | 80 | 1 (RBSTART=0) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 1 (RBSTART=107) | 2635 | N/A | TDD | N/A |
| Note x: this DL band is also subject to IMD5 that is not specified Note y: this second test point is optional |  |  |  |  |  |  |  |  |

- 3.2) For fully UL RB allocation test case 2, RAN4 further discuss whether MSD due to IMD3 or ACLR1 cross band isolation is considered for DL_n40A-n41C_UL_n41C.

  - Option 1:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 34.9 | TDD | IMD3xy |
|  | n41 | 2555 | 80 | 216 (RBSTART=1) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 216 (RBSTART=0) | 2635 | N/A | TDD | N/A |
| Note x: this DL band is also subject to IMD5 that is not specified Note y: this second test point is optional |  |  |  |  |  |  |  |  |

  - Option 2:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| CA_n41C | n40 | 2595 | 190 | 30 | 270 (RBstart=0) + 243 ((RBstart=0) | 2397.5 | 5 | 34 | ACLR1 |

  - Option 3: The cross band isolation MSD caused by fully allocated maximum aggregated BW of UL CA_n41C falls into n40 is 31.5dB and 21.7dB for CA ACLR1 and CA ACLR2, respectively.

  - Other proposal and value are also welcome.

NOTE: In the next meeting, RAN4 can further evaluate the MSD values based on the following channel bandwidth configurations:

Band n41: one 100MHz BW CC at 2565MHz centre frequency and another one 60MHz BW CC at 2645MHz centre frequency with 30kHz SCS.

Band n40: one 5MHz BW CC at 2397.5MHz centre frequency.

Proposal 4: For the MSD value DL_n41C-n79A_UL_n41C, the following inputs from companies can be further checked considering the assumption of filter performance and PA interference.

- 4.1) 1RB+1RB test case 1:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| CA_n41-n79 | n41 | 2555 | 80 | 1 (RBSTART=0) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 1 (RBSTART=0) | 2635 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4950 | 8.4 | TDD | IMD4x |
| Note x: this DL band is also subject to IMD6 that is not specified Note y: this second test point is optional |  |  |  |  |  |  |  |  |

- 4.2) For fully UL RB allocation test case 2, RAN4 further discuss whether MSD due to IMD4 or >ACLR2 cross band isolation is considered for DL_n41C-n79A_UL_n41C.

  - Option 1:

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| CA_n41-n79 | n41 | 2555 | 80 | 216 (RBSTART=1) | 2555 | N/A | TDD | N/A |
|  |  | 2635 | 80 | 216 (RBSTART=0) | 2635 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4950 | 4.6 | TDD | IMD4xy |
| Note x: this DL band is also subject to IMD6 that is not specified Note y: this second test point is optional |  |  |  |  |  |  |  |  |

  - Option 2:

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| CA_n41C | n79 | 2595 | 190 | 30 | 270 (RBstart=0) + 243 ((RBstart=0) | 4420 | 40 | 0.5 | >ACLR2 |

  - Option 3: There is no need to define additional cross band isolation MSD requirements caused by fully allocated maximum aggregated BW of UL CA_n41C falls into n79.

  - Other proposal and value are also welcome.

NOTE: In the next meeting, RAN4 can further evaluate the MSD values based on the following channel bandwidth configurations:

Band n41: one 100MHz BW CC at 2565MHz centre frequency and another one 60MHz BW CC at 2645MHz centre frequency with 30kHz SCS.

Band n79: one 10 or 40MHz BW CC at 4405 or 4420MHz centre frequency. Companies may provide input for both channel bandwidths.

Observation:

- We recognize that the 100MHz+60MHz CA_n41C configuration is more valid for China which is the worst case for the two regions where CA_n40-n41 and CA_n41-n79 are valid (China and Japan). Thus, it is a better approach for the test point.

- However, it seems that full band n40 and full band n41 may be available in the Philippines. If this does not change the order of the IMD of CA_n41C falling in the band n40, one may expect a slightly increased MSD due to a lower frequency gap between the bands.

- For the band n79 victim channel, considering that the China spectrum goes up to 5GHz and that the smallest CBW of 10MHz is not optional, a 10MHz channel at 4995MHz is appropriate. This is the worst case as it is the closest to the CA_n41C H2 which is just above the IMD4 product of interest. The worst position is not the lowest channel in band n79.

Based on the above, in this contribution we discuss how the two test points are best captured and then make formal MSD proposals for MSD test points for CA_n40A-n41C and CA_n41C-n79A with n41C UL.

## 2.2 Capturing the two MSD test points and associated guidelines

In the last RAN4 meeting, it was agreed that beyond the 1RB+1RB test point used since Release 17 for the MSD due to the 1 band 2CC intra-band contiguous UL CA, a second test point can be introduced with a Full+Full RB allocation. It has been suggested that this second test point could be introduced in the 1UL cross-band isolation MSD table while the legacy 1RB+1RB test point is in the 2UL IMD MSD table.

In our view, it is not possible to separate those two test points and, even in the case of the fully allocated RB allocation in both CC, there are IMD products corresponding to the example Table 1 calculated ranges. This can also be observed in the measured spectrums; thus, the MSD should be related to an IMD order rather than an ACLR range (>ACLR2 would cover 10 separate IMD products of Table 1). In any case, the single CC ACLR range is also related to the IMD range of the UL allocation and its image, while for intra-band UL CA the spectrum is directly related to the IMD products of each CC allocation.

However, we also recognize that this MSD is linked to the interference of one UL band.

Table 1: example of IMD products of contiguous intra-band ULCA

| all in MHz | flow/Min | fhigh/Max | BB IMD range3 |  |  |
| --- | --- | --- | --- | --- | --- |
| fUL5 | 2515 | 2675 | order | flow | fhigh |
| 2CCBW1 | 0 | 160 | IMD2 (1-1) | Min2CCBW | Max2CCBW |
| fDL | 2300 | 2400 |  | 0 | 160 |
| close to UL IMD range2 |  |  | IMD4 (2-2) | 2*Min2CCBW | 2*Max2CCBW |
| order | flow | fhigh |  | 0 | 320 |
| IMD3 (2-1) | fULlow-MaxULBW | fULhigh+MaxULBW | IMD6 (3-3) | 3*Min2CCBW | 3*Max2CCBW |
|  | 2355 | 2835 |  | 0 | 480 |
| IMD5 (3-2) | fULlow-2*MaxULBW | fULhigh+2*MaxULBW | close to H2 IMD range4 |  |  |
|  | 2195 | 2995 | order | flow | fhigh |
| IMD7 (4-3) | fULlow-3*MaxULBW | fULhigh+3*MaxULBW | IMD4 (3-1) | 2*fULlow-Max2CCBW | 2*fULhigh+Max2CCBW |
|  | 2035 | 3155 |  | 4870 | 5510 |
| IMD9 (5-4) | fULlow-4*MaxULBW | fULhigh+4*MaxULBW | IMD6 (4-2) | 2*fULlow-2*Max2CCBW | 2*fULhigh+2*Max2CCBW |
|  | 1875 | 3315 |  | 4710 | 5670 |
| IMD11 (6-5) | fULlow-5*MaxULBW | fULhigh+5*MaxULBW | close to H3 IMD range4 |  |  |
|  | 1715 | 3475 | order | flow | fhigh |
| IMD13 (7-6) | fULlow-6*MaxULBW | fULhigh+6*MaxULBW | IMD5 (4-1) | 3*fULlow-Max2CCBW | 3*fULhigh+Max2CCBW |
|  | 1555 | 3635 |  | 7385 | 8185 |
| analysis | The lower n41C IMD3 overlaps with band n40 DL (IMD5 also) |  |  |  |  |

Observation: the 2CC IMD products that are not covered by the single CC case and harmonics covers the spectrum next to the UL signal, and also around the BB frequencies and the second and third harmonic. The BB frequencies are only of interest for non-contiguous ULCA with large gaps.

Proposal on mandatory and optional test points for 1 band 2CC contiguous ULCA MSD:

- The two test points are part of the same MSD table, the 1RB+1RB test point first.

  - To avoid rework, they are in the 2UL IMD MSD table as is currently for the mandatory 1RB+1RB test point.

  - Alternatively, they are both captured in a separate table after the cross-band isolation table, but all such test points should also be moved there.

- The two test points use the same configuration (UL/DL frequencies, UL/DL CBW) except the UL CCs RB allocations:

  - The FULs are chosen the closest to the victim band and the contiguous CC BWs are such that the maximum aggregated BW that is obtained still fits within the band.

  - The victim band FDL is chosen the closest to the considered IMD product frequency range, using the smallest DL CBW.

  - The 1RB+1RB RBstart (U,V) are chosen to best center the target IMD in the victim DL channel. This test point is mandatory.

  - The Full+Full RBstart of the lowest UL channel is chosen such that the Full+Full allocation is contiguous. This test point is optional.

- The 1RB+1RB test point MSD is calculated based on the assumption that SEM is just met (back-off is allowed) with IMD3=-13dBm/MHz, IMD5=-30dBm/MHz.

- The Full+Full test point MSD is calculated based on the assumption that SEM and ACLR are just met (back-off is allowed) with IMD3=-13dBm/MHz, IMD5=-30dBm/MHz, ACLR=30dBc for PC3 and 31dBc for PC2/PC1.5.

- The two test points are captured in the following test point table format:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| nAA | N/A | Min DL CBW | N/A | Closest to IMD | MSD1 | TDD | IMDX |
| nBB | CC1 FUL | CC1 CBW | 1 (RBSTART=U) | CC1 FDL | N/A | TDD | N/A |
|  | CC2 FUL | CC2 CBW | 1 (RBSTART=V) | CC2 FDL | N/A | TDD | N/A |
| nAA | N/A | Min DL CBW | N/A | Closest to IMD | MSD2 | TDD | IMDX |
| nBB | CC1 FUL | CC1 CBW | Full (RBSTART=W) | CC1 FDL | N/A | TDD | N/A |
|  | CC2 FUL | CC2 CBW | Full (RBSTART=0) | CC2 FDL | N/A | TDD | N/A |

## 2.3 CA_n40A-n41C DL with n41C UL MSD.

To evaluate the interference level in the victim DL channel, we reuse the wideband measurements we conducted for 1RB+1RB and Full+Full allocations in previous RAN4 meetings. These measurements were based on 80MHz+80MHz contiguous channels. In this contribution we are now designing the test point for a 100MHz+60MHz channel configuration.

For the 1RB+1RB cases this makes no difference in the spectrum since the 1RB are positioned at the maximum spacing which only changes marginally between the two aggregated channels configurations.

For the Full+Full case, we are comparing 216RBs+216RBs=432RBs with 270RBS+162RBs=432 RBs equally, given that equal PSD is used, there is little changes between the two aggregated channel configurations.

This allows us to reuse our former measurements and avoids another low noise/wideband measurement that requires specific filtering and careful test setup calibration. The former measurements are shifted in Figure 1 to represent the spectrum in bands n40 and band CA_n41C spectrum:

- The light blue curve is the 1RB+1RB spectrum with the required back off to meet the spectrum mask (black dashed line) in dBm/MHz.

  - The red spot is the IMD3 level in band n40 in dBm/MHz and since it is <4.5MHz this is also the power in a 5MHz n40 DL channel.

- The dark blue curve is the contiguous Full+Full spectrum with the required back-off to meet the spectrum mask (black dashed line) and ACLR in dBm/MHz.

  - The red line at the top of band n40 represents the integrated Full+Full spectrum in the top of the band n40 5MHz channel.

- The dark green line represents the band n40 frequency range.

- Note that the Chinese band n41 range is assumed compared to the full band n41 which would result in larger MSD.

Figure 1: CA_n41C UL spectrum for Full+Full and 1RB+1RB allocations overlapping with band n40.

Observation:

- The 1RB+1RB IMD3 (index +2-1) level is at -13dBm/MHz at the antenna, and IMD5 also meets the -30dBm/MHz SEM, thus interfering power at the PA output in n40 5MHz DL channel is -9dBm

- The Full+Full IMD3 (index +2-1) level is at -21.1dBm at the antenna, thus the interfering power at the PA output in n40 5MHz DL channel is -17.1dBm

- The difference between the two allocation types is ~8dB thus the 1RB+1RB test point is representative.

Based on the above measurements, the post MRC MSD can be calculated for both type of allocations as in Table 1. The RF front-end assumptions are as follows:

- 4dB post PA and pre LNA losses

- 35dB rejection from band n41 filter (assumes some rebound as the maximum rejection targets the 2.4GHz ISM band just above n40).

Table 2: Post MRC band n40 MSD calculation.

| 1+1RB at MPR Tx noise in band n40 5 MHz CBW | [dBm/MBW] | -9.0 |  |
| --- | --- | --- | --- |
| n41 SAW Tx to Ant. port rejection at n40 DL freq. | [dB] | 35 |  |
| Diplexer rejection of PA Tx noise at n40 DL freq. | [dB] | 0 |  |
| Main LNA to ant insertion loss / Post PA diplexer to ant IL | [dB] | 4 | 0 |
| Antenna-to-Antenna isolation | [dB] | 0 | 10 |
| In channel levels: primary/diversity (antenna) | [dBm/MBW] | -48.0 | -58.0 |
| n40 5MHz CBW ideal REFSENS | [dBm/MBW] | -100.0 |  |
| noise floor | [dBm/MBW] | -96.0 | -96.0 |
| IMD2 noise due to Tx leakage - referred to antenna connector | [dBm/MBW] | -999.0 | -999.0 |
| Total noise level at antenna connector | [dBm/MBW] | -48.0 | -58.0 |
| n40 5MHz degradation | [dB] | 48.0 | 38.0 |
| MSD afterMRC | uncor [dB] | 40.60 |  |
| MSD afterMRC | cor [dB] | 42.50 |  |
| Full+full at MPR0 Tx noise in band n40 5 MHz CBW | [dBm/MBW] | -17.1 |  |
| n41 SAW Tx to Ant. port rejection at n40 DL freq. | [dB] | 35 |  |
| Diplexer rejection of PA Tx noise at n40 DL freq. | [dB] | 0 |  |
| Main LNA to ant insertion loss / Post PA diplexer to ant IL | [dB] | 4 | 0 |
| Antenna-to-Antenna isolation | [dB] | 0 | 10 |
| In channel levels: primary/diversity (antenna) | [dBm/MBW] | -56.1 | -66.1 |
| n40 5MHz CBW ideal REFSENS | [dBm/MBW] | -100.0 |  |
| noise floor | [dBm/MBW] | -96.0 | -96.0 |
| IMD2 noise due to Tx leakage - referred to antenna connector | [dBm/MBW] | -999.0 | -999.0 |
| Total noise level at antenna connector | [dBm/MBW] | -56.1 | -66.1 |
| n40 5MHz degradation | [dB] | 39.9 | 29.9 |
| MSD afterMRC | uncor [dB] | 32.50 |  |
| MSD afterMRC | cor [dB] | 34.40 |  |

Proposal on IMD3 n40 MSD for CA_n40A-n41C DL with CA_n41C UL:

- The following test points, MSD value, and associated note are added to 38.101-1 Table 7.3A.5-1.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 42.5 | TDD | IMD3 |
|  | n41 | 2565 | 100 | 1 (RBSTART=0) | 2565 | N/A | TDD | N/A |
|  |  | 2645 | 60 | 1 (RBSTART=52) | 2645 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2397.5 | 34.4 | TDD | IMD3x |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2645 | 60 | 162 (RBSTART=0) | 2645 | N/A | TDD | N/A |
| Note X: This second test point is optional. NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz. |  |  |  |  |  |  |  |  |

## 2.4 CA_n41C-n79A DL with n41C UL MSD.

To evaluate the interference level in the victim DL channel, we reuse the wideband measurements we conducted for 1RB+1RB and Full+Full allocations in previous RAN4 meetings. These measurements were based on 80MHz+80MHz contiguous channels. In this contribution we are now designing the test point for a 100MHz+60MHz channel configuration.

The same reasoning than for CA_n40A-n41C applies here and allows us to reuse our former measurements. The former measurements are just shifted in Figure 2 to represent the spectrum in bands n79, and CA_n41C harmonic 2 and IMD4:

- The light blue curve is the 1RB+1RB spectrum with the required back-off to meet the spectrum mask (black dashed line) in dBm/MHz.

  - The red spot is the IMD4 (3, -1) level in band n79 in dBm/MHz and since it is <9.4MHz this is also the power in a 10MHz n79 DL channel.

- The dark blue curve is the contiguous Full+Full spectrum with the required back-off to meet the spectrum mask (black dashed line) and ACLR in dBm/MHz.

  - The red line at the top of band n79 represents the integrated Full+Full spectrum in the top of the band n79 10MHz channel.

- The dark green line represents the band n79 frequency range.

- Note that the Chinese band n41 range is assumed compared to the full band n41 which would result in larger MSD as it would be closer to the H2 spectrum.

Figure 2: CA_n41C UL spectrum near H2 for Full+Full and 1RB+1RB allocations overlapping with band n79.

Observation:

- The 1RB+1RB IMD4 (index +3-1) level is at -37.6dBm/MHz at the antenna, thus interfering power at the PA output in n79 10MHz DL channel is -33.6dBm

- The Full+Full IMD4 (index +3-1) level is at -42.8dBm at the antenna, thus interfering power at the PA output in n79 10MHz DL channel is -38.8dBm

- The difference between the two allocation types is ~5dB thus the 1RB+1RB test point is representative.

Based on the above measurements, the post MRC can be calculated for both type of allocations as in Table 2. The RF front-end assumptions are as follows:

- 4dB post PA and pre LNA losses

- 27dB rejection from band n41 filter (assumes rebound close to second harmonic frequency)

- 15dB rejection from the >2.7GHz/<3.3GHz diplexer

Table 3: Post MRC band n79 MSD calculation.

| 1+1RB at MPR Tx noise in band n79 10 MHz CBW | [dBm/MBW] | -33.6 |  |
| --- | --- | --- | --- |
| n41 SAW Tx to Ant. port rejection at n79 DL freq. | [dB] | 27 |  |
| Diplexer rejection of PA Tx noise at n79 DL freq. | [dB] | 15 |  |
| Main LNA to ant insertion loss / Post PA diplexer to ant IL | [dB] | 4 | 0 |
| Antenna-to-Antenna isolation | [dB] | 0 | 10 |
| In channel levels: primary/diversity (antenna) | [dBm/MBW] | -79.6 | -89.6 |
| n79 10MHz CBW ideal REFSENS | [dBm/MBW] | -95.8 |  |
| noise floor | [dBm/MBW] | -91.8 | -91.8 |
| IMD2 noise due to Tx leakage - referred to antenna connector | [dBm/MBW] | -999.0 | -999.0 |
| Total noise level at antenna connector | [dBm/MBW] | -79.6 | -89.6 |
| n79 10MHz degradation | [dB] | 12.5 | 4.2 |
| MSD afterMRC | uncor [dB] | 6.65 |  |
| MSD afterMRC | cor [dB] | 8.40 |  |
| Full+full at MPR0 Tx noise in band n79 10 MHz CBW | [dBm/MBW] | -38.8 |  |
| n41 SAW Tx to Ant. port rejection at n79 DL freq. | [dB] | 27 |  |
| Diplexer rejection of PA Tx noise at n79 DL freq. | [dB] | 15 |  |
| Main LNA to ant insertion loss / Post PA diplexer to ant IL | [dB] | 4 | 0 |
| Antenna-to-Antenna isolation | [dB] | 0 | 10 |
| In channel levels: primary/diversity (antenna) | [dBm/MBW] | -84.8 | -94.8 |
| n79 10MHz CBW ideal REFSENS | [dBm/MBW] | -95.8 |  |
| noise floor | [dBm/MBW] | -91.8 | -91.8 |
| IMD2 noise due to Tx leakage - referred to antenna connector | [dBm/MBW] | -999.0 | -999.0 |
| Total noise level at antenna connector | [dBm/MBW] | -84.8 | -94.8 |
| n79 10MHz degradation | [dB] | 7.8 | 1.8 |
| MSD afterMRC | uncor [dB] | 3.81 |  |
| MSD afterMRC | cor [dB] | 5.30 |  |

Proposal on IMD4 n79 MSD for CA_n41C-n79A DL with CA_n41C UL:

- The following test points, MSD value, and associated note are added to 38.101-1 Table 7.3A.5-1.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n41 | 2565 | 100 | 1 (RBSTART=112) | 2555 | N/A | TDD | N/A |
|  |  | 2645 | 60 | 1 (RBSTART=161) | 2635 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 8.4 | TDD | IMD4 |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2645 | 60 | 162 (RBSTART=0) | 2645 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 5.3 | TDD | IMD4x |
| Note X: This second test point is optional. NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz. |  |  |  |  |  |  |  |  |

# Conclusion

In this contribution, based on our previous measurements and after clarifying the mandatory first test point and optional second test point parameters, we make the following proposals.

Proposal on mandatory and optional test points for 1 band 2CC contiguous ULCA MSD:

- The two test points are part of the same MSD table, the 1RB+1RB test point first.

  - To avoid rework, they are in the 2UL IMD MSD table as is currently for the mandatory 1RB+1RB test point.

  - Alternatively, they are both captured in a separate table after the cross-band isolation table, but all such test points should also be moved there.

- The two test points use the same configuration (UL/DL frequencies, UL/DL CBW) except the UL CCs RB allocations:

  - The FULs are chosen the closest to the victim band and the contiguous CC BWs are such that the maximum aggregated BW that is obtained still fits within the band.

  - The victim band FDL is chosen the closest to the considered IMD product frequency range, using the smallest DL CBW.

  - The 1RB+1RB RBstart (U,V) are chosen to best center the target IMD in the victim DL channel. This test point is mandatory.

  - The Full+Full RBstart of the lowest UL channel is chosen such that the Full+Full allocation is contiguous. This test point is optional.

- The 1RB+1RB test point MSD is calculated based on the assumption that SEM is just met (back-off is allowed) with IMD3=-13dBm/MHz, IMD5=-30dBm/MHz.

- The Full+Full test point MSD is calculated based on the assumption that SEM and ACLR are just met (back-off is allowed) with IMD3=-13dBm/MHz, IMD5=-30dBm/MHz, ACLR=30dBc for PC3 and 31dBc for PC2/PC1.5.

- The two test points are captured in the following test point table format:

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| nAA | N/A | Min DL CBW | N/A | Closest to IMD | MSD1 | TDD | IMDX |
| nBB | CC1 FUL | CC1 CBW | 1 (RBSTART=U) | CC1 FDL | N/A | TDD | N/A |
|  | CC2 FUL | CC2 CBW | 1 (RBSTART=V) | CC2 FDL | N/A | TDD | N/A |
| nAA | N/A | Min DL CBW | N/A | Closest to IMD | MSD2 | TDD | IMDX |
| nBB | CC1 FUL | CC1 CBW | Full (RBSTART=W) | CC1 FDL | N/A | TDD | N/A |
|  | CC2 FUL | CC2 CBW | Full (RBSTART=0) | CC2 FDL | N/A | TDD | N/A |

Proposal on IMD3 n40 MSD for CA_n40A-n41C DL with CA_n41C UL:

- The following test points, MSD value, and associated note are added to 38.101-1 Table 7.3A.5-1.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n40-n41 | n40 | N/A | 5 | N/A | 2397.5 | 42.5 | TDD | IMD3 |
|  | n41 | 2565 | 100 | 1 (RBSTART=0) | 2565 | N/A | TDD | N/A |
|  |  | 2645 | 60 | 1 (RBSTART=52) | 2645 | N/A | TDD | N/A |
|  | n40 | N/A | 5 | N/A | 2397.5 | 34.4 | TDD | IMD3x |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2645 | 60 | 162 (RBSTART=0) | 2645 | N/A | TDD | N/A |
| Note X: This second test point is optional. NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz. |  |  |  |  |  |  |  |  |

Proposal on IMD4 n79 MSD for CA_n41C-n79A DL with CA_n41C UL:

- The following test points, MSD value, and associated note are added to 38.101-1 Table 7.3A.5-1.

| Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR CA band combination | NR band | UL Fc (MHz) | UL/DL BW (MHz) | UL LCRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_n41-n79 | n41 | 2565 | 100 | 1 (RBSTART=112) | 2555 | N/A | TDD | N/A |
|  |  | 2645 | 60 | 1 (RBSTART=161) | 2635 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 8.4 | TDD | IMD4 |
|  | n41 | 2565 | 100 | 270 (RBSTART=3) | 2565 | N/A | TDD | N/A |
|  |  | 2645 | 60 | 162 (RBSTART=0) | 2645 | N/A | TDD | N/A |
|  | n79 | N/A | 10 | N/A | 4995 | 5.3 | TDD | IMD4x |
| Note X: This second test point is optional. NOTE 14: Applicable when n41 spectrum is restricted to 2515-2675MHz. |  |  |  |  |  |  |  |  |

# References

[1] R4-2417180 CA_n40A-n41C and CA_n41C-n79A MSD due to n41C UL, Skyworks Solutions Inc., RAN4#112bis

[2] R4-2417181 WF on MSD requirements with intra-band contiguous UL CA_n40A-n41C and CA_n41C-n79A, Huawei, RAN4#112bis

[3] R4-2416012, On MSD requirements with intra-band contiguous UL CA, ZTE Corporation, Sanechips, RAN4#112bis

[4] R4-2416029, Discussion on MSD for CA_n40A-n41C with intra-band UL CA_n41C, Huawei, HiSilicon, RAN4#112bis

[5] R4-2416030, Discussion on MSD for CA_n41C-n79A with intra-band UL CA_n41C, Huawei, HiSilicon, RAN4#112bis