3GPP TSG-RAN WG4 Meeting#113							         		R4-2417793

Orlando, Florida, USA, 18 – 22 November 2024

Title: 	Observations on HPUE 2UL IMD MSDs

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.1.1.1.1 General aspects (incl. duty cycle solutions, MSD rules) [NR_ENDC_RF_Ph4-Core]

Document for:	Discussion

# 1	Background

In RAN#112bis, we discussed how to provide delta MSD values for 1UL HPUE MSDs in [1], the different options on how to simplify the HPUE MSD requirement process, or whether to remove this requirement were captured in way forward [2]. In this contribution, we focus our observations on the 2UL HPUE MSD cases that require further elaboration in order to provide delta MSD LUT based on calculations, while other papers [3,4] make similar observations and proposals for calculation based LUTs for 1UL HPUE MSDs. Based on this paper’s observations, a first phase of proposals for 2UL HPUE MSD will be covered in [5].

# Discussion

## 2.1 Way forward on HPUE MSD rules

The way forward [2] captured several options in italics below with some agreements on how to proceed further.

Option 1:

  - RAN4 agrees on the following directions in this meeting and continues on details in subsequent meetings.

  - RAN 4 defines the default power class inter-band CA/DC and intra-band CA/DC MSD requirements, and RAN4 specifies equations/simplified guidelines/lookup tables “LUT” that define the relationship between the default power class MSD and the HPUE MSD requirements to replace the existing HPUE MSD tables:

  - Equations/LUT/Simplified guidelines identifying the HPUE MSD requirements are expected to be captured in RAN4 spec as normative requirements, in Rel-19 if possible, the details are FFS

  - FFS the expected spec changes and procedure changes (i.e., continue/stop providing TP’s for each combination, remove/keep all existing HPUE notes/ HPUE MSD tables etc) are recommended to be applicable from Rel-20, or the late stage of Rel-19

- Decision can be deferred until the Equations/LUT/Simplified guidelines are getting done

  - Inform RAN5 after all the discussion are finalized, and it is up to RAN5 to decide how to simplify HPUE CA/DC MSD conformance testing, [and RAN4 can make recommendation if any consensus]

- [Agree to capture just single worst case MSD in TS for HPUE PC which is agnostic to number of TX, for each kind of MSD]

NOTE 1: RAN4 continues normal procedure specifying MSD for HPUE band combination supports at least until down-selection is done.

NOTE 2: RAN4 discuss/consider all HPUE scenarios/configurations as a package for sake of consistence. For the scenarios out of this WI scope, the agreements (if any) are recommended for band combination HPUE basket WIs to follow.

Option 2:

  - Current working procedure of HPUE CA/DC introduction are kept unchanged

  - HPUE MSD tables are kept

  - Unified Excel tool of calculating HPUE MSD are being developed with following details to be addressed

- Companies to evaluate if the interferer level for PC2/PC1.5 can be scaled by less than current 100% to achieve more meaningful PC2/PC1.5 IMD4 and IMD5.

- [Agree to capture just single worst case MSD value in TS for each PC which is agnostic to number of TX].

- Agree to use the following rules in deriving PC2/PC1.5 MSD for cases where PC3 MSD is 0dB:

- If one of the rules above is met, PC2 1TX MSD=0.4dB, PC2 2TX MSD=0.5dB, PC1.5 2TX MSD=0.8dB.

- If both rules above are met, PC2 1TX MSD=0.8dB, PC2 2TX MSD=0.9dB, PC1.5 2TX MSD=1.5dB.

- In case it is discovered that the PC3 MSD is omitted or is illogical compared to respective similar band combinations, then naturally the PC3 MSD can be further evaluated before PC2/PC1.5 assessment.

- Agree that the assumed PRX/DRX difference is 6dB, and additional noise into PRX is 1dB due to 2 TX case for PC2/PC1.5 NR CA and EN-DC.

- The tool is published in 3GPP in Excel format if all proposals are agreeable.

NOTE: Freedom for any company to provide PC3 MSD evaluations using companies’ calculation methods and other assumptions fully remains because PC3 is the baseline to derive PC2/PC1.5 MSD. It is not precluded to provide evaluations for PC2/PC1.5.

Option 3:

  - PC2/PC1.5 MSD requirements are not specified. UE only need to be verified under the Rx requirements of default power class, with default power class transmission

  - Add the Equations/LUT/Simplified guidelines into the Annex of TS to derive the HPUE MSD for information, in Rel-19 if possible, the details are FFS

  - FFS the expected spec changes and procedure changes (i.e., continue/stop providing TP’s for each combination, remove/keep all existing HPUE notes/ HPUE MSD tables etc) are recommended to be applicable from Rel-20, or the late stage of Rel-19

-Decision can be deferred until the Equations/LUT/Simplified guidelines are getting done

  - Inform RAN5 after all the discussion are finalized, and it is up to RAN5 to decide how to simplify HPUE CA/DC MSD conformance testing, [and RAN4 can make recommendation if any consensus]

NOTE 1: RAN4 continues normal procedure specifying MSD for HPUE band combination supports at least until down-selection is done.

NOTE 2: RAN4 discuss/consider all HPUE scenarios/configurations as a package for sake of consistence. For the scenarios out of this WI scope, the agreements (if any) are recommended for band combination HPUE basket WIs to follow.

Option 4:

  - PC2/PC1.5 MSD requirements are not specified, MSD for PC2 and PC1.5 are only specified in TR or Annes of TS which is not mandate, UE shall meet the Rx requirements of default power class, with default power class transmission

Agreement:

- To clarify the Equations/LUT/Simplified guidelines mentioned in Option 1 and Option 3

  - Some examples are provided in the Annex of this WF

- Other options such as Option2 are not precluded

The annex of the way forward provided examples on how the HPUE delta MSD could be either:

- based on MSD statistics and provided in tables in Annex A1.

- based on calculating delta MSD from the increased interference level and presented in the form of curves in Annex A2 that could be simplified to LUTs. Note that it was shown in [1] that there is good accordance between the calculated delta MSD from the interference increase model and the 1UL specified values.

Observation: All options except Option 2 rely on having a uniform approach by providing delta MSD tables or curves based on statistics or calculations and captured in tables, curves, or equations. The main differences are:

- Delta MSD based on statistics or calculations. Although examples were provided the data is more mature for 1UL than 2UL

- Delta MSD being normative or informative.

- Whether or not these are captured as a requirement in TS, in TRs, in annex of TS, ect.

Based on the above, in this contribution we focus our observations on the 2UL HPUE MSD cases that require further elaboration in order to provide delta MSD LUT based on calculations, while other papers [3,4] make similar observations and proposals for calculation based LUTs for 1UL HPUE MSDs. Based on this paper observations, a first phase of proposals for 2UL HPUE MSD will be covered in [5].

## 2.2 HPUE cases with issues

HPUE cases with incomplete or incorrect requirements

Table 1 collects issues with the two UL band MSDs for PC2 and PC1.5 with the following parameters/content:

- The first two columns provide the DLCA and ULCA configurations.

- The next three columns provide the specified inter-band HPUE power class for PC2 2Tx, PC2 3Tx and PC1.5 3Tx

- The next column provides the victim band (FDD band in these FDD+TDD cases)

- The next two columns provide the specified MSDs for PC3 and PC2

- The next column “Source of IMD” provides the IMD order and type for the MSDs.

Table 1: HPUE combinations with issues

| NR CA configuration |  | 2Tx PC2 | 3Tx PC2 | 3Tx PC1.5 | NR band | PC3MSD | PC2MSD | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DLCA | ULCA |  |  |  |  | 20+20 | 23+23/2x20 |  |
| n25-n77 | n25-n77 | 23F+23/26T | missing? | 23F+29T | n25 | 26 | 32.1 | 2B_IMD2 |
| n70-n77 | n70-n77 | missing? | 23F+23/26T | 23F+29T | n70 | 31 | 37 | 2B_IMD2 |
| n7-n20 | n7-n20 | this is probably PC3 in a PC2 table |  |  | n20 | missing | 12 | 2B_IMD3 |
| n25-n41C | n25-n41C | 23F+23/26T | 23F+23/26T | 23F+29T | n25 | missing | 15.3 | 3CC_IMD3 |
| n8-n77 | n8-n77 | 23F+23/26T |  |  | n8 | missing | 15.5 | 2B_IMD4 |
| n3-n28-n78 | n28-n78 | 23F+23/26T | 23F+23/26T |  | n3 | missing | 29.2 | IMD3toB3 |
| n18-n77 | n18-n77 | 23F+23/26T |  |  | n18 | 8.4 | 17.5 | 2B_IMD4 |
| n18-n77 | n18-n77 | 23F+23/26T |  |  | n18 | N/A | 18.4 | 2B_IMD4 |
| n18-n77 | n18-n77 | 23F+23/26T |  |  | n18 | 4.5 | 10.5 | 2B_IMD5 |
| n18-n77 | n18-n77 | 23F+23/26T |  |  | n18 | N/A | 11.7 | 2B_IMD5 |
| n20-n78 | n20-n78 | probably a PC3 case? |  |  | n20 | 11 | missing? | 2B_IMD4 |
| n20-n78(2A) | n20-n78(2A) | probably a PC3 case |  |  | n20 | missing | 8.6 | 2CC_IMD4 |
| n28-n78(2A) | n28-n78(2A) | 23F+23/26T | is this PC3 only? |  | n28 | missing | 18.5 | 2CC_IMD4 |
| n28-n78 | n28-n78 | 23F+23/26T | 23F+23/26T |  | n28 | 5.5 | missing | 2B_IMD5 |
| n5-n77 | n5-n77 | 23F+23/26T | 23F+23/26T | 23F+29T | n5 | 5.5 | missing | 2B_IMD5 |
| n5-n7-n78 | n5-n78 | 23F+23/26T |  |  | n5 | 3.3 | missing | IMD5toB3 |
| n7-n28-n78 | n7-n78 | 23F+23/26T |  |  | n28 | 3 | missing | IMD5toB3 |

Observations:

- CA_n7-n20 is an FDD-FDD combination which was not in scope for R18 HPUE and is not in the HPUE power class table and has a missing PC3 MSD and a PC2 MSD. It seems that this is a PC3 combination MSD that was captured in the PC2 table instead of PC3.

- CA_n20-n78/n78(2A) have missing PC2 or PC3 MSD while the other MSD is specified and is not in the HPUE power class table. It seems that this is a PC3 combination MSD that was captured in the PC2 table instead of PC3 for the CA_n20-n78(2A) case.

- CA_n28-n78/n78(2A) have missing PC2 or PC3 MSD while the other MSD is specified and is valid in the HPUE power class table. The CA_n28-n78(2A) is possibly only for PC3, but then CA_n28-n78 is missing PC2 MSD.

- CA_n25-n77 has PC1.5 3Tx power class configuration but not PC2 3Tx power class configuration which should be there as a fallback.

- CA_n70-n77 has PC2 3Tx power class configuration but not PC2 2Tx power class configuration which seems to be an omission compared to other cases where combinations supporting 3Tx PC2 also support 2Tx PC2.

- CA_n18-n77 has two sets of MSDs for both IMD4 and IMD5 with one having N/A for the PC3 case. The valid cases should be sorted.

- Some cases have a large PC2 MSD but no PC3 MSD: CA_n25-n41C, CA_n8-n77, CA_n3-n28-n78, and CA_n28-n78(2A). This needs to be understood.

- Some cases have a PC3 MSD but no PC2 MSD: CA_n28-n78, CA_n5-n77, CA_n5-n7-n78, and CA_n7-n28-n78. These cases are incomplete.

Incorrect HPUE power class

Observations:

- There are HPUE cases specified in the power class table but that are not valid in the BCS table: CA_n28A-n41C, CA_n28A-n79C, CA_n40A-n79C, CA_n41A-n79C and CA_n41C-n79A.

- There are HPUE cases that have an HPUE MSD specified but are not in the power class table, notably the cases where one band supports intra-band ULCA: CA_n1-n77(2A), CA_n1-n78(2A), CA_n3-n77(2A), CA_n3-n78(2A), CA_n5-n77(2A), CA_n5-n78(2A), CA_7-n77(2A), CA_n7-n78(2A), CA_n25-n41C, CA_n25-n77(2A), CA_n25-n78(2A), CA_n28-n77(2A), CA_n28-n78(2A), CA_n41C-n66, CA_n41C-n77, CA_n41-n77(2A), CA_n66-n77(2A), CA_n66-n78(2A) and CA_n71-n77(2A).

HPUE IMD cases with incorrect MSD

Table 2 collects all the PC2 cases for inter-band DL CA with one band UL with intra-band ULCA with the following parameters/content:

- The first two columns provide the DLCA and ULCA configurations.

- The next two columns provide the specified inter-band HPUE power class for PC2 2Tx and PC2 3Tx

- The next column provides the victim band (FDD band in these FDD+TDD cases)

- The next two columns provide the specified MSDs for PC3 and PC2

- The next column “Source of IMD” provides the IMD order and type for the MSDs.

- The next two columns provide the specified versus calculated delta MSD between PC3 and PC2.

Table 2: One UL band with 2CC MSD

| NR CA configuration |  | 2Tx PC2 | 3Tx PC2 | NR band | PC3MSD | PC2MSD | Source of IMD | PC2 Delta MSD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DLCA | ULCA |  |  |  | 20+20 | 23+23/2x20 |  | spec | calc |
| n5-n77(2A) | n5-n77(2A) | 23F+23/26T |  | n5 | 8.6 | 18.5 | 2CC_IMD4 | 9.9 | 0 |
| n5-n78(2A) | n5-n78(2A) | 23F+23/26T |  | n5 | 8.6 | 18.5 | 2CC_IMD4 | 9.9 | 0 |
| n20-n78(2A) | n20-n78(2A) | probably a PC3 case |  | n20 | missing | 8.6 | 2CC_IMD4 | #! | 0 |
| n28-n77(2A) | n28-n77(2A) | 23F+23/26T |  | n28 | 8.6 | 18.5 | 2CC_IMD4 | 9.9 | 0 |
| n28-n78(2A) | n28-n78(2A) | 23F+23/26T | is this PC3 only? | n28 | missing | 18.5 | 2CC_IMD4 | #! | 0 |
| n71-n77(2A) | n71-n77(2A) | 23F+23/26T |  | n71 | 8.6 | 18.5 | 2CC_IMD4 | 9.9 | 0 |
| n1-n77(2A) | n1-n77(2A) | 23F+23/26T |  | n1 | 17 | 31 | 2CC_IMD5 | 14 | 0 |
| n7-n77(2A) | n7-n77(2A) | 23F+23/26T |  | n7 | 15 | 29.9 | 2CC_IMD5 | 14.9 | 0 |
| n7-n78(2A) | n7-n78(2A) | 23F+23/26T |  | n7 | 15 | 29.9 | 2CC_IMD5 | 14.9 | 0 |
| n41C-n66 | n41C-n66 | 23F+23/26T | 23F+23/26T | n66 | 32.5 | 32.5 | 2CC_IMD5 | 0 | 0 |
| n41-n77(2A) | n41-n77(2A) | 26/23T+23/26T |  | n41 | 17 | 32 | 2CC_IMD5 | 15 | 0 |
| n66-n77(2A) | n66-n77(2A) | 23F+23/26T |  | n66 | 15 | 31 | 2CC_IMD5 | 16 | 0 |
| n1-n78(2A) | n1-n78(2A) | 23F+23/26T |  | n1 | 1.7 | 13.6 | 2CC_IMD7 | 11.9 | 0 |
| n3-n77(2A) | n3-n77(2A) | 23F+23/26T |  | n3 | 2.2 | 13.6 | 2CC_IMD7 | 11.4 | 0 |
| n3-n78(2A) | n3-n78(2A) | 23F+23/26T |  | n3 | 2.2 | 13.6 | 2CC_IMD7 | 11.4 | 0 |
| n25-n41C | n25-n41C | 23F+23/26T | 23F+23/26T | n25 | 8.5 | 8.5 | 2CC_IMD7 | 0 | 0 |
| n25-n77(2A) | n25-n77(2A) | 23F+23/26T |  | n25 | 2.7 | 13.6 | 2CC_IMD7 | 10.9 | 0 |
| n25-n78(2A) | n25-n78(2A) | 23F+23/26T |  | n25 | 2.7 | 13.6 | 2CC_IMD7 | 10.9 | 0 |
| n66-n78(2A) | n66-n78(2A) | 23F+23/26T |  | n66 | 1.7 | 13.6 | 2CC_IMD7 | 11.9 | 0 |
| n41C-n77 | n41C-n77 | 26/23T+23/26T | 26T+26T | n77 | 2.7 | 2.7 | 2CC_IMD9 | 0 | 0 |

Observations:

- Except for CA_n41C-n66, CA_n25-n41C and CA_n41C-n77 which are correct, all the band combinations have a significant delta MSD between PC3 and PC2 that seems proportional to the IMD order as with the two-band IMD case.

- In fact, the delta should be zero as the IMD levels for the 1UL band intra-band CA is limited by the SEM mask (the UE can apply MPR for this) which is constant for all power classes and thus the IMD levels are constant:

  - IMD3 is limited to -13dBm/MHz and IMD5 to -30dBm/MHz for contiguous intra-band ULCA. Thus, higher order IMDs are also limited to the same power level independently of the band power class.

  - IMD3 is limited to -30dBm/MHz for non-contiguous intra-band ULCA. Thus, higher order IMDs are also limited to the same power level independently of the band power class.

- Consequently, the HPUE MSD related to 1UL band with intra-band CA should be equal to the PC3 value and the delta MSD is 0dB.

## 2.3 Observations on PC2 cases

Figure 1 and Figure 2 are derived from the annex Tables A1 and A2 covering PC2 two DL and three DL bands respectively, where the cases with issues described above are removed.

The two figures illustrate the delta MSD between PC2 and PC3 versus PC3 MSD with the following aspects:

- Calculated curves based on equations provided on the annex of [2] which assume that:

  - PC2 interference level = PC3 interference level + 3 * the IMD order, with:

  - Blue curve for IMD2, green curve for IMD3, orange curve for IMD4. and red curve for IMD5

- The delta MSD based on the specified PC3 and PC2 MSDs with:

  - Blue triangle for IMD2, green diamond for IMD3, orange dots for IMD4, and red squares for IMD5.

Figure 1: Two DL band PC2 delta MSD versus PC3 MSD. Specification points and calculated curves versus IMD order.

Figure 2: Three DL band PC2 delta MSD versus PC3 MSD. Specification points and calculated curves versus IMD order.

Observations:

- In both cases there is only one PC2 MSD value while there are multiple implementations possible:

  - 2TX PC2 with 1Tx PC3 FDD+ 1Tx PC3 TDD

  - 2TX PC2 with 1Tx PC3 FDD+ 1Tx PC2 TDD

  - 3TX PC2 with 1Tx PC3 FDD+ 2Tx PC3 TDD

  - 3TX PC2 with 1Tx PC3 FDD+ 2Tx PC2 TDD

- All the specified values seem consistent with a PC3+PC3 implementation which is valid for both PC3 and PC2 inter-band power class, and see the power of the two PAs rising by 3dB from 20dBm. and The IMD interference thus rises with 3xIMD order.

  - There is no consideration of MRC imbalance change between 2Tx and 3Tx, higher headroom for a PC2 TDD PA versus a PC3 TDD PA, or different coupling between PAs in a 2Tx and 3Tx architecture.

- The three bands case has more data points than the two bands case and has a better correlation to the calculated values.

  - IMD2 has good matching, but with its high value MSD for PC3, is in the saturated 6dB delta MSD part.

  - IMD3 has very good matching and is also mostly in the saturated 9dB delta MSD part.

  - IMD4 has good matching and is at the beginning of the saturated 12dB delta MSD part. One outlier seems to have been given the 12dB delta while, due to its low PC3 MSD, it should have been around 5dB.

  - IMD5 shows a lot more dispersion in delta MSD while the PC3 MSD spread is very narrow:

    - For two band MSD, the PC3 MSD is within 1dB while the delta MSD is 6dB to 15dB. If the latter can be explained by allocating the saturated 15dB value to cases where, with the low PC3 MSD, the delta should rather be in the 12dB; the 6dB value cannot be explained. These will probably require corrections.

    - For three band, the correlation is better with only a few cases that can be explained by allocating the saturated 15dB value to cases where, with the low PC3 MSD, the delta should rather be in the 12dB.

- The calculated curves based on 3xIMD order increase in interference power is valid for all R18 PC2 cases which, although the TDD band may have different power capability (PC3 or PC2) or architecture (1Tx or 2Tx), is consistent with the fact that the worst case is for the FDD PC3 PA that is unchanged.

## 2.4 Observations on PC1.5 cases

Figure 3 is derived from the annex Tables A1 covering PC1.5 two DL bands, where the cases with issues described above are removed. The figure illustrates the delta MSD between PC1.5 and PC3 versus PC3 MSD with the following aspects:

- Calculated curves based on equations provided on the annex of [2] but modified to account for the fact that the TDD PA is increased to 27.7dBm but in fact by 24.7dBm per PA with one coupling more tightly to the FDD PA:

  - PC1.5 interference level 	= PC3 interference level + 3 * the IMD order + 1.7*(IMDorder-1)

= PC3 interference level + 4.7 * the IMDorder – 1.7dB, with:

  - Blue curve for IMD2, green curve for IMD3, orange curve for IMD4, and red curve for IMD5

- The delta MSD based on the specified PC3 and PC1.5 MSDs with:

  - Blue triangle for IMD2, green diamond for IMD3, orange dots for IMD4, and red squares for IMD5.

Figure 3: Two DL band PC1.5 delta MSD versus PC3 MSD. Specification points and calculated curves versus IMD order.

Observations:

- There is only one PC1.5 MSD value in Release 19 as the only valid implementation is PC3 1Tx FDD + PC1.5 2TX TDD. But in release 19 more cases are anticipated.

- All the specified values seem to account for the sum of the TDD PAs power increase (this may be assuming fully separate antennas for the two TDD PAs from the PC3 FDD PA?). Further checking on PC1.5 assumptions is needed but a modified calculation based on an interference power increasing with the order of the IMD will work.

  - There is no consideration of MRC imbalance change between 2Tx and 3Tx, 1.3dB headroom for 2xPC2 TDD PA versus a PC3 TDD PA, or different coupling between PAs in a 2Tx and 3Tx architecture.

- The PC1.5 case has a limited number of data points and a lot of duplicated values. In general, the specified values are higher than the calculated values but that may be related to different coupling assumptions. Aside from that shift upwards:

  - IMD2 has good matching but with its high value MSD for PC3 is in the saturated delta MSD part.

  - There is no IMD3 case.

  - IMD4 has relatively good matching and is at the beginning of the saturated delta MSD part. One outlier seems to have been given a large delta.

  - IMD5 show a lot more dispersion in delta MSD while the PC3 MSD spread is very narrow: the PC3 MSD is within 1dB while the delta MSD is 10dB to 23dB. If the latter can be explained by allocating the saturated value to cases where, with the low PC3 MSD, the delta should lower; the 10dB value cannot be explained. These will probably require corrections.

# Conclusion

In this contribution, we analysed all the available HPUE 2UL MSDs in the latest available R18 specification and made several observations.

Observations on incorrect or incomplete cases:

- One FDD-FDD combination which was not in scope for R18 HPUE that is not in the HPUE power class table

- Some have missing PC2 or PC3 MSD while the other MSD is specified

- Some have PC1.5 3Tx power class configuration but not PC2 3Tx power class configuration which should be there as a fallback.

- Some have PC2 3Tx power class configuration but not PC2 2Tx power class configuration

- Some cases have a large PC2 MSD but no PC3 MSD

- Some cases have a PC3 MSD but no PC2 MSD

- There are HPUE cases that are specified in the power class table but are not valid in the BCS table

- There are HPUE cases that have an HPUE MSD specified but are not in the power class table, notably the cases where one band supports intra-band ULCA

Observations on 1UL band 2CC MSD:

- The HPUE MSD related to 1UL band with intra-band CA should be equal to the PC3 value and the Delta MSD is 0dB.  Many cases should be corrected.

Observations on PC2 HPUE MSD: The calculated curves based on the 3xIMD order increase in interference power is valid for all R18 PC2 cases which, although the TDD band may have different power capability (PC3 or PC2) or architecture (1Tx or 2Tx), is consistent with the fact that the worst case is for the FDD PC3 PA that is unchanged.

Observations on PC1.5 HPUE MSD:

- There is only one PC1.5 MSD value in Release 19 as the only valid implementation is PC3 1Tx FDD + PC1.5 2TX TDD. More cases are anticipated in release 19.

- All the specified values seem to account for the sum of the TDD PAs power increase (may be assuming fully separate antennas for the two TDD PAs from the PC3 FDD PA?). Further checking on PC1.5 assumptions is needed but a modified calculation based on an interference power increasing with the order of the IMD will work.

# References

[1] R4-2416252 HPUE CADC MSD simplification, Skyworks Solutions Inc., RAN4#112bis

[2] R4-2417098 WF on HPUE RF requirements, Samsung, Skyworks, Nokia, RAN4#112bis

[3] R4-2419712 1UL HPUE NRCA MSD observations, Skyworks Solutions Inc., RAN4#113

[4] R4-2419699 1UL HPUE NRCA MSD Simplifications for Rel-19, Skyworks Solutions Inc., RAN4#113

[5] R4-2417794 Simplification of HPUE 2UL IMD MSDs, Skyworks Solutions Inc., RAN4#113

# Annex

In this contribution, we show a number of graphs for different types of HPUE MSDs, in this annex we provide the tables from which these graphs are derived.

Table A1 covers the two UL band IMD2/3/4/5 MSDs for PC2 and PC1.5 with either:

- 1CC per band (2B_IMDX in “source of IMD” column) or

- 1CC in one band and 2CC on the other band (triple beat denominated 3CC_IMD3 in “source of IMD” column)

The table have the following parameters/content:

- The first two columns provide the DLCA and ULCA configurations.

- The next three columns provide the specified inter-band HPUE power class for PC2 2Tx, PC2 3Tx and PC1.5 3Tx

- The next column provides the victim band (FDD band in these FDD+TDD cases)

- The next three columns provide the specified MSDs for PC3, PC2 and PC1.5

- The next column “Source of IMD” provides the IMD order and type for the MSDs

- The next two or four columns provides the delta MSD specified and calculated based on the proportional to IMD order equations provided in annex of [2] and slightly modified for PC1.5 for PC2 and PC1.5

- The table order is with increasing IMD order.

Table A 1: PC2 and PC1.5 HPUE MSD for two band UL with 2 or 3CCs

| NR CA config |  | 2Tx PC2 | 3Tx PC2 | 3Tx PC1.5 | NR band | PC3 MSD | PC2 MSD | PC1.5 MSD | Source of IMD | PC2 Delta MSD |  | PC1.5 Delta MSD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DLCA | ULCA |  |  |  |  | 20+20 | 23+ 23/2x20 | 23+ (2x24.7) |  | spec | calc | spec | calc |
| n1-n77 | n1-n77 | 23F+23/26T |  |  | n1 | 29.8 | 35.8 |  | 2B_IMD2 | 6 | 6 | N/A | N/A |
| n2-n77 | n2-n77 | 23F+23/26T | 23F+23/26T | 23F+29T | n2 | 26 | 32.1 | 35.2 | 2B_IMD2 | 6.1 | 6 | 9.2 | 7.7 |
| n3-n77 | n3-n77 | 23F+23/26T |  |  | n3 | 26 | 31.9 |  | 2B_IMD2 | 5.9 | 6 | N/A | N/A |
| n3-n78 | n3-n78 | 23F+23/26T |  |  | n3 | 26 | 31.9 |  | 2B_IMD2 | 5.9 | 6 | N/A | N/A |
| n25-n77 | n25-n77 | 23F+23/26T | missing | 23F+29T | n25 | 26 | 32.1 | 37.5 | 2B_IMD2 | 6.1 | 6 | 11.5 | 7.7 |
| n25-n78 | n25-n78 | 23F+23/26T |  |  | n25 | 26 | 32.1 |  | 2B_IMD2 | 6.1 | 6 | N/A | N/A |
| n66-n77 | n66-n77 | 23F+23/26T | 23F+23/26T | 23F+29T | n66 | 31 | 34.33 | 40 | 2B_IMD2 | 3.33 | 6 | 9 | 7.7 |
| n70-n77 | n70-n77 | missing? | 23F+23/26T | 23F+29T | n70 | 31 | 37 | 41.8 | 2B_IMD2 | 6 | 6 | 10.8 | 7.7 |
| n7-n20 | n7-n20 | this is probably PC3 in a PC2 table |  |  | n20 | missing | 12 |  | 2B_IMD3 | #! | 6.4 | N/A | N/A |
| n18-n41 | n18-n41 | 23F+23/26T |  |  | n18 | 15.6 | 24.6 |  | 2B_IMD3 | 9 | 9 | N/A | N/A |
| n3-n41C | n3-n41C | 23F+23/26T |  |  | n3 | 15.3 | 23.3 |  | 3CC_IMD3 | 8 | 6 | N/A | N/A |
| n25-n41C | n25-n41C | 23F+23/26T | 23F+23/26T | 23F+29T | n25 | missing | 15.3 |  | 3CC_IMD3 | #! | 0 | N/A | N/A |
| n1-n77 | n1-n77 | 23F+23/26T |  |  | n1 | 8 | 17.8 |  | 2B_IMD4 | 9.8 | 10.4 | N/A | N/A |
| n1-n78 | n1-n78 | 23F+23/26T |  |  | n1 | 8 | 17.8 |  | 2B_IMD4 | 9.8 | 10.4 | N/A | N/A |
| n2-n77 | n2-n77 | 23F+23/26T | 23F+23/26T | 23F+29T | n2 | 8 | 19.1 | 26.4 | 2B_IMD4 | 11.1 | 10.4 | 18.4 | 15.3 |
| n3-n41 | n3-n41 | 23F+23/26T |  |  | n3 | 8.2 | 18.4 |  | 2B_IMD4 | 10.2 | 10.4 | N/A | N/A |
| n3-n77 | n3-n77 | 23F+23/26T |  |  | n3 | 8 | 18.5 |  | 2B_IMD4 | 10.5 | 10.4 | N/A | N/A |
| n3-n78 | n3-n78 | 23F+23/26T |  |  | n3 | 8 | 18.5 |  | 2B_IMD4 | 10.5 | 10.4 | N/A | N/A |
| n5-n77 | n5-n77 | 23F+23/26T | 23F+23/26T | 23F+29T | n5 | 8.3 | 18.6 | 25.5 | 2B_IMD4 | 10.3 | 10.4 | 17.2 | 15.3 |
| n5-n78 | n5-n78 | 23F+23/26T |  |  | n5 | 8.3 | 18.6 |  | 2B_IMD4 | 10.3 | 10.4 | N/A | N/A |
| n7-n77 | n7-n77 | 23F+23/26T | 23F+23/26T | 23F+29T | n7 | 7.1 | 15.8 | 29.7 | 2B_IMD4 | 8.7 | 10 | 22.6 | 15.3 |
| n8-n77 | n8-n77 | 23F+23/26T |  |  | n8 | missing | 15.5 |  | 2B_IMD4 | #! | 9.4 | N/A | N/A |
| n8-n78 | n8-n78 | 23F+23/26T | 23F+23/26T |  | n8 | 8.3 | 15.5 |  | 2B_IMD4 | 7.2 | 10.4 | N/A | N/A |
| n18-n77 | n18-n77 | 23F+23/26T |  |  | n18 | 8.4 | 17.5 |  | 2B_IMD4 | 9.1 | 10.4 | N/A | N/A |
| n18-n77 | n18-n77 | 23F+23/26T |  |  | n18 | N/A | 18.4 |  | 2B_IMD4 | N/A | 10.4 | N/A | N/A |
| n20-n78 | n20-n78 | probably a PC3 case? |  |  | n20 | 11 | missing? |  | 2B_IMD4 | #! | 11.2 | N/A | N/A |
| n25-n77 | n25-n77 | 23F+23/26T | missing | 23F+29T | n25 | 8 | 19.1 | 25.8 | 2B_IMD4 | 11.1 | 10.4 | 17.8 | 15.3 |
| n26-n78 | n26-n78 | 23F+23/26T | 23F+23/26T |  | n26 | 11.1 | 23.8 |  | 2B_IMD4 | 12.7 | 11.2 | N/A | N/A |
| n30-n77 | n30-n77 | 23F+23/26T |  |  | n30 | 8 | 17.6 |  | 2B_IMD4 | 9.6 | 10.4 | N/A | N/A |
| n41-n71 | n41-n71 | 23F+23/26T | 23F+23/26T | 23F+29T | n71 | 11 | 16.3 | 25.4 | 2B_IMD4 | 5.3 | 11.2 | 14.4 | 16.3 |
| n2-n77 | n2-n77 | 23F+23/26T | 23F+23/26T | 23F+29T | n2 | 5 | 20 | 28 | 2B_IMD5 | 15 | 11.5 | 23 | 18.1 |
| n5-n77 | n5-n77 | 23F+23/26T | 23F+23/26T | 23F+29T | n5 | 5.5 | missing | 24.3 | 2B_IMD5 | #! | 11.9 | 18.8 | 18.1 |
| n8-n79 | n8-n79 | 23F+23/26T |  |  | n8 | 4.8 | 21.5 |  | 2B_IMD5 | 16.7 | 11.3 | N/A | N/A |
| n12-n77 | n12-n77 | 23F+23/26T |  |  | n12 | 5.5 | 11.7 |  | 2B_IMD5 | 6.2 | 11.9 | N/A | N/A |
| n13-n77 | n13-n77 | 23F+23/26T |  |  | n13 | 5.5 | 20.5 |  | 2B_IMD5 | 15 | 11.9 | N/A | N/A |
| n14-n77 | n14-n77 | 23F+23/26T |  |  | n14 | 5.5 | 11.7 |  | 2B_IMD5 | 6.2 | 11.9 | N/A | N/A |
| n18-n77 | n18-n77 | 23F+23/26T |  |  | n18 | 4.5 | 10.5 |  | 2B_IMD5 | 6 | 11 | N/A | N/A |
| n18-n77 | n18-n77 | 23F+23/26T |  |  | n18 | N/A | 11.7 |  | 2B_IMD5 | N/A | 9.1 | N/A | N/A |
| n25-n77 | n25-n77 | 23F+23/26T | missing | 23F+29T | n25 | 5 | missing | 29.8 | 2B_IMD5 | #! | 11.2 | 24.8 | 18.1 |
| n28-n77 | n28-n77 | 23F+23/26T |  |  | n28 | 5.5 | 19.2 |  | 2B_IMD5 | 13.7 | 11.9 | N/A | N/A |
| n28-n78 | n28-n78 | 23F+23/26T | 23F+23/26T |  | n28 | 5.5 | missing |  | 2B_IMD5 | #! | 11.9 | N/A | N/A |
| n66-n77 | n66-n77 | 23F+23/26T | 23F+23/26T | 23F+29T | n66 | 5 | 11.27 | 26 | 2B_IMD5 | 6.27 | 11.2 | 21 | 18.1 |
| n66-n78 | n66-n78 | 23F+23/26T |  |  | n66 | 5 | 11.27 |  | 2B_IMD5 | 6.27 | 11.2 | N/A | N/A |
| n70-n77 | n70-n77 | missing? | 23F+23/26T | 23F+29T | n70 | 5 | 18.4 | 27.9 | 2B_IMD5 | 13.4 | 11.2 | 22.9 | 18.1 |
| n71-n77 | n71-n77 | 23F+23/26T | 23F+23/26T | 23F+29T | n71 | 5.5 | 11.4 | 16 | 2B_IMD5 | 5.9 | 11.9 | 10.5 | 18.1 |
| n71-n78 | n71-n78 | 23F+23/26T |  |  | n71 | 5.5 | 11.4 |  | 2B_IMD5 | 5.9 | 11.9 | N/A | N/A |
| n77-n85 | n77-n85 | 23F+23/26T |  |  | n85 | 5.5 | 11.7 |  | 2B_IMD5 | 6.2 | 11.9 | N/A | N/A |

Table A2 covers the two UL band IMD2/3/4/5 MSDs for PC2 3 band DL combinations:

- The first two columns provide the DLCA and ULCA configurations.

- The next two columns provide the specified inter-band HPUE power class for PC2 2Tx and PC2 3Tx.

- The next column provides the victim band (FDD band in these FDD+TDD cases)

- The next two columns provide the specified MSDs for PC3 and PC2

- The next column “Source of IMD” provides the IMD order and type for the MSDs.

- The next two columns provide the delta MSD specified and calculated based on the proportional to IMD order equations provided in annex of [2] PC2.

- The table order is with increasing IMD order.

Table A 2: PC2 HPUE MSD for one band UL with 2CCs

| NR CA configuration |  | 2Tx PC2 | 3Tx PC2 | NR band | PC3MSD | PC2MSD | Source of IMD | PC2 Delta MSD |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DLCA | ULCA |  |  |  | 20+20 | 23+23/2x20 |  | spec | calc |
| n1-n3-n77 | n1-n77 | 23F+23/26T |  | n3 | 31.5 | 37.5 | IMD2toB3 | 6 | 6 |
| n1-n3-n77 | n3-n77 | 23F+23/26T |  | n1 | 31 | 37 | IMD2toB3 | 6 | 6 |
| n1-n3-n78 | n1-n78 | 23F+23/26T |  | n3 | 27.9 | 33.9 | IMD2toB3 | 6 | 6 |
| n1-n28-n41 | n1-n41 | 23F+23/26T |  | n28 | 29.3 | 36.6 | IMD2toB3 | 7.3 | 6 |
| n2-n66-n77 | n2-n77 | 23F+23/26T | 23F+23/26T | n66 | 29.2 | 34.7 | IMD2toB3 | 5.5 | 6 |
| n2-n66-n77 | n66-n77 | 23F+23/26T | 23F+23/26T | n2 | 32.1 | 37.6 | IMD2toB3 | 5.5 | 6 |
| n3-n28-n41 | n3-n41 | 23F+23/26T |  | n28 | 26 | 32 | IMD2toB3 | 6 | 6 |
| n3-n28-n41 | n28-n41 | 23F+23/26T | 23F+23/26T | n3 | 26 | 32 | IMD2toB3 | 6 | 6 |
| n5-n7-n77 | n7-n77 | 23F+23/26T | 23F+23/26T | n5 | 30.2 | 35.2 | IMD2toB3 | 5 | 6 |
| n5-n7-n77 | n5-n77 | 23F+23/26T | 23F+23/26T | n7 | 30.1 | 35.1 | IMD2toB3 | 5 | 6 |
| n5-n7-n78 | n7-n78 | 23F+23/26T |  | n5 | 30.2 | 35.2 | IMD2toB3 | 5 | 6 |
| n5-n7-n78 | n5-n78 | 23F+23/26T |  | n7 | 30.1 | 35.1 | IMD2toB3 | 5 | 6 |
| n7-n71-n77 | n71-n77 | 23F+23/26T | 23F+23/26T | n7 | 29.6 | 34.6 | IMD2toB3 | 5 | 6 |
| n7-n26-n78 | n7-n78 | 23F+23/26T |  | n26 | 30.2 | 36.2 | IMD2toB3 | 6 | 6 |
| n7-n26-n78 | n26-n78 | 23F+23/26T | 23F+23/26T | n7 | 30.1 | 36.1 | IMD2toB3 | 6 | 6 |
| n7-n28-n78 | n7-n78 | 23F+23/26T |  | n28 | 28.8 | 33.8 | IMD2toB3 | 5 | 6 |
| n7-n28-n78 | n28-n78 | 23F+23/26T | 23F+23/26T | n7 | 30.5 | 35.5 | IMD2toB3 | 5 | 6 |
| n18-n41-n77 | n18-n41 | 23F+23/26T |  | n77 | 30.1 | 33 | IMD2toB3 | 2.9 | 6 |
| n18-n41-n77 | n18-n77 | 23F+23/26T |  | n41 | 28.5 | 32.6 | IMD2toB3 | 4.1 | 6 |
| n18-n41-n77 | n41-n77 | 26/23T+23/26T | 26T+26T | n18 | 29.3 | 33.8 | IMD2toB3 | 4.5 | 6 |
| n25-n66-n77 | n25-n77 | 23F+23/26T | missing | n66 | 29.2 | 35.2 | IMD2toB3 | 6 | 6 |
| n25-n66-n77 | n66-n77 | 23F+23/26T | 23F+23/26T | n25 | 32.1 | 38.1 | IMD2toB3 | 6 | 6 |
| n25-n66-n78 | n66-n78 | 23F+23/26T |  | n25 | 32.1 | 37.6 | IMD2toB3 | 5.5 | 6 |
| n28-n41-n77 | n28-n41 | 23F+23/26T | 23F+23/26T | n77 | 28.2 | 34.2 | IMD2toB3 | 6 | 6 |
| n28-n41-n77 | n41-n77 | 26/23T+23/26T | 26T+26T | n28 | 30.8 | 36.8 | IMD2toB3 | 6 | 6 |
| n28-n41-n77 | n28-n77 | 23F+23/26T |  | n41 | 29.5 | 35.5 | IMD2toB3 | 6 | 6 |
| n28-n77-n79 | n77-n79 | 26/23T+23/26T |  | n28 | 16.2 | 22.7 | IMD2toB3 | 6.5 | 6 |
| n30-n66-n77 | n66-n77 | 23F+23/26T | 23F+23/26T | n30 | 29.2 | 34.2 | IMD2toB3 | 5 | 6 |
| n41-n71-n77 | n41-n71 | 23F+23/26T | 23F+23/26T | n77 | 29.1 | 35.1 | IMD2toB3 | 6 | 6 |
| n41-n71-n77 | n71-n77 | 23F+23/26T | 23F+23/26T | n41 | 28.7 | 34.7 | IMD2toB3 | 6 | 6 |
| n41-n71-n77 | n41-n77 | 26/23T+23/26T | 26T+26T | n71 | 30.8 | 36.8 | IMD2toB3 | 6 | 6 |
| n1-n28-n77 | n28-n77 | 23F+23/26T |  | n1 | 15.7 | 24.7 | IMD3toB3 | 9 | 8.8 |
| n1-n28-n78 | n28-n78 | 23F+23/26T | 23F+23/26T | n1 | 15.7 | 24.6 | IMD3toB3 | 8.9 | 8.8 |
| n1-n28-n79 | n1-n79 | 23F+23/26T |  | n28 | 15.2 | 24.2 | IMD3toB3 | 9 | 8.7 |
| n1-n77-n79 | n77-n79 | 26/23T+23/26T |  | n1 | 15.6 | 24.6 | IMD3toB3 | 9 | 8.8 |
| n1-n41-n77 | n1-n41 | 23F+23/26T |  | n77 | 19.6 | 28.2 | IMD3toB3 | 8.6 | 8.9 |
| n2-n5-n77 | n5-n77 | 23F+23/26T | 23F+23/26T | n2 | 16.5 | 24.8 | IMD3toB3 | 8.3 | 8.9 |
| n2-n12-n77 | n12-n77 | 23F+23/26T |  | n2 | 16.5 | 24.8 | IMD3toB3 | 8.3 | 8.9 |
| n2-n14-n77 | n14-n77 | 23F+23/26T |  | n2 | 16.5 | 24.8 | IMD3toB3 | 8.3 | 8.9 |
| n3-n7-n78 | n7-n78 | 23F+23/26T |  | n3 | 17.6 | 26.6 | IMD3toB3 | 9 | 8.9 |
| n3-n18-n77 | n18-n77 | 23F+23/26T |  | n3 | 15.7 | 24.2 | IMD3toB3 | 8.5 | 8.8 |
| n3-n28-n77 | n3-n77 | 23F+23/26T |  | n28 | 15.3 | 24.2 | IMD3toB3 | 8.9 | 8.7 |
| n3-n28-n77 | n28-n77 | 23F+23/26T |  | n3 | 17 | 25.8 | IMD3toB3 | 8.8 | 8.9 |
| n3-n28-n78 | n28-n78 | 23F+23/26T | 23F+23/26T | n3 | missing | 29.2 | IMD3toB3 | #! | 8.9 |
| n3-n41-n77 | n3-n41 | 23F+23/26T |  | n77 | 16.8 | 25.6 | IMD3toB3 | 8.8 | 8.9 |
| n3-n41-n77 | n41-n77 | 26/23T+23/26T | 26T+26T | n3 | 16.4 | 25.1 | IMD3toB3 | 8.7 | 8.9 |
| n3-n77-n79 | n77-n79 | 26/23T+23/26T |  | n3 | 15.7 | 24.2 | IMD3toB3 | 8.5 | 8.8 |
| n5-n25-n77 | n5-n77 | 23F+23/26T | 23F+23/26T | n25 | 16.5 | 24.8 | IMD3toB3 | 8.3 | 8.8 |
| n5-n25-n78 | n5-n78 | 23F+23/26T |  | n25 | 16.5 | 24.8 | IMD3toB3 | 8.3 | 8.8 |
| n5-n30-n77 | n30-n77 | 23F+23/26T |  | n5 | 15.2 | 23.5 | IMD3toB3 | 8.3 | 8.7 |
| n5-n30-n77 | n5-n77 | 23F+23/26T | 23F+23/26T | n30 | 13.2 | 21.4 | IMD3toB3 | 8.2 | 8.6 |
| n5-n66-n77 | n5-n77 | 23F+23/26T | 23F+23/26T | n66 | 13.2 | 22.2 | IMD3toB3 | 9 | 8.6 |
| n12-n30-n77 | n30-n77 | 23F+23/26T |  | n12 | 15.2 | 23.5 | IMD3toB3 | 8.3 | 8.7 |
| n12-n30-n77 | n12-n77 | 23F+23/26T |  | n30 | 13.2 | 21.4 | IMD3toB3 | 8.2 | 8.6 |
| n12-n66-n77 | n66-n77 | 23F+23/26T | 23F+23/26T | n12 | 15.2 | 23.5 | IMD3toB3 | 8.3 | 8.7 |
| n12-n66-n77 | n12-n77 | 23F+23/26T |  | n66 | 13.2 | 21.4 | IMD3toB3 | 8.2 | 8.6 |
| n13-n25-n77 | n13-n77 | 23F+23/26T |  | n25 | 16 | 24 | IMD3toB3 | 8 | 8.8 |
| n13-n66-n77 | n13-n77 | 23F+23/26T |  | n66 | 17.1 | 26 | IMD3toB3 | 8.9 | 8.9 |
| n13-n66-n77 | n66-n77 | 23F+23/26T | 23F+23/26T | n13 | 15.2 | 24.1 | IMD3toB3 | 8.9 | 8.7 |
| n14-n30-n77 | n30-n77 | 23F+23/26T |  | n14 | 15.2 | 23.5 | IMD3toB3 | 8.3 | 8.7 |
| n14-n30-n77 | n14-n77 | 23F+23/26T |  | n30 | 13.2 | 21.4 | IMD3toB3 | 8.2 | 8.6 |
| n14-n66-n77 | n66-n77 | 23F+23/26T | 23F+23/26T | n14 | 15.2 | 23.5 | IMD3toB3 | 8.3 | 8.7 |
| n14-n66-n77 | n14-n77 | 23F+23/26T |  | n66 | 13.2 | 21.4 | IMD3toB3 | 8.2 | 8.6 |
| n25-n41-n77 | n25-n41 | 23F+23/26T | 23F+23/26T | n77 | 14.8 | 23.7 | IMD3toB3 | 8.9 | 8.6 |
| n25-n41-n77 | n41-n77 | 26/23T+23/26T | 26T+26T | n25 | 17.6 | 26.5 | IMD3toB3 | 8.9 | 8.8 |
| n25-n71-n77 | n71-n77 | 23F+23/26T | 23F+23/26T | n25 | 16.5 | 25.4 | IMD3toB3 | 8.9 | 8.8 |
| n29-n30-n77 | n30-n77 | 23F+23/26T |  | n29 | 15.2 | 23.5 | IMD3toB3 | 8.3 | 8.7 |
| n29-n66-n77 | n66-n77 | 23F+23/26T | 23F+23/26T | n29 | 15.2 | 23.5 | IMD3toB3 | 8.3 | 8.7 |
| n41-n66-n77 | n41-n66 | 23F+23/26T | 23F+23/26T | n77 | 16.1 | 25 | IMD3toB3 | 8.9 | 8.8 |
| n41-n71-n77 | n41-n71 | 23F+23/26T | 23F+23/26T | n77 | 16.3 | 25.2 | IMD3toB3 | 8.9 | 8.8 |
| n41-n71-n77 | n71-n77 | 23F+23/26T | 23F+23/26T | n41 | 15.5 | 24.4 | IMD3toB3 | 8.9 | 8.7 |
| n66-n71-n77 | n71-n77 | 23F+23/26T | 23F+23/26T | n66 | 15.5 | 24.4 | IMD3toB3 | 8.9 | 8.7 |
| n66-n71-n77 | n66-n77 | 23F+23/26T | 23F+23/26T | n71 | 15.3 | 24.2 | IMD3toB3 | 8.9 | 8.6 |
| n1-n7-n78 | n1-n78 | 23F+23/26T |  | n7 | 9.1 | 20.6 | IMD4toB3 | 11.5 | 10.8 |
| n1-n7-n78 | n7-n78 | 23F+23/26T |  | n1 | 8.7 | 20.1 | IMD4toB3 | 11.4 | 10.6 |
| n1-n28-n79 | n28-n79 | 23F+23/26T |  | n1 | 1.2 | 13.4 | IMD4toB3 | 12.2 | 4.9 |
| n1-n41-n77 | n1-n77 | 23F+23/26T |  | n41 | 11.5 | 22 | IMD4toB3 | 10.5 | 11.2 |
| n1-n41-n77 | n41-n77 | 26/23T+23/26T | 26T+26T | n1 | 9.3 | 21.4 | IMD4toB3 | 12.1 | 10.8 |
| n2-n30-n77 | n30-n77 | 23F+23/26T |  | n2 | 8.6 | 19.3 | IMD4toB3 | 10.7 | 10.6 |
| n2-n30-n77 | n2-n77 | 23F+23/26T | 23F+23/26T | n30 | 10.6 | 22.2 | IMD4toB3 | 11.6 | 11.1 |
| n3-n7-n78 | n7-n78 | 23F+23/26T |  | n3 | 8.6 | 17 | IMD4toB3 | 8.4 | 10.6 |
| n3-n28-n79 | n3-n79 | 23F+23/26T |  | n28 | 10.3 | 21.5 | IMD4toB3 | 11.2 | 11.1 |
| n5-n14-n77 | n5-n77 | 23F+23/26T | 23F+23/26T | n14 | 11.6 | 20.3 | IMD4toB3 | 8.7 | 11.2 |
| n7-n25-n77 | n7-n77 | 23F+23/26T | 23F+23/26T | n25 | 8.6 | 20 | IMD4toB3 | 11.4 | 10.6 |
| n7-n66-n77 | n7-n77 | 23F+23/26T | 23F+23/26T | n66 | 8.7 | 20.5 | IMD4toB3 | 11.8 | 10.6 |
| n7-n66-n78 | n7-n78 | 23F+23/26T |  | n66 | 8.7 | 20.5 | IMD4toB3 | 11.8 | 10.6 |
| n25-n41-n66 | n41-n66 | 23F+23/26T | 23F+23/26T | n25 | 11 | 22.7 | IMD4toB3 | 11.7 | 11.2 |
| n25-n41-n77 | n41-n77 | 26/23T+23/26T | 26T+26T | n25 | 8.6 | 20 | IMD4toB3 | 11.4 | 10.6 |
| n25-n66-n77 | n25-n77 | 23F+23/26T | missing | n66 | 10.4 | 22 | IMD4toB3 | 11.6 | 11.2 |
| n25-n66-n77 | n66-n77 | 23F+23/26T | 23F+23/26T | n25 | 9.1 | 20.6 | IMD4toB3 | 11.5 | 10.8 |
| n25-n66-n78 | n25-n78 | 23F+23/26T |  | n66 | 10.4 | 21.1 | IMD4toB3 | 10.7 | 11.2 |
| n30-n66-n77 | n30-n77 | 23F+23/26T |  | n66 | 8.7 | 19.2 | IMD4toB3 | 10.5 | 10.6 |
| n41-n66-n77 | n41-n77 | 26/23T+23/26T | 26T+26T | n66 | 9 | 20.5 | IMD4toB3 | 11.5 | 10.8 |
| n41-n71-n77 | n41-n71 | 23F+23/26T | 23F+23/26T | n77 | 10.3 | 21.9 | IMD4toB3 | 11.6 | 11.2 |
| n1-n3-n79 | n3-n79 | 23F+23/26T |  | n1 | 3.6 | 18.7 | IMD5toB3 | 15.1 | 10.1 |
| n1-n28-n77 | n1-n77 | 23F+23/26T |  | n28 | 4.2 | 16.5 | IMD5toB3 | 12.3 | 10.8 |
| n1-n28-n78 | n1-n78 | 23F+23/26T |  | n28 | 4.2 | 17.2 | IMD5toB3 | 13 | 10.8 |
| n2-n5-n77 | n2-n77 | 23F+23/26T | 23F+23/26T | n5 | 3.8 | 13.6 | IMD5toB3 | 9.8 | 10.2 |
| n2-n30-n77 | n2-n77 | 23F+23/26T | 23F+23/26T | n30 | 3.4 | 12.9 | IMD5toB3 | 9.5 | 10.1 |
| n3-n28-n79 | n28-n79 | 23F+23/26T |  | n3 | 5.7 | 20.5 | IMD5toB3 | 14.8 | 12 |
| n3-n41-n77 | n3-n77 | 23F+23/26T |  | n41 | 5.3 | 13 | IMD5toB3 | 7.7 | 11.8 |
| n5-n7-n78 | n5-n78 | 23F+23/26T |  | n5 | 3.3 | missing | IMD5toB3 | #! | 9.9 |
| n5-n12-n77 | n12-n77 | 23F+23/26T |  | n5 | 3.9 | 14 | IMD5toB3 | 10.1 | 10.5 |
| n5-n12-n77 | n5-n77 | 23F+23/26T | 23F+23/26T | n12 | 4.4 | 14.9 | IMD5toB3 | 10.5 | 11 |
| n5-n14-n77 | n14-n77 | 23F+23/26T |  | n5 | 3.9 | 14 | IMD5toB3 | 10.1 | 10.5 |
| n5-n25-n77 | n25-n77 | 23F+23/26T | missing | n5 | 3.8 | 13.6 | IMD5toB3 | 9.8 | 10.5 |
| n5-n25-n78 | n25-n78 | 23F+23/26T |  | n5 | 3.8 | 13.6 | IMD5toB3 | 9.8 | 10.5 |
| n5-n29-n77 | n5-n77 | 23F+23/26T | 23F+23/26T | n29 | 4.4 | 14.9 | IMD5toB3 | 10.5 | 11 |
| n7-n25-n77 | n25-n77 | 23F+23/26T | missing | n7 | 5.3 | 18.8 | IMD5toB3 | 13.5 | 11.8 |
| n7-n66-n77 | n66-n77 | 23F+23/26T | 23F+23/26T | n7 | 3.4 | 18.8 | IMD5toB3 | 15.4 | 11.8 |
| n7-n26-n78 | n7-n78 | 23F+23/26T |  | n26 | 3.3 | 15.7 | IMD5toB3 | 12.4 | 9.9 |
| n7-n28-n78 | n7-n78 | 23F+23/26T |  | n28 | 3 | missing | IMD5toB3 | #! | 9.6 |
| n18-n28-n77 | n18-n77 | 23F+23/26T |  | n28 | 4.4 | 17.5 | IMD5toB3 | 13.1 | 11 |
| n18-n28-n77 | n28-n77 | 23F+23/26T |  | n18 | 3.9 | 12.1 | IMD5toB3 | 8.2 | 10.5 |
| n25-n41-n77 | n25-n41 | 23F+23/26T | 23F+23/26T | n77 | 4.2 | 17.2 | IMD5toB3 | 13 | 10.8 |
| n25-n41-n77 | n25-n77 | 23F+23/26T | missing | n41 | 5.3 | 18.8 | IMD5toB3 | 13.5 | 11.8 |
| n25-n66-n77 | n25-n77 | 23F+23/26T | missing | n66 | 4 | 16.9 | IMD5toB3 | 12.9 | 10.5 |
| n25-n66-n77 | n66-n77 | 23F+23/26T | 23F+23/26T | n25 | 2.1 | 13.2 | IMD5toB3 | 11.1 | 8.5 |
| n30-n66-n77 | n66-n77 | 23F+23/26T | 23F+23/26T | n30 | 3.4 | 12.9 | IMD5toB3 | 9.5 | 10.1 |
| n41-n66-n77 | n66-n77 | 23F+23/26T | 23F+23/26T | n41 | 5.2 | 18.7 | IMD5toB3 | 13.5 | 11.8 |