3GPP TSG-RAN WG4 Meeting #113                      R4-2417797

Orlando, US, 18th – 22nd November, 2024

Source:	China Telecom

Title:	Discussion on BS demodulation performance requirements for MMSE-IRC

Agenda Item:	7.16.3

Document for:	discussion

Introduction

In RAN#104 meeting, the Rel-19 new WID on NR demodulation performance: Phase 5 is approved.

The following objectives are approved for BS requirements:

- Define FR1 PUSCH performance requirements with inter-cell interference in homogeneous and heterogeneous deployment based on the following assumptions:

  - Receiver type: MMSE-IRC receiver

  - Scenarios:

    - FDD synchronous deployments

    - TDD synchronous deployments with aligned UL:DL configurations among cells

    - Slot-based transmission and aligned SCS among cells

  - Reuse LTE based interference profile in TR 36.884 as a starting point. Other interference profiles are not precluded.

The WF was approved in [1]. In this contribution, our views on the open issues are given.

Discussion

Interference profile

Status in the WF [1]: - On how to account for HPUE in existing LTE profiles: - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE. - It is not precluded for companies to bring system level simulation on other profiles. - Proposals on how to account for HPUE in existing LTE profiles. - Proposal 1: Perform new system-level simulations with the inclusion of HPUEs - Proposal 2: Consider artificially increasing DIP/INR with additional 3 or 7 dB

As for coverage of HPUE, on the one hand, we agree with the motivation of considering HPUE in the test scope since it is widely used in 5G NR system. On the other hand, such approach requires system level simulation from companies to derive the interference profile.

We do not expect different implementations of BS MMSE-IRC receiver under HPUE scenario or unaligned TDD pattern scenarios either. Thus, to avoid system level simulation work, if the performance gain of MMSE-IRC over baseline MMSE can be confirmed under legacy LTE settings, we are fine not to cover HPUE or un-aligned TDD pattern scenarios.

At the same time, to try to cover the practical deployment of HPUE, we are also fine to perform some artificial evaluation within RAN4 for MMSE-IRC performance study only, e.g., to check the performance gain of IRC over baseline MRC by increasing INR by 3 or 7 dB.

Proposal 1: Reuse the legacy LTE interference power level for requirement definition if the performance gain of MMSE-IRC over baseline MMSE can be confirmed under legacy LTE settings.

At the same time, to account for HPUE deployment, RAN4 could also perform evaluation to check performance gain of IRC over baseline MRC by artificially increasing INR by 3 or 7 dB.

Test scope

Status in the WF [1]: - Candidate options for DFT-s-OFDM: - Option 1: - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz - Other options are not precluded - Agreement: - For CP-OFDM: - For performance evaluation purpose, Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz. - Further discuss the CHBW for requirement definition

The BS MMSE-IRC feature is a baseband feature thus we think its requirements should be applicable for all SCS, TDD patterns, CHBW, PUSCH mapping type if supported.

Therefore, as for the test scope, it is proposed to align with the same test scope for Rel-15 PUSCH requirements.

- Channel bandwidth for CP-OFDM:

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz channel bandwidth

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz channel bandwidth

- Channel bandwidth for DFT-s-OFDM:

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

Proposal 2: For requirement definition, align with the same test scope for Rel-15 normal PUSCH requirements:

- Channel bandwidth for CP-OFDM:

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz channel bandwidth

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz channel bandwidth

- Channel bandwidth for DFT-s-OFDM:

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

MCS

Status in the WF [1]: - Option 1: Cover QPSK/16QAM/64QAM - Option 1A: Use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024) - Option 1B: MCS index 2, 12, 20 in MCS table 1 - Option 1C: Cover MCS 2, 16, 20 - Option 2: Choose only one MCS configuration - Option 3: Consider MCS 5(QPSK, R=379/1024) and MCS 13(16QAM, R=490/1024) and FFS 64QAM

We support to cover QPSK, 16QAM and 64QAM in the test. It is proposed to decide the final MCS based on the simulation results considering the performance gain over baseline receiver.

Proposal 3: Cover QPSK, 16QAM and 64QAM in the test and decide the final MCS based on the simulation results considering the performance gain over baseline receiver.

Waveform

Status in the WF [1]: - Cover CP-OFDM and FFS DFT-s-OFDM

Considering the BS MMSE-IRC performance verification should be applicable for almost all BS products, it is proposed to also cover DFT-s-OFDM waveform for the test coverage.

Proposal 4: Also cover DFT-s-OFDM waveform.

Channel condition

Status in the WF [1]: - For serving cell: - Option 1: TDLA30-10 and TDLC300-100 for HetNet and HomNet respectively - Option 2: TDLA30-10 and TDLB100-100 for HetNet and HomNet respectively - Option 3: Cover TDLA30-10, TDLB100-400, TDLC300-100 - Option 4: Use TDLB100-400 - For interference cell: - Option 1: Introduce new propagation channel for testing with desired delay spread 1000ns for the interference PUSCH, i.e., TDLC1000 - Option 2: Same propagation condition for serving and interference PUSCH - Option 3: Use TDLC300-100

The LTE BS MMSE-IRC test requirements use EPA 5 and EVA 70 channel model for the serving cell for HetNet and HomNet respectively. And ETU channel model is used for the interference cell(s).

Number of TX antennas (Note 1); Number of RX antennas (Note 1); Propagation conditions and correlation matrix (Annex B) (Note 2); DIP set. Tested signal; Interferer 1; Interferer 2. 1; 2; EPA 5 Low; ETU 5 Low; N/A; Set 2. EVA 70 Low; ETU 70 Low; N/A; Set 1*. 4; EPA 5 Low; ETU 5 Low; ETU 5 Low; Set 2. EVA 70 Low; ETU 70 Low; ETU 70 Low; Set 1*. 8; EPA 5 Low; ETU 5 Low; ETU 5 Low; Set 2. EVA 70 Low; ETU 70 Low; ETU 70 Low; Set 1*.

Since we have proposed to reuse the same interference power level (i.e., DIPs) for the interference cell, we should also use channel models with similar delay spread and Doppler frequency. Therefore, we propose to use TDLA30-10 and TDLC300-100 propagation channel model for the serving cell for HetNet and HomNet respectively.

As for the nonboring cell, currently TS38.104 has not yet modeled a propagation channel with delay spread as large as ETU model (i.e., 991ns) and with maximum excess tap delay as 5000ns. It is proposed to introduce new propagation channel for testing with desired delay spread 1000ns, i.e., TDLC1000.

Proposal 5: Use TDLA30-10 and TDLC300-100 propagation channel model for the serving cell for HetNet and HomNet respectively.

Proposal 6: Introduce new propagation channel for testing with desired delay spread 1000ns for the nonboring cell, i.e., TDLC1000.

DMRS configuration

Status in the WF [1]: - Candidate options on DMRS type: - Option 1: Cover DMRS configuration type 1 and type 2 - Option 2: DMRS configuration type 1 - Candidate options on DMRS sequence generation: - Option 1: - NID0=0, nSCID =0 for CP-OFDM - NID0=0, group hopping and sequence hopping are disabled for DFT-s-OFDM - Option 2: - nSCID =0 for all cells - Different NID0=0 for serving and interference cells

For the DMRS type, we prefer to align with the legacy test configuration, i.e., consider DMRS type 1 only.

For the configurations for DMRS sequence generation for the serving PUSCH, we support to reuse the same configuration for Rel-15 PUSCH requirements for CP-OFDM and DFT-s-OFDM respectively:

- CP-OFDM: NID0=0, nSCID =0

- DFT-s-OFDM: NID0=0, group hopping and sequence hopping are disabled

As for the neighboring PUSCH, we recognize that in the LTE enhanced requirements type A, different cell IDs are configured for different base stations, which results in differentand thus different DMRS sequence among cells. To align with LTE configuration, we propose to configure different DMRS sequence for each gNBs.

Proposal 7: For the serving PUSCH, reuse the same configuration for Rel-15 PUSCH requirements for CP-OFDM and DFT-s-OFDM respectively:

- CP-OFDM: NID0=0, nSCID =0

- DFT-s-OFDM: NID0=0, group hopping and sequence hopping are disabled

For the interfering PUSCH, configure different DMRS sequence for each gNBs, e.g., NID0=1 and 2 for the 2 neighboring cells.

Test requirement definition

Status in the WF [1]: - Option 1: Using SINR based requirement definition - Option 2: SNR based requirement definition

We have already agreed to use INR based interference modeling criteria for requirement definition, thus we support to use the SNR based requirement definition.

Proposal 8: Use the SNR based requirement definition

Conclusion

Proposal 1: Reuse the legacy LTE interference power level for requirement definition if the performance gain of MMSE-IRC over baseline MMSE can be confirmed under legacy LTE settings.

At the same time, to account for HPUE deployment, RAN4 could also perform evaluation to check performance gain of IRC over baseline MRC by artificially increasing INR by 3 or 7 dB.

Proposal 2: For requirement definition, align with the same test scope for Rel-15 normal PUSCH requirements:

- Channel bandwidth for CP-OFDM:

- For 15kHz SCS, cover 5MHz, 10MHz, 20MHz channel bandwidth

- For 30kHz SCS, cover 10MHz, 20MHz, 40MHz, 100MHz channel bandwidth

- Channel bandwidth for DFT-s-OFDM:

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

Proposal 3: Cover QPSK, 16QAM and 64QAM in the test and decide the final MCS based on the simulation results considering the performance gain over baseline receiver.

Proposal 4: Also cover DFT-s-OFDM waveform.

Proposal 5: Use TDLA30-10 and TDLC300-100 propagation channel model for the serving cell for HetNet and HomNet respectively.

Proposal 6: Introduce new propagation channel for testing with desired delay spread 1000ns for the nonboring cell, i.e., TDLC1000.

Proposal 7: For the serving PUSCH, reuse the same configuration for Rel-15 PUSCH requirements for CP-OFDM and DFT-s-OFDM respectively:

- CP-OFDM: NID0=0, nSCID =0

- DFT-s-OFDM: NID0=0, group hopping and sequence hopping are disabled

For the interfering PUSCH, configure different DMRS sequence for each gNBs, e.g., NID0=1 and 2 for the 2 neighboring cells.

Proposal 8: Use the SNR based requirement definition.

Reference

R4-2416548, WF on [112bis][319] NR_demod_Ph5_Part1_General_BS, China Telecom, RAN4#112bis, Oct 2024.