3GPP TSG-RAN WG4 Meeting #113                                                                                             R4-2417809

Orlando, US, Nov. 18 – Nov. 22, 2024

Source: 	CATT

Title: 	Discussion on UE RF requirements for the NTN new S-band

Agenda item:	6.17.2

Document for:	Approval

- Introduction

In RAN4 #112-bis meeting, a WF [1] on NR NTN support new S-band UE RF requirement was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some views on UE RF requirements for introducing the NTN new S-band.

- Discussion

2.1 Synchronization Raster

The agreements of UE synchronization raster has been made during previous meeting, as shown below:

#### Issue 1-2-1: SS Raster - Agreement - Specify both Case A and [Case B] as follows NTN satellite operating band; SS Block SCS; SS Block pattern1; Range of GSCN (First – <Step size> – Last). n252; 15 kHz; Case A; 5456 – <1> – 5494. [30 kHz]; [Case B]; [5460 – <1> – 5488]. NOTE : SS Block pattern is defined in clause 4.1 in 3GPP TS 38.213 [7]..

As described in TS 38.213, there are a total of three SSB patterns defined for FR1 and FR1-NTN:

- Case A: 15 kHz SSB SCS, for situations where operator requires a minimum system CBW of 5 MHz. The SSB CBW will be less than 5 MHz so the minimum system CBW could be 5 MHz.

- Case B: 30 KHz SSB SCS, for situations where operator requires coexistence with certain LTE bands. The SSB transmission will not be impacted by LTE PDCCH / CRS.

- Case C: 30 KHz SSB SCS, for situations where operator requires 60 kHz SCS sharing channel configuration.

In general, the SSB patterns for brand new operating bands are usually requested by operators. For SSB pattern Case A, it is supported by default in almost all FR1 operating bands, unless 15 kHz SCS is not defined.

For Case B, it is introduced if the operator needs to guarantee the coexistence of the new band with the LTE legacy band, as at least one SSB transmission will be protected from interference from LTE PDCCH / CRS transmissions. However, looking back at the discussions at previous meetings, we do not see a strong demand for case B from operators.

For Case C, since the maximum Tx CBW for new NTN S band is 20 MHz, the incentive to introduce 60 kHz SCS does not seem to be too strong, where 60 KHz SCS is usually used to support 100 MHz CBW scenarios.

Therefore, we believed that for UE synchronization raster of new NTN S band, the SSB pattern Case A should be defined for synchronization raster entries, as shown below:

Table 2.1-1: Applicable SS raster entries per operating band (FR1-NTN)

| NTN satellite operating band | SS Block SCS | SS Block pattern1 | Range of GSCN (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n256 | 15 kHz | Case A | 5429 – <1> – 5494 |
| n255 | 15 kHz | Case A | 3818 – <1> – 3892 |
|  | 30 kHz | Case B | 3824 – <1> – 3886 |
| n254 | 15 kHz | Case A | 6215 – <1> – 6244 |
|  | 30kHz | Case C | 6218 – <1> – 6241 |
| [n252] | 15 kHz | Case A | 5456 – <1> – 5494 |
| NOTE: SS Block pattern is defined in clause 4.1 in TS 38.213 [7]. |  |  |  |

Proposal 1: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the synchronization raster entries with SSB pattern Case A should be defined as Table 2.1-1.

Proposal 2: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the synchronization raster entries with SSB pattern Case B or Case C could be introduced only if the operator has an explicit request.

2.2 Reference sensitivity

The REFSENS for NTN new S band UE UE was also discussed at the last meeting. According to the definition of REFSENS, the maximum Tx CBW of n252 band is the same as other FR1 NTN, which means that the factor affecting REFSENS is the NF of the UE, i.e. the design of the duplexer.

Considering the suggestion in WID that the work of this WI shall leverage the studies and requirements for NR NTN n256, n255, n254, the duplexer of NTN new S band may also need to consider whether to reuse the design of the existing band. However, since the NTN new S band does not have a fully matched FR1 NR TN frequency range, a design that directly reuses NR TN lacks reference. Unfortunately, unlike the n256 band, the bandwidth of the NTN new S band is 20MHz, which seems to cause some obstacles to designs that directly reuse the n256 band. In addition, if the n256 duplexer design is reused, the REFSENS requirement would have to be relaxed by 0.5 dB, and the blocking requirement would have to be relaxed as well, which will result in performance degradation for NTN new S band UE.

Another approach is to reuse the LTE designs, such as band 23. In any case, we do not wish to limit the implementation that UE could apply, but given that the NTN new S band will be only applicable in North America, the option of a dedicated duplexer implementation could also be considered.

Therefore, we believe that REFSENS of NTN new S band may not be suitable by reusing the value of n256, the following values could be considered:

Table 2.2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
|  | 15 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| n252 | 15 | -100 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
|  | 30 |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |

Proposal 3: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the REFSENS should be defined as Table 2.2-1.

2.3 Out-of-band Blocking

Similar to the discussion of REFSENS above, we believed that for UE of new NTN S band, the OOBB requirements should be defined as shown in the table below:

Table 2.3-1: Out of-band blocking for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| Operating Band | Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- | --- |
|  | Pinterferer | dBm | -44 | -30 | -15 |
| n252 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n2542 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n255 | Finterferer (CW) | MHz | -60 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -85 < f – FDL_low ≤ -60 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 85 or FDL_high + 85 ≤ f ≤ 12750 |
| n2561 | Finterferer (CW) | MHz | -100 < f – FDL_low < -15 or 15 < f – FDL_high < 60 | -145 < f – FDL_low ≤ -100 or 60 ≤ f – FDL_high < 85 | 1 ≤ f ≤ FDL_low – 145 or FDL_high + 85 ≤ f ≤ 12750 |
| NOTE 1: Band n256 lower frequency ranges are modified to enable specific implementations NOTE 2: Band n254 power level of the interferer (Pinterferer) for Range 3 shall be modified to -20 dBm for Finterferer > 2585 MHz and FInterferer < 2775 MHz. NOTE 3: void NOTE 4: void |  |  |  |  |  |

Proposal 4: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the OOBB should be defined as Table 2.3-1.

- Conclusion

This contribution provides our views on UE requirements for introduction of the NTN new S-band. The following proposals are concluded as follows:

Proposal 1: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the synchronization raster entries with SSB pattern Case A should be defined as Table 2.1-1.

Proposal 2: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the synchronization raster entries with SSB pattern Case B or Case C could be introduced only if the operator has an explicit request.

Proposal 3: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the REFSENS should be defined as Table 2.2-1.

Proposal 4: For the NTN FDD band with UE transmitting at 2000 - 2020 MHz and SAN transmitting at 2180 - 2200 MHz, the OOBB should be defined as Table 2.3-1.

- References

- R4-2417206, WF on NTN and IoT-NTN bands, Inmarsat, RAN4 #112-bis