3GPP TSG-RAN WG4 Meeting #113                                                                                             R4-2417814

Orlando, US, Nov. 18 –Nov. 22, 2024

Source: 	CATT

Title: 	Discussion on UE Tx requirement for NTN support HPUE

Agenda item:	7.8.2.2.1

Document for:	Approval

- Introduction

In RAN4 #112-bis meeting, a WF [1] on UE Tx requirement for NTN support HPUE was agreed. However, there are still some remaining issues need to be discussed in the next meeting.

In this contribution, we would like to share some initial views on UE Tx requirement for NTN support HPUE for NR NTN HPUE.

- Discussion

The open issues on UE Tx requirements for NTN support HPUE was concluded during the previous meetings, as shown below:

Issue 2-3: UE architecture/type considerations for NR-NTN PC1.5 UE Agreement: - Both 1Tx and 2Tx for PC1.5 non-handheld UE can be considered as starting point for requirements discussions: - FFS whether PC1.5 non-handheld UE need to be specified if PC1 non-handheld UE be specified - FFS whether PC1.5 handheld UE can be specified or not Issue 2-4: UE architecture/type considerations for NR-NTN PC1 UE Agreement: - Confirm feasibility of PC1 1Tx for non-handheld UE. - For PC1, at least 1Tx to be considered for requirements. - Further discuss consideration of PC1 1Tx, 2Tx, 3Tx for handheld UE. - Further discuss consideration of PC1 2Tx, 3Tx for non-handheld UE. Issue 2-6: MPR - Option 1: RAN4 to consider applicable regulatory requirements in ECC/CEPT in MPR and A-MPR evaluations. (Qualcomm) - Option 2: Reusing NR-NTN PC3 MPR values (both single Tx and Tx diversity) for PC2 MPR as starting point (Mediatek, Samsung) - Option 3: MPR for NTN HPUE should be further evaluated according to the agreed ACLR value, PA model and other limiting factor for MPR evaluation. (ZTE) Way forward: - Further discuss with ACLR candidate values, following agreement and above options. Issue 3-3-2: MPR - Agreement: - FFS the MPR requirements. Following potential factors to be further discussed: - ACLR from co-existence study - Current TN MPR requirements - MPR simulation assumptions - Other factors not precluded if identified.. Issue 2-7: A-MPR - Option 1: RAN4 to consider applicable regulatory requirements in ECC/CEPT in MPR and A-MPR evaluations. (Qualcomm) - Option 2: To reduce number of power back-off simulations for the FR1 satellite bands, we suggest not considering scenarios/combinations that are not expected for the satellite bands: large RB allocations, CP-OFDM, higher-order modulations. (Apple) - Option 3: Regarding the A-MPR requirements, once HPUE e.g. PC2/PC1.5or PC1 are supported in any specific bands, then the A-MPR requirement should be revisited again to identify the appropriate the A-MPR requirements. (ZTE) Way forward: - Further discuss with following agreement and above options. Issue 3-3-3: A-MPR - Agreement - FFS A-MPR requirements. Following potential factors to be further discussed: - Simulation reduction by not considering large RB allocations, CP-OFDM, higher-order modulations - A-MPR simulation assumption including ACLR from co-existence study - Applicable bands - Regional requirements for A-MPR - Other factors.

For TN HPUE that support higher power levels such as PC 1.5, only non-handheld UEs will be deployed. However, similar to PC 2 handheld HPUE, the 2Tx PC 1.5 handheld UE is also possible since the state-of-the-art RF front-end implementations already made some progress (such as deployed by using 2 Tx RF components with 26 dBm capability). For PC 1.5 non-handheld UE, the implementation could use more Tx chains. However, as mentioned above, there are no significant challenges to adopting 2 Tx implementation for non-handheld PC 1.5 HPUE. Therefore, 2 Tx architecture could be considered for both handheld and non-handheld NR NTN HPUE.

Proposal 1: If NR NTN PC2 HPUE with 1Tx architecture is commercially available and widely supported, then for NR NTN PC 1.5 HPUE, the 2 Tx architecture could be considered for both handheld and non-handheld.

For NTN HPUE MPR and A-MPR requirements, the actual power backoff of the NTN HPUE Tx during transmission will be affected by both MPR and A-MPR. According to the UE transmission power description in TS 38.101-1, the power backoff for a UE transmitting in a certain band will be max {MPR, A-MPR}, where MPR is affected by the UE ACLR (i.e., the result of a coexistence study), and A-MPR is usually defined by certain regional regulation (if needed). In general, A-MPR requirements is more stringent than MPR requirements, so we believe that RAN4 could wait for the conclusion of the coexistence study, because if the ACLR target is more stringent, the MPR requirements might be able to meet the regional regulatory requirements. If regional regulatory requirements are more stringent, A-MPR requirements may be considered, as large power backoff may result in the transmit power of UE no longer meeting the NTN HPUE transmit power thresholds.

Proposal 2: MPR and A-MPR requirements for NTN HPUE could be determined according to the agreed ACLR value. When HPUE was supported, the MPR and A-MPR should not offset the benefit of HPUE.

- Conclusion

This contribution provides our views on how to conduct the UE RF requirements for NR NTN HPUE. The following proposals are concluded as follows:

Proposal 1: If NR NTN PC2 HPUE with 1Tx architecture is commercially available and widely supported, then for NR NTN PC 1.5 HPUE, the 2 Tx architecture could be considered for both handheld and non-handheld.

Proposal 2: MPR and A-MPR requirements for NTN HPUE could be determined according to the agreed ACLR value. When HPUE was supported, the MPR and A-MPR should not offset the benefit of HPUE.

- References

- R4-2417122, WF on UE RF requirements for NTN HPUE, Samsung, RAN4 #112-bis