3GPP TSG-RAN WG4 Meeting #113                                                                                                      R4-2417826

Orlando, US, 18th –22nd November, 2024

Source: 	CATT

Title: 	Further discussion on BS demodulation performance for MMSE-IRC

Agenda item:	7.16.3

Document for:	Approval

- Introduction

In RAN4#112bis meeting, a WF [1] on NR demodulation performance: Phase 5 was agreed and the remaining issues on BS demodulation performance for MMSE-IRC in WF require further discussion.

In this contribution, we provide our views on BS demodulation performance for MMSE-IRC.

- Discussion

2.1 Interference modelling

Interference profile

As per the WF [1], the remaining open issue concerning interference profile is shown as below:

Interference profile - On interference modelling: - Reuse the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH - On how to account for HPUE in existing LTE profiles: - Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE. - It is not precluded for companies to bring system level simulation on other profiles. - Proposals on how to account for HPUE in existing LTE profiles. - Proposal 1: Perform new system-level simulations with the inclusion of HPUEs - Proposal 2: Consider artificially increasing DIP/INR with additional 3 or 7 dB

For inter-cell interference signal type, random 16QAM symbol for neighbour PUSCH captured in B.6.2 in TS36.104 are sufficient and can be reused for NR.

Proposal 1: For interference modelling:

  - Reuse the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH

For HPUE,  to avoid system level simulation work,  the approach “Reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE.” and  the approach “Proposal 2: Consider artificially increasing DIP/INR with additional 3 or 7 dB” can be acceptable. We prefer to not to cover HPUE scenario if the performance gain of MMSE-IRC over baseline MMSE can be confirmed under legacy LTE settings.

Observation 1: Not to cover HPUE scenario, reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE.

2.2 Test parameters

Channel bandwidth

As per the WF [1], the remaining open issue concerning channel bandwidth is shown as below:

- Candidate options for DFT-s-OFDM: - Option 1: - For 15kHz SCS, 5MHz - For 30kHz SCS, 10MHz - Other options are not precluded - Agreement: - For CP-OFDM: - For performance evaluation purpose, Minimum channel bandwidth for each SCS, i.e., 5MHz for 15kHz SCS and 10MHz for 30kHz. - Further discuss the CHBW for requirement definition

The channel bandwidth for DFT-s-OFDM is dependent on waveform decision. For DFT-s-OFDM, the Option 1 is sufficient. So we prefer Option 1 if DFT-s-OFDM waveform is agreed.

For channel bandwidth for requirement definition for CP-OFDM, the 5MHz for 15kHz SCS and 10MHz for 30kHz for performance evaluation can be adopted.

Proposal 2: Adopt option 1 for CBW for DFT-s-OFDM if DFT-s-OFDM waveform is agreed as below

  - Option 1:

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

Propagation condition

As per the WF [1], the remaining open issue concerning propagation condition is shown as below:

Propagation condition - Candidate options: - For serving cell: - Option 1: TDLA30-10 and TDLC300-100 for HetNet and HomNet respectively - Option 2: TDLA30-10 and TDLB100-100 for HetNet and HomNet respectively - Option 3: Cover TDLA30-10, TDLB100-400, TDLC300-100 - Option 4: Use TDLB100-400 - For interference cell: - Option 1: Introduce new propagation channel for testing with desired delay spread 1000ns for the interference PUSCH, i.e., TDLC1000 - Option 2: Same propagation condition for serving and interference PUSCH - Option 3: Use TDLC300-100

We can refer to propagation condition for normal PUSCH demodulation, the TDLB100-400 Low, TDLC300-100 Low, TDLA30-10 Low, and TDLA30-10 Low are defined for QPSK, 16QAM, 64QAM and 256QAM respectively. The propagation condition choosing is dependent on MCS choosing, so for QPSK, 16QAM, 64QAM are chosen, the TDLB100-400 Low, TDLC300-100 Low, and TDLA30-10 Low are needed.  So we prefer Option 3 for serving cell.

Proposal 3: Adopt Option 3 for propagation condition for serving cell as below:

- Option 3: Cover TDLA30-10, TDLB100-400, TDLC300-100

For interference PUSCH, same propagation condition for serving and interference PUSCH in Option 2 and TDLC300-100 in Option 3 can be acceptable. In LTE, the different propagation condition for serving and interference PUSCH are defined. For random 16QAM symbol for the neighbour PUSCH, so we think that TDLC300-100 in Option 3 is best choice.

Proposal 4: Adopt Option 3 for propagation condition for interference cell as below:

- Option 3: Use TDLC300-100

MCS

As per the WF [1], the remaining open issue concerning MCS is shown as below:

MCS - Candidate options: - Option 1: Cover QPSK/16QAM/64QAM - Option 1A: Use MCS 5(QPSK, R=379/1024), MCS 13(16QAM, R=490/1024) and MCS 24(64QAM, R=772/1024) - Option 1B: MCS index 2, 12, 20 in MCS table 1 - Option 1C: Cover MCS 2, 16, 20 - Option 2: Choose only one MCS configuration - Option 3: Consider MCS 5(QPSK, R=379/1024) and MCS 13(16QAM, R=490/1024) and FFS 64QAM (ZTE)

For option 1A, the modulation and code rate are close to that of MCS 6, 15, 20 for LTE PUSCH MMSE-IRC demodulation, for option 1B and 1C, the MCSs are same with that for NR PUSCH normal demodulation. Option 1B and 1C are same. Option 3 is similar with Option 1A, Option 2 can reduce workload, but too few requirements will be defined for MMSE-IRC. All options can be acceptable. From NR consistence, Option 1B is best choice,

Proposal 5 Adopt Option 1B for MCS as below:

- Option 1B: MCS index 2, 12, 20 in MCS table 1

Waveform

As per the WF [1], the remaining open issue concerning waveform is shown as below:

Waveform - Agreement - Cover CP-OFDM and FFS DFT-s-OFDM

Since the CP-OFDM and DFT-s-OFDM were defined for NR PUSCH normal demodulation, so it is reasonable to define NR PUSCH MMSE-IRC demodulation for CP-OFDM and DFT-s-OFDM, so we prefer to define both CP-OFDM and DFT-s-OFDM waveform. In order to reduce workload, we can accept not to define DFT-s-OFDM demodulation for MMSE-IRC.

Proposal 6: Adopt DFT-s-OFM waveform for MMSE-IRC.

DMRS configuration

As per the WF [1], the remaining open issue concerning DMRS configuration is shown as below:

DMRS configuration - Candidate options on DMRS type: - Option 1: Cover DMRS configuration type 1 and type 2 - Option 2: DMRS configuration type 1 - Candidate options on DMRS sequence generation: - Option 1: - NID0=0, nSCID =0 for CP-OFDM - NID0=0, group hopping and sequence hopping are disabled for DFT-s-OFDM - Option 2: - nSCID =0 for all cells - Different NID0=0 for serving and interference cells

We think that only DMRS configuration type 1 is sufficient because the density of DMRS configuration type 1 is greater than that of DMRS configuration type 2. So we prefer Option 2: DMRS configuration type 1.

Proposal 7: Adopt Option 2 for DMRS type as below:

  - Option 2: DMRS configuration type 1

The Option 1 on DMRS sequence generation is from normal PUSCH demodulation in TS 38.104, and Option 2 on DMRS sequence generation to configure different NID0 for serving and interference cells is reasonable, because physical cell ID (NIDcell) is configured as 0, 1 and 2 for Cell 1, Cell 2 and Cell 3 respectively for MMSE-IRC in TS 38.101-4, TS 36.104 and TS 36.101. So we can combine the Option 1 and Option 2 as new option as below:

  - New Option:

- NID0=0, 1 and 2 for Cell 1, Cell 2 and Cell 3, nSCID =0 for CP-OFDM

- NID0=0, 1 and 2 for Cell 1, Cell 2 and Cell 3, group hopping and sequence hopping are disabled for DFT-s-OFDM

We prefer new option for DMRS sequence generation configuration.

Proposal 8: Adopt new option for DMRS sequence generation configuration as below:

  - New Option:

- NID0=0, 1 and 2 for Cell 1, Cell 2 and Cell 3, nSCID =0 for CP-OFDM

- NID0=0, 1 and 2 for Cell 1, Cell 2 and Cell 3, group hopping and sequence hopping are disabled for DFT-s-OFDM

Test requirement definition

As per the WF [1], the remaining open issue concerning test requirement definition is shown as below:

Test requirement definition - Proposals: - Option 1: Using SINR based requirement definition - Option 2: SNR based requirement definition - Proposals on SINR definition: - Option 1: - For BS type 1-C and BS type 1-H: The SINR used in this clause is specified based on a single carrier and defined as: SINR = S / N’ Where: S is the total signal energy of one intra-cell UE in the slot on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). N’ is the summation of the received energy of the strongest inter-cell interfering signals explicitly defined in a test procedure plus the white noise energy N in a bandwidth corresponding to the transmission bandwidth over the same duration where signal energy exists on a single antenna connector (for BS type 1-C) or on a single TAB connector (for BS type 1-H). The respective energy of each inter-cell interfering signal relative to N is defined by its associated INR value.. - For BS type 1-O The SINR used in this clause is specified based on a single carrier and defined as: SINR = S / N’ Where: S is the total signal energy of one intra-cell UE in a slot on a RIB. N’ is the summation of the received energy of the strongest inter-cell interfering signals explicitly defined in a test procedure plus the white noise energy N in a bandwidth corresponding to the transmission bandwidth over the same duration where signal energy exists on a RIB. The respective energy of each inter-cell interfering signal relative to N is defined by its associated INR value..

For INR-based modelling in TS 38.101-4, the INR is defined as a test parameter, INR has shown the relationship between interference and noise, so SNR as test requirement definition is sufficient, theoretically SINR-based requirement and SNR-based requirement are equivalent.

SINR based requirement was used for DIP-based modelling for LTE PUSCH and PDSCH MMSE-IRC demodulation requirement in TS 36.104 and 36.101. In last meeting, RAN4 agreed that INR-based modelling for NR PDSCH MMSE-IRC demodulation requirement in TS 38.101-4 will be used for NR PUSCH MMSE-IRC demodulation requirement, in  TS 38.101-4, NR PDSCH MMSE-IRC demodulation requirement was defined as SNR based requirement, so it is reasonable to use SNR based requirement for NR PUSCH MMSE-IRC demodulation requirement. So we prefer Option 2: SNR based requirement definition.

Proposal 9: Adopt Option 2 for test requirement definition as below:

  - Option 2: SNR based requirement definition

- Conclusion

This contribution provides analysis on NR BS MMSE-IRC demodulation. The following proposals and observations are concluded as follows:

Proposal 1: For interference modelling:

  - Reuse the same interference modelling as captured in B.6.2 in TS36.104, to transmit random 16QAM symbol for the neighbour PUSCH

Observation 1: Not to cover HPUE scenario, reuse the LTE interference power level as a start point, and check performance gain of MMSE-IRC over baseline MMSE.

Proposal 2: Adopt option 1 for CBW for DFT-s-OFDM if DFT-s-OFDM waveform is agreed as below

  - Option 1:

- For 15kHz SCS, 5MHz

- For 30kHz SCS, 10MHz

Proposal 3: Adopt Option 3 for propagation condition for serving cell as below:

- Option 3: Cover TDLA30-10, TDLB100-400, TDLC300-100

Proposal 4: Adopt Option 3 for propagation condition for interference cell as below:

- Option 3: Use TDLC300-100

Proposal 5 Adopt Option 1B for MCS as below:

- Option 1B: MCS index 2, 12, 20 in MCS table 1

Proposal 6: Adopt DFT-s-OFM waveform for MMSE-IRC.

Proposal 7: Adopt Option 2 for DMRS type as below:

  - Option 2: DMRS configuration type 1

Proposal 8: Adopt new option for DMRS sequence generation configuration as below:

  - New Option:

- NID0=0, 1 and 2 for Cell 1, Cell 2 and Cell 3, nSCID =0 for CP-OFDM

- NID0=0, 1 and 2 for Cell 1, Cell 2 and Cell 3, group hopping and sequence hopping are disabled for DFT-s-OFDM

Proposal 9: Adopt Option 2 for test requirement definition as below:

  - Option 2: SNR based requirement definition

- References

- R4-2416548, WF on [112bis][319] NR_demod_Ph5_Part1_General_BS, China Telcom, RAN4#112bis

- R4-2413570, WF on NR demodulation performance: Phase 5, China Telcom, RAN4#112

- RP-240831, New WID: WI resulting from discussion on Demodulation performance requirements enhancements, Moderator (vivo), RAN#103

- R4-2411118, Discussion on BS demodulation performance for MMSE-IRC, CATT, RAN4#112