3GPP TSG-RAN WG4 Meeting #113                                                                                                      R4-2417828

Orlando, US, 18th –22nd November, 2024

Source: 	CATT

Title: 	Further discussion on coexistence evaluation for Ku-band for NR NTN

Agenda item:	7.9.2

Document for:	Approval

- Introduction

In RAN4#112 meeting, a WF[1] on NTN Ku band Coexistence study was agreed and remaining issues on Ku-band coexistence assumptions in WF require further discussion.

In this contribution, we provide our views on coexistence evaluation for Ku-band for NR NTN.

- Discussion

Noise figure and G/T for Ku-band SAN

As per the WF [1], the SAN parameters except noise figure and G/T were agreed.  We think that noise figure defined for Ka-band SAN in Table 6a.2.2.1-4 of TR 38.863 as below can be reused for Ku-band SAN.

Table 6a.2.2.1-4: NTN satellite Noise Figure in dB

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| NF (dB) | 3.5 | 3.5 | 3.5 |
| NOTE: The NF of 5.9dB is used for all satellite types for calibration. |  |  |  |

Proposal 1:  Adopt 3.5dB noise figure for satellite GEO, LEO 600 and LEO1200 for Ku-band.

As per the WF[1], the agreed SAN RX antenna maximum gain is copied as below:

The G/T in dB can be calculated using following equation:

G/T = Ga – NF – 10*LOG (To)                                                                                        (Eq.1)

Where:

-	Antenna Gain: Ga in dBi

-	Ambient Temperature: T0 (usually 290 K)

-	Noise Figure: NF in dB.

Using Eq.1 with above proposed noise figure and agreed SAN Rx antenna gain, the G/T for GEO, LEO-1200, and LEO-600 for Ku-band is shown as below:

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ku-band (i.e. 14 GHz for UL) | 7.14 m | 0.714 m | 0.714 m |
| G/T |  | 30.4 dB/K | 10.4 dB/K | 10.4 dB/K |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |

Proposal 2: Adopt 30.4 dB/K G/T for satellite GEO, and 10.4 dB/K G/T for satellite LEO-1200 and LEO-600 for Ku-band.

Noise figure and G/T for Ku-band NTN UE

As per the WF [1], the remaining open issue concerning noise figure and G/T for Ku-band NTN UE is shown as below:

Since 40dBm(10W) VSAT max Tx power was agreed in WF[1], and 36.2 dBi (option 2 10W) is for 10W, so 36.2 dBi (option 2 10W) VSAT max Tx antenna gain is reasonable.

Proposal 3: Adopt 36.2 dBi (option 2 10W) as VSAT max Tx antenna gain.

Noise figure for Ka-band NTN UE defined in Table 6a.2.2.2-1 of TR 38.863 is copied as below:

Table 6a.2.2.2-1: NTN UE characteristics for system level simulations Characteristics; Fixed VSAT; L-ESIM. Frequency band; 27GHz UL/17GHz DL; 27GHz UL/17GHz DL. Antenna type and configuration; [TBD]; [TBD]. Polarisation; Circular; Circular. Rx Antenna gain; [TBD]; [TBD]. Antenna temperature; [TBD]; [TBD]. Noise figure1; 2.5dB for GEO [2,5dB and 6dB for LEO]; 2.5dB for GEO [2,5dB and 6dB for LEO]. Tx transmit power; [TBD]; [TBD]. Tx antenna gain; [TBD]; [TBD]. Antenna elevation angle2; 25˚ and 90˚; 25˚ and 90˚. NOTE1: The NF of 4dB is used for calibration. NOTE2: 30 and 90 degree are used for calibration..

From above table, for Ka-band NTN VSAT, the 2.5dB is for GEO, and 2,5dB and 6dB is for LEO. Since the 14GHz in Ku-band is smaller than 27GHz in Ka-band, so we think the 2.5dB for LEO is sufficient. We prefer to adopt 2.5dB noise figure for VSAT GEO and LEO.

Proposal 4: Adopt 2.5dB noise figure for VSAT GEO and LEO.

Referring to Table 4.4-1 of TR 38.811, the G/T for VSAT for Ku-band can use the following equation to calculate G/T.

NOTE 1: For the computation of G/T or figure of merit, following formula applies in dB: G/T = Ga – NF – 10*LOG (To+(Ta-To)/(100.1*NF)) Where: - Antenna Gain : Ga in dBi - Ambient Temperature : T0 (usually 290 K) - Antenna temperature : Ta (typically 290 K with 0 dBi gain and 150 K with >30 dBi gain) - Noise Figure: NF in dB

According to agreement in WF[1], VSAT max Rx antenna gain in Ku-band is 34.3dBi,  so Ga is 34.3dBi, since Ga > 30dBi, so antenna temperature Ta is equal to 150K. Using proposed 2.5dB noise figure for VSAT, so G/T for VSAT GEO and LEO is 23.2 dB/K.

Proposal 5: Adopt 23.2 dB/K G/T for VSAT GEO and LEO.

- Conclusion

This contribution provides analysis on coexistence evaluation parameters for Ku-band for NR NTN, and the following proposals and observations are concluded as follows:

Proposal 1:  Adopt 3.5dB noise figure for satellite GEO, LEO 600 and LEO1200 for Ku-band.

Proposal 2: Adopt 30.4 dB/K G/T for satellite GEO, and 10.4 dB/K G/T for satellite LEO-1200 and LEO-600 for Ku-band.

Proposal 3: Adopt 36.2 dBi (option 2 10W) as VSAT max Tx antenna gain.

Proposal 4: Adopt 2.5dB noise figure for VSAT GEO and LEO.

Proposal 5: Adopt 23.2 dB/K G/T for VSAT GEO and LEO.

- References

- R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#112bis

- R4-2413519, Way Forward for [112][314] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group), RAN4#112

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat, Eutelsat Group, Thales, CHTTL, Hispasat, RAN#104

- R4-2411507,Ku Band Co-existence, Intelsat, RAN4#112