3GPP TSG-RAN WG4 Meeting #113                                                                                                      R4-2417834

Orlando, US, 18th –22nd November, 2024

Source: 	CATT

Title: 	Discussion on FR2-NTN inclusion to specifications

Agenda item:	8.2

Document for:	Approval

- Introduction

In RAN4#112bis meeting, a WF[1] on FR2-NTN inclusion to specifications was agreed and remaining issues on FR2-NTN inclusion to specifications in WF require further discussion.

In this contribution, we provide our views on FR2-NTN inclusion to specifications.

- Discussion

As per the WF[1], the remaining open issue concerning FR2-NTN inclusion to specifications is shown as below:

- Whether FR2-NTN is regarded as FR2-1 when referenced from other specification RAN4 confirms to RAN1 that FR2-NTN follows FR2-1 procedure. - Whether and how to modify Note 2 in the table (38.101-5 and 38.105) RAN4 agrees to further consider the following options: - Option 1: Remove both Note 1 and Note 2 - Option 2: Keep Note 1 and remove Note 2 - Option 3a: Revise Note 2 (FR2 -> FR2-1) - Option 3b: Update NOTE 2 to “NTN bands within this frequency range are to follow FR2-1 band procedure when referenced from other specifications.” - Option 3c: Update NOTE 2 to: In other specifications, when FR2-NTN is not explicitly defined, NTN bands within this frequency range are regarded as FR2, or FR2-1 when there is a differentiation between FR2-1 and FR2-2.” - Option 3d: Update NOTE 2 to “NTN bands within this frequency range are regarded as a FR2 band when referenced from other specifications, if applicable.”

There is not ambiguous for Note 1 since the FR1-NTN and FR1 are identical, so keeping Note 1 is feasible.

Observation 1: There is not ambiguous for Note 1 since the FR1-NTN and FR1 are identical, so keeping Note 1 is feasible.

As above, removing Note 1 is not necessary, so Option 1 is not best choice. For Option 2, keeping Note 1 and removing Note 2 would cause that use of FR1-NTN and FR2-NTN in other specifications is inconsistent.

Observation 2: For Option 2, keeping Note 1 and removing Note 2 would cause that use of FR1-NTN and FR2-NTN in other specifications is inconsistent.

Option 3a, 3b, 3c and 3d can be regarded as: Keep Note1 and update Note 2. As above analysis for Option 2, there is not ambiguous for Note 1 since the FR1-NTN and FR1 are identical, so keeping Note 1 is feasible, but it is not convenient when adding FR1NTN-only features.

Currently, FR2-NTN term was added in RAN1 specifications, but it is not added in RAN2 specifications, so only revising FR2->FR2-1 in Note 2 is incorrect since revised Note 2(FR2-FR2-1) is not applicable for RAN1. Thus Option 3a and Option 3b is not applicable.

For Option 3c, the description “In other specifications, when FR2-NTN is not explicitly defined” is incorrect, since the FR2-NTN is defined in RAN4 specifications instead of other specifications. It is sufficient that NTN bands in FR2-NTN are only regarded as FR2-1 band, since the “Whenever FR2 is referred, both FR2-1 and FR2-2 frequency sub-ranges shall be applied, unless otherwise stated.” in sub-clause 5.1 of TS 38.104 indicate that FR2-1 should be applied for FR2-band. So the “NTN bands within this frequency range are regarded as FR2, or FR2-1 when there is a differentiation between FR2-1 and FR2-2.”  in Option 3c is not necessary. Thus, Option 3c is not applicable.

For Option 3d, “if applicable” solves the issue that Note2 is not applicable for RAN1 specifications. According to the agreement in WF[1], the “FR2 band” in Option 3d should be revised to “FR2-1 band”. So we prefer the new Option 3d as below:

- New Option 3d: Update NOTE 2 to “NTN bands within this frequency range are regarded as a FR2-1 band when referenced from other specifications, if applicable.”

So, we propose,

Proposal 1: Adopt following new Option 3d:

- New Option 3d: Update NOTE 2 to “NTN bands within this frequency range are regarded as a FR2-1 band when referenced from other specifications, if applicable.”

If new Option 3d is not agreed by RAN4, we also can compromise Option 1, which can make other specification clearer.

- Conclusion

This contribution provides analysis on notes for FR1-NTN and FR2-NTN, and the following proposals and observations are concluded as follows:

Observation 1: There is not ambiguous for Note 1 since the FR1-NTN and FR1 are identical, so keeping Note 1 is feasible.

Observation 2: For Option 2, keeping Note 1 and removing Note 2 would cause that use of FR1-NTN and FR2-NTN in other specifications is inconsistent.

Proposal 1: Adopt following new Option 3d:

- New Option 3d: Update NOTE 2 to “NTN bands within this frequency range are regarded as a FR2-1 band when referenced from other specifications, if applicable.”

- References

- R4-2417185, WF on FR2-NTN inclusion to specifications, Apple, RAN4#112bis