3GPP TSG-RAN WG4 Meeting #113                                                                                                      R4-2417840

Orlando, US, 18th –22nd November, 2024

Source: 	CATT

Title: 	Discussion on NTN system parameters for Ka-band NTN co-existence

Agenda item:	5.21.1

Document for:	Approval

- Introduction

In this contribution, we provide our views on NTN system parameters for Ka-band NTN coexistence evaluation for TR 38.863.

- Discussion

In Ku-band co-existence study, we checked the parameters for Ka-band NTN co-existence and found that some important parameters, such as equivalent satellite antenna aperture, etc., are missing in TR 38.863, so it is necessary to add them to TR 38.863.

Equivalent satellite antenna aperture

The Ka-band satellite antenna parameters for 20GHz DL and 30GHz UL in Table 6.1.1.1-1 of TR 38.821 are copied as following table:

| Satellite orbit |  | GEO | LEO-1200 | LEO-600 |
| --- | --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 1200 km | 600 km |
| Satellite antenna pattern |  | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 | Section 6.4.1 in TR 38.811 |
| Payload characteristics for DL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note 1) | Ka-band (i.e. 20 GHz for DL) | 5 m | 0.5 m | 0.5 m |
| Satellite Tx max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |
| 3dB beamwidth |  | 0.1765 deg | 1.7647 deg | 1.7647 deg |
| Payload characteristics for UL transmissions |  |  |  |  |
| Equivalent satellite antenna aperture (Note1) | Ka-band (i.e. 30 GHz for UL) | 3.33 m | 0.33 m | 0.33 m |
| Satellite RX max Gain |  | 58.5 dBi | 38.5 dBi | 38.5 dBi |

The 17GHz (DL)/ 27GHz (UL) were agreed for Ka-band NTN coexistence study, since the 17GHz (DL)/ 27GHz (UL) is different from 20GHz (DL)/ 30GHz (UL) for Ka-band in able 6.1.1.1-1 of TR 38.821. So TX equivalent satellite antenna aperture and RX equivalent satellite antenna aperture for 20GHz DL and 30GHz UL need to be modified for 17GHz DL and 27GHz UL. For example, the TX equivalent satellite antenna aperture for 17GHz DL can be calculated with 5m*20GHz/17GHz, which is equal to 5.8824m. So, RAN4 have an agreement on equivalent satellite antenna aperture values for co-existence study in WF[1] in RAN4#108bis as below:

- Equivalent satellite antenna aperture values for co-existence study:

| GEO | LEO-1200 | LEO-600 |
| --- | --- | --- |
| 5.9/3.66m (DL/UL) | 0.6/0.36 m (DL/UL) | 0.6/0.36 m (DL/UL) |

Equivalent satellite antenna aperture is important parameter that determines the 3dB beamwidth and max gain. So we propose to add equivalent satellite antenna aperture values for Ka-band NTN co-existence study to TR 38.863.

Proposal 1: Add the following equivalent satellite antenna aperture values for Ka-band NTN co-existence study to TR 38.863.

| GEO | LEO-1200 | LEO-600 |
| --- | --- | --- |
| 5.9/3.66m (DL/UL) | 0.6/0.36 m (DL/UL) | 0.6/0.36 m (DL/UL) |

G/T

The Set-1 satellite parameters for Ka-band NTN co-existence study in Table 6a.2.2.1-2 of TR 38.863 is copied as below:

Table 6a.2.2.1-2: Set-1 satellite parameters for co-existence study Satellite orbit; GEO; LEO-1200; LEO-600. Satellite altitude; 35786 km; 1200 km; 600 km. Payload characteristics for DL transmissions. Satellite EIRP density; 17GHz; 40 dBW/MHz; 10 dBW/MHz; 4 dBW/MHz. Satellite max TX power in dBm; BW (MHz); 200; 400; 200; 400; 200; 400. SCS; 120; 120; 120; 120; 120; 120. NRB; 132; 264; 132; 264; 132; 264. Satellite Tx max Gain; 58.5 dBi; 38.5 dBi; 38.5 dBi. 3dB beamwidth or HPBW (Half-Power BandWidth) of main central beam; 0.1765 deg; 1.7647 deg; 1.7647 deg. ABS (Adjacent Beam Spacing) of adjacent beams from the central beam; Note 1; Note 1; Note 1. Satellite beam diameter; 110 km; 40 km; 20 km. Payload characteristics for UL transmissions. Satellite Rx max Gain; 27GHz; 58.5 dBi; 38.5 dBi; 38.5 dBi. NOTE1: ABS[rad] = sqrt(3) x sin(HPBW[degrees]/2) or ABS[rad] = sqrt(3) x sinr(HPBW[rad]/2) with ABS [degree]=180/pi x ABS[rad] and with HPBW the Half-Power BandWidth of the main lobe from the satellite antenna pattern..

The NTN satellite Noise Figure in dB for Ka-band NTN co-existence study in Table 6a.2.2.1-4 of TR 38.863 is copied as below:

Table 6a.2.2.1-4: NTN satellite Noise Figure in dB Satellite; GEO; LEO 600; LEO 1200. G/T (dB K-1); 58.5; 38.5; 38.5. NF (dB); 3.5; 3.5; 3.5. NOTE: The NF of 5.9dB is used for all satellite types for calibration..

It is incorrect that the G/T (dB K-1) value (58.5, 38.5) in above Table 6a.2.2.1-4 is equal to Satellite Rx max Gain value  (58.5, 38.5) in above Table 6a.2.2.1-2.

The G/T in dB for satellite can be calculated using following equation:

G/T = Ga – NF – 10*LOG (To)                                                                                        (Eq.1)

Where:

-	Antenna Gain: Ga in dBi

-	Ambient Temperature: T0 (usually 290 K)

-	Noise Figure: NF in dB.

Using Eq.1 with above proposed noise figure in above Table 6a.2.2.1-4 and agreed SAN Rx antenna gain in above Table 6a.2.2.1-4, the G/T for GEO, LEO-1200, and LEO-600 for Ka-band are calculated in the following table.

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 30.4 | 10.4 | 10.4 |
| NF (dB) | 3.5 | 3.5 | 3.5 |
| NOTE: The NF of 5.9dB is used for all satellite types for calibration. |  |  |  |

Proposal 2: Update the G/T (dB K-1) for Ka-band NTN co-existence study in Table 6a.2.2.1-4 of TR 38.863 as below:

Table 6a.2.2.1-4: NTN satellite Noise Figure in dB

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 30.4 | 10.4 | 10.4 |
| NF (dB) | 3.5 | 3.5 | 3.5 |
| NOTE: The NF of 5.9dB is used for all satellite types for calibration. |  |  |  |

Based on above proposals, a CR[2] for TR 38.863 on NTN system parameters for Ka-band NTN co-existence is provided for agreement.

- Conclusion

This contribution provides analysis on coexistence evaluation parameters for Ku-band for NR NTN, and the following proposals and observations are concluded as follows:

Proposal 1: Add the following equivalent satellite antenna aperture values for Ka-band NTN co-existence study to TR 38.863.

| GEO | LEO-1200 | LEO-600 |
| --- | --- | --- |
| 5.9/3.66m (DL/UL) | 0.6/0.36 m (DL/UL) | 0.6/0.36 m (DL/UL) |

Proposal 2: Update the G/T (dB K-1) for Ka-band NTN co-existence study in Table 6a.2.2.1-4 of TR 38.863 as below:

Table 6a.2.2.1-4: NTN satellite Noise Figure in dB

| Satellite | GEO | LEO 600 | LEO 1200 |
| --- | --- | --- | --- |
| G/T (dB K-1) | 30.4 | 10.4 | 10.4 |
| NF (dB) | 3.5 | 3.5 | 3.5 |
| NOTE: The NF of 5.9dB is used for all satellite types for calibration. |  |  |  |

Based on above proposals, a CR[2] for TR 38.863 on NTN system parameters for Ka-band NTN co-existence is provided for agreement.

- References

- R4-2316906, WF on NR NTN Co-existence, Samsung, RAN4#108bis

- R4-2417841, CR for TR 38.863, On NTN system parameters for Ka-band co-existence, CATT, RAN4#113