3GPP TSG RAN WG4 Meeting #113		R4-2417865

Orlando, USA, Nov 18th – 22nd , 2024

Title: 	Further discussion on other RF requirements for LP-WUS

Agenda Item:	7.23.2.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112bis, some progress has been made on other RF requirements for LP-WUS with the following agreements [1]:

Issue 3-2-2: Maximum input level requirements Agreements: - The same maximum input power of [-25 dBm] could be starting point for LP-WUR. - FFS whether this value should be scaled-down based on the number of LP-WUS RBs. Issue 3-2-3: Reference channel for LP-WUR requirements Agreements: - RAN4 should define reference measurement channel for LP-WUR RF requirements. - FFS details.

In this contribution, we present our views on these open issues.

# 2 Discussion

RAN1 has just agreed that the number of PRBs for LP-WUS is 11, applicable to all SCSs. And according to the transmission bandwidth configuration table below [2], it is reasonable to consider scaling down from -25dBm for maximum input level:

First of all, these configurations should be excluded where all PRBs are occupied by LP-WUS and no PRB left for NR, i.e., for NR channel bandwidth 5MHz with SCS 30kHz, and NR channel bandwidth 10MHz with SCS 60kHz.

Proposal 1: The configurations should be excluded for the use of LP-WUS where all PRBs are occupied by LP-WUS and no PRB left for NR.

Under such assumptions, the maximum input level could be scaled down according to NR channel bandwidth 3MHz, i.e., 10*log10(NRB = 15/11) = 1.35dB, and if we take round to 0.5dB, then the maximum input level for LP-WUR should be -25-1.5 = -26.5dBm.

Proposal 2: RAN4 to specify maximum input level for LP-WUR as -26.5dBm.

Since LP-WUS signal design is still ongoing in RAN1, it is not possible for RAN4 to accomplish reference channel definition, hence RAN4 should hold on and wait for the completion of LP-WUS signal design.

Proposal 3: RAN4 to hold on discussion on reference channel definition until RAN1 finalizes LP-WUS signal design.

Since the LP-WUS is tasked with waking up the MR, its reliability is vital. On one hand, a relaxed IBB compared with that for MR may guarantee the coverage of LP-WUR in blocking scenarios, on the other hand, if the LP-WUS fails to effectively filter out in-band or out-of-band signals resulting from a relaxed requirement, it may lead to missed wake-up events, which would negatively impact the system's performance and negate the benefits of implementing LP-WUS/WUR. Having the same IBB and OBB for both LP-WUS and MR creates a unified set of performance standards across the system. This consistency can simplify design, testing, and regulatory compliance processes, and ensure interoperability for different components of the system including LP-WUS and MR without unexpected issues arising from differences in interference tolerance, mitigate the risk of false wake-ups caused by unwanted interference ensuring that the MR is activated only when necessary.

Hence, we propose to reuse the IBB and OBB for legacy UEs specified in TS 38.101-1 for LP-WUS if no other issue is identified.

Proposal 4: RAN4 to reuse the IBB and OBB requirements for legacy UEs specified in TS 38.101-1 for LP-WUS if no other issue is identified.

# 3 Conclusion

In this contribution, we have the following proposals on other RF requirements for LP-WUS:

Proposal 1: The configurations should be excluded for the use of LP-WUS where all PRBs are occupied by LP-WUS and no PRB left for NR.

Proposal 2: RAN4 to specify maximum input level for LP-WUR as -26.5dBm.

Proposal 3: RAN4 to hold on discussion on reference channel definition until RAN1 finalizes LP-WUS signal design.

Proposal 4: RAN4 to reuse the IBB and OBB requirements for legacy UEs specified in TS 38.101-1 for LP-WUS if no other issue is identified.

# Reference

[1] R4-2417112, “WF on UE RF requirements for LP-WUS”, vivo

[2] 3GPP TS 38.101-1 v18.7.0