3GPP TSG RAN WG4 Meeting #113		R4-2417869

Orlando, USA, Nov 18th – 22nd , 2024

Title: 	On remaining issues on UE RF requirements for RedCap PC2

Agenda Item:	7.7.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112bis, a draft CR was endorsed for RedCap PC2, which is only for TDD bands [1]. RAN#106 will check whether or not FDD bands should be applicable [2]. However, literally the Tx RF requirements for RedCap are not complete. In this contribution, we propose to use a similar way as Rx RF requirements for RedCap to complete Tx RF requirements.

# 2 Discussion

The current specs structure for Tx and Rx RF requirements are shown in the following:

## 6.1I (Reserved) ## ------ ## 6.2I Transmitter power for RedCap ### 6.2I.1 Maximum output power for RedCap …

## 7.1I General For a Redcap UE the requirements in Section 7 shall be verified with the channel bandwidth up to 20MHz and REFSENS specified in clause 7.3I.

Thanks to subclause 7.1I, the whole set of Rx RF requirements are defined for a RedCap UE. However, strictly speaking, there is no other explicit other Tx RF requirements for a RedCap UE, except maximum output power.

Observation: In current specs, only maximum output power for RedCap is specified with other Tx RF requirements undefined.

Therefore, with the addition of RedCap PC2, RAN4 should make it very clear and complete the whole Tx RF requirements set.

One possible way is to add a general sub-clause like 7.1I:

6.1I General For a Redcap UE the requirements in Section 6 shall be verified with the channel bandwidth up to 20MHz and requirements specified in clause 6.2I.

Proposal: To complete the missing piece in current specification, RAN4 should add one sub-clause as following:

6.1I General For a Redcap UE the requirements in Section 6 shall be verified with the channel bandwidth up to 20MHz and requirements specified in clause 6.2I.

# 3 Conclusion

In this contribution, we have the following observation and proposal on UE RF requirements for RedCap PC2:

Observation: In current specs, only maximum output power for RedCap is specified with other Tx RF requirements undefined.

Proposal: To complete the missing piece in current specification, RAN4 should add one sub-clause as following:

6.1I General For a Redcap UE the requirements in Section 6 shall be verified with the channel bandwidth up to 20MHz and requirements specified in clause 6.2I.

# Reference

[1] R4-2417086, “Draft CR to 38.101-1: power class 2 RedCap for TDD bands”, China Telecom, ZTE, CATT, China Unicom, Huawei, HiSilicon, Ericsson

[2] RP-241997, “Revised WID on NR power class 2 RedCap (Reduced Capability) UE in FR1”, China Telecom