3GPP TSG RAN WG4 Meeting #113		R4-2417880

Orlando, USA, Nov 18th – 22nd , 2024

Title: 	TP to PRD01 on the maximum aggregated bandwidth for intra-band CA with BCS4/BCS5

Agenda Item:	9.1.1.1

Source: 	CATT

Document for:	Discussion

# 1 Introduction

For the first version of the first PRD in RAN4 [1], the existing texts might need to be improved. For example, in sub-clause 5.7.3.2 on the maximum aggregated bandwidth for intra-band CA with BCS4/BCS5, the first bullet on intra-band contiguous CA is not in the right place, and the bullet icon ‘-‘ could be mixed the minus operator ‘-‘ for intra-band non-contiguous CA.

The proposed changes are presented below.

# 2 Text proposal

------- < Start of change> -----

#### 5.7.3.2	The maximum aggregated bandwidth for intra-band CA with BCS4/BCS5

To guarantee the BCS4/BCS5 can cover all the possible bandwidth configurations for intra-band CA, the maximum aggregated bandwidth chosen for BCS4/BCS5 should equal to:

- min{n*max channel bandwidth of each carrier, BWChannel_CA of each CA bandwidth class, Maximum frequency range of each band} for intra-band contiguous CA.

- min{n*max channel bandwidth of each carrier, Maximum frequency range of each band - Minimum sub-block gaps} for intra-band non-contiguous CA.

where

- n is the number of aggregated CCs,

- minimum sub-block gaps indicates the sum of the min sub-block gap between the upper edge of lower component carrier and lower edge of higher component carrier that UE can support per band combination in two adjacent non-contiguous component carriers.

The value of min sub-block gaps could be clarified by the request operator but it should try to cover the needs of all possible operators.

------- < End of change> ------

# Reference

[1] R4-2416422, “draft PRD on rules guidelines and ways of working for introduction of band combinations”, Ericsson