3GPP TSG RAN WG4 Meeting #113		R4-2417884

Orlando, USA, Nov 18th – 22nd , 2024

Title: 	MPR applicability for FR2

Agenda Item:	7.1.1.2.3

Source: 	CATT

Document for:	Discussion

# 1 Introduction

In RAN4#112bis, it was agreed to introduce an optional UE capability to indicate that MPR is based on UL aggregated bandwidth instead of cumulative aggregated channel bandwidth [1]:

#### Issue 3-1-2: Applicable MPR for FR2 UL CA with DL intra band CA Agreement in main session: - In general, with UE indication of new capability [FR2 MPR enhancements-R19] for MPR improvement, MPR based on UL BWchannel_CA applies instead that based on cumulative aggregated channel BW (CABW). - If only 1 UL CC is activated, the MPR requirements of single carrier could be reused. - A note should be added to the MPR table, e.g. - For FR2-1, for 200MHz BW granularity, RAN4 to change the MPR calculation for DFT-s-BPSK or DFT-s-QPSK from “MPRC_CA = MAX(MPR1, MPR2)” to “MPRC_CA = MPR2”, for UE with UE indication of new capability for MPR improvement. #### Issue 3-2-1: Optional UE capability Agreement in main session: - An optional per UE capability for FR2 is introduced - The capability is implementation agnostic - Supporting FR2 MPR enhancement is applicable only from Rel-19

However, there are still some open issues identified:

#### Issue 3-2-2: CC activation-based MPR improvement - WF - Further discuss activated based MPR improvement. - FFS using common UE capability between configuration based and activation based, or another UE capability for activated-based improvements. - FFS MPR can be determined based on activated UL CCs BW instead of Cumulative aggregated channel bandwidth (CABW). (i.e., activation based also applies to UL CA case) - FFS limitation the enhancement to cases where the activated UL CCs form a contiguous block.

In this contribution, we present our views on these open issues.

# 2 Discussion

If a UE can further improve MPR based on activated CCs instead of configured CCs, then a new UE capability should be introduced to indicate its capability for activation-based improvement.

Proposal 1: RAN4 to introduce a new UE capability to indicate the support of activation-based improvement for FR2.

When the new UE capability is introduced for the support of activation-based improvement, whether cumulative aggregated channel bandwidth (CABW) or UL CCs BW is based can be represented by previously introduced FR2 capability [FR2 MPR enhancements-R19]. The below table illustrates different cases:

Table – 1 MPR improvement capabilities for FR2

| MPR improvement capability | Reported (Support) | Not reported |
| --- | --- | --- |
| [FR2 MPR enhancements-R19] | UL aggregated BW replacing CABW | CABW based |
| [FR2 MPR activation-based improvements-R19 | UL aggregated BW replacing CABW | CABW based |

Proposal 2: Whether UL aggregated BW or CABW is based for MPR is dependent on the new capability [FR2 MPR enhancements-R19].

If the activated UL CCs cannot form a contiguous block, more analysis and evaluation is required to determine the feasible MPR values. Considering the limited time in this release, we suggest to limit the MPR improvement to the cases where activated UL CCs form a contiguous block so that the previous works can be reused.

Proposal 3: Limit activation-based MPR improvements to the cases where UL CCs forms a contiguous block in Rel-19.

# 3 Conclusion

In this contribution, we have the following proposals for MPR applicability for FR2:

Proposal 1: RAN4 to introduce a new UE capability to indicate the support of activation-based improvement for FR2.

Proposal 2: Whether UL aggregated BW or CABW is based for MPR is dependent on the new capability [FR2 MPR enhancements-R19].

Proposal 3: Limit activation-based MPR improvements to the cases where UL CCs forms a contiguous block in Rel-19.

# Reference

[1] R4-2417204, “WF on power domain enhancements”, Huawei