3GPP TSG-RAN WG4 Meeting # 113	                                       R4-2417890

Orlando, US, Nov. 18 – Nov. 22, 2024

Agenda Item: 7.24.2.1

Source: MediaTek Inc.

Title: Discussion on event triggered L1 report for LTM

Document for: Discussion

# Introduction

RAN#105 agreed on the revised WID “NR mobility enhancements Phase 4” [1]. According to WID, RAN4 is supposed to define requirements for the following measurement enhancements:

- Event triggered L1 report,

- CSI-RS L1 measurements,

- CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch.

Last meeting, there were some discussions [2]. In the next, we would like to provide our views on the open issues of event triggered L1 report.

# Discussion

## 2.1 Capabilities for Support of Event Triggering L1 Reporting

In legacy, capabilities for Support of Event Triggered L3 measurement Reporting had been specified in section 9.1.4. Similarly, for event triggered L1 measurement reporting, the number of events supported shall be defined too. Table 9.1.4.2-1 is copied here for information.

38.133 Table 9.1.4.2-1: Requirements for reporting criteria per measurement category

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1,2,3,4,5 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 2,3,4,5 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) Note 2,4,5 | 10 | Only applicable for UE with this (inter-RAT) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSTD Note 2,4,5 | 1 | Inter-RAT RSTD measurement reporting for UE supporting OTDOA; 1 report capable of minimum 16 inter-RAT cell measurements. Only applicable for UE with this (inter-RAT RSTD via LPP [22]) capability. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Inter-RAT (E-UTRA FDD, E-UTRA TDD) RSRP and RSRQ measurements for E-CID Note 2,4,5 | 1 | Inter-RAT RSRP and RSRQ measurements for E-CID reported to E-SMLC via LPP [22]. One report capable of at least in total 10 inter-RAT RSRP and RSRQ measurements. Applicable to UE capable of reporting inter-RAT RSRP and RSRQ to E-SMLC via LPP. These reporting criteria apply for any E-UTRA carrier frequencies other than the carrier frequency of the E-UTRA PSCell or E-UTRA SCell. |
| Intra-frequency RSSI and channel occupancy measurements with CCA Note 1,2,3 | 1 | One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA. |
| Inter-frequency RSSI and channel occupancy measurements with CCA Note 2,3 | 1 | One report capable of one RSSI and one channel occupancy measurements over a channel [TS 37.213] with CCA. Applicable for UE capable of performing and reporting RSSI and channel occupancy on carrier frequencies under CCA. |
| Intra-frequency SSB-based measurements for NR E-CID Note 1,2,3,4,5 | 1 | Intra-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 9 intra-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP. |
| Intra-frequency CSI-RS based measurements for NR E-CID Note 1,2,3,4,5 | 1 | Intra-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 9 intra-frequency CSI-RSRP and/or CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16. |
| Inter-frequency SSB-based measurements for NR E-CID Note 2,3,4,5 | 1 | Inter-frequency SS-RSRP and SS-RSRQ measurements for NR E-CID reported to LMF via LPP [34]. One report capable of at least in total 10 inter-frequency SS-RSRP and SS-RSRQ measurements. Applicable to UE capable of reporting at least one of SS-RSRP and SS-RSRQ to LMF via LPP. |
| Inter-frequency CSI-RS based measurements for NR E-CID Note 2,3,4,5 | 1 | Inter-frequency CSI-RSRP and CSI-RSRQ measurements for NR E-CID reported to LMF via LPP [22]. One report capable of at least in total 10 inter-frequency CSI-RSRP and CSI-RSRQ measurements. Applicable to UE capable of reporting any of CSI-RSRP and CSI-RSRQ to LMF via LPP, as indicated in nr-ECID-MeasSupported-r16. |
| DL RSTD Note 2,4,5 | 1 | DL RSTD measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-TDOA-MeasCapability, indicated via LPP [34]) DL RSTD measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for DL-TDOA. Only applicable for UE capable of reporting measurements for DL-TDOA to LMF via LPP [34]. |
| UE Rx-Tx Note 2,4,5 | 1 | UE Rx-Tx measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for multi-RTT) UE Rx-Tx measurements and if supported also multiple corresponding DL PRS-RSRP measurements configured for multi-RTT. Only applicable for UE capable of reporting measurements for multi-RTT to LMF via LPP [34]. |
| DL PRS-RSRP Note 2,4,5 | 1 | DL PRS-RSRP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34]. |
| DL PRS-RSRPP Note 2,4,5 | 1 | DL PRS-RSRPP measurement reporting; 1 report capable of multiple (within the UE PRS measurement capability, nr-DL-PRS-MeasCapability, indicated via LPP [34] for AoD) DL PRS-RSRPP measurements configured for DL-AoD. Only applicable for UE capable of reporting measurements for DL-AoD to LMF via LPP [34]. |
| SRS-RSRP Note 2,3,4,5 | 1 | SRS-RSRP measurement reporting for CLI; 1 report capable of up to 32 SRS resources measurements. Only applicable for UE supporting cli-SRS-RSRP-Meas-r16. |
| CLI-RSSI Note 2,3,4,5 | 1 | CLI-RSSI measurement reporting for CLI; 1 report capable of up to 64 CLi-RSSI resources measurements. Only applicable for UE supporting cli-RSSI-Meas-r16. |
| NOTE 1: When the UE is configured with PSCell and SCell carrier frequencies, Ecat for Intra-frequency is applied per corresponding NR serving frequency. NOTE 2: Applicable for UE configured with SA NR operation mode. NOTE 3: Applicable for UE configured with EN-DC operation mode. NOTE 4: Applicable for UE configured with NE-DC operation mode. NOTE 5: Applicable for UE configured with NR-DC operation mode. |  |  |

Regarding how to define the capability for event triggered L1 measurement reporting, we think at least the following aspects shall be discussed.

Whether to share the capability with Event Triggered L3 measurement Reporting

In our understanding, the goal of event triggered L1 measurement reporting is to replace L3 report and used to trigger LTM. With this understanding, for the frequency layers configured with event triggered L1 measurement reporting, it is not necessary to have event triggered L3 measurement reporting.

But on the other hand, NW may configure and adjust event triggered L1 measurement reporting based on L3 report. Then there will be both L3 event and L1 event for the same frequency layer. We think it is reasonable to introduce extra UE capability to support event triggered L1 measurement reporting.

Proposal 1: Define a separate capability for supported number of events triggered L1 measurement reporting.

Whether and how to differentiate intra-frequency and inter-frequency events

As shown in Table 9.1.4.2-1 in 38.133, for L3 measurement, the events for intra-frequency and inter-frequency are viewed as different events. It is straightforward for L3 measurement, as each report configuration is linked to a measObject which is for a single frequency layer (as shown in the copied L3 measurement and report configurations from 38.331).

L3 measurement and report configurations MeasIdToAddMod ::= SEQUENCE { measId MeasId, measObjectId MeasObjectId, reportConfigId ReportConfigId } MeasObjectToAddMod ::= SEQUENCE { measObjectId MeasObjectId, measObject CHOICE { measObjectNR MeasObjectNR, ..., measObjectEUTRA MeasObjectEUTRA, measObjectUTRA-FDD-r16 MeasObjectUTRA-FDD-r16, measObjectNR-SL-r16 MeasObjectNR-SL-r16, measObjectCLI-r16 MeasObjectCLI-r16, measObjectRxTxDiff-r17 MeasObjectRxTxDiff-r17, measObjectRelay-r17 SL-MeasObject-r16, measObjectNR-SL-r18 MeasObjectNR-SL-r18 } } MeasObjectNR ::= SEQUENCE { ssbFrequency ARFCN-ValueNR OPTIONAL, -- Cond SSBorAssociatedSSB ssbSubcarrierSpacing SubcarrierSpacing OPTIONAL, -- Cond SSBorAssociatedSSB smtc1 SSB-MTC OPTIONAL, -- Cond SSBorAssociatedSSB smtc2 SSB-MTC2 OPTIONAL, -- Cond IntraFreqConnected refFreqCSI-RS ARFCN-ValueNR OPTIONAL, -- Cond CSI-RS referenceSignalConfig ReferenceSignalConfig, absThreshSS-BlocksConsolidation ThresholdNR OPTIONAL, -- Need R absThreshCSI-RS-Consolidation ThresholdNR OPTIONAL, -- Need R …… }

But for L1 measurement and report, if we follow the configurations in R18 LTM. That is: each LTM-CSI-ReportConfig is linked to a LTM-CSI-ResourceConfigId. In one LTM-CSI-ResourceConfig, the SSB to measure can be from any LTM candidate cell (intra-f or inter-f).

So LTM-CSI-ReportConfig may be linked to multiple cells on different frequency layers. It seems not so straightforward on how to count the number of events configured for intra-frequency and inter-frequency L1 measurement.

R18 LTM L1 measurement and report configurations LTM-CSI-ReportConfig-r18 ::= SEQUENCE { ltm-CSI-ReportConfigId-r18 LTM-CSI-ReportConfigId-r18, ltm-ResourcesForChannelMeasurement-r18 LTM-CSI-ResourceConfigId-r18, ltm-ReportConfigType-r18 CHOICE { periodic-r18 SEQUENCE { reportSlotConfig-r18 CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUCCH-r18 SEQUENCE { reportSlotConfig-r18 CSI-ReportPeriodicityAndOffset, pucch-CSI-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofBWPs)) OF PUCCH-CSI-Resource }, semiPersistentOnPUSCH-r18 SEQUENCE { reportSlotConfig-r18 CSI-ReportPeriodicityAndOffset, reportSlotOffsetList-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-2-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-1-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), p0alpha P0-PUSCH-AlphaSetId }, aperiodic-r18 SEQUENCE { reportSlotOffsetList-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-2-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128), reportSlotOffsetListDCI-0-1-r18 SEQUENCE (SIZE (1.. maxNrofUL-Allocations-r16)) OF INTEGER (0..128) } }, ltm-ReportContent-r18 LTM-ReportContent-r18, ... } LTM-CSI-ResourceConfig-r18 ::= SEQUENCE { ltm-CSI-ResourceConfigId-r18 LTM-CSI-ResourceConfigId-r18, ltm-CSI-SSB-ResourceSet-r18 LTM-CSI-SSB-ResourceSet-r18, ... } LTM-CSI-SSB-ResourceSet-r18 ::= SEQUENCE { ltm-CSI-SSB-ResourceList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF SSB-Index, ltm-CandidateIdList-r18 SEQUENCE (SIZE (1..maxNrofLTM-CSI-SSB-ResourcesPerSet-r18)) OF LTM-CandidateId-r18, ... }

There may be two ways to solve the issue if we only consider SSB based L1 measurement. One is that if the LTM-CSI-ReportConfig is linked to multiple cells on different frequency layer, then the number of events should be counted no less than the number of frequency layers provided in this LTM-CSI-ReportConfig. Another way is to inform RAN2 to take this issue into consideration when design the signaling, e.g., when configuring event triggered L1-RSRP events, all candidate cells in the same LTM-CSI-ReportConfig are limited to the same frequency layer.

Proposal 2: For SSB based event triggered L1-RSRP measurement reporting, regard the events related to different candidate cells on different frequency layers as different events. Discuss how to differentiate intra-frequency and inter-frequency events. Further discuss the following options:

- Option 1: if the LTM-CSI-ReportConfig is linked to multiple cells on different frequency layer, then the number of events related to this LTM-CSI-ReportConfig should be counted no less than the number of frequency layers.

- Option 2: inform RAN2 that the events related to different candidate cells on different frequency layers are regarded as different events and ask RAN2 to take this into consideration when design the signaling.

How to coordinate with R19 MIMO

RAN2#126 Agreement - Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

RAN1#117 Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs FFS: Whether the above is captured in RAN1 or RAN2 specification. RAN1#118 Working Assumption On UE-initiated/event-driven beam reporting, regarding trigger events, besides for Event-2, Event-1 and Event-7 are both supported. - Event-1: Quality of the current beam is worse than a certain threshold. - Event-7: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the M-th best quality. - M is RRC configured with subjective to UE capability signalling - UE may only indicate a single candidate value or not support Event-7. - The additionally supported events will reuse the same design as event 2 – unless there is consensus to do otherwise - The additionally supported events will be lower priority compared to event 2.

As shown above, Events LTM2-5 are supported in R19 mobility. Similar events are also supported in R19 MIMO. It is likely that NW configures separate events for mobility and MIMO. As far as we know, the evaluation method of events configured in mobility and MIMO may be different, so the events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell.

We believe the capability of supported number of events for L1 measurement will be discussed in R19 MIMO too. The question is to define separate capabilities for R19 mobility and MIMO or define a unified capability. We prefer to define a capability that indicates the total number of L1 events supported for mobility and MIMO. If to define separate capabilities, the number of supported events for both mobility and MIMO should be smaller as the efforts to support events for mobility and MIMO are similar for UE. Defining a single one for both can make the best use of UE capability. In addition, although the evaluation methods and reporting mechanisms for mobility and MIMO are different, it is a waste of UL resources as well as UE buffer and power to configure basically the same two events for both mobility and MIMO.

Proposal 3: Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

Proposal 4: Define the capability for L1 events shared by mobility L1 events and MIMO L1 events.

## 2.2 L1 event triggered reporting delay

RAN2 agreed to support event triggered periodic MR based on NW configuration. We see no big difference between event triggered periodic L1 MR and event triggered L1 MR. RAN4 can define the report delay requirements for event triggered periodic L1 report too.

RAN2#126bis - Agreements on L1 event triggered MR - Event triggered periodic MR can be supported, based on NW configuration.

Proposal 5: Define report delay requirements for both event triggered periodic L1 MR and event triggered L1 MR.

Last meeting, RAN4 discussed how to define L1 event triggered reporting delay and reached consensus on the starting point. Regarding the exact reporting delay requirements, there are several candidates.

#### Issue 1-8: starting point in event triggered L1 reporting requirements - Agreement: - Use the following as starting point. Wording can be discussed in CR. - Starting point in event triggered L1 reporting requirements is the time when a condition exists at the UE which will trigger the reporting. #### Issue 1-9: L1 event triggered reporting delay - Candidate solutions: - Option 1: The existing framework in L3 event triggered reporting RRM requirements can be as a starting point. (CTC, Samsung, ZTE) - Option 2: RAN4 to discuss how to consider the total measurement time if multiple cells are measured with different measurement period. (Xiaomi) - It’s possible that the periodicity of RS is different from each other and then the measurement period is different. How to define total measurement time for multiple cells is FFS. - Option 3: (CMCC) - for SSB based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements. - The delay is less than TL1-RSRP_Measurement_Period or TL1-RSRP_Measurement_Perioa + TSSB_time_index, which are the L1-RSRP measurement period defined for Rel-18 LTM. - for CSI-RS based event triggered L1 measurement reporting, it is proposed to define L1 measurement reporting delay requirements, and L1 measurement reporting accuracy requirements - Option 4: Reporting delay is defined as one L1-RSRP measurement period on target cell. (Apple) - Option 4a: The event triggered measurement reporting delay, shall be less than LTM L1-RSRP measurement period based on SSB or CSI-RS. (Samsung) - Option 5: SSB-based LTM L1 event triggered reporting delay is TL1-RSRP_Measurement_Period_SSB_intra, TL1-RSRP_Measurement_Period_SSB_inter or TL1-RSRP_Measurement_Period_SSB_inter_withoutGap (Nokia) - Option 6: RAN4 to discuss and define the event evaluation time (i.e., the minimum time required to complete the event evaluation and be ready to send on the first reporting occasion from the measurement occasion). (Ericsson)

In our understanding, we can follow L3 MR requirements. For reference, we copy the requirements for intra-frequency L3 event MR here: The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or T identify intra without index defined in clause 9.2.5.1 or clause 9.2.6.2. When L3 filtering is used an additional delay can be expected.

We also copy part of intra-frequency L1 measurement requirements defined in R18 LTM here too:

If a neighbor cell is known according 9.14.2, the UE shall be capable of performing L1-RSRP measurements based on the configured SSB resource for L1-RSRP computation, and the UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_intra if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell. Otherwise, UE physical layer shall be capable of reporting L1-RSRP measured over the measurement period of TL1-RSRP_Measurement_Period_SSB_intra+TSSB_time_index_intra, where TSSB_time_index_intra is the time period used to acquire the index of the SSB being measured given in Table 9.2.5.1-3. …… Table 9.14.5.1-2: Intra-frequency L1-RSRP measurement period TL1-RSRP_Measurement_Period_SSB_intra in FR1 for UE capable of [capability of measurement with RTD>CP] Configuration; TL1-RSRP_Measurement_Period_SSB_intra (ms). non-DRX; max(TReport, ceil(M*P)*TSSB_NBC*NLayer). DRX cycle ≤ 320ms; max(TReport, ceil(K *M*P)*max(TDRX,TSSB_NBC) *NLayer). DRX cycle > 320ms; ceil(M*P)*TDRX*NLayer. Note 1: TSSB_NBC is the periodicity of the neighbor cell SSB-Index configured for intra-frequency L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. [NLayer = the number of intra-frequency layers configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18] + the number of inter-frequency layers without measurement gaps which are configured for L1-RSRP measurement with [LTM-CSI-ResourceConfig-r18]]. Note 2: K = 1.5. Note 3: UE shall at least measure neighbour cells which are activated for TCI state within UE capability and additional cells to measure is up to UE implementation..

As L3 filtering is not introduced for event triggered L1 measurement report, we don’t need to consider the impact of L3 filtering here. But TTT is also supported for L1 measurement, the impact should be reflected in the spec. For L3 measurement, cell search delay is also included in Tidentify intra with index or T identify intra without index. But for L1 measurement, we only defined the measurement delay for known cells. Last meeting, R4 agreed that the starting point is the time when the triggering channel condition is met. But when the triggering channel condition is met, UE may not have detected that cell yet. This should be considered when writing the CR. In our understanding, event triggered L1 measurement report requirements will be captured in 9.14 and 9.15. The applicable rules in 9.14.2 and 9.15.2 are also applicable. In addition, as shown above, the measurement period is lower bounded by TReport. For event triggered report, TReport should be assumed as 0.

Take SSB based intra-frequency L1 measurement as an example, we suggest to define the event triggered L1 measurement reporting delay as:

- The event triggered L1 measurement reporting delay shall be up to TL1-RSRP_Measurement_Period_SSB_intra if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell. Otherwise the event triggered L1 measurement reporting delay shall be up to TL1-RSRP_Measurement_Period_SSB_intra+TSSB_time_index_intra.

  - In TL1-RSRP_Measurement_Period_SSB_intra, TReport is assumed as 0.

Proposal 6: For SSB based SSB based intra-frequency L1 measurement, report delay requirements is defined as:

- The event triggered L1 measurement reporting delay shall be up to TL1-RSRP_Measurement_Period_SSB_intra if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell. Otherwise the event triggered L1 measurement reporting delay shall be up to TL1-RSRP_Measurement_Period_SSB_intra+TSSB_time_index_intra. When TTT is used, an additional reporting delay can be expected.

  - In TL1-RSRP_Measurement_Period_SSB_intra, TReport is assumed as 0.

# Summary

In this paper, we provide some views on event triggered L1 report in R19 mobility. We have the following proposals:

Proposal 1: Define a separate capability for supported number of events triggered L1 measurement reporting.

Proposal 2: For SSB based event triggered L1-RSRP measurement reporting, regard the events related to different candidate cells on different frequency layers as different events. Discuss how to differentiate intra-frequency and inter-frequency events. Further discuss the following options:

- Option 1: if the LTM-CSI-ReportConfig is linked to multiple cells on different frequency layer, then the number of events related to this LTM-CSI-ReportConfig should be counted no less than the number of frequency layers.

- Option 2: inform RAN2 that the events related to different candidate cells on different frequency layers are regarded as different events and ask RAN2 to take this into consideration when design the signaling.

Proposal 3: Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold.

Proposal 4: Define the capability for L1 events shared by mobility L1 events and MIMO L1 events.

Proposal 5: Define report delay requirements for both event triggered periodic L1 MR and event triggered L1 MR.

Proposal 6: For SSB based SSB based intra-frequency L1 measurement, report delay requirements is defined as:

- The event triggered L1 measurement reporting delay shall be up to TL1-RSRP_Measurement_Period_SSB_intra if deriveSSB-IndexFromCell is enabled or UE has reported SSB index in L3 measurement report of the same cell. Otherwise the event triggered L1 measurement reporting delay shall be up to TL1-RSRP_Measurement_Period_SSB_intra+TSSB_time_index_intra. When TTT is used, an additional reporting delay can be expected.

  - In TL1-RSRP_Measurement_Period_SSB_intra, TReport is assumed as 0.

# 4 Reference

RP-242356, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.

R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple, RAN4#112bis.